|  | DISTRICT WATER & SANITATION MISSION DISTRICT MOHLA-MANPUR-A.CHOWKI (C.G) |  |
| --- | --- | --- |

##### SINGLE VILLAGE  OF PIPED WATER SUPPLY SCHEME FOR

##### VILLAGE MARKATOLA, DHANEGAON

##### BLOCK MOHLA DISTRICT- MOHLA-MANPUR-A.CHOWKI.

TENDER DOCUMENT FOR

Balance Work of Construction, Testing and Commissioning of R.C.C. Elevated Service Reservoir 40 KL capacities & staging 12 mt. (35% Balance Work) at Markatola including providing and fixing of CI Double flange pipe, double flange specials & DI double flange soft seated sluice valve including construction of suitable R.C.C. Valve Chamber

&

Providing, laying, jointing, interconnection, testing and commissioning of 90/110 mm dia UPVC class 06/10 kg/sqcm Pipe, 63/75 mm dia HDPE 06 kg/sqcm Pipe with suitable joints, valve specials and all fittings including all allied civil works,  including Electric connection from CSPDCL, Road Cutting, Switch room cum pump House, Information Board, construction of boundary wall around the over head tank, installation of electro chlorinator, construction of chlorinator room, providing functional household tap connection (F.H.T.C) in individual households and government institution  etc. complete with 03 months trial run, operation & maintenance for 06 months under Single Village  of Piped Water Supply Scheme under Jal Jeevan Mission For

Village Markatola, Dhanegaon

Block Mohla District- Mohla-Manpur-A.Chowki.

ON FORM – A

Risk & Cost

Estimated Cost Rs. 19.36 Lakhs

EXECUTIVE ENGINEER AND MEMBER SECRETARY,

DWSM, MOHLA-MANPUR-A.CHOWKI (C.G.)

I N D E X

| S. No. | Contents | Pages From - To |
| --- | --- | --- |
| 1. | Salient Feature of Tender | 3 |
| 2 | Notice Inviting Tender (In English) | 4-8 |
| 3 | Notice Inviting Tenders (In Hindi) | 9-13 |
| 4 | Detailed Notice Inviting Tenders | 14-25 |
| 5 | Annexure - “A”- Model Rules relating to labour, water supply and sanitation etc | 26 |
| 6 | Annexure - “B”- Contractor’s Labour Regulations | 27 |
| 7 | Annexure “C”- Statement showing lead of material | 28 |
| 8 | Annexure “D” - Form of certificate on Income Tax | 29 |
| 9 | Appendix 2.13 | 30 |
| 10 | ANNEXURE “E-I to E-VI” – Specifications | 45 |
| 11 | Important points of for Tenderer | 65 |
| 12 | Annexure "F"- Schedule of Quantity | 68 |
| 13 | Annexure "G, G-I to G-IX" - Schedule of payment | 101 |
| 14 | ANNEXURE “H” - Pre-Contract Integrity Pact | 106 |
| 12 | Annexure "I" - Special Condition | 110 |
| 16 | ANNEXURE “J” -Suspension, demotion, Non-renewal & De-registration of contract/firms 2014. | 111 |
| 17 | ANNEXURE “K” - Information & Instruction to the bidders for online electronic government procurement system. | 118 |
| 18 | Annexure "L" - Annexure 2.10 G-III | 121 |
| 19 | Annexure "M”- Single Tender | 123 |
| 20 | Annexure 1 to 4 | 124 |
| 21 | Annexure "N” Payment of wages through digital mode | 128 |
| 22 | Annexure "O" Unemployed Engineer circular | 129 |
| 23 | Annexure "P" GST Order Copy | 132 |
| 24 | Annexure "Q" Drawings | 133 |

SALIENT FEATURE OF TENDER

| 1. | Name of work | Balance Work of Construction, Testing and Commissioning of R.C.C. Elevated Service Reservoir of 40 KL capacities & staging 12 mt. (35% Balance Work) at Markatola including providing and fixing of CI Double flange pipe, double flange specials & DI double flange soft seated sluice valve including construction of suitable R.C.C. Valve Chamber & Providing, laying, jointing, interconnection, testing and commissioning of 90/110 mm dia UPVC class 06/10 kg/sqcm Pipe, 63/75 mm dia HDPE 06 kg/sqcm Pipe with suitable joints, valve specials and all fittings including all allied civil works, including Electric connection from CSPDCL, Road Cutting, Switch room cum pump House, Information Board, construction of boundary wall around the over head tank, installation of electro chlorinator, construction of chlorinator room, providing functional household tap connection (F.H.T.C) in individual households and government institution etc. complete with 03 months trial run, operation & maintenance for 06 months under Single Village of Piped Water Supply Scheme under Jal Jeevan Mission For Village Markatola, Dhanegaon Block Mohla District- Mohla-Manpur-A.Chowki . |
| --- | --- | --- |
| 2. | Name of contractors A. Class of Contractor registered in Unified Registration system, E-Registration. B. Postal Address C. Telephone Number & Fax number if any |  |
| 3. | Probable amount of contract | Rs. 19.36 Lakhs |
| 4. | Earnest Money | Rs. 14600.00 |
| 5. | Date of Issue |  |
| 6. | Money receipt no. and date |  |

Note - This tender document with Annexure and notice inviting tender shall be part of the agreement.

Office of Executive Engineer and Member Secretary,

DWSM, Mohla-Manpur-A.Chowki , (C.G.).

NIT No./32/DWSM, MOHLA-MANPUR-A.CHOWKI /2026					Dated : 12-06-2026

NOTICE OF INVITING TENDER

Risk & Cost

On behalf of the DWSM, MOHLA-MANPUR-A.CHOWKI online percentage rate tender in form "A" is invited by Executive Engineerand member secretary for the following work from contractors registered in Class “D” & above Under Unified Registration System, E-Registration in Govt. of Chhattisgarh.

In respect of percentage rate tenders, contractor should quote his separate tender percentage rate above or below the following schedules of rate "The Schedule of rates issued by the Engineer-in-Chief, C.G. Raipur PHE Raipur for water supply and sewerage inforce in PHE Department with effective form dated 01-06-2020 & all amendments issued up to the 05/2021-22.

“The schedule of rates issued by the Engineer-in-Chief, CG PWD Raipur of year 2015 for building works inforce in PHE Department from 07.04.2016 with all amendments issued up to the date of issue of N.I.T.”

"The SOR applicable for different item of work is mentioned against each item of work".

NAME OF WORK :-

| Group no. | Name of work | Probable amount of contract ( In lacs) | Earnest money | Time allowed for completion | Bid Submission fees | Validty of tender |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | Balance Work of Construction, Testing and Commissioning of R.C.C. Elevated Service Reservoir of 40 KL capacities & staging 12 mt. (35% Balance Work) at Markatola including providing and fixing of CI Double flange pipe, double flange specials & DI double flange soft seated sluice valve including construction of suitable R.C.C. Valve Chamber & Providing, laying, jointing, interconnection, testing and commissioning of 90/110 mm dia UPVC class 06/10 kg/sqcm Pipe, 63/75 mm dia HDPE 06 kg/sqcm Pipe with suitable joints, valve specials and all fittings including all allied civil works, including Electric connection from CSPDCL, Road Cutting, Switch room cum pump House, Information Board, construction of boundary wall around the over head tank, installation of electro chlorinator, construction of chlorinator room, providing functional household tap connection (F.H.T.C) in individual households and government institution etc. complete with 03 months trial run, operation & maintenance for 06 months under Single Village of Piped Water Supply Scheme under Jal Jeevan Mission For Village Markatola, Dhanegaon Block Mohla DISTRICT- Mohla-Manpur-A.Chowki | 19.36 Lakh | 14600.00 | 03 months including rainy season | Rs. 311.00 | 4 months ( from the bid due date |

M/S mjunction sevices Limited

Office no. 202, 2nd floor

Anjani Tower, Near Hotel

Laal Bagh Inn, Telibandha

Raipur -492001`Toll free no – 1800 258 2502, http://eproc.cgstate.gov.in

The Bidders are also invited to get themselves trained on the operations of the e-Procurement System. Bidders may get in touch with the Service Provider of the e-Procurement System for confirming the time and date for their training session.

Tender Schedule:-

| - Date-Time Detail(s) |  |
| --- | --- |
| Online Release NIT | 12-06-2026 10:30 A:M |
| Bid Start Date | 19-06-2026 10:30 AM |
| Pre Bid Discussion Details |  |
| Pre-Bid Discussion Type | OFFLINE (Office of the Executive Engineer/Member secretary DWSM, MOHLA-MANPUR-A.CHOWKI ) |
| Start Date | 25-06-2026 10:30 AM |
| End Date | 25-06-2026 12:30 PM |
| Corrigendum (if any) | _ |
| Bid Due Date | 03-07-2026 5:30 PM |
| Physical Doc Submission End Date | 06-07-2026 5:30 PM |
| Bid Open Date for Envelope “A” & “B” (Scheduled) | 07-07-2026 10:30 AM |

- All the Tenders are required to submit Envelope “A” physically containing the followings:

- Valid registration certificate in appropriate class under unified registration system - e-registration of Chhattisgarh govt.

- The Duly noterified affidavit regarding the details of works in hand indicating name of work, Agreement no., work order no. and date, amount of contract, period of completion, value of work and balance work in hand with details of work on the date of submission of Tender.

- EMD &  security deposit -

- The Earnest Money online Mode. The Earnest Money of the successful tenderer will be retained as part of the Security Deposit. The Earnest Money deposit will be  returned to the unsuccessful tenderer

- Each Bidder should submit Bid Document Fee and Earnest Money Deposit in the amount specified in the bid through Online Payment Gateway (https://eproc.cgstate.gov.in) or by RTGS/NEFT only.

- Tender Document Fee and EMD submitted in any other form e.g. Cash/Bank Guarantee/FDR/TDR etc. shall not be accepted.

- The EMD of unsuccessful bidder will be returned to their registered Account Number.

- EMD of the successful bidder shall be adjusted against Security Deposit and shall be relased as per provision of agreement clause-19 General Condition of Contract (GCC) i.e. Security Deposit deduction consists of Earnest Money plus a deduction of 5% SD amount from the payment made in each running bill, till the two together amounts to 5% of cost of work put to tender or 5% of the cost of the work executed when the same exceeds the cost of the work put to tender.

- Security Deposit of the contractor to the extent of 50% shall be refunded on his getting the Complition Certificate which includes Trail Run Period, provided that all the recoveries outstanding against him are realised. 25% of the amount shall be refunded on maintenance period being over i.e. 6 months may be extended for increase warranty which is extended due to non-performance of the system. Balance 25% shall be refunded after the final bill is passed (including successful completetion of O&M period).

- No interest shall be payble on SD.

II. All the Tenders are required to submit Envelope “B” Online containing the followings: -

- Experience certificate of successful completion of work of same nature in contractor's/firm's/company's own name indicating agreement no., work order no. and date, amount of contract, stipulated period of completion, & actual period of completion during last ten year i.e. 2016-17 to 2025-26.The certificate should be issued by an officer not below the rank of Executive Engineer.Only the work experience for laying and providing drinking water pipeline work will be validated.

- Financial turn over during last ten years i.e. 2016-17 to 2025-26.

3.	foHkkxh; fufonkvksa esa fu;ekuqlkj Hkkx ysus okys Bsdsnkj ;fn fdlh futh@lkoZtfud {ks=ksa esa dk;Z fd;s gS] rks os vius vuqHko izek.k i= laikfnr dk;Z{ks= ds] lacaf/kr v/kh{k.k vfHk;ark ;k mPp Lrj ds dk;kZy; esa tek djsaA bl izdkj izLrqr fd;s x;s vuqHko izek.k i= dh lR;rk dh iqf’V l{ke vf/kdkjh }kjk vo”; dj fy;k tkos rFkk ;FkklaHko ftu dk;ksZZ ds laca/k esa izek.k i= izLrqr fd;k x;k gS] LFky  fujh{k.k }kjk bldk lR;kiu Hkh dj fy;k tkos rkfd dk;Z dh xq.koRrk ,oa vkadyu gks ldsA bl izdkj futh@lkoZtfud midze {ks=ksa esa fd;s x;s dk;Z gsrq foHkkxh; vf/kdkjh v/kh{k.k vfHk;ark ;k mPp Lrj }kjk tkjh izek.k i= dks lacaf/kr Bsdsnkj fufonk dh izfd;k esa dk;Z vuqHko izek.k i= ds :Ik esa izLrqr dj ldsaxs] ftls fufonk izfd;k esa ekU; fd;k tkosxkA

III. All the Tenders are required to submit Envelope “C” Online containing the Price Bid

ELIGIBILTY CRITERIA FOR TENDER DOCUMENT

1-	yksd fuekZ.k foHkkx N-x- esa ,dhd`r iath;u O;oLFkk&bZ jftLVsª'ku esa mi;qDr Js.kh es aiath;u     	vko';d gSA rFkk bZ&VsaMfjax esa Hkkx ysus gsrq N-x- “kklu ds bZ&izksD;ksjesaV iksVZy esa Hkh iath;u 	vko”;d gSA

2-		dk;Z fu"iknu {kerk %&

1½ ;fn dksbZ fufonkdkj igyh ckj fufonk esa Hkkx ys jgk gS rks mlds fy;s :-100-00 yk[k rd ds dk;Z gsrq dksbZ Bid Capacity dh x.kuk dh vko”;drk ugh gksxh A

2½ ;fn fufonkdkj ty thou fe”ku ds dk;ksZ esa fdlh Hkh ,d dk;Z dk dk;kZns”k izkIr dj fy;k gks] rks mls Bid Capacity dh x.kuk esa dksbZ NqV izkIr ugh gksxh A

dk;Z fu"iknu {kerk = 3 x A x (N/12) – C

tgka ij

A =	fiNys 10foRrh; o’kksZ esa  fd;s x;s dk;Z ds okf’kZd VuZ vksOgj pkVZM ,dkmUVsaV }kjk tkjh

fd;s x;s VuZ vksOgj esa 10 izfr”kr  vf/kHkkj izfro”kZlfgr esa vf/kdreA foxr 10 o’kksZ ds VuZvksoj dh tkudkjh lh-,- }kjk xks’kokjk ds :i esa tkjh fd;k x;k gksuk pkfg,A

N =	fufonk esa dk;Z iw.kZ djus dh fu/kkZfjr vof/k eghuksa esaA fufonk esa dk;Z dh vof/k ¼ o’kkZ _rqqq jfgr ½ dk mYYks[k djrs gq, fu/kkZfjr dh tkosaA

C =	fufonk tkjh gksus dh frfFk rd fufonkdkj }kjk leLr vuqcaf/kr Bsdksa ds ’ks"k dk;ksZ dh ykxr 20%vf/kHkkj izfro’kZlfgr dqy ykxrA

-		fdlh Hkh vuqca/k esa eki iqfLrdk esa fufonk tkjh gksus dh frfFk rd ntZ dk;ksaZ dks laikfnr 	dk;Z ekuk tkosxk ,oa ;g ‘ks"k dk;ksaZ ds ykxr dh x.kuk dk vk/kkj gksxkA

“A” ,oa  “C” gsrq Vscy fuEukuqlkj gS %&

STANDARD TABLE FOR CALCULATION OF ‘A’ & ‘C’ FOR BID CAPACITY

(EXAMPLE FOR YEAR 2026-27)

- Table for calculation of ‘A’

¼lkj.kh mnkgj.k Lo:i rS;kj dj iznf’kZr dh xbZ gSA pkyw foRrh; o’kZ 2026&27 ds vk/kkj ij½

| S.no. | Financial Year | Yearly Turnover | Factor of multiplication for calculation of projected value of Turnover | Projected value of Turnover (3x4) | Maximum value of projected turnover i.e. ‘A’ |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | 2025-26 |  | 1.10 |  |  |
| 2 | 2024-25 |  | 1.21 |  |  |
| 3 | 2023-24 |  | 1.33 |  |  |
| 4 | 2022-23 |  | 1.46 |  |  |
| 5 | 2021-22 |  | 1.61 |  |  |
| 6 | 2020-21 |  | 1.77 |  |  |
| 7 | 2019-20 |  | 1.95 |  |  |
| 8 | 2018-19 |  | 2.14 |  |  |
| 9 | 2017-18 |  | 2.36 |  |  |
| 10 | 2016-17 |  | 2.59 |  |  |

2. Table for calculation of ‘C’ ¼pkyw foRrh; o’kZ 2026&27 ds vk/kkj ij½

| S.no. | Financial Year | Total Balance work of Particular Year | Factor of multiplication for projected value of balance work | Projected value of balance work (3x4) |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 |
| 1 | 2026-27 |  | 1.00 |  |
| 2 | 2025-26 |  | 1.2 |  |
| 3 | 2024-25 |  | 1.44 |  |
| 4 | 2023-24 |  | 1.728 |  |
| 5 | 2022-23 |  | 2.074 |  |
| 6 | 2021-22 |  | 2.49 |  |
| 7 | 2020-21 |  | 2.99 |  |
| 8 | 2019-20 |  | 3.59 |  |
| 9 | 2018-19 |  | 4.31 |  |
| 10 | 2017-18 |  | 5.17 |  |
| 11 | 2016-17 |  | 6.20 |  |
| 12 | and so on |  | and so on |  |
| Total ‘C’ :- |  |  |  |  |

C= Summation of projected cost of all balance works

“ks’k dk;ksaZ dh ykxr dks vuqca/kokj fooj.k] Bsdsnkj fufonk gsrq vkosnu djrs le;@fufonk izi= izkIr djus gsrq dk;Zokgh bZ&Vs.Mfjax izkslsl esa mi;qDr LVst esa] U;wure :- 100@& jkf”k ds uksVjkbZTM “kiFk i= ftldh frfFk fufonk tkjh gksus ds ckn dh frfFk esa gksA “kiFk i= esa Li’V :i ls fufonk Øekad ,oa fnukad ds mYys[k ds lkFk fn;k tkos ,oa QeZ }kjk leLr vuqcaf/kr Bsdksa ds “ks’k dk;ksaZ dh ykxr dh tkudkjh izlrqr dh tk jgh gS] og iwjh rjg tkap iM+rky dj nh tk jgh gS ,oa dksbZ rF; ugha Nqik;k x;k gS ,oa xyrh ik;s tkus ij fufonkdkj dh fufonk gsrq ik=rk fujLr ekuh tkosxh] ftls os Lohdkj djus gsrq ck/; gksaxsA fufonkdkj }kjk izLrqr izi= esa odZ bu gS.M dh jkf’k vfuok;Z :i ls mYysf[kr fd;k x;k gks] odZ bu gS.M dh lwph vuqca/kokj ykxr lfgr uksVjh }kjk izekf.kr gksA

3-	Lo;a ds uke ij vFkok QeZ@daiuh ds uke ij dk;Z dk fufonkdkj dks vuqHko gksuk pkfg,A ftl dk;Z dk vuqHko fufonkdkj }kjk izLrqr fd;k tk jgk gS] ml dk;Z dk dk;kZns”k dh frfFk] fufonk tkjh gksus dh frfFk ls foxr 10 foRrh; o’kZ ds Hkhrj dk gksA

vuqHko ds ijh{k.k gsrq vuqHko izek.k i= esa fuEukuqlkj tkudkjh ;qDr vuqHko izek.k i= Bsdsnkj }kjk izLrqr dh tkosxh ,oa fuEufyf[kr izk:i esa vko”;d izfof’V dj Bsdsnkj }kjk vuqHko izek.k i=ksa ds lkFk fufonk gsrq izLrqr djuk gksxkA %&

| Ø- | dk;Z dk uke ,oa rduhdh fooj.k | dk;kZns”k Ø- ,oa fnukad | dk;Z dh ykxr | dk;Z iw.kZ djus dh fu/kkZfjr vof/k vuqca/k ds vuqlkj | dk;Z iw.kZ djus dh okLrfod frfFk | lQyrkiwoZd iw.kZ fd;s x;s dk;Z dk rduhdh fooj.k ¼lacaf/kr dk;Z dh ih-D;w- dh “krksaZ ds vuqlkj½ | vuqHko izek.k i= tkjh djus okys rduhdh vf/kdkjh@dk;kZy; dk fooj.k U;wure dk;Zikyu vfHk;ark Lrj dk gks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

fufonk esa nf'kZr dk;ksZ dh ek=k ds x.kuk dk vkadyu fuEukuqlkj gksxk %&

foxr nl o"kksZ esa %&

ikbZi ykbZu dk;Z %&

1	;w-ih-Ogh-lh- ikbZi ykbZu %

¼i½	7500 eh- rd dh yackbZ ds dk;Z gsrq dsoy mi;qDr Js.kh esa iath;u vko’;d gksxkA

¼ii½	7500 eh- ls vf/kd yackbZ ds dk;Z gsrq ;w-ih-Ogh-lh- ikbZi ykbZu dh dqy yackbZ ds 25 izfr”kr  dk;Z dk vuqHko fdlh ,d vuqca/k esa lQyrkiwoZd VsfLVax lfgr fd;k tkuk vko’;d gksxkA

Minimum Diameter of distribution & Rising main pipe shall be 63 mm.

mPpLrjh; tykxkj %&

1½ 	mPpLrjh; tykxkj gsrq fufonk esa nf”kZr vf/kdre {kerk ds mPpLrjh; tykxkj dh {kerk dk 50 izfr'kr {kerk rFkk 50 izfr”kr LVsftax vFkok 10 ehVj nksuksa esa tks Hkh vf/kd gks] ds fuekZ.k dk lQyrk iwoZd iw.kZ djus dk vuqHko vko';d gSA

2½		lEiosy gsrq fufonk esa nf’kZr vf/kdre {kerk ds lEiosy dh {kerk dk 50 izfr”kr {kerk ds fuekZ.k vFkok        50 izfr”kr {kerk ds mPpLrjh; tykxkj fuekZ.k dk lQyrkiwoZd iw.kZ djus dk vuqHko vko”;d gSA

3½	75 fd-yh- rd dh {kerk 12 eh LVsftax ds mPpLrjh; tykxkj ds fy;s mi;qDr Js.kh esa iath;u i;kZIr gksxkA

fo'ks"k Vhi %&

  - mijksDr leLr dk;ksZ ds fy, fufonkdkj }kjk fufonk esa nf'kZr vuqlkj fMIyksek@fMxzh /kkjh vfHk;ark ds fu;qfDr dk izek.k&i= layXu djuk vfuok;Z gksxkA dk;Z laiknu dh vof/k ds nkSjku mUgs fd;s x;s Hkqxrku ls lacaf/kr cSad ,dkmaV dh Nk;kizfr izR;sd ns;d ds lkFk izLrqr djuk gksxkA

  - fo'ks"k ifjfLFkfr;ksa esa fufonkvksa esa vgZrk] ekinaM ,oa 'krsZ 'kklu ls vuqefr i'pkr SWSM }kjk ifjorZu fd;k tk ldsxkA

  - mijksDr ik=rk fu/kkZj.k dh “krksZ ds vuq:Ik :Ik;s 5-00 djksM+ rd dh fufonkvksa esa DWSM, MOHLA MANPUR A.CHOWKI  ik=rk fu/kkZj.k djus esa l{ke gksxsaA

  - Contrctor should arrange for IEC work and training to VWSC members/gram panchayat members for operation and maintainance of piped water supply scheme during O&M period.

Note:-

- For details on tendering procedure through the electronic tendering system, please refer to “Guide lines for bidders on using intergrated eprocurment system” document available along with the tender documents as Annexure-K

- This NIT shall form part of the agreement.  The other conditions, for the pre qualification and details of works can be seen in the office of the undersigned and also on the PHE sub portal(https://eproc.cgstate.gov.in)

- The DWSM, MOHLA MANPUR A.CHOWKI shall reserves the right to change the key dates of the tender process.

|  | Executive Engineer and member secretary , DWSM, MOHLA MANPUR A.CHOWKI (C.G.) |
| --- | --- |

Endt./		 /DWSM, MOHLA MANPUR A.CHOWKI /2022	………, Dated :

Copy forwarded to -

- P.A. to Minister of P.H.E.Deptt. C.G. Govt.,  Raipur C.G.

- Secretary, Govt. of Chhattisgarh, Public Health Engg. Department, Mantralaya Naya Raipur

- Director JJM , Neer Bhavan Raipur.

- The Engineer-in-Chief, Public Health Engineering Department, Chhattisgarh Raipur

- The Chief Engineer, PHED, Raipur Zone, Raipur Zone

- The Superintending Engineer, PHE Deptt Durg Circle Durg.

- The Collector,and President DWSM, Mohla Manpur A.Chowki,  District - Mohla Manpur A.Chowki

- Chief Executive Officer, Zila Panchayat, District Mohla Manpur A.Chowki

- The Executive Engineer and member secretary DWSM, Mohla Manpur A.Chowki, Public Health Engg. Division Mohla Manpur A.Chowki

- ...................... member  DWSM, Mohla Manpur A.Chowki,  District - Mohla Manpur A.Chowki

- The Assistant Engineer, Public Health Engg. Sub-Division, Mohla Manpur/A.Chowki District Mohla Manpur A.Chowki

- The Sarpanch, Gram Panchayat...................Block.................District Mohla Manpur A.Chowki

- Chiarman, VWSC, Gram Panchayat...................Block.................District Mohla Manpur A.Chowki

- E.E. MIS, O/o the E-in-C PHED Raipur.  It is requested to make available this NIT on Public Health Engineering Department C.G. Website (https://eproc.cgstate.gov.in) and other related state govt. websites too for wide publicity.

# Notice Board

|  | Executive Engineer and member secretary DWSM, MOHLA MANPUR A.CHOWKI (C.G.) |
| --- | --- |

dk;kZy; dk;Zikyu vfHk;ark ,oa lnL; lfpo

¼ftyk ty ,oa LoPNrk fe”ku½

yksd LokLF; ;kaf=dh [kaM] eksgyk ekuiqj va-pkSdh ¼N-x-½

NIT No./32/DWSM, MOHLA-MANPUR-A.CHOWKI /2025				ated : 12-06-2026

fufonk vkea=.k lwpuk fjLd ,aM dkLV

vkWuykbZu fufonk,a **izi=&,** esa izfr'kr nj ij uhps mYysf[kr dk;Z ds fy;s DWSM, MOHLA-MANPUR-A.CHOWKI dh vksj ls dk;Zikyu vfHk;ark ,oa lnL; lfpo DWSM, MOHLA-MANPUR-A.CHOWKI  }kjk +,dhdz`r iath;u O;oLFkk bZ&jftLVªs'ku dh Js.kh “D” ,oa mPp esa iath—r Bsdsnkjksa ls vkeaf=r dh tkrh gS

çfr'kr nj fufonkvksa ds lacaèk esa] Bsdsnkj dks viuh vyx fufonk çfr'kr nj fuEufyf[kr nj vuqlwfp;ksa ds Åij ;k uhps m)`r djuh pkfg,A eq[; vfHk;ark] lh-th- jk;iqj ih,pbZ jk;iqj }kjk ih,pbZ foHkkx esa ty vkiwfrZ vkSj lhojst ds fy, tkjh njksa dh vuqlwph] fnukad 01&06&2020 ls çHkkoh vkSj 06@2021&22 rd tkjh lHkh la'kksèkuksa ds lkFkA

eq[; vfHk;ark] lhth ihMCY;wMh jk;iqj }kjk o"kZ 2015 esa ih,pbZ foHkkx esa Hkou fuekZ.k dk;ksZa ds fy, tkjh njksa dh vuqlwph] fnukad 07-04-2016 ls ,uvkbZVh tkjh gksus dh frfFk rd tkjh lHkh la'kksèkuksa ds lkFkA

fofHkUu dk;Z enksa ds fy, ykxw ,lvksvkj çR;sd dk;Z en ds lkeus mYysf[kr gSA

| Øa- | dk;Z dk uke | vuqekfur ykxr ¼:- yk[k esa½ | /kjksgj jkf'k | dk;Z iw.kZ djus dh vof/k | fufonk Tkek djus dk ewY; | fufonk dh oS/krk |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | ty thou fe”ku varxZr v/kwjs NksM+s dk;Z gsrq fjLd ,aM dkLV esa ,dy ;kstuk uy ty iznk; ;kstukarxZr jktukanxkao ftys ds fodkl[kaM eksgyk ds xzke ejdkVksyk esa 40 fd-yh- {kerk 12 eh- LVsftax ¼35 izfr”kr dk;Z “ks’k½ dh vkj-lh-lh- mPp Lrjh; Vadh dk fuekZ.k dk lEiw.kZ dk;Z ,oa 90/110 mm dia UPVC class 06/10 kg/sqcm Pipe, 63/75 mm dia HDPE 06 kg/sqcm Pipe Lis”kYl lfgr iznk; dj tksM+us] fcNkus] bUVjdusD”ku ,oa CSPDCL, ROAD CUTTING, Lohp :e] ckm.Mªhoky] bysDVªksDyksjhusVj LFkkiuk] DyksjhusVj :e] ?kjksa ,oa “kkldh; laLFkkuksa esa dk;Zjr uy dusD”ku iznk; djus laca/kh vkfn leLr dk;Z] VsfLVax] 03 ekg dk Vªk;y ju] 6 ekg rd lapkyu la/kkj.k dk dk;ZA ¼lEiw.kZ dk;Z lkexzh lfgr½ | 19.36 yk[k | 14600.00 | 03 ekg ¼o"kkZ _rq lfgr½ | 311@& | 04 ekg ¼vafre fcM frfFk ls½ |

M/S mjunction sevices Limited

Office no. 202, 2nd floor

Anjani Tower, Near Hotel

Laal Bagh Inn, Telibandha

Raipur -492001`Toll free no – 1800 258 2502, http://eproc.cgstate.gov.in

The Bidders are also invited to get themselves trained on the operations of the e-Procurement System. Bidders may get in touch with the Service Provider of the e-Procurement System for confirming the time and date for their training session.

Tender Schedule:-

| - Date-Time Detail(s) |  |
| --- | --- |
| Online Release NIT | 12-06-202610:30 A:M |
| Bid Start Date | 19-06-202610:30 AM |
| Pre Bid Discussion Details |  |
| Pre-Bid Discussion Type | OFFLINE (Office of the Executive Engineer/Member secretary DWSM, MOHLA-MANPUR-A.CHOWKI ) |
| Start Date | 25-06-202610:30 AM |
| End Date | 25-06-202612:30 PM |
| Corrigendum (if any) | _ |
| Bid Due Date | 03-07-20265:30 PM |
| Physical Doc Submission End Date | 06-07-2026 5:30 PM |
| Bid Open Date for Envelope “A” & “B” (Scheduled) | 07-07-202610:30 AM |

- All the Tenders are required to submit Envelope “A” physically containing the followings:

- Valid registration certificate in appropriate class under unified registration system - e-registration of Chhattisgarh govt.

- The Duly noterified affidavit regarding the details of works in hand indicating name of work, Agreement no., work order no. and date, amount of contract, period of completion, value of work and balance work in hand with details of work on the date of submission of Tender.

- EMD &  security deposit -

- The Earnest Money online Mode. The Earnest Money of the successful tenderer will be retained as part of the Security Deposit. The Earnest Money deposit will be  returned to the unsuccessful tenderer

- Each Bidder should submit Bid Document Fee and Earnest Money Deposit in the amount specified in the bid through Online Payment Gateway (https://eproc.cgstate.gov.in) or by RTGS/NEFT only.

- Tender Document Fee and EMD submitted in any other form e.g. Cash/Bank Guarantee/FDR/TDR etc. shall not be accepted.

- The EMD of unsuccessful bidder will be returned to their registered Account Number.

- EMD of the successful bidder shall be adjusted against Security Deposit and shall be relased as per provision of agreement clause-19 General Condition of Contract (GCC) i.e. Security Deposit deduction consists of Earnest Money plus a deduction of 5% SD amount from the payment made in each running bill, till the two together amounts to 5% of cost of work put to tender or 5% of the cost of the work executed when the same exceeds the cost of the work put to tender.

- Security Deposit of the contractor to the extent of 50% shall be refunded on his getting the Complition Certificate which includes Trail Run Period, provided that all the recoveries outstanding against him are realised. 25% of the amount shall be refunded on maintenance period being over i.e. 6 months may be extended for increase warranty which is extended due to non-performance of the system. Balance 25% shall be refunded after the final bill is passed (including successful completetion of O&M period).

- No interest shall be payble on SD.

II. All the Tenders are required to submit Envelope “B” Online containing the followings: -

- Experience certificate of successful completion of work of same nature in contractor's/firm's/company's own name indicating agreement no., work order no. and date, amount of contract, stipulated period of completion, & actual period of completion during last ten year i.e. 2016-17 to 2025-26.The certificate should be issued by an officer not below the rank of Executive Engineer.Only the work experience for laying and providing drinking water pipeline work will be validated.

- Financial turn over during last ten years i.e. 2016-17 to 2025-26.

3.	foHkkxh; fufonkvksa esa fu;ekuqlkj Hkkx ysus okys Bsdsnkj ;fn fdlh futh@lkoZtfud {ks=ksa esa dk;Z fd;s gS] rks os vius vuqHko izek.k i= laikfnr dk;Z{ks= ds] lacaf/kr v/kh{k.k vfHk;ark ;k mPp Lrj ds dk;kZy; esa tek djsaA bl izdkj izLrqr fd;s x;s vuqHko izek.k i= dh lR;rk dh iqf’V l{ke vf/kdkjh }kjk vo”; dj fy;k tkos rFkk ;FkklaHko ftu dk;ksZZ ds laca/k esa izek.k i= izLrqr fd;k x;k gS] LFky  fujh{k.k }kjk bldk lR;kiu Hkh dj fy;k tkos rkfd dk;Z dh xq.koRrk ,oa vkadyu gks ldsA bl izdkj futh@lkoZtfud midze {ks=ksa esa fd;s x;s dk;Z gsrq foHkkxh; vf/kdkjh v/kh{k.k vfHk;ark ;k mPp Lrj }kjk tkjh izek.k i= dks lacaf/kr Bsdsnkj fufonk dh izfd;k esa dk;Z vuqHko izek.k i= ds :Ik esa izLrqr dj ldsaxs] ftls fufonk izfd;k esa ekU; fd;k tkosxkA

III. All the Tenders are required to submit Envelope “C” Online containing the Price Bid

ELIGIBILTY CRITERIA FOR TENDER DOCUMENT

1-	yksd fuekZ.k foHkkx N-x- esa ,dhd`r iath;u O;oLFkk&bZ jftLVsª'ku esa mi;qDr Js.kh es aiath;u     	vko';d gSA rFkk bZ&VsaMfjax esa Hkkx ysus gsrq N-x- “kklu ds bZ&izksD;ksjesaV iksVZy esa Hkh iath;u 	vko”;d gSA

2-		dk;Z fu"iknu {kerk %&

1½ ;fn dksbZ fufonkdkj igyh ckj fufonk esa Hkkx ys jgk gS rks mlds fy;s :-100-00 yk[k rd ds dk;Z gsrq dksbZ Bid Capacity dh x.kuk dh vko”;drk ugh gksxh A

2½ ;fn fufonkdkj ty thou fe”ku ds dk;ksZ esa fdlh Hkh ,d dk;Z dk dk;kZns”k izkIr dj fy;k gks] rks mls Bid Capacity dh x.kuk esa dksbZ NqV izkIr ugh gksxh A

dk;Z fu"iknu {kerk = 3 x A x (N/12) – C

tgka ij

A =	fiNys 10foRrh; o’kksZ esa  fd;s x;s dk;Z ds okf’kZd VuZ vksOgj pkVZM ,dkmUVsaV }kjk tkjh

fd;s x;s VuZ vksOgj esa 10 izfr”kr  vf/kHkkj izfro”kZlfgr esa vf/kdreA foxr 10 o’kksZ ds VuZvksoj dh tkudkjh lh-,- }kjk xks’kokjk ds :i esa tkjh fd;k x;k gksuk pkfg,A

N =	fufonk esa dk;Z iw.kZ djus dh fu/kkZfjr vof/k eghuksa esaA fufonk esa dk;Z dh vof/k ¼ o’kkZ _rqqq jfgr ½ dk mYYks[k djrs gq, fu/kkZfjr dh tkosaA

C =	fufonk tkjh gksus dh frfFk rd fufonkdkj }kjk leLr vuqcaf/kr Bsdksa ds ’ks"k dk;ksZ dh ykxr 20%vf/kHkkj izfro’kZlfgr dqy ykxrA

-		fdlh Hkh vuqca/k esa eki iqfLrdk esa fufonk tkjh gksus dh frfFk rd ntZ dk;ksaZ dks laikfnr 	dk;Z ekuk tkosxk ,oa ;g ‘ks"k dk;ksaZ ds ykxr dh x.kuk dk vk/kkj gksxkA

“A” ,oa  “C” gsrq Vscy fuEukuqlkj gS %&

STANDARD TABLE FOR CALCULATION OF ‘A’ & ‘C’ FOR BID CAPACITY

(EXAMPLE FOR YEAR 2026-27)

- Table for calculation of ‘A’

¼lkj.kh mnkgj.k Lo:i rS;kj dj iznf’kZr dh xbZ gSA pkyw foRrh; o’kZ 2026&27 ds vk/kkj ij½

| S.no. | Financial Year | Yearly Turnover | Factor of multiplication for calculation of projected value of Turnover | Projected value of Turnover (3x4) | Maximum value of projected turnover i.e. ‘A’ |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | 2025-26 |  | 1.10 |  |  |
| 2 | 2024-25 |  | 1.21 |  |  |
| 3 | 2023-24 |  | 1.33 |  |  |
| 4 | 2022-23 |  | 1.46 |  |  |
| 5 | 2021-22 |  | 1.61 |  |  |
| 6 | 2020-21 |  | 1.77 |  |  |
| 7 | 2019-20 |  | 1.95 |  |  |
| 8 | 2018-19 |  | 2.14 |  |  |
| 9 | 2017-18 |  | 2.36 |  |  |
| 10 | 2016-17 |  | 2.59 |  |  |

2. Table for calculation of ‘C’ ¼pkyw foRrh; o’kZ 2026&27 ds vk/kkj ij½

| S.no. | Financial Year | Total Balance work of Particular Year | Factor of multiplication for projected value of balance work | Projected value of balance work (3x4) |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 |
| 1 | 2026-27 |  | 1.00 |  |
| 2 | 2025-26 |  | 1.2 |  |
| 3 | 2024-25 |  | 1.44 |  |
| 4 | 2023-24 |  | 1.728 |  |
| 5 | 2022-23 |  | 2.074 |  |
| 6 | 2021-22 |  | 2.49 |  |
| 7 | 2020-21 |  | 2.99 |  |
| 8 | 2019-20 |  | 3.59 |  |
| 9 | 2018-19 |  | 4.31 |  |
| 10 | 2017-18 |  | 5.17 |  |
| 11 | 2016-17 |  | 6.20 |  |
| 12 | and so on |  | and so on |  |
| Total ‘C’ :- |  |  |  |  |

C= Summation of projected cost of all balance works

“ks’k dk;ksaZ dh ykxr dks vuqca/kokj fooj.k] Bsdsnkj fufonk gsrq vkosnu djrs le;@fufonk izi= izkIr djus gsrq dk;Zokgh bZ&Vs.Mfjax izkslsl esa mi;qDr LVst esa] U;wure :- 100@& jkf”k ds uksVjkbZTM “kiFk i= ftldh frfFk fufonk tkjh gksus ds ckn dh frfFk esa gksA “kiFk i= esa Li’V :i ls fufonk Øekad ,oa fnukad ds mYys[k ds lkFk fn;k tkos ,oa QeZ }kjk leLr vuqcaf/kr Bsdksa ds “ks’k dk;ksaZ dh ykxr dh tkudkjh izlrqr dh tk jgh gS] og iwjh rjg tkap iM+rky dj nh tk jgh gS ,oa dksbZ rF; ugha Nqik;k x;k gS ,oa xyrh ik;s tkus ij fufonkdkj dh fufonk gsrq ik=rk fujLr ekuh tkosxh] ftls os Lohdkj djus gsrq ck/; gksaxsA fufonkdkj }kjk izLrqr izi= esa odZ bu gS.M dh jkf’k vfuok;Z :i ls mYysf[kr fd;k x;k gks] odZ bu gS.M dh lwph vuqca/kokj ykxr lfgr uksVjh }kjk izekf.kr gksA

3-	Lo;a ds uke ij vFkok QeZ@daiuh ds uke ij dk;Z dk fufonkdkj dks vuqHko gksuk pkfg,A ftl dk;Z dk vuqHko fufonkdkj }kjk izLrqr fd;k tk jgk gS] ml dk;Z dk dk;kZns”k dh frfFk] fufonk tkjh gksus dh frfFk ls foxr 10 foRrh; o’kZ ds Hkhrj dk gksA

vuqHko ds ijh{k.k gsrq vuqHko izek.k i= esa fuEukuqlkj tkudkjh ;qDr vuqHko izek.k i= Bsdsnkj }kjk izLrqr dh tkosxh ,oa fuEufyf[kr izk:i esa vko”;d izfof’V dj Bsdsnkj }kjk vuqHko izek.k i=ksa ds lkFk fufonk gsrq izLrqr djuk gksxkA %&

| Ø- | dk;Z dk uke ,oa rduhdh fooj.k | dk;kZns”k Ø- ,oa fnukad | dk;Z dh ykxr | dk;Z iw.kZ djus dh fu/kkZfjr vof/k vuqca/k ds vuqlkj | dk;Z iw.kZ djus dh okLrfod frfFk | lQyrkiwoZd iw.kZ fd;s x;s dk;Z dk rduhdh fooj.k ¼lacaf/kr dk;Z dh ih-D;w- dh “krksaZ ds vuqlkj½ | vuqHko izek.k i= tkjh djus okys rduhdh vf/kdkjh@dk;kZy; dk fooj.k U;wure dk;Zikyu vfHk;ark Lrj dk gks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

fufonk esa nf'kZr dk;ksZ dh ek=k ds x.kuk dk vkadyu fuEukuqlkj gksxk %&

foxr nl o"kksZ esa %&

ikbZi ykbZu dk;Z %&

1	;w-ih-Ogh-lh- ikbZi ykbZu %

¼i½	7500 eh- rd dh yackbZ ds dk;Z gsrq dsoy mi;qDr Js.kh esa iath;u vko’;d gksxkA

¼ii½	7500 eh- ls vf/kd yackbZ ds dk;Z gsrq ;w-ih-Ogh-lh- ikbZi ykbZu dh dqy yackbZ ds 25 izfr”kr  dk;Z dk vuqHko fdlh ,d vuqca/k esa lQyrkiwoZd VsfLVax lfgr fd;k tkuk vko’;d gksxkA

Minimum Diameter of distribution & Rising main pipe shall be 63 mm.

mPpLrjh; tykxkj %&

1½ 	mPpLrjh; tykxkj gsrq fufonk esa nf”kZr vf/kdre {kerk ds mPpLrjh; tykxkj dh {kerk dk 50 izfr'kr {kerk rFkk 50 izfr”kr LVsftax vFkok 10 ehVj nksuksa esa tks Hkh vf/kd gks] ds fuekZ.k dk lQyrk iwoZd iw.kZ djus dk vuqHko vko';d gSA

2½		lEiosy gsrq fufonk esa nf’kZr vf/kdre {kerk ds lEiosy dh {kerk dk 50 izfr”kr {kerk ds fuekZ.k vFkok        50 izfr”kr {kerk ds mPpLrjh; tykxkj fuekZ.k dk lQyrkiwoZd iw.kZ djus dk vuqHko vko”;d gSA

3½	75 fd-yh- rd dh {kerk 12 eh LVsftax ds mPpLrjh; tykxkj ds fy;s mi;qDr Js.kh esa iath;u i;kZIr gksxkA

fo'ks"k Vhi %&

- mijksDr leLr dk;ksZ ds fy, fufonkdkj }kjk fufonk esa nf'kZr vuqlkj fMIyksek@fMxzh /kkjh vfHk;ark ds fu;qfDr dk izek.k&i= layXu djuk vfuok;Z gksxkA dk;Z laiknu dh vof/k ds nkSjku mUgs fd;s x;s Hkqxrku ls lacaf/kr cSad ,dkmaV dh Nk;kizfr izR;sd ns;d ds lkFk izLrqr djuk gksxkA

- fo'ks"k ifjfLFkfr;ksa esa fufonkvksa esa vgZrk] ekinaM ,oa 'krsZ 'kklu ls vuqefr i'pkr SWSM }kjk ifjorZu fd;k tk ldsxkA

- mijksDr ik=rk fu/kkZj.k dh “krksZ ds vuq:Ik :Ik;s 5-00 djksM+ rd dh fufonkvksa esa DWSM, MOHLA MANPUR A.CHOWKI  ik=rk fu/kkZj.k djus esa l{ke gksxsaA

- Contrctor should arrange for IEC work and training to VWSC members/gram panchayat members for operation and maintainance of piped water supply scheme during O&M period.

Note:-

- For details on tendering procedure through the electronic tendering system, please refer to “Guide lines for bidders on using intergrated eprocurment system” document available along with the tender documents as Annexure-K

- This NIT shall form part of the agreement.  The other conditions, for the pre qualification and details of works can be seen in the office of the undersigned and also on the PHE sub portal(https://eproc.cgstate.gov.in)

- The DWSM, MOHLA MANPUR A.CHOWKI shall reserves the right to change the key dates of the tender process.

|  | Executive Engineer and member secretary , DWSM, MOHLA MANPUR A.CHOWKI (C.G.) |
| --- | --- |

Endt./		 /DWSM, MOHLA MANPUR A.CHOWKI /2022	………, Dated :

Copy forwarded to -

- P.A. to Minister of P.H.E.Deptt. C.G. Govt.,  Raipur C.G.

- Secretary, Govt. of Chhattisgarh, Public Health Engg. Department, Mantralaya Naya Raipur

- Director JJM , Neer Bhavan Raipur.

- The Engineer-in-Chief, Public Health Engineering Department, Chhattisgarh Raipur

- The Chief Engineer, PHED, Raipur Zone, Raipur Zone

- The Superintending Engineer, PHE Deptt Durg Circle Durg.

- The Collector,and President DWSM, Mohla Manpur A.Chowki,  District - Mohla Manpur A.Chowki

- Chief Executive Officer, Zila Panchayat, District Mohla Manpur A.Chowki

- The Executive Engineer and member secretary DWSM, Mohla Manpur A.Chowki, Public Health Engg. Division Mohla Manpur A.Chowki

- ...................... member  DWSM, Mohla Manpur A.Chowki,  District - Mohla Manpur A.Chowki

- The Assistant Engineer, Public Health Engg. Sub-Division, Mohla Manpur/A.Chowki District Mohla Manpur A.Chowki

- The Sarpanch, Gram Panchayat...................Block.................District Mohla Manpur A.Chowki

- Chiarman, VWSC, Gram Panchayat...................Block.................District Mohla Manpur A.Chowki

- E.E. MIS, O/o the E-in-C PHED Raipur.  It is requested to make available this NIT on Public Health Engineering Department C.G. Website (https://eproc.cgstate.gov.in) and other related state govt. websites too for wide publicity.

# Notice Board

|  | Executive Engineer and member secretary DWSM, MOHLA MANPUR A.CHOWKI (C.G.) |
| --- | --- |

(CHAPTER II)                                         APPENDIX 2.10                                  (Appendix 2.10)

(See paragraph 2.079)

DISTRICT WATER AND SANITATION MISSION, DISTRICT MOHLA-MANPUR-A.CHOWKI

DETAILED NOTICE INVITING TENDERS

(In Form A.B.C. )

INTRODUCTION

On behalf of the DWSM, MOHLA-MANPUR-A.CHOWKI , online percentage rate tender in form "A" is invited by Executive Engineer and member secretary for the following work from contractors registered in Class “D” & above Under Unified Registration System, E-Registration in Govt. of Chhattisgarh.

In respect of percentage rate tenders, contractor should quote his separate tender percentage rate above or below the following schedules of rate "The Schedule of rates issued by the Engineer-in-Chief, C.G. Raipur PHE Raipur for water supply and sewerage inforce in PHE Department with effective form dated 01-06-2020 & all amendments issued up to the 06/2021-22.

“The schedule of rates issued by the Engineer-in-Chief, CG PWD Raipur of year 2015 for building works inforce in PHE Department from 07.04.2016 with all amendments issued up to the date of issue of N.I.T.”

"The SOR applicable for different item of work is mentioned against each item of work".

NAME OF WORK :-

| Group no. | Name of work | Probable amount of contract ( In lacs) | Earnest money | Time allowed for completion | Bid Submission fees | Validty of tender |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | Balance Work of Construction, Testing and Commissioning of R.C.C. Elevated Service Reservoir of 40 KL capacities & staging 12 mt. (35% Balance Work) at Markatola including providing and fixing of CI Double flange pipe, double flange specials & DI double flange soft seated sluice valve including construction of suitable R.C.C. Valve Chamber & Providing, laying, jointing, interconnection, testing and commissioning of 90/110 mm dia UPVC class 06/10 kg/sqcm Pipe, 63/75 mm dia HDPE 06 kg/sqcm Pipe with suitable joints, valve specials and all fittings including all allied civil works, including Electric connection from CSPDCL, Road Cutting, Switch room cum pump House, Information Board, construction of boundary wall around the over head tank, installation of electro chlorinator, construction of chlorinator room, providing functional household tap connection (F.H.T.C) in individual households and government institution etc. complete with 03 months trial run, operation & maintenance for 06 months under Single Village of Piped Water Supply Scheme under Jal Jeevan Mission For Village Markatola, Dhanegaon Block Mohla District- Mohla-Manpur-A.Chowki . | 19.36 Lakh | 14600.00 | 03 months including rainy season | Rs. 311.00 | 4 months ( from the bid due date |

M/S mjunction sevices Limited

Office no. 202, 2nd floor

Anjani Tower, Near Hotel

Laal Bagh Inn, Telibandha

Raipur -492001`Toll free no – 1800 258 2502, http://eproc.cgstate.gov.in

The Bidders are also invited to get themselves trained on the operations of the e-Procurement System. Bidders may get in touch with the Service Provider of the e-Procurement System for confirming the time and date for their training session.

Tender Schedule:-

| - Date-Time Detail(s) |  |
| --- | --- |
| Online Release NIT | 12-06-202610:30 A:M |
| Bid Start Date | 19-06-202610:30 AM |
| Pre Bid Discussion Details |  |
| Pre-Bid Discussion Type | OFFLINE (Office of the Executive Engineer/Member secretary DWSM, MOHLA-MANPUR-A.CHOWKI ) |
| Start Date | 25-06-202610:30 AM |
| End Date | 25-06-202612:30 PM |
| Corrigendum (if any) | _ |
| Bid Due Date | 03-07-20265:30 PM |
| Physical Doc Submission End Date | 06-07-2026 5:30 PM |
| Bid Open Date for Envelope “A” & “B” (Scheduled) | 07-07-202610:30 AM |

- All the Tenders are required to submit Envelope “A” physically containing the followings:

- Valid registration certificate in appropriate class under unified registration system - e-registration of Chhattisgarh govt.

- The Duly noterified affidavit regarding the details of works in hand indicating name of work, Agreement no., work order no. and date, amount of contract, period of completion, value of work and balance work in hand with details of work on the date of submission of Tender.

- EMD &  security deposit -

- The Earnest Money online Mode. The Earnest Money of the successful tenderer will be retained as part of the Security Deposit. The Earnest Money deposit will be  returned to the unsuccessful tenderer

- Each Bidder should submit Bid Document Fee and Earnest Money Deposit in the amount specified in the bid through Online Payment Gateway (https://eproc.cgstate.gov.in) or by RTGS/NEFT only.

- Tender Document Fee and EMD submitted in any other form e.g. Cash/Bank Guarantee/FDR/TDR etc. shall not be accepted.

- The EMD of unsuccessful bidder will be returned to their registered Account Number.

- EMD of the successful bidder shall be adjusted against Security Deposit and shall be relased as per provision of agreement clause-19 General Condition of Contract (GCC) i.e. Security Deposit deduction consists of Earnest Money plus a deduction of 5% SD amount from the payment made in each running bill, till the two together amounts to 5% of cost of work put to tender or 5% of the cost of the work executed when the same exceeds the cost of the work put to tender.

- Security Deposit of the contractor to the extent of 50% shall be refunded on his getting the Complition Certificate which includes Trail Run Period, provided that all the recoveries outstanding against him are realised. 25% of the amount shall be refunded on maintenance period being over i.e. 6 months may be extended for increase warranty which is extended due to non-performance of the system. Balance 25% shall be refunded after the final bill is passed (including successful completetion of O&M period).

- No interest shall be payble on SD.

II. All the Tenders are required to submit Envelope “B” Online containing the followings: -

- Experience certificate of successful completion of work of same nature in contractor's/firm's/company's own name indicating agreement no., work order no. and date, amount of contract, stipulated period of completion, & actual period of completion during last ten year i.e. 2016-17 to 2025-26.The certificate should be issued by an officer not below the rank of Executive Engineer.Only the work experience for laying and providing drinking water pipeline work will be validated.

- Financial turn over during last ten years i.e. 2016-17 to 2025-26.

3.	foHkkxh; fufonkvksa esa fu;ekuqlkj Hkkx ysus okys Bsdsnkj ;fn fdlh futh@lkoZtfud {ks=ksa esa dk;Z fd;s gS] rks os vius vuqHko izek.k i= laikfnr dk;Z{ks= ds] lacaf/kr v/kh{k.k vfHk;ark ;k mPp Lrj ds dk;kZy; esa tek djsaA bl izdkj izLrqr fd;s x;s vuqHko izek.k i= dh lR;rk dh iqf’V l{ke vf/kdkjh }kjk vo”; dj fy;k tkos rFkk ;FkklaHko ftu dk;ksZZ ds laca/k esa izek.k i= izLrqr fd;k x;k gS] LFky  fujh{k.k }kjk bldk lR;kiu Hkh dj fy;k tkos rkfd dk;Z dh xq.koRrk ,oa vkadyu gks ldsA bl izdkj futh@lkoZtfud midze {ks=ksa esa fd;s x;s dk;Z gsrq foHkkxh; vf/kdkjh v/kh{k.k vfHk;ark ;k mPp Lrj }kjk tkjh izek.k i= dks lacaf/kr Bsdsnkj fufonk dh izfd;k esa dk;Z vuqHko izek.k i= ds :Ik esa izLrqr dj ldsaxs] ftls fufonk izfd;k esa ekU; fd;k tkosxkA

III. All the Tenders are required to submit Envelope “C” Online containing the Price Bid

ELIGIBILTY CRITERIA FOR TENDER DOCUMENT

1-	yksd fuekZ.k foHkkx N-x- esa ,dhd`r iath;u O;oLFkk&bZ jftLVsª'ku esa mi;qDr Js.kh es aiath;u     	vko';d gSA rFkk bZ&VsaMfjax esa Hkkx ysus gsrq N-x- “kklu ds bZ&izksD;ksjesaV iksVZy esa Hkh iath;u 	vko”;d gSA

2-		dk;Z fu"iknu {kerk %&

1½ ;fn dksbZ fufonkdkj igyh ckj fufonk esa Hkkx ys jgk gS rks mlds fy;s :-100-00 yk[k rd ds dk;Z gsrq dksbZ Bid Capacity dh x.kuk dh vko”;drk ugh gksxh A

2½ ;fn fufonkdkj ty thou fe”ku ds dk;ksZ esa fdlh Hkh ,d dk;Z dk dk;kZns”k izkIr dj fy;k gks] rks mls Bid Capacity dh x.kuk esa dksbZ NqV izkIr ugh gksxh A

dk;Z fu"iknu {kerk = 3 x A x (N/12) – C

tgka ij

A =	fiNys 10foRrh; o’kksZ esa  fd;s x;s dk;Z ds okf’kZd VuZ vksOgj pkVZM ,dkmUVsaV }kjk tkjh

fd;s x;s VuZ vksOgj esa 10 izfr”kr  vf/kHkkj izfro”kZlfgr esa vf/kdreA foxr 10 o’kksZ ds VuZvksoj dh tkudkjh lh-,- }kjk xks’kokjk ds :i esa tkjh fd;k x;k gksuk pkfg,A

N =	fufonk esa dk;Z iw.kZ djus dh fu/kkZfjr vof/k eghuksa esaA fufonk esa dk;Z dh vof/k ¼ o’kkZ _rqqq jfgr ½ dk mYYks[k djrs gq, fu/kkZfjr dh tkosaA

C =	fufonk tkjh gksus dh frfFk rd fufonkdkj }kjk leLr vuqcaf/kr Bsdksa ds ’ks"k dk;ksZ dh ykxr 20%vf/kHkkj izfro’kZlfgr dqy ykxrA

-		fdlh Hkh vuqca/k esa eki iqfLrdk esa fufonk tkjh gksus dh frfFk rd ntZ dk;ksaZ dks laikfnr 	dk;Z ekuk tkosxk ,oa ;g ‘ks"k dk;ksaZ ds ykxr dh x.kuk dk vk/kkj gksxkA

“A” ,oa  “C” gsrq Vscy fuEukuqlkj gS %&

STANDARD TABLE FOR CALCULATION OF ‘A’ & ‘C’ FOR BID CAPACITY

(EXAMPLE FOR YEAR 2026-27)

- Table for calculation of ‘A’

¼lkj.kh mnkgj.k Lo:i rS;kj dj iznf’kZr dh xbZ gSA pkyw foRrh; o’kZ 2026&27 ds vk/kkj ij½

| S.no. | Financial Year | Yearly Turnover | Factor of multiplication for calculation of projected value of Turnover | Projected value of Turnover (3x4) | Maximum value of projected turnover i.e. ‘A’ |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | 2025-26 |  | 1.10 |  |  |
| 2 | 2024-25 |  | 1.21 |  |  |
| 3 | 2023-24 |  | 1.33 |  |  |
| 4 | 2022-23 |  | 1.46 |  |  |
| 5 | 2021-22 |  | 1.61 |  |  |
| 6 | 2020-21 |  | 1.77 |  |  |
| 7 | 2019-20 |  | 1.95 |  |  |
| 8 | 2018-19 |  | 2.14 |  |  |
| 9 | 2017-18 |  | 2.36 |  |  |
| 10 | 2016-17 |  | 2.59 |  |  |

2. Table for calculation of ‘C’ ¼pkyw foRrh; o’kZ 2026&27 ds vk/kkj ij½

| S.no. | Financial Year | Total Balance work of Particular Year | Factor of multiplication for projected value of balance work | Projected value of balance work (3x4) |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 |
| 1 | 2026-27 |  | 1.00 |  |
| 2 | 2025-26 |  | 1.2 |  |
| 3 | 2024-25 |  | 1.44 |  |
| 4 | 2023-24 |  | 1.728 |  |
| 5 | 2022-23 |  | 2.074 |  |
| 6 | 2021-22 |  | 2.49 |  |
| 7 | 2020-21 |  | 2.99 |  |
| 8 | 2019-20 |  | 3.59 |  |
| 9 | 2018-19 |  | 4.31 |  |
| 10 | 2017-18 |  | 5.17 |  |
| 11 | 2016-17 |  | 6.20 |  |
| 12 | and so on |  | and so on |  |
| Total ‘C’ :- |  |  |  |  |

C= Summation of projected cost of all balance works

“ks’k dk;ksaZ dh ykxr dks vuqca/kokj fooj.k] Bsdsnkj fufonk gsrq vkosnu djrs le;@fufonk izi= izkIr djus gsrq dk;Zokgh bZ&Vs.Mfjax izkslsl esa mi;qDr LVst esa] U;wure :- 100@& jkf”k ds uksVjkbZTM “kiFk i= ftldh frfFk fufonk tkjh gksus ds ckn dh frfFk esa gksA “kiFk i= esa Li’V :i ls fufonk Øekad ,oa fnukad ds mYys[k ds lkFk fn;k tkos ,oa QeZ }kjk leLr vuqcaf/kr Bsdksa ds “ks’k dk;ksaZ dh ykxr dh tkudkjh izlrqr dh tk jgh gS] og iwjh rjg tkap iM+rky dj nh tk jgh gS ,oa dksbZ rF; ugha Nqik;k x;k gS ,oa xyrh ik;s tkus ij fufonkdkj dh fufonk gsrq ik=rk fujLr ekuh tkosxh] ftls os Lohdkj djus gsrq ck/; gksaxsA fufonkdkj }kjk izLrqr izi= esa odZ bu gS.M dh jkf’k vfuok;Z :i ls mYysf[kr fd;k x;k gks] odZ bu gS.M dh lwph vuqca/kokj ykxr lfgr uksVjh }kjk izekf.kr gksA

3-	Lo;a ds uke ij vFkok QeZ@daiuh ds uke ij dk;Z dk fufonkdkj dks vuqHko gksuk pkfg,A ftl dk;Z dk vuqHko fufonkdkj }kjk izLrqr fd;k tk jgk gS] ml dk;Z dk dk;kZns”k dh frfFk] fufonk tkjh gksus dh frfFk ls foxr 10 foRrh; o’kZ ds Hkhrj dk gksA

vuqHko ds ijh{k.k gsrq vuqHko izek.k i= esa fuEukuqlkj tkudkjh ;qDr vuqHko izek.k i= Bsdsnkj }kjk izLrqr dh tkosxh ,oa fuEufyf[kr izk:i esa vko”;d izfof’V dj Bsdsnkj }kjk vuqHko izek.k i=ksa ds lkFk fufonk gsrq izLrqr djuk gksxkA %&

| Ø- | dk;Z dk uke ,oa rduhdh fooj.k | dk;kZns”k Ø- ,oa fnukad | dk;Z dh ykxr | dk;Z iw.kZ djus dh fu/kkZfjr vof/k vuqca/k ds vuqlkj | dk;Z iw.kZ djus dh okLrfod frfFk | lQyrkiwoZd iw.kZ fd;s x;s dk;Z dk rduhdh fooj.k ¼lacaf/kr dk;Z dh ih-D;w- dh “krksaZ ds vuqlkj½ | vuqHko izek.k i= tkjh djus okys rduhdh vf/kdkjh@dk;kZy; dk fooj.k U;wure dk;Zikyu vfHk;ark Lrj dk gks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

fufonk esa nf'kZr dk;ksZ dh ek=k ds x.kuk dk vkadyu fuEukuqlkj gksxk %&

foxr nl o"kksZ esa %&

ikbZi ykbZu dk;Z %&

1	;w-ih-Ogh-lh- ikbZi ykbZu %

¼i½	7500 eh- rd dh yackbZ ds dk;Z gsrq dsoy mi;qDr Js.kh esa iath;u vko’;d gksxkA

¼ii½	7500 eh- ls vf/kd yackbZ ds dk;Z gsrq ;w-ih-Ogh-lh- ikbZi ykbZu dh dqy yackbZ ds 25 izfr”kr  dk;Z dk vuqHko fdlh ,d vuqca/k esa lQyrkiwoZd VsfLVax lfgr fd;k tkuk vko’;d gksxkA

Minimum Diameter of distribution & Rising main pipe shall be 63 mm.

mPpLrjh; tykxkj %&

1½ 	mPpLrjh; tykxkj gsrq fufonk esa nf”kZr vf/kdre {kerk ds mPpLrjh; tykxkj dh {kerk dk 50 izfr'kr {kerk rFkk 50 izfr”kr LVsftax vFkok 10 ehVj nksuksa esa tks Hkh vf/kd gks] ds fuekZ.k dk lQyrk iwoZd iw.kZ djus dk vuqHko vko';d gSA

2½		lEiosy gsrq fufonk esa nf’kZr vf/kdre {kerk ds lEiosy dh {kerk dk 50 izfr”kr {kerk ds fuekZ.k vFkok        50 izfr”kr {kerk ds mPpLrjh; tykxkj fuekZ.k dk lQyrkiwoZd iw.kZ djus dk vuqHko vko”;d gSA

3½	75 fd-yh- rd dh {kerk 12 eh LVsftax ds mPpLrjh; tykxkj ds fy;s mi;qDr Js.kh esa iath;u i;kZIr gksxkA

fo'ks"k Vhi %&

- mijksDr leLr dk;ksZ ds fy, fufonkdkj }kjk fufonk esa nf'kZr vuqlkj fMIyksek@fMxzh /kkjh vfHk;ark ds fu;qfDr dk izek.k&i= layXu djuk vfuok;Z gksxkA dk;Z laiknu dh vof/k ds nkSjku mUgs fd;s x;s Hkqxrku ls lacaf/kr cSad ,dkmaV dh Nk;kizfr izR;sd ns;d ds lkFk izLrqr djuk gksxkA

- fo'ks"k ifjfLFkfr;ksa esa fufonkvksa esa vgZrk] ekinaM ,oa 'krsZ 'kklu ls vuqefr i'pkr SWSM }kjk ifjorZu fd;k tk ldsxkA

- mijksDr ik=rk fu/kkZj.k dh “krksZ ds vuq:Ik :Ik;s 5-00 djksM+ rd dh fufonkvksa esa DWSM, MOHLA MANPUR A.CHOWKI  ik=rk fu/kkZj.k djus esa l{ke gksxsaA

- Contrctor should arrange for IEC work and training to VWSC members/gram panchayat members for operation and maintainance of piped water supply scheme during O&M period.

Note:-

- For details on tendering procedure through the electronic tendering system, please refer to “Guide lines for bidders on using intergrated eprocurment system” document available along with the tender documents as Annexure-K

- This NIT shall form part of the agreement.  The other conditions, for the pre qualification and details of works can be seen in the office of the undersigned and also on the PHE sub portal(https://eproc.cgstate.gov.in)

- The DWSM, MOHLA MANPUR A.CHOWKI shall reserves the right to change the key dates of the tender process.

After Comparison of rates (As mentioned on the Key dates), the status of tender will be informed to qualified bidders. Not more than one tender shall be submitted by a contractor or by a firm of contractors.

- The electrical work shall be executed only through the contractors who possess proper valid electric 	license from the Chief Electrical Advisor to the Government.  He should also attach a copy of the 	license.

- Not more than one tender shall be submitted by a contractor or by a firm of contractors.

- No two or more concerns in which an individual is interested as a proprietor and/or partner shall 	tender for the execution of the same work.  If they do so, all such tenders shall be liable to be rejected.

- The Chhattisgarh Govt. is  accepting authority here in after referred to as such for the purpose of this 	contract

- Applications for issue of tender documents shall be downloaded online from the portal 	https://eproc.cgstate.gov.in as per the dates mentioned in the Key Dates.

- Tender documents will be downloaded online and has to be completely filled online by the vendor 	including  Terms and Conditions & commercial bid and will opened online by the Department.

- The copies of other drawings and documents pertaining to the work signed for the purpose of 	identification by the accepting officer or his accredited representative and samples of materials to be 	arranged by the contractor will be open for inspection by tenderers at the following office during 	working hours.

- R A T E S :

- The schedule of items :-

The schedule of main items of work to be executed is enclosed as Annexure (F).

- Percentage rate tender in form “A”

- In respect of percentage rate tenders, contractor should quote his separate tender percentage rate 	above or below the following schedules of rate

"    The Schedule of rates issued by the Engineer-in-Chief, C.G. Raipur PHE Raipur for water 	supply  and sewerage inforce in PHE Department with effective form  dated 01.06.2020 with all 	amendment issued up to the date of issue of N.I.T.

“  	The schedule of rates issued by the Engineer-in-Chief, CG PWD Raipur of year 2015 for 	building works inforce in PHE Department from 07.04.2016 with all amendments issued up to the 	date of issue of N.I.T.”

"The SOR applicable for different item of work is mentioned against each item of work".

- (For Form “A” only) The percentage of tender above/below or as per with the relevant schedule rates 	inclusive of amendments and correction slips issued up to the date of the notice inviting tenders 	should be expressed on the tender form itself both in words and figures in such a way that 	interpolation is not possible and all over writings should be neatly scored out and rewritten and the 	corrections should be duly attested prior to the submission of tender. Tenders not specifying 	percentage in words will summarily be rejected. Any amendments to the schedule of rates after the 	date of issue of this tender notice or the date of issue of any amendments to the N.I.T., specifically 	notifying the said amendment to the current schedule of rates , shall not apply to this tender.

- The percentage tendered by the contractor will apply to those rates which find place in the current 	schedule of rates mentioned in clause 2.2.1, or have been derived from the said current schedule of 	rates and not to other items of work.

- The percentage quoted by the contractor shall not be altered by the contractor during the terms of 	contract.  The deduction or addition as the case may be of the percentage will be calculated on the 	amount of the bill for the work done, after deducting the cost of materials supplied departmentally at 	rates specified in the agreement.

- The rates quoted in the tender for various items of work will not be altered by the contractor during 	the term of contract.

- LEAD AND LIFT OF WATER - No lead and lift for carting of water will be paid.

- LEAD AND LIFT OF MATERIALS - No lead and lift for carting of materials shall be payable to the 	contractor except in case of such items for which specific lead and lift are provided in the current 	schedule of rates mentioned in clause 2.2.1 of the N.I.T. or in the schedule of items in respect of item 	rate tenders.

- NON SCHEDULE ITEMS OF WORKS - During the execution of the work there is likelihood of such items of work, which do’not find place in the current schedule of rates, referred to above in respect of percentage rate contracts or such items which are given in the schedule of items in respect of item rate contracts, for which contractor has not quoted his rates. Contractor will have to carry out these items of work. Rates of such items of work which do not find place in the current schedule of rate referred to above, in respect of percentage rate contracts or such items in respect of item rates contracts shall be decided by the District Water and Sanitation Mission of the district..................., and the decision of the DWSM, MOHLA-MANPUR-A.CHOWKI  shall be binding on the contractor.  The quantum of such work will not exceed 10% of amount of contract unless accepted by the department and the contractor.

- SUBMISSION OF TENDER :

| 3.01 | The Physical Earnest money which is to be submitted manually in Physical Envelope - A where it should be clearly written on the envelope as under :- ENVELOPE - A EARNEST MONEY From - (… Name of Contractor…) and should reach Executive Engineer(PHED) and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI , PHE Division .......... , as per date and time mentioned in the key dates. |
| --- | --- |
| 3.02 | ENVELOPE - B This online envelope shall contain as specified document should be submitted in envelope "B". |
| 3.03 | ENVELOPE - C This Envelope should contain only the Percent Rate Offer. The Tenderer shall fill their Percent Rate in the online form. Rate (At par SOR or ...............percent below/above SOR) From - (… Name of Contractor…) |

3.5  ADJUSTMENT OF EARNEST MONEY :- Earnest money, which has been deposited for a particular work, will not, ordinarily, be adjusted towards the earnest money for another work, but if the tender of contractor for a work, in the same division has been rejected and the earnest money has not been refunded to him due to any reason, it may be so adjusted by the Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI .

3.6  SECURITY DEPOSIT :-

- The security deposit shall be recovered from the running bills @ 5 (Five) percent as per clause-1 of the agreement read with para 3.5 of the N.I.T.

- The amount of the E.M. shall not be adjusted when value of work done reaches the limit of the amount of contract or exceeds the probable amount of contract.

3.7  IMPLICATION OF SUBMISSION OF TENDER :- Tenderers are advised to visit site sufficiently in advance of the date fixed for admission of the tender.  A tenderer shall be deemed to have full knowledge of the relevant documents, samples, site etc., whether he inspects them or not.

3.8    The submission of a tender by a contractor implies that he has read the notice, conditions of tender and all other contract documents and made himself aware of the standards and procedure, in this respect, laid down in the National Building Code of India 1940/Indian Standards the scope and specification of the work to be done and the conditions and rates at which stores, tools and plants etc., will be issued to him by the……………………… has seen the quarries with their approaches, site of work, etc. and satisfied himself regarding the suitability and availability of site of work etc. and satisfied himself regarding the suitability and availability of the materials at the quarries.  The responsibility of opening new quarries and construction and maintenance of approaches there to shall lie wholly with the contractor.

3.9  	INCOME TAX: Contractor have to submit copy of PAN card issued by income tax department.

3.10 	LIST OF WORKS IN PROGRESS - Tenders must be accompanied by a list of contracts already held by the tenderer at the time of submitting the tender,  in the Department and elsewhere showing therein.

- The amount of each contract.

- Balance of work remaining to be done, and

- The amount of solvency-certificate produced by him at the time of enrollment in the - - - - - - - - - - - - - - - - - - - -

3.11 RELEATIONSHIP - The contractor shall not be permitted to tender for works in the Division (responsible for award and Execution of contracts) in which his near relative is posted as Divisional Accountant.  He shall intimate the names of his near relative working in P.H.E. Secretariat and P.H.E.Divisions.  He shall also intimate the name of persons who are working with him in any capacity or subsequently employed by him and who are near relatives to any gazetted officer in the P.H.E. Secretariat. Any breach of this condition by the contractor would render himself liable to be removed from the approved list of the contractors of the …………………………

Note -  By the terms near relative is meant wife, husband, parents and son, grand son, brothers, sisters, 	brothers in laws, father in law and mother in law.

3.12 	The tender for the works shall be witnessed by a contractor. Failure to observe this condition shall 	render the tender of the contractor liable to rejection.

- OPENING AND ACEPTANCE OF TENDER -

Tender shall be floated online by the Engineer in charge. Prospective bidders are permitted to 	raise query related to various issue or permitted to put technical conditions on floated Tenders through 	off line mode. Hard copy of such objections/suggestions should reach the division office and pre-bid 	discussion office within the time frame mentioned in floated tender. Competent Departmental officers 	will examine received technical conditions/query about various issues with reference to floated tender 	raised by prospective bidders in pre-bid discussion (in which tenderers may also participate) and 	approve modification if any required in floated tender. Modification will be uploaded online as 	corrigendum by the Engineer incharge. Prospective bidders then submit their online offer as per 	schedule key date.

The tenders shall be opened at the time and place started in Para 1  by the Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI 	in the first instances, the first online Pre qualification envelope containing the scanned copies of the 	Pre Qualification Documents as mentioned in the Tender Document will be opened then the Physical envelope containing the earnest money shall be opened. If the earnest money is found proper, the 	online envelope-A containing the Earnest Money details, its scanned copy and scan copy of 	documents required as minimum qualification to bid shall be opened. If the tenderer found qualified  	as per pre qualification criteria, the online Envelope containing the price offer shall be opened. The 	Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI  under unavoidable circumstance may depute another officer in his absence to 	receive and open tenders on his behalf.

The tenderer shall have to duly fill their offer in appropriate online form meant for it. The Bid 	hashes 	of offer shall be submitted on or before the date and time specified.

- POWER OF EXECUTIVE ENGINEER AND MEMBER SECRETARY, DWSM, MOHLA-MANPUR-A.CHOWKI  - The Executive Engineerand member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI  does not bind himself to accept or recommend for the acceptance to the DWSM, MOHLA-MANPUR-A.CHOWKI  or other higher authority, 	the lowest or any tender or to give any reasons for his decision.

- CONDITIONAL TENDER -  Conditional tenders are liable to be rejected.

- CANVASSING - Canvassing for support in any form for the acceptance of any tender is strictly 	prohibited.  Any tenderer doing so will render himself liable to penalties which may include removal 	of his name from the register of approved contractors or penal action under section 8 of the M.P. 	Vinirdishtta Bharasta Acharan Nivaran Vidheyak, 1982. (As adopted by C.G. State).

- ONLINE TENDER - Only Online Tenders shall be accepted . The tenders submitted in hard copy 	shall be returned unopened.

- AUTHORITY OF EXECUTIVE ENGINEER AND MEMBER SECRETARY , DWSM, MOHLA-MANPUR-A.CHOWKI  - The authority competent to accept a tender, reserve 	the right of accepting the tender for the whole work or for a distinct part of it, or distributing the work 	between one or more tenderers.

- VALIDITY OF OFFER - Tender shall remain open up to four months from the bid due date of tender 	and in the event of the tenderer withdrawing the offer before the aforesaid date, for any reason 	whatsoever, earnest money deposited with the tender shall be forfeited by the Executive 	Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI  after approval by DWSM, MOHLA-MANPUR-A.CHOWKI .

- SPECIFICATIONS :

- BRIEF SPECIFICATION -   A brief note on construction and specifications of the work is enclosed 	in Annexure-E-I, E-II, E-III, E-IV, E-V, -VI, E-VII .

- MATERIAL OF CONSTRUCTION - The materials of construction to be used in the work shall be 	governed by the provision of part V of the National Building code of India, 1940 and the relevant 	Indian Standard Specification with amendments & revisions issued up to the date of tender notice.

- WORKMANSHIP - The work shall be carried out according to the specifications referred to 	hereinafter and according to sound engineering practice.  The decision of the Executive Engineer and  member secretary, DWSM, MOHLA-MANPUR-A.CHOWKI , in 	respect of workmanship will be final.

- SPECIFICATION FOR BUILDING WORKS - (Including water supply and sanitary fittings)                 d.1 The contractor shall execute the work in conformity with the standards and procedure laid down in the National Building Code of India, 1940, and as per Chhattisgarh P.W.D. specification or specifications in force or special specification wherever enclose separately, and in accordance with the approved drawings.

d.2 CONCRETE - All concrete shall be mixed in concrete mixers and compacted by mechanical Vibrators.  Slumps test shall be carried out during concreting and sample test cubes prepared and tested in due course.  The testing will be carried out by the Department. The results of the tests shall confirm with the required standard and if the Engineer-in-charge considers that a structural test is necessary, the same shall be carried out as instructed by the Engineer-in-charge at the contractors expense and should the result of this be unsatisfactory the contractor will be bound to take down and reconstruct the particular portion of work which has given unsatisfactory test results.

d.3 BRICKS - The contractor should use the bricks manufactured on the metric measures as for as possible.

d.4 	All timber used in the wood work for all new works must be properly seasoned. In case of important buildings mechanical seasoning should be done in good seasoning plants.

In case the contractor does not procure good seasoned wood,  he may be asked to get it seasoned in plant at his own expense but no certificate is required where no additional rate is paid.

- SPECIFICATION FOR WORKS -

(Excluding bridges and culverts)

The road works and collection of materials for road works shall be carried out according to  the Chhattisgarh P.W.D. specifications, as adopted for……………………………… or specifications inforce or special specifications, wherever enclose separately, or the relevant specifications published by the Indian Road Congress.

- CONTRADICTIONS OR AMENDMENTS - In the event of contradictions between the stipulations of the current schedule of rates (vide part of this N.I.T.) and aforesaid specification (vide para 5.1 to 5.6 above) the stipulation of the current schedule of rates shall gain precedence. In the event of contradictions, if any between different specifications and or codes of practice, refer to above, the decision of the DWSM, MOHLA-MANPUR-A.CHOWKI  shall be final subjects to appeal in case of dispute before within one month of decision.

6.	MISCELLANEOUS CONDITIONS :

  - SUBLETTING - The contractor shall not, without the prior approval of the competent authority in 	writing, sublet or assign to any other party or parties, the whole or any portion of the work under the 	contract. Where such approval is granted, the contractor shall not be relieved of any obligation or duty 	or responsibility which he undertakes under the contract.

  - TAXES - The rate quoted by the contractor shall be deemed to be inclusive of the sales and other levies, duties, royalties, cess, toll, taxes of Central and State Governments including GST (Goods and Services Tax), taxes of local Bodies and Authorities that the Contractor wii have to pay for the performance of this Contract. The Government will perform such duties in regard to the deduction of such taxes at source as per applicable law.

However, if any other New Tax (not increase or decrease in existing all taxes, other levies, 	duties, royalties, cess, toll) is levied on the Contractor (under the agreement) either by Central Govt. 	or State Govt. then the Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI  shall reimburse the “New Tax” amount; on submission of proof of such payment by the contractor.

  - Mineral extracted for works carried out on behalf of the Government of India, from the quarries in 	possession of controlled by the State Government is subject to payment of Royalty by the contractor to whom it shall not be refundable.

  - RULES OF LABOUR CAMPS - The contractor will be bound to follow the Chhattisgarh Model Rule relating to layout water supply and sanitation on labour camps (vide Annnexure-A) and the provision of the National Building Code of India, in regard to constructions and safety.

  - FAIR WAGES - The contractor shall pay not less than fair wages to labourers engaged by him on the works(rules enclosed vide Annexure-B)

  - WORKS IN THE VICINITY - The Executive Engineer and member secretary, DWSM, MOHLA-MANPUR-A.CHOWKI  reserves the right to take up departmental 	work or to award work on contract in the vicinity without prejudice to the terms of contract.

  - BEST QUALITY OF CONSTRUCTION MATERIALS - Materials of the best quality will be used as 	approved by the Executive Engineer and member secretary, DWSM, MOHLA-MANPUR-A.CHOWKI .

  - REMOVAL OF UNDESIRED PERSONS - The contractor shall on receipt of the requisition from the 	Executive Engineer and member secretary, DWSM, MOHLA-MANPUR-A.CHOWKI  at once remove any person employed by him on the work who, if in the opinion of the Executive Engineer and member secretary, DWSM, MOHLA-MANPUR-A.CHOWKI  is unsuitable or undesirable.

  - AMOUNT DUE FROM CONTRACTOR - Any amount due to from the contractor on any account 	concerning works may be recovered from him as arrear of land revenue.

  - TOOLS AND PLANTS - The contractor shall arrange at his own cost tools and plant required for 	proper execution of the work.  Certain plants may however be issued to the contractor as a special 	case.

  - RIGHT TO INCREASE OR DECREASE WORK - The competent authority (DWSM, MOHLA-MANPUR-A.CHOWKI ) reserve the right to increase or decrease work. The competent authority reserves the right to increase or decrease any item of the work during the currency of the contract and the contractor, will be bound to comply with the order of the competent authority without any claim for compensation.

  - TIME SCHEDULE - The work shall be done by the contractor according to the time schedule fixed by the competent authority (DWSM, MOHLA-MANPUR-A.CHOWKI ).

  - TIME OF CONTRACT - Time allowed for carrying out the work as entered in the N.I.T. shall be strictly observed by the contractor and shall be reckoned from the date of work order to commence the work.

  - PAYMENT BY PFMS - After evalution of work by third party appointed by competant authority and as per JJM Guidline, the payment will be made by PFMS only. No Bank commission charges on realising such payment will be borned by the contractor.

6.12	(a)In any case no interest shall be paid to contractor for delay in payments, irrespective of the type of 	payment like running bills, advance payments, final bills, release of SDs etc.

6.16	TRANSPORT OF MATERIALS - The contractor shall make his own arrangement for transport of all 	materials.  The contractor is not bound to arrange for priority in getting wagon or any other material 	through all possible assistance by way of recommendation will be given if it is found necessary in the 	operation by the Engineer-in-charge.  If it proves to be in effective, the contractor shall have no claim 	for any compensation on that account.

- SPECIAL CONDITIONS :-

To be in inserted the N.I.T. of a particular work if found necessary in the interest of the work

  - AGREEMENT

7.1.1 	EXECUTION OF AGREEMENT - The tenderer whose tender has been accepted herein after referred to as the contractor, shall produce on appropriate solvency certificate, is so required by the Executive Engineer and member secretary, DWSM, MOHLA-MANPUR-A.CHOWKI  and will execute the tri party (PHED/Gram Panchayat/Contractror) agreement in the prescribed form, within a fortnight of the date of communication of the acceptance of his tender by competent authority.  Failure to do so will result in the eanrest money being forfeited to Government and tender being cancelled.

7.1.2 (a) The contractor shall employ the following Technical Staff during the Execution of work

- One Graduate Engineer when the work to be executed is more than Rs.1 Crore.

- One Diploma Holder, Sub Engineer when the cost of work to be executed is from Rs.20 Lakhs.

- The technical staff should be available at site whenever required by the Engineer-in-charge to take instructions.

- In case the contractor fails to employ the technical staff as aforesaid, the E.E.shall have the right to take suitable remedial measures.

- The contractor should give the names and other detail of the Graduate Engineer/Diploma Holder Sub Engineer whom he intends to employ or who is under employment on the work at the time he commences the work.

- The contractor should give a certificate to the effect that the Engineer/Diploma Holder Sub Engineer is exclusively in his employment.

Provided that :-

- An Engineer or Sub-Engineer may look after more than one work in the same locality but the total value of such work under him should not exceed Rs.25 Lakhs in the case of an Engineer and Rs.5 Lakhs in the case of a Sub Engineer.

- It is not necessary for the contractor’s  partner in case of firm/company, who is himself an Engineer/Sub-Engineer to employ another Engineer, Sub-Engineer for the Supervision of work.

- The Retired Assistant Engineer who is holding a Diploma may be treated at per with a Graduate for the operation of the above clause.

In case the contractor fails to employ the Technical staff as aforesaid he shall be liable to pay sum of Rs.30000.00 (Rs. Thirty  thousand only) for each month of default in the case of graduate Engineer and Rs.25000.00 (Rs. Twenty five Thousand only) for each month of default in the case of Diploma holder Sub Engineer.

  - CONDITIONS APPLICABLE FOR CONTRACTOR -  All the conditions of the tender notice will be binding on the contractors in additions to the condition of the contract in the prescribed form :-

7.2.1 In case of overhead tank specification annexed as annexure E-II & E-III shall given precedence 	over SOR specification.

7.2.2  Defect liability period shall be of 12 month after completion of the work. Trial run, operation 	and maintanance period shall also be of 06 months. Hence both defect liability period and trial run, 	operation and maintanance period shall commence simualtaneousaly. S.D. release shall govern by the 	clause-19.

Following documents along with form “A” annexed with this N.I.T. shall form a part of contract.

| Annexure “A” | : | Model Rules relating to labour, water supply etc. |
| --- | --- | --- |
| Annexure “B” | : | Contractors labour regulations. |
| Annexure “C” | : | Statement showing the lead of materials |
| Annexure “D” | : | Form of Income Tax Clearance Certificate (Applicable to works costing more than Rs. 2.00 Lakhs) |
| Annexure “E-I” | : | Specification for PVC Pipe |
| Annexure “E-II | : | Specification for Civil Work |
| Annexure”E-III” | : | Specification for FHTC |
| Annexure”E-IV” | : | Specification for pump works |
| Annexure”E-V” | : | Specification of RCC Elevated Service Reservoir |
| Annexure”E-VI” | : | Special condition of RCC Service Reservoir |
| Annexure “F” | : | Schedule of Quantity |
| Annexure “G” | : | Break-Up Schedule for valuation of work |
| Annexure “H” | : | Integrity Pact |
| Annexure “I” | : | Specials Condition |
| Annexure “J” | : | Suspension, Demotion, Non-Renewal & De-registration of Contractors/Firm, 2014 |
| Annexure “K” | : | Online electronics government procurement system (e-GPS) |
| Annexure “L” |  | Annexure 2.10 G-III |
| Annexure “M” | : | Single Tender |
| Annexure “1 to 4” | : | Affidavit & other formats |
| Annexure “N” | : | Payment of wages through digital mode |
| Annexure “O” | : | Unemployed Engineer circular |
| Annexure “P” | : | GST Order Copy |
| Annexure “Q” | : | Drawings |

ANNEXURE - “A”

Model Rules Relating to Labour water supply and sanitation in labour camps.

NOTE - These model rules are intended primarily for labour camps which are not of a permanent nature.  They lay down the minimum desirable standard which should be adhered to. Standards in permanent or semi-permanent labour camps should not obviously be lower than those for temporary camps.

- LOCATION - The camps should be located in elevated and well drained ground in the locality.

- Labour huts to be constructed for one family of 5 persons each.  The lay out to be shown in the prescribed sketch.

- HUTTING - The huts to be built of local materials.  Each hut should provide at least 20 sq.mt. of living space.

- SANITARY FACILITIES - Latrines and urinals shall be provided at least 12 meters away from the nearest quarters separately for men and women and specially so marked on the following scale.

- LATRINE -  Pit provided at rate of 10 users or two families per seat, separate urinals as required as the privy can also be used for this purpose.

- DRINKING WATER -  Adequate arrangements shall be made for the supply of drinking water. If practicable filtered and chlorinated supplies shall be arranged, when supplies are from intermittent sources over head storage tanks shall be provided with a capacity of 5 Litres a person per day. Where the supply is to be made from a well it shall confirm to the sanitary standard laid down in the report of Rural Sanitation Committee.  The well should be at least 30 metres away from any latrine or other source of pollution.  If possible hand pump should be installed for drawing the water from well.  The well should be effectively disinfected once every month and the quality of the water should be got tested at the Public Health Institution between each work of disinfecting.

- BATHING AND WASHING - Separate bathing and washing place shall be provided for men and women for every 25 persons in the camp.  There shall be one gap and space of 2 sq. meters for washing and bathing, proper drainage for the waste water should be provided.

- WASTE DISPOSAL - Dustbin shall be provided at suitable places in camp and the residents shall be directed to throw all rubbish in to those dustbins. The dustbins shall be provided with cover. The contents shall be removed every day and disposed off by trenching.

- MEDICAL FACILITIES (A) Every camp where, 1,000 or more person reside shall be provided with whole time doctor and dispensary.  If there are women in the camp a whole time nurse shall be employed.

- Every camp where less than 1000, but more than 250 person reside shall be provided with a dispensary and part time nurse/midwife.

- If there are less than 250 persons in any camp a first aid kit shall be maintained in charge of whole time persons, trained in first aid.

All the medical facilities mentioned above shall be for all residents in the camp including a dependent of the worker if any, free of cost.

SANITARY STAFF - For each labour camp there should be qualified sanitary inspector and sweeper should be provided in the following scales :

- For camps with strength over 200but not exceeding 500 persons one sweeper for every 75 persons above the First 200 for which 3 sweepers shall be provided.

- For campus with a strength over 500 persons one sweeper for every 100 person above first 500 for which 6 sweepers should be provided..

ANNEXURE - “B”

CONTRACTORS LABOUR REGULATION

The contractor shall not pay less than fair wage to labours engaged by him in the work.

EXPLANATION - (a)  “Fair wages” means wages whether for time or piece works as notified on the date of inviting tenders  and where such wages have not been so notified the wages prescribed by the……………………………………………. department for the division in which the work is done.

- The contractor shall notwithstanding the provision of any contract to the contrary, cause to be paid a fair wage to labourers indirectly engaged on the work including any labour engaged by his sub-contractors in connection with the said work as if labourers had been immediately employed by him.

- In respect of all labour directly or indirectly employed on the works or the performance of his contract,  the contractor shall comply with or cause to be complied with the Labour Act in force.

- The Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI /Sub Divisional Officer shall have the right to deduct from the money due to the contractor any sum required or estimated to be required for making good the loss suffered by a worker or workers by reason of non fulfilment of the conditions of the contract for the benefit of the workers, non payment of wages or of deductions made from his or their wages which or not justified by their terms of the contract or non observance of regulations.

- The contractor shall be primarily liable for all payments to be made under and for the observance of the regulations aforesaid without prejudice to his right to claim indemnity from his sub contractors.

- The regulations aforesaid shall be deemed to be a part of this contract and any breach thereof shall be deemed to be a breach of this contract.

- The contractor shall obtain a valid license under the contract (Regulation and Abolition) Act, inforce and rules made there under by competent authority from time to time before commencement of work, and continue to have a valid license until the completion of the work.

Any failure to fulfil this requirement shall attract the penal provisions of this contract arising out of the resulted non-execution of the work assigned to the contractor.

Special Additional Condition :-

A.	Cess@1% (one percent only) shall be deducted at source, from every bill of contractor by Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI  under "Building and other construction for workers welfare, cess Act-1996"

B.	The contractor will get him self/them selves registered with "Chhattisgarh Building and other Construction Welfare Board" for work amounting to Rs. 10.00 Lacs (Ten Lacs) and above and enclose a true copy of such registration certificate within 1 month after award of work.

ANNEXURE - C

Statement showing the Lead of Materials

S.No.                                      Description                                             Lead

- ……………………………………………………………………………………

- ……………………………………………………………………………………

- ……………………………………………………………………………………

- ……………………………………………………………………………………

- ……………………………………………………………………………………

Note - This statement is only for guidance of the contractor.  The tenderer should satisfy himself regarding the availability of the required quality and quantity of materials.

ANNEXURE – D

Form of certificate on Income Tax to be submitted by contractor.  Tendering for works costing Rs2.00 Lakh or more.

- Name and style (of the company, firm, H.U.F. or individual) in which the applicant assessed to income tax and address for purposes of assessment.

- The income tax Circle/Ward/District in which the applicant is assessed to income tax.

- Following particulars concerning the last Income tax assessment made :-

- Reference No. (Or. G.I.R.No.) of the assessment.

- Assessment year and accounting year.

- Amount of total income assessed.

- Amount of tax assessed IT,ST.EPT,BPT.

- Amount of tax paid IT,ST,EPT,BPT.

- Balance being tax not yet paid and reasons for such arrears.

- Whether any attachment or certificate proceeding pending in respect of the arrears.

- Whether the company or firm or H.U.F. on which the assessment was made has been or is being liquidated, wound up, dissolved, partitioned or being declared insolvent, as the case may be.

- The position about later assessment namely, whether returns submitted under section 22 (1) of (2) of the income-tax act and whether tax paid under section 18 A of the act. and the amount of tax so paid or in arrears.

- In case there has been no income tax assessment at all in the past, whether returns submitted under section 21 (1) or (2) and 18-A (3) and if so, the amount of income tax returned or tax paid and the income tax circle/ward/district concerned.

- The name and addresses of branch(es) verified the particulars set out above and found correct subject to the following remarks.

Signature of I.T.O.

Dated……………………….						Circle/Ward/District

APPENDIX 2.13

(See Paragraph 2.091)

FORM A

GOVERNMENT OF CHHATTISGARH

PUBLIC HEALTH ENGINEERING DEPARTMENT

Issued to Shri/M/s ………………………………………………………………………………..

Balance Work of Construction, Testing and Commissioning of R.C.C. Elevated Service Reservoir of 40 KL capacities & staging 12 mt. (35% Balance Work) at Markatola  including providing and fixing of CI Double flange pipe, double flange specials & DI double flange soft seated sluice valve including construction of suitable R.C.C. Valve Chamber & Providing, laying, jointing, interconnection, testing and commissioning of 90/110 mm dia UPVC class 06/10 kg/sqcm Pipe, 63/75 mm dia HDPE 06 kg/sqcm Pipe  with suitable joints, valve specials and all fittings including all allied civil works,  including Electric connection from CSPDCL, Road Cutting, Switch room cum pump House, Information Board, construction of boundary wall around the over head tank, installation of electro chlorinator, construction of chlorinator room, providing functional household tap connection (F.H.T.C) in individual households and government institution  etc. complete with 03 months trial run, operation & maintenance for 06 months under Single Village  of Piped Water Supply Scheme under Jal Jeevan Mission For Village Markatola, Dhanegaon    Block Mohla District- Mohla-Manpur-A.Chowki .

Amount of Contract Rs. -	19.36 Lakhs

Amount of E.M.D. Rs. 	–	14600.00

Cost of Tender Form Rs.-	311.00

Vide M.R.No. & Date…………………………………………………………………………….

Time allowed for Completion……. 03 months including rainy season

Date of opening Tender ………………………………………………………………………….

Division		- Public Health Engineering Division …………

Sub- Division ………………………………………………………………………….

PERCENTAGE RATE TENDER AND CONTRACT FOR WORKS

General Rules and Direction for the Guidance of Contractors

1.     Tenders must be invited for all works proposed to be given on contract unless the amount of works proposed to be given on contract is Rs. 12,000 or less.  The N.I.T. shall be posted in public places signed by the authority inviting the tenders.

N.I.T. will state the work to be carried out as well as the date for submitting and opening tenders and the time allowed for carrying out the work, also the amount of earnest money to be deposited with the tender and the amount of the security deposit to be deposited by the successful tenderer & the percentage, if any, to be deducted from bills. It will also state whether a refund of quarry fees, royalties and ground rents will be granted.  Copies of the specifications, designs and drawings and a schedule of items and rates of the various description of work and any other documents required in connection with the work signed for the purpose of identification by the authority competent to approve the tender, shall also be open for inspection by the contractor at the office of the authority selling the tender forms during office hours.

Further that the schedule of items alongwith the rates payable shall be attached to the tender documents and in the event of variation in rates given in such list with the Current schedule of Rates, the rates given in the C.S.R. approved by the competent authority shall prevail.

2.       In the event of the tender being submitted by a firm, it must be signed separately by each member thereof. In the event of the absence of any partner it must be signed on its behalf by a person holding a power of attorney fficient him to do so. Such power of attorney should be produced, with the tender and it must disclose that the firm is duly registered under the Indian Partnership Act.

3.        Any person, who submits a tender, shall fill up above or below the C.S.R. specified in rule-I, he is willing to undertake the work.  Only one rate of percentage above or below the C.S.R. on all the schedule & items shall be named.  Tenders which propose any alteration in the work specified in the said N.I.T.  or in the time allowed for carrying out the work, or which contain any other conditions of any sort, will be liable to rejection.  No single tender shall include more than one work, but contractors, who wish to tender for two or more works shall submit a separate tender for each Tenders shall have the name and number of the work to which they refer, written outside the envelope.

4.       The authority receiving the tenders or his duly fficient assistant, will open tenders in the presence of any intending contractors who may be present at the time, and will enter the amount of the several tenders in a comparative statement in a suitable form.  Receipts for earnest money will be given to all tenderers except those whose tenders are rejected and whose earnest money is refunded on the day the tenders are opened.

5. The officers competent to dispose of the tenders shall have right of rejecting all or any of the tenders.

6.    The receipt of a clerk for any money paid by the contractor will not be considered as any acknowledgement of payment to the Sub-Divisional/Divisional authority selling the tender form and the contractor shall be responsible for seeing that he procures a receipt signed by that authority or any other person duly fficient by him.

7.The memorandum of work tendered for, and the schedule of materials to be supplied by the Department and their issue rates be filled in and completed before the tender form is issued.  If a form is issued to an intending tenderer without having been so filled in and completed he shall request the office to have this done before he completes and delivers his tender.

Tender For Works

I/We hereby tender for the execution, for the Governor of Chhattisgarh of the work specified in the under written memorandum within the time specified in such memorandum at (in figures) ……………………………(in words)…………………………………………………………………….. .percent below/above the rates entered in the schedule mentioned in rule 1 and in accordance in all respects with the specifications, designs, drawings and instructions in writing reffered to in rule 1 thereof and in clause 12 of the annexed conditions and with such materials as are provided for, by, and in all other respects in accordance with such conditions as far as applicable.

Memorandum

- Name of work …………………………………………………………………………………

- Cost of work put to Tender……………………………………………………………………

- Earnest money…………………………………………………………………………………

- Security deposit (including earnest money)……………………………………………………

- Percentage, if any to be deducted from bills…………….……………………………………..

- Time allowed for the work from date of written order to commence…………………………..

Should this tender be accepted, I/We hereby agree to abide by and fulfill all terms and provisions of the said condition of the contract annexed hereto as far as applicable, or in default, thereof to forfeit & pay to the Governor of Chhattisgarh or his successors in office the sums of money mentioned in the said condition.  A separate sealed cover duly superscribed containing the sum of Rs………………… as earnest money the full value of which is to be absolutely forfeited to the said. Governor or his successors in office without prejudice to any other rights or remedies of the said Governor or his successors in office, should I/We fail to commence the work specified in the above memorandum or should I/We not deposit the full amount of security deposit specified in the above memorandum, in accordance with clause 1 of the said conditions of the contract, otherwise the said sum of Rs…………………………shall be retained by Government on account of such security deposit as aforesaid or the full value of which shall be retained by Government on account of the security deposit specified in clause 1 of the said conditions of the contract.

Signature of Witness to Contractor’s signature                      Signature of the Contractor before

Submission of tender

Dated the                          day of                                            Dated the                         day of

………………………………….201                                     ……………………201

Address of the witness

…………………………………………………………………………………………………………. Occupation of the witness

………………………………………………………………………………………………………….

The above tender is hereby accepted by me on behalf of the Governor of Chhattisgarh

Dated the…………………..201   day of ……………..201

….…………………………………………

Signature of the Officer by whom accepted

CONDITIONS OF CONTRACT

Definition:-

1.  The “contract” means the documents, forming the notice inviting tenders and tender documents submitted by the tenderer and the acceptance 47thereof including the formal agreement executed between the Government of Chhattisgarh and the contractor.

2.    In the contract the following expressions shall unless otherwise required by the context, have the meanings hereby respectively assigned to them:-

- The expression “works” or “work” shall unless thereby mean something either in the subject or context repugnant to such construction, be constructed and taken to mean the works by virtue of the contract contracted to be executed whether temporary or permanent, and whether original, altered, substituted or additional

- The “site” shall mean the land and/or other places on, into or through which work is to be executed under the contract or any adjacent land, path or street through which work is to be executed under the contract or any adjacent land, path, or street which may be allotted or used for the purpose of carrying out the contract.

- The “Governor” means Governor of Chhattisgarh and his successors in office.

- The “Engineer-in-charge” means the Divisional Officer or the Sub-Divisional Officer, as the case may be, who shall supervise and be in charge of the work and who shall sign the contract on behalf of the Governor.

- “Government” shall mean the Government of Chhattisgarh.

- The term “Chief Engineer” means the Chief Engineer of the basin/zone/project and the Engineer-in-Chief, in case he is in charge of any basin/zone/project.

Note –“Words” importing the singular number include plural number and vise-versa.

Security Deposit Clause 1 -  The person whose tender may be accepted (hereinafter called the contractors, which expression shall unless excluded by or repugnant to the context, include his heirs executors administrators, representatives and assigns) shall permit Government at the time of making any payments to him for the value of work done under the contract to deduct the security deposit as under:-

The Security Deposit to be taken for the due performance of the contract under the terms and conditions printed on the tender form will be the earnest money plus a deduction of 5 percent from the payment made in the running bills, till the two together amount to 5 percent of the cost of works put to tender or 5 percent of the cost of the works executed when the same exceeds the cost of work put to tender.

Compensation for Delay Clause 2 -  The time allowed for the carrying out the work, as entered in the tender, form shall be strictly observed by the contractor and shall be deemed to be the essence of the contract and shall be reckoned from the fifteenth days after the date on which the order to commence the work is issued to the contractor, for a work  where completion is up to 6 months.

For works, for which the completion period is beyond six months -  The period will be reckoned from the thirtieth days after the date on which the order to commence the work is issued to the contractor.  The work shall through out the stipulated period of contract be proceeded with all due diligence, keeping in view that time is the essence of the contract.  The contractor shall be bound in all cases, in which the time allowed for any work exceeds one month to complete 1/8th of the whole work before ¼th of the whole time allowed under the contract has elapsed, 3/8   of the work before ½ of such time has elapsed and ¾th of the work before ¾ of such time has elapsed. In the event of the contractor falling to comply with the above conditions,  the Executive Engineerand member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI  shall levy on the contractor, as compensation an amount equal to :-

- ½ percent of the value of work per week in respect of work costing up to Rs.2,00,000

- 3/8 percent of the value of work per week in respect of work costing above Rs.2,00,000 and up to Rs.5,00,000

- ¼ percent of the value of work per week in respect of work costing above Rs.5,00,000 and up to Rs.10,00,000

- 1/8 percent of the value of work per week in respect of work costing above Rs.10,00,000 and up to Rs.25,00,000

- 1/16 percent of the value of work per week in respect of work costing above Rs.25,00,000 and above.

The total amount of compensation under the provision of the clause shall be limited to 6 percent of the value of work.

The decision of the DWSM, MOHLA-MANPUR-A.CHOWKI  shall be final.

The delay in departmental assistance ingrained in the contract will be taken duly into account while recovering any compensation for the delay in the scales prescribed above.  Where the Engineer-in-charge decides that the contractor is liable to pay compensation for not giving proportionate progress under this clause and the compensation is recommended during the intermediate period, such compensation shall be kept in deposit and shall be refunded if the contractor subsequently makes up the progress for the lost time, within the period of contract including extension granted, if any.

Action when the work is Left Incomplete, Abandoned or Delayed Beyond the Permitted, Limit Allowed by the Divisional Officer

CLAUSE 3–In any case in which under any clause or clauses of this contract the contractor shall have rendered himself liable to pay compensation amounting to the whole of his security deposit (whether paid in one sum or deducted by installments) or committed a breach of any of the rules contained in clause 24 or in the case of abandonment of the work, except due to permanent disability or death of the contractor or any other cause the Divisional Officer on behalf of the Governor of Chhattisgarh shall give a notice before 12 days for works costing up to Rs.10.00 lacs and before 30 days for works costing above 10.00 lacs and in the event of the contractor failing to comply with the direction contained in the said notice, shall have power to adopt any of the following courses, as he may deem best in the interests of the Government.

- To rescind the contract (of which rescission notice in writing to the contractor under the hand of the Divisional Officer shall be conclusive evidence) and in which case the security deposit of the contractor shall stand forfeited and be absolutely at the disposal of Government.

- To employ labour paid the Works Department and to supply material to carry out the work or any part of the work, debiting the contractor with the cost of the labour and the price of the materials (of the amount of which cost and price certificate of the Divisional Officer shall and conclusive against the contractor) and crediting him with the value of the work done in all respects in the same manner and the same rates as if it had been carried out by the contractor under the terms of his contract or the cost of the labour and the price of the materials as certified by the Divisional Officer, which ever is  less.  The certificate of the Divisional Officer as to the value of the work done shall be final and conclusive against the contractor.

- To measure up the work of the contractor and to take such part thereof as shall be unexecuted out of his hands, and to give it to another contractor to complete in which case any expenses which may be incurred in excess of the sum which would have been paid to the original contractor, if  the whole work had been executed by him (of the amount of which excess certificate in writing of the Divisional Officer shall be final and conclusive) shall be borne and paid by the original contractor and may be deducted from any money due to him by Government under the contract or otherwise or from his security deposit or the proceeds of sale thereof or a sufficient part thereof.

In the event of any of the above courses being adopted by the Divisional Officer, the contractor shall have no claim to compensation for any loss sustained by him by reason of his having purchased or procured any materials or entered in to any agreements or made any advances on account of, or with a view to, the execution of the work or the performance of the contract.  And incase the contract shall be rescinded under the provision aforesaid the contractor shall not be entitled to recover or be paid any sum for any work thereto for actually performed under the contract unless and ffic the Sub-Divisional/Divisional Officer will have certified in writing the performance of such work and the value payable in respect thereof, and he shall only be entitled to be paid the value so certified.

Whenever action is taken under clause 3 (a) the contractor’s bill shall be fficient up within 3 months from the date of rescission both in the case of building works and road and bridge works.

Office of Executive Engineer and Member Secretary,

DWSM, MOHLA-MANPUR-A.CHOWKI , (C.G.).

(SHOW CAUSE NOTICE UNDER CLAUSE 3 OF THE AGREEMENT)

NO. ................./ 							Dated .....................

To,

Name/ M/s ......................................

.........................................................

........................................................

Sub :- 		Name of work ...........................

Agreement No. .................................

Dear Sir,

WHEREAS it appears to the undersigned that by reason of your wrongful delay or fficient of work or slow progress, the work entrusted to you under the agreement referred to above will not be completed/has not been completed (strike out whichever not applicable) within the stipulated/extended date of completion. (strike out whichever not applicable)

Therefore, I, in exercise of the powers conferred on me by the aforesaid agreement, for and on behalf of the Gsovernor of Chhattisgarh, hereby give you notice to show cause within............................................. to my fficient why an action under clause 3 (a), (b) & (c) (strike out whichever not applicable) of the above agreement will not be taken against you on account of the breach of contract on your part. Please note that in case no cause is shown by you within the stipulated period or the cause shown is not to my satisfaction, I shall take such ctions against you as are contemplated under clause 3 (a), (b) & (c) ( strike out whichever not applicable) thereunder of the said agreement and/or other clauses thereof, without further notice.

|  | Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI ,(C.G.) |
| --- | --- |

Note : while determining the contract under any of the sub-clause of clause 3 for causes other than the causes as mentioned above (viz. wrongful delay or fficient of work or slow progress) suitable modifications may be made.

Office of Executive Engineer and Member Secretary,

DWSM, MOHLA-MANPUR-A.CHOWKI , (C.G.).

(SHOW ON FINAL ACTION UNDER CLAUSE 3 OF THE AGREEMENT)

NO. ................./ 							Dated .....................

To,

Name/ M/s ......................................

.........................................................

........................................................

Dear Sir,

Whereas under Clause 3 of the aforesaid agreement the Engineer-in-charge shall have powers to take action under clause 3 in the event of delay or suspension in the execution of the aforesaid work by the contractor so that in the opinion of the Engineer-in-charge (which shall be final and binding) the contractor will be unable to secure completion of the work by the stipulated/have already failed to complete the work by the extended* date of completion, whereas you have delayed/suspended the execution of the aforesaid work and as per the opinion of the undersigned, the Engineer-in-charge (which is final and binding), you will be unable to secure completion of the work by the stipulated/have already failed to complete the work by the extended* date of completion and, and whereas you were served with show cause in this regard under this office no. ................................... date .......................... but no reply has been given by you so far/your reply vide letter no. ............................. received in response to show cause has been considered carefully but not found to the satisfaction of the Engineer-in-charge (strike out whichever is not applicable, the arguments/facts claimed by the contractor be indicated in brief) or reply not received by the date Therefore under powers delegated to me under sub-clause 3 (a) 3 (b) & 3 (c), I ................. the Engineer-in-Charge for the aforesaid work under the aforesaid agreement, for and on behalf of the Governor of Chhattisgarh hereby

    - Determine the contract as aforesaid upon which determination your earnest money deposit, security deposit already recovered and performance Guarantee stand abosolutely forfeited to the I and shall be absolutely at the disposal of Government, and

    - Take out such part of the work out of your hand, as remains unexecuted, for giving it to  another contractor to complete the work, and you shall have no claim to compensation for any loss sustained by you by reasons of your having purchased or procured any materials or entered into any engagements or made any advances on account of or with a view to the execution of the work or the performance of the contract. You are also hereby served with notice to the effect that the work executed by you will be measured up on ........................... for which you are asked to attend for joint measurement failing which the work will be measured by the department unilaterally in your absence and result of measurement will be final and will be binding on you.

    - You shall not be allowed to participate in the tendering process for the balance work.

This is without prejudice to Government’s right to take action under any other clauses or sub-clauses of the agreement and to realize Government dues and losses and damages whatsoever under such clauses or sub-clauses.

| * Score out which is not applicable. | Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI (C.G.) |
| --- | --- |

Note : while determining the contract any of the sub-clause of clause 3 for causes other than the causes as mentioned above (viz. wrongful delay or fficient of work or slow progress) suitable modifications may be made.

Power to take possession of or require removal of Materials Tools and

Plants or sale of Contractor’s Plants etc.

CLAUSE 4 -  In any case in which any of the powers, conferred upon the Divisional Officer by clause 3 hereof shall have become exercisable and the same shall not be exercised, the non-exercise thereof shall not constitute a waiver of any of the conditions hereof and such powers shall not with standing be exercisable in the event of any future case of default by the contractor for which by any clause or clauses hereof he is declared liable to pay compensation amounting to the whole of his security deposit, and the liability of the contractor for the past & future compensation shall remain unaffected.  In the event of the Divisional Officer  putting in force either of the power (a), (b) or (c) vested in him under the preceding clause he may, if he so desires, take possession of all or any tools, plant, materials, and stores, in or upon

the works, or the site thereof or belonging to the contractor or procured by him and intended to be used for the execution of the work or any part thereof, paying or allowing for the same in account at the contract rates, or in case of these not being applicable, at current market rates, to be certified by Divisional Officer, whose certificate thereof shall be final; otherwise the Divisional Officer may be notice in writing to the contractor or his clerk of the works, foreman or other authorized agent require him to remove such tools, plant, materials or stores from the premises (within a time to be specified in such notice) and in the event of the contractor failing to comply with any such requisition, the Divisional Officer may remove them at the contractor’s expense or sell them by auction or private sale on account of the contractor & at his risk in all respects and the certificate of the Divisional Officer as to the expense of any such removal and the amount of the proceeds and expense of any such sale shall be final and conclusive against the contractor.

EXTENSION OF TIME

CLAUSE 5– If the contractor shall desire an extension of time or completion of the work on the grounds of his having been unavoidably hindered in its execution or any other ground he shall apply in writing to the Divisional Officer/Sub-Divisional Officer, within 30 days of the date of hindrance on account of which he desires such extension as aforesaid and the Divisional Officer/Sub-Divisional Officer, with whom he has signed the agreement shall if in his opinion, (which shall be final) reasonable grounds are shown therefore, may authorized such extension for a period not exceeding three months.  Any further extension shall be subject to previous sanction of the DWSM, MOHLA-MANPUR-A.CHOWKI  (grounds to be shown therefore) provided always where the Divisional Officer/Sub-Divisional Officer has recommended the grant of the extension/permitted the contractor to carry out the work reserving the right of the Department to impose the liquidated damages (as provided for under the agreement) the running bills shall continue to be paid to him.

Provided further if any extension applied for is proposed to be refused, the competent authority shall give the contractor an opportunity to be heard before taking final decision.

FINAL CERTIFICATE

CLAUSE 6– On completion of the work, the contractor shall be furnished with a certificate by the Sub-Divisional Officer/Divisional officer (hereinafter called the Engineer-in-charge) of such completion in the form appended at the end, but no such certificate shall be given, nor shall the work be considered to be complete until the contractor shall have removed from the premises on which the works shall be executed, all scaffolding, surplus materials and rubbish and cleaned off the dirt from all wood-work, doors, windows, walls, floors or other parts of any building in upon or about which the work is to be executed or of which he may have had possession for the purpose of the execution thereof, nor until the works shall have been measured by the Engineer-in-charge whose measurements shall be binding and conclusive against the contractor.  If the contractor shall fail to comply with the requirements of this clause as to removal of scaffolding surplus materials and rubbish and cleaning of dirt on or before the date fixed for the completion of the work,  the Engineer-in-charge may,  at the expense of the contractor remove such scaffolding, surplus materials and rubbish and dispose of the same as he thinks fit and clean off such dirt as aforesaid and the contractor shall forthwith pay the amount of all expenses so incurred, and shall have no claim in respect of any such scaffolding or surplus materials as aforesaid, except for any sum actually realized by the sale thereof.

Payment on intermediate Certificate to be Regarded as Advances

CLAUSE 7 –No payments shall ordinarily be made for work estimated to cost less than Rs.1000 (Rs. One thousand) till after the whole of the works shall have been completed and certificate of completion given, but if intermediate payment during the course of execution of works is considered desirable in the interest of works, the contractor may be paid at the discretion of the Engineer-in-charge.  But in the case of works estimated to cost more then rupees one thousand, the contractor shall on submitting the bill therefore be entitled to receive a monthly payment proportionate to the part thereof then approved and passed by the Engineer-in-Charge, whose certificate of such approval and passing of the sum so payable shall be final and conclusive against the contractor.  But all such intermediate payments shall be regarded as payments by way of advance against the final payment for works actually done and completed and shall not preclude the requiring of bad, unsound and imperfect or unskillful work to be removed and taken away and reconstructed, or erected or be considered  as an admission of the due performance of the contract, or any such part, thereof, in any respect, or the accruing of any claim, nor shall it conclude, determine, or affect in any way the powers of the Engineer-in-charge under these conditions or any of them as to the final settlement and adjustment of the accounts or otherwise or in any other way vary or affect the contract.  The final bill shall be submitted by the contractor within one month of the date fixed for completion of the work, otherwise the Engineer-in-charge’s certificate of the measurement and of the total amount payable for work accordingly shall be final and binding on all parties.

Bills to be submitted Monthly

CLAUSE 8 –A bill shall be submitted by the contractor each month on or before the date fixed by the Engineer-in-Charge for all work executed in the previous month, and the Engineer-in-charge shall takes or cause to be taken the requisite measurement for the purpose of having the same verified and the claim, as for as admissible, adjusted, if possible, before expiry of ten days from the presentation of the bill.  If the contractor does not submit the bill within the time fixed as aforesaid,  the Engineer-in-Charge may depute a subordinate to measure up the said work in the presence of the contractor, whose counter signature to the measurement list will be sufficient warrant and the Engineer-in-Charge may prepare a bill from such list which shall be binding on the contractor in all respects.

Bills to be on Printed Forms

CLAUSE 9 –The contractor shall submit all bills on printed forms to be had on application at the office of the Engineer-in-charge, and the charges in the bills shall always be entered at the rates specified in the tender or in the case of any extra work ordered in pursuance of these conditions, and not mentioned or provided far in the tender at the rates here-in-after provided for such work.

The deduction or addition as the case may be of the percentage will be calculated on the amount of the bill for the work done, after deducting the cost of materials supplied departmentally at rates specified in the agreement.

Receipts to be signed by Partners or Persons having authority to do so

CLAUSE 10 -  Receipts for payments made on account of a work when executed by a firm must also be signed by the several partners, except where the contractors are described in their tender as a firm in which case the receipt must be signed in the name of the firm by one of the partners, or by some other person having authority to give effectual receipt for the firm.

Advances to Contractors

CLAUSE 11 –Advancesto contractors are as a rule prohibited, and every endeavor should be made to maintain a system, under which no payments are made except for work actually done.  Exceptions are, however, permitted in the following cases :-

Cases in which a contractor, whose contract is for finished work, requires an advance on the security of materials brought to sites,  Divisional Officer may in such cases, sanction advances up to an amount not exceeding 75% of the value but 90% in the case of steel (as assessed by the Divisional Officer) provided that the rate allowed in no case in more than the rate payable for the finished item as stipulated in the contract of such materials, provided that they are of imperishable nature and that a formal agreement is drawn up with the contractor under which Government secures a lien on the materials and is safeguarded against losses due to the contractor postponing the execution of the work or to the shortage or misuse of the materials, and against the expense entitled for there proper watch and safe custody.

Payment of such advances should be made only on the certificate of an officer not below the rank of Sub-Divisional Officer, that the quantities of materials upon which the advances are made have actually been brought to site, that the contractor has not previously received any advance on that security and that all the materials are required by the contractor for use on items of work for which rates for finished work have been agreed upon.  Recoveries of advances so made should not be postponed ffic the whole of the work entrusted to the contractor is completed.  They should be made from his bills, for work done as the materials are used, the necessary deductions being made when ever the item of work in which they are used are billed for.

Before granting the above secured advance the contractor shall sign the Indenture Bond in the prescribed form.

Work to be Executed in Accordance with Specification, Drawing, Order etc.

CLAUSE 12 –The contractor shall execute the whole and every part of work in the most substantial and workman like manner, and both as regards materials and otherwise in every respect in strict accordance with the specifications.  The contractor shall also conform exactly fully and faithfully to the designs, drawings and instructions in writing relating to the work signed by the Engineer-in-charge and lodged in his office and to which the contractor shall be entitled to have access at such office or on the site of the work for the purpose of inspection during office hours and the contractor shall, if he so requires, be entitled at his own expense to take or cause to be made copies of the specifications, and of all such designs, drawings and instructions as aforesaid.

Chhattisgarh P.W.D. Specifications shall apply along with the various I.S.I. Codes in the case of any variance, the following order of precedence shall prevail.

- Specification as per N.I.T.

- Specifications asperC.S.R. of the circle

- I.S.I. code/I.R.C. specification

- C.G P.W.D. Specification

- Mode of measurements for building shall be as provided in the C.S.R. applicable to the contract.  Where such mode of measurement is not specified in the C.S.R. it shall be done as per I.S.I. code of building measurement.  However, if any mode of measurement is specifically mentioned in the N.I.T. the same will get precedence over all the above.

CLAUSE 12 A –In respect of all bearing, hinges or similar part intended for use in the superstructure of any bridge, the contractor shall, whenever required, in the course of manufacture, arrange and afford all facilities for the purpose of inspection and test of all or any of the part and the material used therein to any officer of the Directorate of inspection of the Ministry of works, production and supply of the Government of India and such bearing, hinges or similar parts shall not be used in the superstructure of any bridge except on production of a certificate of acceptance thereof from the Directorate of Inspection.  All inspection charges will be payable by the contractors.

(This clause may be struck off, if the tender is not for bridge work)

Additions Alterations is Specifications and Designs

CLAUSE 13 –The Engineer-in-charge shall have power to make any alterations in, omissions, from additions to, or substitutions for, the original specifications, drawings, designs and instructions, that my appear to him to be necessary or advisable during the progress of the work, and the contractor shall be bound to carry out the work in accordance with any instruction which may be given to him in writing signed by the Engineer-in-charge and such alterations omissions additions or substitution shall not invalidate the contract and any altered, additional or substituted work which the contractor may be directed to do in the manner above specified as part of the work shall be carried out by the contractor on  the same conditions in all respects on which he agreed to do the main work and at the same rates as are specified in the tender for the main work, provided the total value of such increased or altered or substituted work does not exceed 25% of the amount put to tender, inclusive of contractor’s percentage. If such value exceeds 25% it shall be open to the contractor either to determine the contract or apply for extension.

Extension of time in Consequence of Alterations

The time for the completion of the work shall be extended in the proportion that the altered, additional or substituted work bear to the original contractor’s work and certificate of the Engineer-in-charge shall be conclusive as to such proportion.

Rates for works not in Unified Schedule of Rates

And if the altered, additional or substituted work includes any class of work, for which no rate is specified in this contract, then such classes of work shall be carried out at the rates entered in the applicable schedule of rates which was in force on the date of tender provided that when the tender for the original work as a percentage below/above the schedule of rate, the altered, additional or substituted work required as aforesaid shall be chargeable at the said schedule of rates, minus/plus the same percentage deduction, addition and if such class of work is not entered in and arrange to carry if out in such manner as may be considered advisable provided always and if the contractor shall commence work or incur any expenditure in regard thereto before the rates shall have been determined as lastly herein before mentioned then and in such case he shall only be entitled to be paid in respect of the work carried out on Expenditure incurred by him prior to the date of the determination of the rates as aforesaid according to such rate or rates as shall be fixed by the Engineer-in-charge.  In the event of a dispute the decision of the S.E. of the circle shall be final.

If during the course of execution, where it is found necessary that certain item/items of work not provided for in the C.S.R. of the Circle required to be carried out then the Engineer-in-charge shall identify such item/items including approximate quantity of the contract and ask the contractor to submit his rates in writing supported by the requisite data within a period of 7 days.  The Engineer-in-charge shall obtain approval/modification of the proposed rate from the competent authority and communicate the same within a period of 4 Weeks to the contractor.  In case the contractor agrees to the above rates as fixed by the competent authority then they shall form a part of supplementary schedule of the contract agreement.  If the contractor does not agree to the rate of the competent authority then it shall be open for the Engineer-in-charge to get the work executed through any other agency.  The contractor will not however be entitled to any compensation due to delay or hindrance or loss of profit accruing on account of this extra work executed by alternative agency.

If the contractor commences non schedule work or incur expenditure in regard there to before the rates shall have been determined by the competent authority, then he shall be entitled for payment for the work done as may be finally decided by the competent authority. In the event of dispute, the decision of the DWSM, MOHLA-MANPUR-A.CHOWKI shall be final.

No Claim to any Payment or compensation for alteration in or restriction of works

CLAUSE 14 –If at any time after the execution of the contract documents, the Engineer-in-charge shall for any reason whatsoever require the whole or any part of the work as specified in the tender to be stopped for any period or shall not require the whole or part of the work to be carried out at all or to be carried out by the contractor he shall give notice in writing of the fact to the contractor  who shall there upon suspend or stop the work totally or partially, as the case may be.

In any such case, except as provided hereunder, the contractor shall have no claim to any payment or compensation what so ever on account of any profit or advantage which he might have derived from the execution of the work in full, but which he did not so derive in consequence of the full amount of the work not having been carried out or on account of any loss that he may be put to on account of materials purchased or agreed to be purchased or for, unemployment of labour recruited by him.  He shall not also have any claim for compensation by reason of any alteration having been made in the original specifications, drawing, designs and instructions, which may involve any curtailment of the work as originally contemplated. Where, however, materials have already been purchased or agreed to be purchased by the contractor shall be paid for such materials at the rates determined by the Engineer-in-charge provided they are not in excess of requirement and or of  approved quality and/or shall be compensated for the loss if any, that he may be put to, in respect of materials agreed to be purchased by him, the amount of such compensation to be determined by the Engineer-in-charge  whose decision shall be final.  If the contractor suffers any loss on account of his having to pay labour charges during the period during which the stoppage of work has been ordered under this clause, the contractor shall, on application be entitled to such compensation on account of labour charges as the Engineer-in-charge, whose decision shall be final, may consider reasonable provided that the contractor shall not be entitled to any compensation on account of labour charges, if in the opinion of the Engineer-in-charge, the labour could have been employed by the contractor else where for the whole or part of the period. During which the stoppage of the work has been ordered as aforesaid.

If the total duration suspension of the work is more than the six months, then this suspension of the work will be considered as permanent stoppage of the work, and the contractor can determine the contract, if he so desires.

Time Limit for Unforeseen Claims

CLAUSE 12– Under no circumstances whatever shall the contractor be entitled to any compensation from Government on any account unless the contractor shall have submitted a claim in writing to the Engineer-in-charge within one month of the cause of such claim occurring.

Action and Compensation Payable in case of Bad Work

CLAUSE 16 –If at any time before the security deposit is refunded to the contractor, it shall appear to the Engineer-in-charge or his subordinate in charge of the work, that any work has been executed with unsound, imperfect or unskillful workmanship or with materials of inferior quality, or that any materials or articles provided by him the execution of  the works are unsound, or of a quality inferior to that contracted for, or are otherwise not in accordance with the contract, it shall be lawful for the Engineer-in-charge to intimate this fact in writing to the contractor and then notwithstanding the fact that the work, materials or articles complained of may have been inadvertently passed, certified and paid for the contractor shall be bound forthwith to rectify, or remove and reconstruct the work so specified in whole or in part, as the case may require, or if so required, shall remove the materials or articles so specified and provide other proper and suitable materials or articles at his own proper charge and cost, and in the event of his failing to do so within a period to be specified by the Engineer-in-charge in the written intimation aforesaid, the contractor shall be liable to pay compensation at the rate of one percent. On the amount of contract put to tender every day not exceeding ten days, during which the failure so, continues, and in the case of any such failure the Engineer-in-charge may rectify or remove and, re-execute the work or remove and replace the materials or articles complained of as the case may be at the risk and expense in all respects of the contractor. Should the Engineer-in-charge consider that any such inferior work or materials as described above may be accepted or made use of, it shall be within his discretion to accept  the same at such reduced rates, as he may fix there for.

Work to be open for inspection – Contractor or Responsible Agent to be present

CLAUSE 17 –All work under or in course of execution or executed in pursuance of the contract shall at all time be open to the inspection and supervision of the Engineer-in-charge and his subordinates and the contractor shall at all times during the usual working hours, and at all other times at which reasonable notice of the intention of the Engineer-in-charge or his subordinate to visit the work shall have been given to the contractor, either himself be present to receive orders and instructions or have a responsible agent duly accredited in writing present for that purpose.  Orders given to the contractor’s agent shall be considered to have the same force as if they had been given to the contractor himself.

Notice to be Given Before Work is Covered Up

CLAUSE 18 –The contractor shall give not less five than days notice in writing to the Engineer-in-charge or his subordinate in charge of the work before  covering up or otherwise placing beyond the reach of measurement any work in order that the same may be measured, and correct dimensions thereof be taken before the same is so covered up or placed beyond the reach of measurement, any work without the consent in writing of the Engineer-in-charge or his subordinate in charge of the work and if any work shall be covered up or placed beyond the reach of measurement without such notice having been given are consent obtained, the same shall be uncovered at the contractor’s expenses, or in default thereof, no payment or allowance shall be made for such work or the materials with which the same was executed.

Contractor Liable for Damage Done and for Imperfections for Twelve Months after Certificate

CLAUSE 19 –If the contractor or his work people or servants shall break, deface, injure or destroy any part of building in which they may be working or any building, road, road curbs, fences, enclosures, water pipes, cabes, drains, electric or telephone posts or wires, trees, grass or grassland or cultivated ground contiguous to the premises on which the work or any part of it is being executed, or if any damage shall happen to the work, while in progress, from any cause whatever, or any imperfections become apparent in it within ffici months after a certificate final or otherwise or its completion shall have been given by the Engineer-in-charge as aforesaid, the contractor shall make good the same at his own expense or in default, the Engineer-in-charge may cause the same to be made good by other workmen and deduct the expense (of which certificate of the Engineer-in-charge shall be final) from any sums that may be then or at any time thereafter, may become due to the contractor or from his security deposits, or the proceeds of sale thereof, or of a sufficient portion thereof.

The contractor hereby also covenants that it shall be his responsibility to see that the buildings constructed under this contract do/does not leak during the period of two consecutive rainy seasons after its (their) completion and if any defects are pointed out to him by the Engineer-in-charge during the said period, the same shall be removed by him at his own expense or in default the Engineer-in-charge may get them removed and deduct the expenses there of from any sum that may be then due to or may become due to the contractor or from the security deposits of the contractor, on amount equal to 20% cost of the roof shall not withstanding anything contained in this clause be retained, till the roofs are tested during two consecutive rainy seasons as aforesaid and the defects are fully removed and if any amount still remains due to this account after making deductions as aforesaid the same may be recovered from him as an arrears of land revenue/cash security.  The security deposit of the contractor to the extent of 50% shall be refunded on his getting the completion certificate, provided that all the recoveries outstanding against him are realized 25% of the amount shall be refunded on maintenance period being over, even if the final bill is not passed, balance 25% shall be refunded after the final bill is passed.

Contractor to Supply Plant, Ladders, Scaffolding Etc.

CLAUSE 20 –The contractor shall supply at his own cost materials (except such special materials if any, as may in accordance with the contract be supplied from the Engineer-in-charge’s Stores) plants, tools, appliances, implements, ladders, cordage, tackle, scaffolding and temporary works, requisite for the proper execution of the work, whether original, altered or substituted, and whether included in the specification or other documents forming part of the contract referred to in these conditions or not or which may be necessary for the purpose of satisfying or complying with the requirements of the Engineer-in-charge as to any matter as to which under these conditions he is entitled to be satisfied, or which he is entitled to require together with carriage therefore to and from the work.  The contractor shall also supply without charge requisite number of persons with the means and materials necessary for the purpose of setting out works, and counting, weighing & assisting in the measurement or examination at any time and from time to time of the work, or materials failing his so doing the same may be provided by the Engineer-in-charge at the expense of the contractor and the expenses may be deducted from any money due to the contractor under the contract, or from his security deposit or the proceeds of sale thereof, or of a sufficient portion thereof.

Contractor is liable for damages arising from non-provision of lights fencing etc. The contractor shall also provide at on his own cost except when the contract specifically provides otherwise and except for payments due under clause all necessary fencing and lights required to protect the public from accident and shall be bound to bear the expenses of defiance of every suit, action or proceedings at law that may be brought by any person for injury sustained owing to neglect of the above precautions & to pay any damage and costs which may be awarded in any such suit, action or proceedings to any such person or which may with the consent of the contractor be paid to compromise any claim by any such person.

Compensation under section 12 sub-section (1) of the workman’s Compensation act 1923

CLAUSE 21– In every case in which by virtue of the provisions of section 12 sub-section (1) of the workman’s compensation Act 1923 Government is obliged to pay compensation to a workman employed by the contractor in execution of the works Government will recover from the contractor the amount of compensation so paid and without prejudice to the rights of Government under section (1) sub-section (2) of the said Act. Government shall be at liberty to recover the amount or any part there of by deducting it from the security deposit or from any sum due by Government to the contractor whether under this contract or otherwise Government may not be bound to contest any claim made against them under section-12 sub-section (1) of the said act. Except on the written request of the contractor and upon his giving to Government full security for all cases for which Government might become liable in consequence contesting such claim.

LABOUR

CLAUSE 22 -  The contractor should get himself registered under-contract-labour regulations and abolition Act’,1940 including its amendments after getting a certificate from the principal employer, who will be the Engineer-in-charge.

CLAUSE 23 -     Labour below the age of 12 years – No labour below the age of 12 years shall be employed on the work.

FAIR WAGE

CLAUSE 24 –The contractor shall pay not less than fair wage to labour engaged by him on the work.

Explanation (a) ‘Fair wage’ means wage whether for time or piece work notified at time of inviting tenders for the work and where such wages have not been so notified, the wages prescribed by the Works Department for the division in which the work is done.

- The contractor shall, notwithstanding the provisions of any contract to the contrary cause to be paid a fair wage to labourers indirectly engaged on the work including any labour engaged by his sub-contractors in connection with the said work, as if the labourers had been immediately employed by him.

I In respect of labour directly or indirectly employed on the works for the performance of the contractor’s part of this agreement the contractor shall comply with or cause to be complied with the labour Act inforce.

(d) The Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI /Sub-Divisional Officer, shall have the right to deduct, from the moneys due to the contractor, any sum required or estimated to be required for making good the loss suffered by a worker or workers by reasons of non-fulfillment of the conditions of the contract for the benefit of the workers, non-payment of wages or deductions made from his or their wages, which are not justified by the terms of the contract or non observance of the regulations.

(e) The contractor shall be primarily liable for all payments to be made under and for the observance of the regulations aforesaid without prejudice to his right to claim indemnity from his sub-contractors.

(f)  The regulations aforesaid shall be deemed to be a part of this contract and any breach thereof shall be deemed to be a breach, of this contract.

Work not to be sublet

CLAUSE 25 –The contract may be rescinded and security deposit forfeited, for subletting, bribing or if contractor becomes insolvent :-

The contract shall not be assigned or sublet without the written approval of the Divisional Officer  and if the contractor shall assign or sublet his contract, or attempt, so to do, or become insolvent commence any insolvency proceedings or make any composition with his creditors, or attempt so to do or if any bribe, gratuity, gift, loan, perquisite, reward of advantage pecuniary or otherwise, shall either directly or indirectly be given, promised or offered by the contractor, or any of his servants or agents  to any public officer or person in the employ of Government in any way relating to his office or employment, or if any such officer or person shall become in any way directly or indirectly interested in the contract, the Divisional Officer may there up by notice in writing record the contract, and the S.D. of the contractor shall there upon stand forfeited and be absolutely at the disposal of Government and the same consequences shall ensure as if the contract had been rescinded under clause 3 thereof, and in addition the contractor shall not be entitled to recover or be paid for any work thereto for actually performed under the contract.

If the contractor gets item/items of work executed on a task rate basis with or without materials, this shall not amount to-subletting of the contract.

Sum payable by way of Compensation to be Considered as Reasonable Compensation

without Reference to Actual Loss

CLAUSE 26 –All sums payable by way of compensation under any of these condition shall be considered as reasonable compensation to be applied to the use of Government without reference to the actual loss or damage sustained, and whether or not any damage shall have been sustained.

Changes in the Constitution of Firm

CLAUSE 27 –In the case of a tender by partners any change in the constitution of the firm shall be forthwith notified by the contractor to the Engineer-in-charge for his information.

Work to be under Direction of Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI

CLAUSE 28 –All works to be executed under the contract shall be executed under the direction and subject to the approval in all respect of the Executive Engineerand member secretaryDWSM, MOHLA-MANPUR-A.CHOWKI  of the Division/ DWSM, MOHLA-MANPUR-A.CHOWKI or the time being who shall be entitled to direct at what point or points and in what manner they are to be commence and from time to time carried on.

Arbitration Clause

CLAUSE 29– Except as otherwise provided in this contract all question and dispute relating to the meaning of the specifications, designs, drawings and instructions herein before mentioned and as to thing whatsoever in any way arising out or relating to the contract designs, drawings, specifications, estimates, concerning the works, or the execution or failure to execute the same, whether arising during the progress of the work or after the completion or abandonment thereof shall be referred to the DWSM, MOHLA-MANPUR-A.CHOWKI  in writing for his decision, within a period of 30 days of such occurrence. Thereupon the DWSM, MOHLA-MANPUR-A.CHOWKI  shall give his written instructions and/or decisions within a period of 60 days of such request.  This period can be extended by mutual consent of the parties.

Upon receipt of written instructions or decisions the parties shall promptly proceed without delay to comply such instructions or decisions, if the DWSM, MOHLA-MANPUR-A.CHOWKI  fails to give his instructions or decisions in writing within a period of 60 days or mutually agreed time after being requested if the parties are aggrieved against the decision of the DWSM, MOHLA-MANPUR-A.CHOWKI , the parties may within 30 days prefer an appeal to the SWSM who shall afford an opportunity to the parties of being heard and to offer evidence in support of his appeal.  The SWSM will give his decision within 90 days.  If any party is not satisfied with the decision of the SWSM he can refer such disputes for arbitration by an Arbitration Board to be constituted by the State Government which shall consist of three members of whom one shall be chosen from among the officers belonging to the Department not below the rank of S.E. one Retired Chief Engineer of any Technical Department, and one serving officer not below the rank of S.E. belonging to another Technical Department.

The following are also the terms of this contract namely :-

- No person other than the aforesaid Arbitration Board constituted by the Government (to handle cases of all Technical Departments) shall act as arbitrator and if for any reason that is not possible, the matter shall not be referred to Arbitration at all.

(b) The State Government may at any time effect any change in the personal of the Board, and the new members or members appointed to the Arbitration Board shall be entitled to proceed with the reference from the stage at which it was left by his or their predecessors.

I The party invoking Arbitration shall specify the dispute or disputes  to be referred to Arbitration under this clause together with the amount or amounts claimed in respect of each such dispute(s).

(d) Where the party invoking Arbitration is the contractor, no reference for Arbitration shall be maintainable, unless the contractor furnishes a security deposit of a sum determined according to

the table given below, and the sum so deposited shall on the determination of Arbitration proceedings be adjusted against the cost, if any awarded by the Board against the party and the balance remaining after such adjustment or in the absence of the such cost being awarded, the whole of the sum shall be refunded to him within one month from the date of the award.

Amount of claim                                      Rate of Security Deposits

For claim below Rs.10,000                      5% of the amount claimed

For claims of Rs.10,000 & above but      3% of the amount claimed subject to minimum of

Below Rs.1,00,000                                   Rs.500

For claims of Rs.1,00,000 & above         2% of the amount claimed subject

to a minimum of Rs.3,000

- If contractor does not make any demand for Arbitration in respect of any claim(s) in writing within 90 days on receiving intimation from the Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI s that the final bills is ready for payment, the claim of the contractor shall be deemed to have been waived and absolutely barred & the Government shall be discharged or released of all liabilities under the contract in respect of such claims.

- The Arbitration Board may from time to time, with the consent of the parties extend the time for making the award.

- A reference to the Arbitration Board shall be no ground for not continuing the work on the part of the contractor and payment as per terms and conditions of the agreement shall be continued by the Department.

- Except where otherwise provided in this contract, the provisions of the Arbitration Act, 1950 and the rules made there under for the time being inforce, shall apply to the Arbitration proceeding under this clause.

Lump Sum in Estimate

CLAUSE 30 –When the SCHEDULE on which a tender is made includes lump sums in respect of part of the works, the contractor shall be entitled to payment in respect of the items of work involved or the part of the work in the question at the same rates as are payable under this contract for such items, or if the part of  the work in question is not, in the opinion of the Engineer-in-charge, capable of measurement, the Engineer-in-charge may at the his desecration pay the lump sum amount entered in the estimates, and the certificate in writing of the Engineer-in-charge shall be final and conclusive against the contractor with regards to any sum or sums payable to him under the provisions of this clause.

Action where no specification

CLAUSE 31 –In the case of any class of work for which there is no such specification as is mentioned in Rule such work shall be carried out in accordance with the specification approved by DWSM, MOHLA-MANPUR-A.CHOWKI  for application to work in the district and in the event of there being no such specification then in such case the work shall be carried out in all respects in accordance with the instructions and requirements of the Engineer-in-charge.

Contractor’s Percentage Whether Applied to Net or Gross Amounts of Bills

CLAUSE 32 –The percentage referred to at para 7 of the tender will be deducted form/added to the gross amount of the bills for work done after deduction of the cost of materials supplied by the department.

Claim for Quantities Entered in the Tender or Estimate

CLAUSE 33 -  Quantities shown in the tender for approximate and no claim shall be entertained for quantities of work executed being either more or less than those entered in the tender of estimate.

Claim for Compensation for Delay in Starting the Work

CLAUSE 34 -  No compensation shall be allowed for any delay caused in the Starting of the work on account of acquisition of land or in the case of clearance works on account of any delay in according sanction to estimates.

Employment of Scarcity Labour

CLAUSE 35 –If Government declare a state of scarcity or famine to exist in any village situated within sixteen kilometers of the work, the contractor, shall employ upon such parts of the work, as are suitable for unskilled labour, any person certified to him by the Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI  or by any person to whom the Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI  may have delegated this duty in writing, to be in need of relief and shall be bound to pay to such persons wages not below the minimum which Government may have fixed in this behalf. Any dispute which may arise in connection with the implementation of this clause shall be decided by the Executive Engineer and member secretary, DWSM, MOHLA-MANPUR-A.CHOWKI whose decision shall be final and binding on the contractor.

CLAUSE 36 – Royalties charges will be borne by the contractor.

Technical Examination

CLAUSE 37 –The Government shall have the right to cause Audit and Technical Examination of the works and the final bills of the contractor including all supporting vouchers, abstracts etc. to be made as per payments of the final bill and if as a results of such Audit & Technical Examination the sum is found to have been overpaid in respect of any work done by the contractor under the contract or any work claimed by him to have been done under contract and found not to have been executed, the contractor shall be liable to refund the amount of over payment and it shall be lawful for the Government to recover the same from the security deposit of the contractor or from any dues payable to the contractor from the Government account.  If it is found that the contractor was paid lesser than what was due to him under the contract in respect of any work executed by him under it, the amount of such under payment shall be duly paid by the Government to the contractor

In the case of any audit examination and recovery consequent on the same the contractor shall be given an opportunity to explain his case and the decision of the DWSM, MOHLA-MANPUR-A.CHOWKI  shall be final.

In the case of Technical Audit, consequent on which there is a recovery from the contractor, no recovery, should be made without orders of the DWSM, MOHLA-MANPUR-A.CHOWKI whose, decision shall be final.  All action under this clause should be initiated and intimated to the contractor within a period of twelve months from the date of completion.

Death of Permanent Invalidity of Contractor

CLAUSE 38 –If the contractor is an individual or a proprietary concern, partnership concern, dies during the currency of the contract or becomes permanently incapacitated, where the surviving partners are only minors, the contract shall be closed without levying any damages/compensation as provided for in clause 3 of the contract agreement.

However, if the competent authority is satisfied about the competence of the survivors , then the competent authority shall enter in to a fresh agreement for the remaining work strictly on the same terms and conditions, under which the contractor was awarded.

Penalty for Breach of Contract

CLAUSE 39 –On the breach of any term or condition of this contract by the contractor the said Governor shall be entitled to forfeit the security deposit or the balance thereof, that may at the time be remaining, and to fficie and retain the same as damages and compensation for the said breach, but without prejudice to the right of the Governor to recover further sums as damages from any sums due or which may become due to the contractor by Government or otherwise howsoever.

Note -  The person or firm submitting the tender should see that the rates in the schedule showing materials to be supplied by the department are filled up by the Engineer-in-charge on the issue of the form prior to the submission of the tender.

Notice to the Contractor to Start Work

Your contract for the……………………………… has been accepted by me/ DWSM, MOHLA-MANPUR-A.CHOWKI  on behalf of the Governor of Chhattisgarh on the …………………….. day of…………………20………….. and you are hereby ordered to commence the work.

Executive Engineer and member secretary, DWSM, MOHLA-MANPUR-A.CHOWKI /Sub-Divisional Officer

The notice to the contractor(s) to start work from the…………… day of……….. 20…………….. was issued vide this office memorandum No……………….. dated the ……….. 20…

Signature of contractor                                    			 Signature of Sub-Divisional Officer/

Divisional Officer

Annexure – E-I

SPECIFICATION FOR PVC PIPE

| 1.1 | Excavation in trenches in all types of strata for laying & jointing of above pipe line with required depth and width as per specification. |  |  |
| --- | --- | --- | --- |
| 1.2 | Construction of valve chambers in all the places where sluice valve, Air valve and non-return valve are provided, Anchor/thrust Blocks, Pedestal/supporting pillars etc. if required as per specification. The internal dimension of chamber shall have clear space of 500 mm all around in valve. This clearness can be modified only after with instruction issued by the Engineer in charge looking to the site conditions. |  |  |
| 1.3 | Back filling of trench as per IS: 12288-1987. |  |  |
| 1.3.1 | Providing and making all interconnections as required to charge the laid pipe lines under scope of work. |  |  |
| 1.4 | In case, the shifting/change of alignment of any existing, water pipe line/sewage line is considered necessary by the Deptt./Municipal Corporation, such service lines will have to be shifted by the contractor for which the payment shall be made for the actual work done as per approved rate of this contract. |  |  |
| 1.5 | (a) No additional payment for transportation of pipe or specials will be paid. (b) On request of contractors charge for inter connection will be decided by S.E. PHE circle .......... separately, inter connection means connection between new pipe line under this NIT and existing old pipe line. |  |  |
| 2.0 | Unplasticized PVC pipes for potable water supply shall be as per IS 4985-2000 duly inspected and tested and having BIS certification mark. |  |  |
| 2.1.1 | Selection, Handling, storage and installation of UPVC pipes shall be as per IS 7634 (part-3)- 2003 |  |  |
| 2.1.2 | Specification of injection moulded PVC socket fittings with solvent cement joints shall be as per IS 7834 (Part-I to VII)-1987. |  |  |
| 2.2 | Visual Appearane |  |  |
| 2.2.1 | The colour of the pipes shall be light grey. Slight variations in the appearance of the colour are permitted. |  |  |
| 2.2.2 | The internal and external surfaces of the pipe shall be smooth, clean and free from grooving and other defects. Slight shallow longitudinal grooves or irregularities in the pipe shall be permissible provided the wall thickness remains wihin the permissible limits. |  |  |
| 2.2.3 | Each pipe may also be marked with the standard mark of BIS certification. |  |  |
| 3.0 | Storage |  |  |
| 3.1.1 | PVC solvent cement should be stored in a cool place except when actually in use at the site. The solvent cement has a limited self life when not stored in hermetically sealed containers. |  |  |
| 3.1.2 | Pipes should be stacked on surface flat and free from sharp object, stones or projection in order to avoid deformation or damage. Ends of pipes should be protected from abrasion and chipping. |  |  |
| 4.0 | In rocky area 12cm cushion of sand or morum below and above the pipe should be provided as per IS 7634 (PartIII): 2003. ( See drawing No. -03) |  |  |
| 5.0 | Marking |  |  |
|  | Each pipe shall be clearly and indelibly marked in ink/paint or hot embossed on white base at intervals of not more than 3 meters, in colours as indicated below. |  |  |
|  | (a) Manufacturer’s name or trade-mark |  |  |
|  | - Out side diameter, |  |  |
|  | - Class of pipe and pressure rating |  |  |
|  | - Batch and lot number |  |  |
|  | - The word plumbing in the case of plumbing pipes. |  |  |
|  | - Each pipe may also be marked with the standard mark BIS certification. |  |  |
|  | Class of Pipe | Working pressure kg/sqcm | Colour of Marking |
|  | Class 2 | 4 | Blue |
|  | Class 3 | 6 | Green |
|  | Class 4 | 8 | Brown |
|  | Class 5 | 10 | Yellow |
| 6.0 | Marking of fittings |  |  |
|  | (i) All fittings shall be clearly and indelibly marked at a prominent place visible even after the installation of the fittings with the following: |  |  |
|  | (a) Manufacturer’s indentification mark, and |  |  |
|  | (b) Size of the fitting and the appropriate class (working pressure) of IS: 4985-1988 to which the pressure rating of the fitting corresponds. |  |  |
|  | (ii) PVC fittings also Conforming to specific requirements as prescribed in the relevant clause of the standard may also be marked with the standard Mark. |  |  |
| 7.0 | The work shall be executed in accordance with the specification and all relevant latest IS codes. |  |  |
| 8.0 | Laying of pipes and fittings/specials includes all precautions to guard against possible to the existing structure/pipe lines, cables etc., taking precautions to prevent dirt form entering the pipe ends, lowering and laying pipe and specials in the trenches with specials arrangement such as cranes, tripods with chain pulley block, use of slings of canvas etc. to fit the endsof pipe and fittings.specials to lift and lower the same. Inspection of pipes and fittings for defects while suspended. Laying of pipes perfectly true in alignment and to gradient etc. |  |  |
| 9.0 | Measurement |  |  |
|  | All measurement should be of the finished work only. The net length of pipes as laid or fixed shall be measured in running meters correct to 10mm. Specials shall be excluded and measured and paid separately. The portion of the pipe inside the joints shall not be included in the length of pipe work. Excavation, refilling, masonry and concrete work wherever required shall be measured and paid for separately under relevant items of work. |  |  |
| 10.0 10.1 | FORMATION : GENERAL : Before pipe line is laid, proper formation be prepared for pipe line |  |  |
| 10.2 | Excavation and Preparation of Trenches for Laying Underground Pipe Lines. |  |  |
| 10.2.1 | The width of the trench at bottom between the faces of sheeting shall be such as to provide not less than 200 mm clearance on either side of the pipe except where rock excavation is involved. Trenches shall be of such extra width, when required, as will permit the convenient placing of timber supports, strutting and planking, and handling of specials. |  |  |
| 10.2.2 | Special consideration should be given to the depth of the trench. In agricultural land, the depth should be sufficient to provide a cover of not less than 900 mm so that the pipe line will not interfere with the cultivation of the land. In rocky ground, rough grazing or swamps, the cover may be reduced provided the water in the pipe line is not likely to freeze due to frost. |  |  |
| 10.2.3 | It may be necessary to increase the depth of pipe line to avoid land drains or in the vicinity of roads, railways or other crossings. |  |  |
| 10.2.4 | Care should be taken to avoid the spoil bank causing an accumulation of rain water. |  |  |
| 10.2.5 | Where pipes are to be bedded directly on the bottom of the trench, it should be trimmed and fficie to permit even bedding of the pipe line and should be free from all extraneous matter which may damage the pipe or the pipe coating. Additional excavation should be made at the joints of the pipe so that the water main is supported along its entire length. |  |  |
| 10.2.6 | Where excavation is through rocks or boulders or at locations of B.C. soils the pipe line should be bedded on concrete bedding or on at least 120 mm of fine grained material, or other proper means are used to protect the pipe and its coating. Material harmful to the pipe line should not be used. |  |  |
| 10.2.7 | Temporary under pining, supports and other protective measures for building structures or apparatus in or adjacent to the trench should be of proper design and sound construction. |  |  |
| 10.3 | ROCK EXCAVATION - The term “rock” wherever used in this standard, shall have the same meaning as given in terminology in IS:1200 (Part 1)- 1974. |  |  |
| 10.3.1 | Blasting– Blasting for excavation shall be permitted only after securing the approval of the authority and only when proper precautions are taken for the protection of persons and property. The hours of blasting shall be fixed by the authority. The procedure of blasting shall conform to the requirements of the authority. |  |  |
| 10.4 | Stacking Excavated Material –All excavated material shall be stacked in such a manner that it will not endanger the work or workmen and it will avoid obstructing footpaths, roads and driveways. Hydrants under pressure, surface boxes, fire or other utility controls shall be left unobstructed and accessible during the construction work. Gutters shall be kept clear or other satisfactory provisions made for street drainage, and natural water-courses shall not be obstructed. |  |  |
| 10.5 | Barricades, Guards and Safety Provisions - To protect persons from injury and to avoid damage to property, adequate barricades, construction signs, torches, red lanterns and guards, as required, shall be placed and maintained during the progress of the construction work and until it is safe for traffic to use the roadways. All materials, piles equipment and pipes which may serve as obstruction to traffic shall be enclosed by fences or barricades and shall be protected by illuminating proper lights when the visibility is poor. The rules and regulations of the local authorities regarding safety provisions shall be observed. |  |  |  |
| 10.6 | Maintenance of Traffic and Closing of Streets - The work shall be carried including closing of road/street in such a manner which will cause the least interruption to traffic. Where it is necessary for traffic to cross the open trenches, suitable bridges shall be provided. |  |  |  |
| 10.6.1 | Suitable signs indicating that a street is closed shall be placed and necessary detour signs for the proper maintenance of traffic shall be provided. |  |  |  |
| 10.7 | Protection of Property and Structures– Trees, shrubbery, fences, poles, and all other property and surface structures shall be protected unless their removal is shown on the drawings or authorized by the authority. When it is necessary to cut roots and tree branches, cutting shall be done under the supervision and direction of the authority. |  |  |  |
|  | Temporary support, adequate protection and maintenance of all underground and surface structures, drains, sewers and other obstructions encountered in the progress of the work, shall be provided under the direction of the authority. The structures, which may have been disturbed, shall be restored after completion of the work. |  |  |  |
| 10.8 | Protection of the Existing Service –As far as possible, the pipe line shall be laid below existing services, like water and gas pipes, cables, cable ducts and drains but not below sewers which are usually laid at greater depth. Where it is unavoidable, pipe line should be suitably protected. A minimum clearance of 120 mm shall be provided between the pipe line and such other services. Where thrust or auger boring is used for laying pipe line across roads, railways or other existing utilities, large clearance as required by the existing utilities, as required shall be provided. Adequate arrangements shall be made to protect and support the existing services during the laying operations. The pipe line shall be so laid as not to obstruct access to other services for inspection, repair and replacement. When such utilities are met with during excavation, the authority concerned shall be intimated and arrangements should be made to support and protect the utilities in consultation with them and in case of such damaged services will be repaired at the cost of contractor himself. |  |  |  |
| 11.0 | TRANSPORTATION, HANDLING AND INSPECTION : |  |  |  |
|  | Transportation –Pipes should be loaded in such a way that they are secured and that no movement should taken place on the vehicle during transit. |  |  |  |
|  | Off-Loading –Cranes should be preferred for off-loading. However, for pipe up to 500 mm nominal bore, skid timbers and ropes may be used. |  |  |  |
| 12.0 12.1 | HYDRAULIC TESTING : The completed pipe line shall be tested for a pressure given in as per code, Annexure “E” |  |  |  |
| 13.0 13.1 | FLUSHING AND DISINFECTION OF MAINS BEFORE COMMISSIONING : The pumping main & distribution mains shall be disinfected before commissioning as per provisions given in CPHEEO manual and IS 5822:1940. |  |  |  |
| 14.0 | REMOVAL, RESTORATION AND MAINTENANCE OF PAVED FOOTPATHS ETC. AFTER LAYING OF PIPE : |  |  |  |
| 14.1 | Allowable Removal of Pavement –Pavement and road surfaces may be removed as a part of the trench excavation, and the amount removed shall depend upon the width of trench specified for the installation of the pipe and the width and length of pavement area required to be removed for the installation of gate valves, specials, manholes or other structures. The width of pavement removed along the normal trench for the installation of the pipe shall not exceed the width of the trench specified by more than 120 mm on each side of trench. The width and the lengths of the area of pavement removed from the installation of gate valves, specials, manholes or other structures should not exceed the maximum linear dimensions of such structures by more than 120 mm on each side. Wherever, in the opinion of the authority, existing conditions make it necessary or advisable to remove additional pavement, it shall be removed as directed by the authority. |  |  |  |
| 14.2 | Restoration of Damaged Surface and Property –Where any pavement, shrubbery, fences poles or other property and surface structures have been damaged, removed or disturbed during the course of work, such property and surface structures shall be replaced or repaired after completion of work. The permanent pavement shall not be restored to a condition equal to that before the work began but the top surface of the removed pavement shall be fficie and finished in such a manner as the traffic may pass smoothly. |  |  |  |
| 14.3 14.1 | Cleaning-up : All surplus materials, and all tools and temporary structures shall be removed from the site as directed by the authority. All dirt, rubbish and excess earth from the excavation shall be hauled to a dump and the construction site left clean to the satisfaction of the authority. |  |  |  |
| 12.0 | APPURTENANCES : BRANCH CONNECTIONS : “T” outlet with manually operated sluice valves shall be provided in the distribution pressure Main for Branch connections. |  |  |  |
| 12.1 | The appurtenances shall be located in such a way that these are clearly and easily accessible for operation and maintenance. |  |  |  |
| 16.0 | CIVIL WORKS : All the allied civil works necessary for laying and jointing of pipeline shall be a part of this contract; therefore, the contractor shall design and carry out the necessary civil works such as thrust blocks, anchor blocks, chambers for appurtenances and necessary earth work. All the civil works shall be designed and carried out as per the relevant Indian Standard Code of practice. All the materials used on civil work should be of a quality approved by Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI . Rejected material shall be removed from the site immediately at the cost of contractor. |  |  |  |
| 17.0 | TESTING, COMMISSIONING OF PIPE LINE, TRAIL RUN AND DEFECT LIABILITY PERIOD :- After completing the job of laying and jointing of pipe line the contractor will do testing, commissioning and operation& maintenance for a period of three months including training of personnel. The repairing of bursting and leakage of pipeline during this period shall be done by the contractor at his cost including cost of all materials. The water for testing and commissioning of pipe line will be made available by the Deptt. Free of cost at the time when convenient to the deptt. In case of delay in making available the water for testing the time extension will be given to the contractor without penalty for such period. Defect liability period shall be 12 months from the date of completion of work. |  |  |  |

18	Disinfection of Pipe line before commissioning:

18.1 Pipeline carrying potable water shall be suitably disinfected before commissioning as per guidelines given in CPHEEO Manual & relevant IS codes.  For this purpose guidance may be obtained from IS:3114-1965 or IS: 5822-1940.

Annexure –II

SPECIFICATION FOR CIVIL WORK

| 1. | Excavation |
| --- | --- |
|  | - All excavation operations shall include excavation and ‘getting out’ the excavated materials. The excavations shall conform to the lines & levels shown in the drawings and as directed by the Engineer-in-Charge. The contractor shall not excavate outside the limits of excavation. Only the excavation shown on the drawings shall be measured and recorded for payment. Extra excavation beyond the dimensions shown in the drawing, shall not be measured for payment, nor the filling/ backfilling resulting from such excavation shall be payable. Any excess depth/width, excavated beyond the specified levels/ dimensions on the drawing shall be made good at the cost of the contractor with the same material or good earth as decided by the Engineer-in-Charge. - Excavation can be done by mechanical or manual means as per choice/ ease of contractor and conditions of site. - The water that may accumulate in excavations during the progress of the work fromsprings, Tidal or river seepage, Broken water mains or drains (not due to the negligence of the contractor) and seepage from subsoil aquifer shall be bailed, pumped out or otherwise removed. - Excavation where directed by the Engineer-in-Charge shall be securely fenced and provided with proper caution signs, conspicuously displayed during the day and properly illuminated with red lights during the night to avoid accidents - 1.5 1.5 Earth work and Excavation shall be done as per IS 1200 (Part-1):1992 and safety codes IS 3764 :1992 |
| 2. | Filling |
|  | - The earth used for filling shall be free from all roots, grass, shrubs, rank vegetation, brushwood, tress, sapling and rubbish. Black cotton soil shall not be used for filling/ back filling. - The space around the foundations and drains in trenches shall be cleared of all debris, brick bats etc. The filling shall be done in layers not exceeding 20 cm in depth. Each layer shall be watered, rammed and consolidated. Ramming shall be done with iron rammers where possible and with blunt end of crow bars where rammers cannot be used. Special care shall be taken to ensure that no damage is caused to the pipes, drains, masonry or concrete in the trenches. In case of filling under floor, the finished level of filling shall be kept to the slope intended to be given to the floor. |
| 3 | Materials |
|  | 3.1Samples of all materials to be used on the work shall be got approved by the contractor from the Engineer-in-Charge well in time. The approved samples duly authenticated and sealed shall be kept in the custody of the Engineer-in-Charge till the completion of the work. All materials to be provided by the contractor shall be brand new and as per the samples approved by the Engineer-in-Charge 3.2 Mandatory tests are required to be carried out as per relevant IS: code. Testing of samples shall be done in the laboratory approved by the Engineer-in-Charge 3.3 The materials, supplied by the Department shall be deemed to comply with the specifications 3.4 Materials stored at site, depending upon the individual characteristics, shall be protected from atmospheric effects due to rain, sun, wind and moisture to avoid deterioration 3.5 All material brought at the site shall be tested, before being used in the work. - Materials like timber, paints etc. shall be stored in such a way that there may not be any possibility of fire hazards. Inflammable materials and explosives shall be stored in accordance with the relevant rules and regulations or as approved by Engineer-in-Charge in writing, so as to ensure desired safety during storage. - The unit weight of materials unless otherwise specified shall be as given in IS: 1911-1967. |
| 4 | Concrete work shall be done as per IS 506 : 2000 |
| 5 | - Cement shall be used as IS standard given below :- When the strength of concrete required is up to M-20, then O.P.C. Conforming to IS 269-1989 or P.S.C. conforming to IS 505 may be used |
| 6 | - S Steel shall be used as per IS standard . - Mild steel and medium tensile steel bars shall Conform to IS :432 (Part- I : 1982 |

- Sand

Sand shall not contain dust, lumps, soft or flaky materials. Fine aggregate having positive alkali silica  reaction shall not be used. All fine aggregate shall confirm to IS :383. The fineness modular of fine aggregate shall neither be less than 2.0 nor greaterthan 3.5. Sand to be used in work shall conform to IS 1242-1960 for plaster and IS 166-1965 for masonry work.

- Coarse aggregate

8.1	Coarse aggregate consist of clear, hard, strong, dense, nonporous and durable pieces ofcrushed stone. They shall not consist pieces of elongated particles salt, alkali, vegetablematter or other deleterious material.

8.2  All coarse aggregate shall conform to IS : 383 & tests for conformity shall be carried out as per  IS: 2386 Part I to VIII. The maximum value of flakiness index for coarse aggregate shall not exceed 35%. The coarse aggregate shall satisfy the following requirement of grading.

9.   Bricks

9.1 Fly ash brick

Clay Fly Ash Bricks: The clay fly ash bricks shall conform to IS 13757. The bricks shall be sound,compact and uniform in shape and colour. Bricks shall have smooth rectangular faces with  sharp and square corners. The bricks shall be free from visible cracks, flaws, warpage,nodules of free lime and organic matter, the bricks shall be hand or machine moulded.Thebricks shall have frog of 100 mm in length 50 mm width and 10 to 20 mm deep on one of its flat sides. Ifmade by extrusion process may not be provided with frogs. Fly Ash shall conform to grade I or grade II of IS 3812.

9.2    Soaking of brick

Bricks shall be soaked in water before use for a period of minimum one hour prior to being laid.   When the bricks are soaked they shall be removed from the tank sufficiently early so than at the time of laying they are skin-dry. Such soaked bricks shall be stacked on a clean place where they are not again spoiled by dirt earth etc.

10. Mortar

10.1 The mortar mixing shall preferably be done in mechanical mixer operated manuallyor by power. Hand mixing can be restored to as long as uniform density of the mix andits strength are assured subject to prior approval of Engineer-in-charge

| 10.2 | Hand mixing operation, if permitted, carried out on clean water tight platform when cement and sand shall be first mixed dry in required proportion several times till the mixture is of uniform. Minimum quantity of water shall be added to bring the mortar to the consistency of stiff paste. |
| --- | --- |
| 10.3 | Mortar shall be mixed only in such quantity as required for immediate use. The mortar normally be considered to use within 30 minutes. Mortar remains unused after 30 minutes shall be rejected and removed from site. |
| 11. | Plaster Plastering shall be started from top towards bottom. Wooden screeds 75mm wide and of the thickness of the plaster shall be fixed vertically 2.5 to 4 meter. Apart to act as gauge and guide in applying plaster. The mortar shall be laid on the wall between the screeds using the plasters float and pressing the mortar so that packed joints are properly filled. The plaster shall there be finished off with a wooden straight edge reaching across the screeds. The straight edge shall be worked on the screeds with small upward and side ways motion 50mm to 75mm at a time. Finally, the surface shall be finished off with a plasters wooden float. Metal floats shall not be used. Curing shall be commenced as soon as mortar used for finishing has hardened sufficiently and not to be damaged during curing. It shall be kept wet for a period of at least 7 days. |
| 12 | Form work |
|  | 11.1 Form work shall include all temporary form for forming concrete of shape with all props, staging, centring required for support. 11.2 All material shall Conform to relevant I.S. specifications 11.3 Form work shall be constructed with metal or timber, for metal all bolts should be counter sunk. 11.4 The form work should be robust and strong and joint shall be leak proof. Staging must have cross bracing and diagonal bracing in both direction 11.5 No plaster is permitted on the concrete surface. Bottom and side surfaces shall give a uniform in textured smooth surface and good appearance. Concrete having rough nonuniform texture and honey combing in more than 5% area shall be rejected and payment for the form work shall not be made. |

Annexure- E- III

SPECIFICATION OF FHTC

| 1 | The supply from the street main to the individual buildings is made through a house service connection . |
| --- | --- |
| 2 | This consists of two parts viz , the communication pipe which runs from the street main to the boundary of the premises and the service pipe which runs inside the premises . |
| 3 | The service connection including the details of the internal plumbing system should confirm to the National Building Code. |
| 4 | Extreme care should be bestowed for the construction of plumbing system. |
| 5 | The rational design criteria evolved by CBRI for plumbing should be followed. |
| 6 | G.I pipes confirming to BIS specification shall be used for service connections. |
| 7 | The communication pipe is attached to Flow control valve or saddles depending on the material of the distribution main in the street . |
| 8 | Flow control valve of suitable sizes are to be provided for adjacent houses . |
| 9 | The FCV shall made of three Components _> Inlet, Outlet and Ball. |
| 10 | FCV Components shall be machined accouratly to ensoure degined flow. |
| 11 | Inlet fficient, Outlet components and Ball shall be made of SS-316 grande. |
| 12 | The FCE shall be designed to give 5 LPM discharge at 0.5 bar Inlet fficien and should not exceed the flow at the range of 7.0 LPM at 1 bar pressure. |
| 13 | The FCV shall have degisned of very simple passage in line such that the valve will pass small floating particles if any entered in the valve to avoid choking. |
| 14 | The ball inside the valve shall act as Non-Return valve to prevent contamination through back flow of house connction. |
| 12 | FCV shall be marked with an Arrow to show the Flow Direction. |
| 16 | Compression Fiting shall conform with ISO 14236:2000 / ISO 17885:2012 Requirements. |
| 17 | Compression Fitting shall have WRAS approval for their fficient to be used with drinking water. |
| 18 | Copression Fitting shall b PN-16 Rated. |
| 19 | Female Threaded Adaptor and Female Threaded Elbow shall have SS-304 Female insert moulded with the main body. |
| 20 | The Body shall be in Black Colour and Compression Cap shall be in Blue Colour. |
| 21 | Compression Side of the Fitting shall be designed such that a seal in eddected on the internal wall surface of the pipe to esure water in the pipe is precluded form coming into contact with the cut end of the pipe. |
| 22 | The Rubber O-Rings shal be Food Grade NBR/EPDM. |
| 23 | The Manufacturer shall have wll equipped, in-house Test Laboratory to carry out the test as per above mentioned Standards. |
| Technical Specifications : - PP Mechanical Clamp Saddles PP Mechanical Clamp Saddle : PP Mechanical Clamp Saddles shall comply with AS/NZS-4129:2008. These saddles shall have following specifications – |  |

1. PP clamp Saddles shall be contoured to fit around the pipe have a Rubber Grommet/ Rubber Bush/ Rubber Seal and not have to sharp edges that dig in.

2. PP Clamp Saddles with O-ring shall not be used.

3. PP clamp Saddles shall have stopper pins to avoid over tightening of the saddle around the pipe.

4. PP clamp Saddles shall have a Positive Locking arrangement to avoid dislocation of saddle on the pipe due to vibrations or axial loading.

5. PP clamp Saddles shall be supplied in blue Colour.

6.  Saddles shall have female threaded Insert in SS-304 material. The thread type shall be BSP.

7. Saddle shall be supplied with 4 Numbers of SS-304 bolts and nuts.

8. Material of construction of saddles shall be food Graded Polypropylene.

9. material of construction of Rubber Grommet shall be food Graded Polypropylene.

10. . PP clamp Saddles shall be able to accommodate the maximum and minimum tolerances of pipe diameter and ovality.

11. . PP clamp Saddles shall be PN-10 rated & testing shall be done as per AS/NZS 4129:2008.

12. The Manufacturer shall maintain complete traceability of production and testing processes. The record shall be available for review to the visiting officer/ Inspector.

13.  Food Graded Polypropylene shall be used to produce PP clamp saddles.

Annexure- E- IV

SPECIFICATION FOR PUMP WORKS

NRrhlx< “kklu }kjk fy;s x;s fu.kZ; vuqlkj NRrhlx<+ LVsV b.MLVªh;y MsOgyiesUV dkiksZjs”ku ls lkexzh iznk; gsrq nj lafonk vuqcaf/kr iznk; drkZ QeZ ls fufonkdkj dks lkexzh izkIr djuk vfuok;Z gksxk A NRrhlx<+ LVsV b.MLVªh;y MsOgyiesUV dkiksZjs”ku dh lafonk dh lkekU; fu;e “krsZ] rduhdh LislhfQds”ku ,oa nj lafonk ds vk/kkj ij lceflZcy iEi ,oa mlls lacaf/kr daVªksy iSuy ,oa dscy bR;kfn Ø; fd;k tkus dh “krsZ fufonk esa “kkfey fd;k tkuk izLrkfor gS A lceflZcy iEi ds dk;Z ukWu ,l-vks-vkj- vk;Ve ds :i esa lfEefyr djrs gq, lkexzh dh nj ij Hkqxrku fd;k tkosxk ,oa bl ij fufonk dh izkIr nj ykxw ugh gksxhA

ANNEXURE - “E-V”

SPECIFICATION FOR CONSTRUCTION OF R.C.C.

ELEVATED SERVICE RESERVOIR

1.0		SCOPE OF WORK:

1.1		THE WORK INVOLVES:

Construction testing and commission of R.C.C. Elevated Service Reservoir of ……..Kilo Liters Capacity over ……….. mtr. Staging at..........  District Village ……… Block ……….Including providing and fixing of double flanged I.S.I. mark C.I. Pipes Class-B, Horizontally casted confirming to IS 7181-1974 for Inlet, Outlet, scour and overflow delivering up to 3.0 meter away from R.C.C. outer columns of reservoir, C.I. double flanged heavy duty specials and duck foot bends conforming to IS 1238-1976, and D.I. double flanged Soft seated sluice valves  etc. and providing and fixing of all accessories such as lightening arrestor, water level indicator, R.C.C. stairs from ground level to tank floor level or balcony and balcony to roof and aluminum ladder from roof to inside floor level of tank, ventilating cowls/shaft manhole covers with frame and G.I. pipe railing for stairs and at roof flooring etc. all complete as per detailed specification on “job work” basis.

This job shall be executed as per detailed specification given in E-II, E-III &provided design &drawing by department& paid as per payment schedule elaborated in annexure G-2.

1.2		The service Reservoir should be as per approved shape supported over columns.  The depth of water in the tank should not be more than 4.0 meter.  The foundation, columns, bracings etc. should be so designed, so as to have a provision for construction of single story building between the G.I. bracing and first bracing with R.C.C. roof and brick walls for residential office, store accommodation.  The   position of bracing hall be so fixed that the height of floor is approximately 3.5 M. and the top of ground level bracing is 0.6 M. above general ground level. This construction of such building is not included in the turn-key-job.

1.3	INTZE TYPE TANK IS NOT PERMITTED.

2.0	GENERAL

2.1	The work of construction of R.C.C. Over Head Service Reservoir involves specialized workmanship, hence requirements of higher standard than general concrete work is essential. The height of staging will be reckoned from an assumed ground level which shall be the higher of the ground level at the site and road level nearest to the site. i.e. the ground level at the site or road level which ever is higher shall be treated as the base level for determination of staging height.

2.2	The tenderers submitting their offer shall submit the drawing of proposed O.H.S. Reservoir with the tender in envelop “B” of technical offer.  The responsibility for construction structural stability and water tightness will, rest solely with the contractor and he shall have to make good, any damage or loss to the Government due to defect if any, of the above mentioned work.  The contractor shall have to make the tank chlorine resistant as heavy does of chlorination is expected to be given in water

2.3	The offer shall include provision for 1.2 meter wide R.C.C. balcony at bottom slab level ( no cut in the balcony shall be allowed ) railing to balcony and at roof level, lightening arrestor, water level indicator, 1.2 meter wide R.C.C. stairs from ground level to tank fficie(balcony) &0.7 m. wide from balcony to top slab with railing ladders and all pipes and fitting (including puddle collars) up to duck foot bends and from duck foot bend upto 3.0 meters out side the supporting structures i.e. R.C.C. column with D.I.  Soft seated sluice valves etc. including their painting and architectural treatment protection and drainage work and all as mentioned in Annexure “E” special condition.

3.0    The contractor shall be required to conduct proper investigations with full field tests for finding bearing capacity & other details at his own cost.  The investigation reports shall be furnished to the engineer-in-charge for verification and record.  No payment will be made to contractor for carrying out the test or an account of any variation in the soil bearing capacity than indicated and charge in design due to strata.  If in case S.B.C test is conducted by department then the amount for this shall  be deducted from the contractor’s bill. On account of drawing and design of the O.H.T provided by the department as per S.B.C test  report submitted by the contractor, amount of Rs.500 shall be deducted from the contractor’s bill .

4.0	STEEL

4.1 	The contractor shall have to arrange himself the entire quantity of steel required for the completion of the work under contract, no steel shall be supplied by the department No extension of time will be granted by the department for non availability or non procurement of steel in time or late supply of steel or for any other reasons what-so-ever.

4.2		The T.M.T. steel for reinforcement shall conform to IS 1786-1985 amended up to date and test certificate shall be required to be furnished to the department in support thereof.  The stresses in steel for design purposes should be taken as specified in IS Code 3340 (Part-II) 1965 amended up to date

4.3		In additions, the contractor shall be required to get tested the random samples of the steel brought at site to see whether they confirm to relevant I.S. specifications.  The cost of such tests shall be borne by the contractor

5.0		The contractor shall have to make his own arrangement for requirement of electric power and telephone connections for construction work, if they desire these facilities.  The department shall provide assistance by way of recommendation only.

6.0		For blasting operation, if required in excavation of foundation, the contractor will make his own arrangement for license, permits and blasting materials from competent authority.

7.0		The contractor will have made his own arrangement for water, required for execution of the works.  For testing of tanks, the department will provide water at ground level near the tank, however all other arrangement for testing of tank shall have to be made by the contractor at his own cost.

8.0		The tank will have a 1.20 meter wide gallery all around at the level of bottom slab. No cut in the balcony/ Gallery shall be allowed

9.0		The RCC stairs of 1.20 meter width from ground level to the gallery should be provided. At landing stair slab should be supported on two radial beam from column on both ends of landing.

10.		G.I. pipe railing in 3 rows (dia 20mm)  should be provided all around the stair case on both side from ground level to the gallery& on one side from gallery to top slab.

10.1		G.I. Pipe  railing of 20 mm dia, medium class, in three rows shall provided all around the gallery and the roof slab top dome and stairs both side.

11.0		The Railing post should consists of 125 mm dia RCC posts having four nos. T.M.T. bars of 8 mm dia, 1.0 m height @ 1.2 m c/c.  These bars shall be welded with the main reinforcement of RCC stairs, balcony and roof slab.  The RCC post shall be very carefully fixed and embedded into concrete. No cut either for stair case or other purpose in balcony should be allowed.

12.0	An aluminum ladder shall provided from man hole to the inside floor or the tank. The aluminum ladder 600 mm wide shall comprise 2 nos.100x44x3 mm rectangular tube (Aluminum) stringers and 80x38x3 mm rectangular tube foot rest at 300 mm c/c.

13.0	R.C.C. ladder of 400 mm width shall be provided from balconyat bottom slab level to the dom top (top slab).

The fficie shall be of 20 mm dia G.I. pipe in three rows connected by R.C.C. post at 1.2 m c/c (specification same as s.no.11).

14.0	To avoid any unauthorized person to climb the over head tank, the entry at the bottom of the stair case should be closed with suitable arrangements. Closing arrangements should be a cage with gate of 2.1 m height.

12.0	Two C.I. Manhole covers and frame of approved quality 505mmx610 mm internal dimension weighing not less than 38 kg shall be provided.  The weight of cover shall be 23 kg. and weight of frame 12 kg.

16.0	Air vents wherever necessary shall be 120 mm dia swan neck type, with minimum 10 nos. shall be provided or ventilation shaft of suitable diameter or ventilation shade of suitable diameter.

17	To avoid any accident at the time of cleaning or maintenance of the tank, the opening of the out let and scour pipes in side the tank should be covered with Hemispherical stainless steel square bar jail having dimension 10 mm x 10mm, 50 mm c/c spacing and embedded in to slab / dome. The diameter of Hemisphere ffi shall be 3 times of diameter of pipe.

18.0	WORKMANSHIP :

18.1 	EXCAVATION :

The depth of excavation will generally be guided by the underground strata and the safe bearing capacity of the foundation soil and as directed by the Engineer-in-Charge. Strata chart of trial pit section is made available just for guidance for the purpose of preliminary design consideration only. Safe B.C. shall have to be verified by contractor by actual site investigation by conducting suitable tests of under Ground strata/soil at his own cost. Test results of soils, foundation shall be submitted along with the detailed structural design. No payment will be made to the contractor for carrying out these tests or on account of  any variation in the soil bearing capacity and design change due to strata.  No de-watering shall be payable under any circumstances where natural, artificial or manmade.

18.2	FILLING FOUNDATION WITH BED CONCRETE

(Leveling course) :

The foundation shall be laid over bed concrete (i.e.) leveling course) of at least 75 mm thick or more, with at least1:3:6 (M-10) concrete with 50 mm gauge grade or the prescribed mix as per instruction of Engineer-in-charge and as per relevant I.S Code.

18.3	REINFORCED CONCRETE WORK :

It shall be strictly as per Annexure “E-III” special condition.  The concrete mix and minimum cement content specified in Annexure “E-III” shall be rigidly followed.

Where the concrete has not fully hardened all laitance shall be removed by scrubbing the wet surface with wire or bristle brushes, care being taken to avoid dislodgement of particle of aggregate.  The surface shall be thoroughly wetted and all free water removed.

The surface shall then be coated with neat cement grout.  The first layer of concrete to be placed on this surface shall not exceed 12 CM (or 6”) in thickness and shall be rammed against old work, particulars attention being paid to corners and close spots.  Concrete should be thoroughly compacted and fully worked around the reinforcement around embedded fixes and into corner of the form work.

18.4	MEASURING : (concrete Mix proportioning)

The quantity of cement shall be determined by weight Ordinary Portland cement conforming to I.S. 269-1976 shall be taken to weight 50 kg/bag.  The quantities of the and course aggregates shall be determined either by volume or by weight,.  The proportion of find and course aggregate shall be in accordance to Para 8 of IS 506-2000 prior to taking execution work.  The contractor shall make own arrangement of the mixed design of concrete as per I.S : relevant code with respect to material available at site and following the same accordingly.

18.5 MIXING :

Concrete shall be mixed in a mechanical mixer. Mixing shall be continued till there is a uniform distribution of the ingredients and the mass is uniform in color and consistency but in no case the mixing shall be done for less than two minutes.

18.6	TRANSPORTING :

Concrete shall be handled from the place of mixing to the place of final deposit as rapidly as practicable by methods which will prevent segregation or loss of any ingredients and maintaining the required workability.

18.7	PLACING AND COMPACTING :

The concrete shall be placed and compacted before setting could commence and should not be subsequently disturbed Methods of placing should be such that there is no segregation (Concreting) shall be carried out continuously up to construction joints, the position and arrangement of which shall be determined by the designer.  When the work has to be resumed on surface.	Which has hardened, such surface shall be roughened.  It shall then be swept clean, then the roughly wetted and covered with a 12-mm layer of mortar which shall be freshly mixed and placed immediately before the placing of the concrete.

18.8	MECHANICAL VIBRATION

When mechanical vibrations for compacting concrete are used, reduced water content should be adopted.  Over vibration or vibration of very wet mixed is harmful and should be avoided whenever vibration has to be applied externally the design of form work and the disposition of vibrators should receive special consideration to ensure efficient compaction and to avoid surface blemishing.

18.9	CURING :

The concrete shall be covered with a layer of old gunny bags or canvass or similar absorbent material and constantly wet for at least twenty eight days from the date of placing of concrete.

18.10	FORM WORK :

18.10.1 	The from work shall confirm to the shape line sand dimensions as shown on the drawings and so constructed as to remain sufficiently rigid during the placing and compacting of concrete, and shall be sufficiently tight to prevent loss of liquid from concrete.  Only well designed and proper steel form work shall be used.

18.10.2	The form work shall be cleaned off.  All rubbish particularly chippings, shaving and saw dust shall be removed from the interior of the forms before the concrete is placed and the form work in contact with the concrete shall be cleaned and thoroughly wetted or treated with a approved composition.

18.11	STRIPPING OF FORM WORK :

18.11.1	In no circumstance form work should be struck off until the concrete reached the strength of at least twice the stress to which the concrete may be subjected at the time of stripping.

18.11.2	In normal circumstances i.e. temperature above 200C from work may be struck after expiry of the following periods as per IS 506-1978

(A)	Vertical sides of slabs, beams and columns 4 hours.

(B)	Bottom of slabs up to 4.5 M Span		7 days

I	Bottoms of slabs over 4.5 M Span		14 days

(D)	Bottoms of beam over 6 M Span			14 days  and

(E)	Bottom of beam over 6 M Span			21 days

The form work should be left longer, as it would assist the curing.  The number of props, their sizes, and position shall be such as to be able to safely carry the full dead load of the slab, beam or arch as the case may be together with any live load likely to occur during curing of further construction.

19.0	MATERIAL :

19.1.1 STEEL

All metal for reinforcement shall be free from loose mill scale, loose rusts, oil and grease or other harmful matter.  The steel used for reinforcement shall be cleaned immediately before placing the concrete.

19.1.2	PLACING :

All reinforcement shall be placed and maintained in position shown in

the drawing. It is very difficult and costly to alterconcrete once placed. It is, therefore very important to check the reinforcement and its placing before being covered.

19.1.3	SIZE AND QUALITY OF STEEL BARS :

The steel bars used for reinforcement shall be strictly as per relevant I.S. specifications, and the contractor shall have to produce the test certificate of the Steel tobe used.

19.2	AGGREGATES :

All aggregates shall confirm to all provisions and test methods of I.S.383-1940

19.3	STORAGE OF MATERIALS :

Cement shall be stored properly in dry ventilated buildings.

20.0	DESIGN MIX :

20.1 	The contractor shall submit mix designs for each strength the proposed slump proportional weight of cement saturated surface, dry aggregates and water. The mixes shall have to be designed as per relevant I.S. specifications.

20.2	The proportion of the concrete shall be such as such to workability into forms angles and ground the reinforcement without excessive manipulation segregation of water gain.

20.3	The water content shall not be increased from the amount required by the design mix unless cement at required water cement ratio added.  The engineer-in-charge may require additional cement without extra compensation to the contractor if he considers that concrete does not produce the required strength.

21	TEST :

21.1 	All tests as specified in the I.S. specifications codes and required for the execution of the work shall be carried out by the contractor at his cost as per instruction of Engineer-in-charge.

21.2 	FILED TESTS :

The contractor shall provide all arrangements for field tests to exercise proper quality control over works specifically for test mentioned under para.

21.3	All the cast iron puddle collars, bell months, horizontly cast double flanged C.I. pipes Class ‘B’ conforming to I.S. 7181 and C.I.D.F. heavy duty specials Conforming to I.S. 1238-1976 required for inlet, outlet, overflow and scour will be supplied and fixed in position by the contractor from desired inside level of the tank of duck foot bend one meter below G.L. and further providing and fixing all double flanged C.I. pipes and specials from duck foot bend on ward up to 3.00 M. outside the supporting structure of the tank (i.e. R.C.C. column) shall also be supplied and fixed by the contractor including testing of the fittings and joints with cost of the materials and joints.  All the fittings shall be as per I.S. specifications.

The arrangement for inlet, overflow and scour pipes shall be such that all these pipes ends independent of each other and each of these shall have bell mouths at their ends.  The top of bell mounts of inlet and over flow shall be at F.T.L. and top of bell mouths of outlet shall be 12 cm. above floor of the tank, where as the bell mouth of scour pipe shall be flushed with floor level.  All these vertical D.F. pipes shall terminate with flanged duck foot bends bottom fixed one meter below ground level.  Further D.F. pipes should be provided up to 3.0 M. away from the supporting structure of the tank.

All these duck foot bends shall be fixed with heavy class sluice valves of makes IVC/Kirloskar/VAG. R.C.C Valve chambers and chamber covers.

All the pipes and specials required for above shall be fix during concreting. Specials which are to be embedded in concrete shall have Puddle collars at the centre of concrete thickness.

The dimensions of the pipes shall be as under :-

dk;kZy; izeq[k vfHk;ark] yksd LokLF; ;kaf=dh foHkkx] uok jk;iqj vVy uxj] dk i= dzaekd 72@iz-v-@2021 fnukad 19-05-2021 esa gq;s la”kks/ku ds vk/kkj ij ¼i= dh Nk;kizfr layXu½

Note :	The top of the inlet/outlet/scour and overflow pipe should have bell mouth. Bye pass arrangement between inlet pipe and out let pipe should be done 	byCI double flanged pipes and specials.

All CI/DI Pipes, special &D.I. valves should be duly inspected by third party i.e. SGS/DGS&D/RITES.

The tank will have to be tested for the water tight press as per I.S. 3340 and it will be the responsibility of the contractor to make it water tight. Any defects, shrinkage or other faults which may appear within 12 months from the completion of the tank arising out of defective or improper materials or workmanship are upon the direction of the Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI  to be amended

and made good by the contractor (s) at his/their own cost unless the Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI  shall decide that he/they ought to be paid for the same and in case of default the Governor of C.G. may recover from the contractor(s) the cost of making good the works.

The arrangements of water for construction shall be done by the contractor at his own cost.  For testing purposes, the contractor will have  to give a test of water tightness of reservoir to the entire satisfaction of the department. The responsibility of structural stability shall also rest solemnly with the contractor.

22.0	Aluminum Lightening conductor shall be provided & fixed with proper earthling arrangements as per relevant I.S. specifications.

23.0	Water level indicator shall comprise of copper float, guide, pulleys with a pointer on the enamel painted indicator plate which shall be calibrated to read the depth of water in the tank in meters. The level indicator and it’s gauge shall be provided on the front face of one of the column just below the bottom slab.

24.0	 Weather coat or exterior emulsion painting of the tank shall be done only after the tank is successfully tested for water tightness.

25.0	No charges for the plastering if required for proper finishing of the surface of structures shall be paid under any circumstances.

26.0	The contractor shall be required to submit detailed completion drawing in six copies of the work immediately on completion of the work

27.0	The work shall be treated as completed when the same is completely tested and handed over to the department including site clearance.

28.0	The tenderer should give at the time of tendering, SCHEDULE of building materials with  + 5%	likely variation factor of safety that will be adopted for separate major components of complete structure.

29.0	The tenderer shall have to produce the complete design and working drawing of each component of structure before starting of work.

30.0	Special considerations should be given in the preparation of design for seismic forces if the structure being located in some mining area.

31.0	The structural design of the R.C.C. elevated service Reservoir shall be done on continuity theory and rotation of joints.

32.0	Due consideration for wind pressure should be taken in design of structure, as per provision of relevant I.S. Specification.  The wind pressure may be taken as per the wind pressure map of India annexed in I.S.875-1964 with special consideration to the previous experience of wind pressure experienced in the locality

33.0	All duck foot bends shall be fixed and anchored properly in concrete blocks, to prevent any displacement of pipe due to water trust.

34.0	The M.S. clamps 50mmx10mm should be provided at 3.0 M. C/C for clamping all the vertical pipes, the clamps should be either fixed in column’s bracing with nut bolts.

35.0	The contractor may note that the department may not be in a position to acquire or make available sufficient land (space) for collection storage of construction materials and equipment’s and working. It shall therefore be contractor’s responsibility to make arrangement of land space for this purpose.

36.0	In case there is any delay in handing over of land to the department by the concerned competent authority the contractor shall only be allowed extension of time or this account.  No compensation for financial claim shall be accepted.

37.0	If under unavoidable circumstances or for reasons beyond control of the department, the proposed site of construction of tank	is required to the changed/shifted, the contractor shall have to take up construction at alternative site, and the contractor shall not have any claim on this account.

38.0	The R.C.C. elevated tank shall be designed, executed and tested in accordance to relevant I.S. specifications.

39.0 	The contractor has to display the details of the construction i.e. name of the project, cost of work, starting date & time for completion. After completion of the work, display board it is to be rewritten stating date of completion. The top height of the sign-board shall be 2.1 meter above the road level and the size shall be 1.8m x 1.2m made of suitable MS pipe. The sign-board shall be placed at the entrance point of the site. The board drawing has be get approved with detailed drawing & design of the structure. In addition to above, in case of Overhead tanks, the display has to be painted on vertical wall of tank & it should be on the highest point of the structure. In case of Intake well & Treatment Plant the suitable displays are to be provided on there main components additionally.

Enclosure :-	The special conditions of annexure “E” shall form a part of the agreement. The tenderers should read these special conditions carefully.

50.0   Providing and applying epoxy paint of approved make to concrete surface of RCC ESR or GSR 	including cleaning the surface by scrapping and air blowers to the satisfaction of Engineer- in- charge, 	necessary scaffolding etc. complete with all elads and lifts and giving satisfactory  hydraulic test for 	water tightness as  per relevant I.S codes.

|  | Executive Engineer and member secretary, DWSM, MOHLA-MANPUR-A.CHOWKI , (C.G.) |
| --- | --- |

Special conditions

ANNEXURE –“E”- VI

CONSTRUCTION OF R.C.C. ELEVATED RESERVOIRS

Public health Engineering Department constructs a large number of elevated   service reservoirs in rural areas as, well as in urban areas.  Many cases of failure of these structures have come to the notice of the department.  To minimize  failures, it is extremely important that a uniform policy with respect to design and construction specifications of these structures is followed throughout the State so that the quality of construction is ensured.  Keeping in view the above objective,  the following instructions are issued for strict compliance in future.  The authorities responsible for enforcing these instructions will be the respective DWSM, MOHLA-MANPUR-A.CHOWKI of the district and member secretary, DWSM, MOHLA-MANPUR-A.CHOWKI s-in-charge of the works.

GENERAL INSTRUCTIONS:

1.0	Elevated Service Reservoirs supported on concrete design shall see that tanks are supported on column structure.

1.1	The R.C.C. columns supporting the tank on round shape.  The top shell i.e. water container, in circular shape as per approved drawing

1.2		On the bottom floor of the water container cast iron or stainless steel grate should be provided over the supply outlet and scour outlet.  This is essential to prevent any accident occurred at Shore when two persons in succession were sucked into the scour pipe, the top of which could not be seen because of calcium  deposits due to regular use of bleaching powder. Cast iron grates 20x20 mm or stainless steel square 20x20 mm can be used with square frames on top of the outlet.

1.3		The over flow outlets should not be connected to the distribution system,  Connection of over flow pipe to the distribution system can result in over

1.4		It is extremely important to make arrangements for supply of sufficient water at the construction site for curing of the concrete.  Continuous and efficient curing is extremely import for development of good compressing strength in any concrete structure.

1.5		It is advisable to use metal derived from igneous rock preferably of basaltic of granite origin.  The coarse sand should be free from soil.  This can be checked easily by half filling a transparent glass with the sand sample and the other half by clean water.  Stir the sand vigorously.  Silt in the sand can then be easily seen in the top water portion.

1.6	It is extremely essential the contractors undertaking the work should have a             concrete mixer with them.  No hand mixing should be allowed.

1.7	In no case the concrete should be laid without vibration.  It is desirable to keep two concrete vibrators at the construction site so that in case of a break down the other vibrator can be used.  It is desirable that the contractor have with them at least two concrete vibrators, which is an essential T & P for laying concrete.

1.8	Intz type over head tank shall not be Permitted.

1.9 	Nominal mix as per IS:506-2000 with upto date amendment shall be permitted for the construction work. Cube test  at the age of 3 , 7 and  28 days of curing is mandatory to be done. Test results should be as per standard value as given in IS 506-2000. If there is any deviation in the test results , then  further decision will be taken by engineer-in charge .

FOLLOWING SPECIFICATIONS SHOULD BE STRICTLY FOLLOWED

2.0	CEMENT AND CONCRETE :

2.1	Minimum Strength of Concrete :

Minimum strength of concrete for components of elevated tank will be below :

Columns staging			-	M 25	(250 Kg/sqcm)

Tank including roof			-	M 30 (300 Kg/sqcm)

  - Minimum Cement Content :

From durability considerations minimum cement content shall be as below :-

Concrete,					-	M25 – 350-Kg/cum

Concrete						-	M30	- 500-kg/cum

  - Cover of concrete :

The minimum cover shall be 50 mm for all the reinforcement.  For foundation

this cover shall be 60 mm.

  - Cement Grade :

Grade 43 cement should be used for columns and grade 53 for the tank portion including the top dome cover.  Only fresh cement should be used in the tank.  It is advisable to use cement manufactured by major plants and reputed firms.  OPC or blast furnace slag cement should be used.

  - Water Cement Ratio :

Water Cement Ratio shall not be more than 0.50 This means 22.5 liters of water per 50 Kg. Bag cement.

  - Testing Machines For Concrete :

Compression testing machines should be installed in each rural and project          divisions.

  - Use  of Construction Chemicals :-

When the water cement ratio is less, the strength and durability of concrete is good.  It is a advisable to use plasticizers in concrete and reduce water cement ration up to 0.4. Plasticizers manufactured by reputed companies are                        recommended.

Proportion of plasticizer to be used shall be as per the instructions manual                          supplied by the manufacturers.

  - Construction joints :

Construction joints be treated in accordance with IS-506.  The surface of already laid concrete be cleaned by water jet and cement slurry be applied.  Cement  mortar 10 mm.  Thick of the same proportion as in concrete be applied and then fresh concrete of the lift be laid.  The form work must overlay 100 mm on the already  laid concrete.

3.0		Minimum Dimensions And shapes :  As per approved drawing and design.

Minimum Dimensions shall be as below :

Circular Column			500 mm

Tank wall				200 mm

Bottom slab				120 mm

Top slab 				125 mm

Note :	If a Dome is provided at the top, the thickness can be limited to 100 mm.			Rectangular/square columns are not allowed Circular shafts are also not 		allowed.  In respect of horizontal braces, corners shall be chamfered by 50x50 mm

4.0	STEEL :

4.1	Minimum steel :  Design requirements as set out in relevant codes in respect of steel be fully satisfied.  However, following minimum steel should be provided. As per approved design & drawing.

(a)	Vertical steel in columns		:	0.8% of cross sectional area 								actually required and 									0.3% when larger

section than actually

required is provided.

(b)	Horizontal link in columns		:	Not less than 8 mm. Dia at 								200 mm c/c or 10 mm dia not 								more than 300 mm c/c.

I	Exposed R.C.C. surface		:	On both faces when 									thickness is 120 mm or 									more.2 Kg/sqm in 										perpendicular direction. The 								above requirement is 									satisfied if. 8 mm bars @ 200 								mm c/c OR 10 mm bars @ 								300 mm c/c are provided.

(d)	Steel in tank						As per provision of I.S. 3340 									subject to minimum as set 										out in I above.

4.2	Maximum spacing of reinforcement :

Maximum spacing of main reinforcement in slab or walls shall not more than 120 mm centre to centre.  The spacing of secondary bars, such as distribution steel or vertical bars in columns.

4.3	Type of Steel :

The steel for reinforcement shall be ISI mark. Thermo mechanically treated bars confirming to relevant I.S.S. .

Detailing of Steel

Before commencing the work, Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI  in-charge should study the            drawing.  It must be insisted that the designer provided details of the shape of each bar its diameter, length and numbers of each category in a schedule of reinforcement.  This must be incorporated in every working drawing.

  - Form Work :

Wherever in type of tanks are adopted or where there is an inclined slab having inclination steeper than one vertical to three horizontals, inside form work must be provided.

(a) Place outer form work.

(b) Place inner form work 300 to 500 mm wide.  Then concrete that portion then fix next strip and concrete.

I In respect of top or bottom domes place upper form work for lowest circular string of width equal to ½th dia of dome and concrete as stated above.  Then concrete central half for which top form may not provide.

6.0	Protection work Drainage:

At the ground level, stone or 100mm thick P.C.C of M-10 pavement should be provided for an area which is 1.5 meter more than the dimensions of tank on all sides.  This should be laid in a slope of 1:60 from the centre and drain be constructed around for outlet of water.

Important Point for Tenderer Related to Works

(Summary of tender document)

| 1. | Security Deposit | : | The Security deposit shall be deducted as per clause 1 of form A given in NIT |
| --- | --- | --- | --- |
| 2. | Mobilization Advance | : | The mobilization advance shall not be paid. |
| 3. | Water for testing, trial run and construction | : | The water for testing and trial run of pipe line will be made available by the Deptt. Free of cost. In case of delay in making available the water for testing the extension in time will be granted to the contractor without penalty for such period. The water for construction shall be arranged by the contractor at his cost. |
| 4. | Price escalation | : | The price variation condition shall not be applicable in this contract. |
| 5. | Right of way for laying of pipe line and constructions | : | The deptt. Shall provide hindrances free alignment and required space for laying of pipe lines and construction of allied works. The deptt. Shall pay the land acquisition charges, if required. |
| 6. | Permissions from other Department | : | The deptt. Shall obtain the permissions for Railway crossing, Telephone line crossing, road cuttings, road crossings from the concerning authority. In case the shifting of Electric Pole is found necessary, the same shall be got shifted by the deptt. Through CSPHCL. The service connections shall not be removed, the contractor will have to lay the pipe lines without any damage to the service lines. In case the shifting of existing water pipe lines/sewage lines is considered necessary, the same shall be shifted by the contractor for which the payment shall be made for actual work done as per tender rate. In case there is damages to service lines, the same shall be restored by the contractor or it will be got restored through concerning agency and the cost will be recovered from the contractor’s bills. In case there is delay in obtaining the permission from other deptt. Including local and private agencies, the extension in time shall be granted, but no claim shall be entertained on this account. No revised rate shall be given for any item of work given in schedule of quantity and applicable SORs. The deptt. Will not provide the underground utility drawings for water lines, electric lines, telephone cables and other utility services. The contractor should see the location of these utility services at site prior to submitting the revised offer. |
| 7. | G.S.T. | : | Proof of registration in G.S.T. |
| 8. | Deviation in Length, dia of pipe line and pipe fittings etc. | : | The deviation in length of pipe line, dia of pipe line and pipe fittings etc. shall be paid/deducted on the approved tender rates As per class 13 of form-A given in the NIT. |
| 9. | Secured Advance | : | The Secured advance against the security of materials brought at site shall be sanctioned up to an amount not exceeding 75% of the value but 90% in case of steel (assessed by the Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI ) provided the material are of imperishable nature and a formal agreement is drawn up with the contractor under which Government secures a lien on the material and is safe guarded against losses due to the contractor postponing the execution of the work or to the shortage or misuse of the materials, and against the expense entitled for their proper watch and safe custody. The recoveries of advance shall be made from the running account bills, for work done as the materials are used, the necessary deductions being made whenever the item of work in which they are used are billed for. The contractor shall sign the prescribed indenture bond in the prescribed form. The provisions of clause 11 “B” of form “A” given in NIT and CPWA code para 10.2.22 (a) shall apply for the sanction of secured advance. Recovery should be done strictly in accordance with provision made in C.G. finance code Vol. II item 68 for “Grant of advance to contractor on the security of materials”. |
| 10. | Location of valves & Nallah crossing | : | The Plan showing the location of Distribution pipe lines, Nallah crossing, road crossing is already enclosed in the compact disk (CD) issued with tender documents. Sluice valves, air valves and scour valves shall be provided at location as per guide lines given in CPHEEO Manual 1999 |
| 11. | Period of completion | : | Time is the essence of this contract & the period of completion is 03 months including rainy season as given in NIT. |
| 12 | ### Changes as per site conditions | : | No separate payments shall be made on account of changes in site condition. The payments shall be made as per condition of contract. |
| 13. | ### Inspection of pipes, valves & DI Specials |  | All pipes, valves and specials shall be ISI mark duly inspected by DGS & D/SGS/RITES. All UPVC pipes and special shall be duly inspected by CIPET. “Providing & fixing of following Ductile iron double flanged sluice valves glandless, resilient (soft seated) non-rising spindle with body bonnet of ductile cast iron of grade GGG 40/SGI 400/12 or equivalent grade or of higher tensile strength grade, as per IS: 3896 part-II-1986 and subsequent revision, wedge fully rubber lined with EPDM food grade quality and seals of NBR. The valve should be with replaceable nut and replaceable sliding shoes valve sten, shall be of single piece thread rolled. Sluice valve shall be compitable for buried applications without valve chambers. The valve should be vacuum tight and 100% leak proof with face to face dimensions as BS: 5163-89/IS: 14846/2000/DIN 3204 F4 and flange connections as per IS: 1538. Valve should be with electrostatic power coating both inside and outside (thickness 250 micron) with pocket less straight thro body passage including jointing and testing with cost of jointing material such as bolts, nuts, rubber insertions etc. all complete.” dk izko/kku fd;k tkuk izLrkfor gS A |
| 14. | ### Taxes | : | The rate quoted by the contractor shall be deemed to be inclusive of the sales and other levies, duties, royalties, cess, toll, taxes of Central and State Governments including GST (Goods and Services Tax), taxes of local Bodies and Authorities that the Contractor wii have to pay for the performance of this Contract. The Government will perform such duties in regard to the deduction of such taxes at source as per applicable law. However, if any other New Tax (not increase or decrease in existing all taxes, other levies, duties, royalties, cess, toll) is levied on the Contractor (under the agreement) either by Central Govt. or State Govt. then the Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI shall reimburse the “New Tax” amount; on submission of proof of such payment by the contractor. |
| 12. | ### Break up schedule of payment | : | This is a percentage rate tender and the intermediate payment shall be paid as per clause 7 and 8 of form “A” given in the NIT.after inpection by TPI & approval by DWSM, MOHLA-MANPUR-A.CHOWKI . |
| 16. | ### Change in Technical Specification | : | It shall be as per clause -13 of form A of NIT |
| 17. | Submission of completion drawing with hard & soft copies in three sets. | : | Contractor will submit progress of the work every month along with drawing showing diameter & center line alignment of completed pipe line with reference to boundary of houses on each road incorporating all necessary details & land-mark in 3 hard copies accompanied with soft copy. |
| 18. | Arrangement of 50 mm dia G.I. Pipe for service connection | : | Contractor has to erect a 50mm (1.5”) dia GI stand pipe (medium class) at every 100 m interval on specified pipe no. of distribution pipe lines up to road level with socket & cap. |
| 19. | Restoration of roads | : | The concrete road shall be restored as per norms & good construction practice using similar mix and specification as adopted by construction agency i.e. Nagar nigam or PWD. |
| 20. | Rates of non scheduled items of work and rates for RCC chambers | : | The rates of non schedule items of work shall be decided as per provision given in S.No. 2.5 of Appendix 2.10 and clause 13 of form “A” given in NIT. SOR Rates :- The SOR rates should be seen by the contractor in SOR’s made applicable in this contract before submitting the revised offer. |
| 21. | ### Back filling & testing of pipe line/joints | : | The back filling of trenches as per IS 12288-1987 specifications shall be done as early as possible after completion of laying of pipe lines. The joints shall not be kept opened for inspection. However in Road crossing the back filling shall be done by totally dry or fully saturated sand for which payment shall be made for actual work done as per instruction of Engineer-in –Charge. |
| 22. | ### Construction of valve chamber | : | Construction of valve chamber and providing and fixing of valve should be done simultaneously along with the work of laying of pipe line as per CPHEEO Manual 1999. |
| 23. | ### Space required for storage of excavated soils, pipes etc. | : | The deptt. Shall not provide the space for storage of pipes, excavated soils and valves. It will be the responsibility of the contractor to arrange the space for storage of such materials. |
| 24. | ### Issues of Pipes, valves and specials. | : | The pipes, valves and specials shall be issued to contractor as per enclosed Annexure H (b). The labour rate shall be given for laying and fixing of these items. The transportation charges for these materials will not be payable to the contractor. |
| 25 | ### Bottom width of trench | : | The width of the trench at bottom shall be such as to provide not less than 200 mm clearance on either side of the pipe except where rock excavation is involved. |
| 26 | ### Insurance | : | Insurance on total contract price for the period of contract shall be done by the contractor. |
| 27 | Procurement of Pipe, Valve | : | Deptt. Shall make payment for the quantities of fitting and other material executed, measured and valued as per the conditions of contract. The contractor should procure the materials after making assessment of the quantities of materials actually required as per site condition. Deptt shall not be responsible for extra quantities brought at site by the contractor. |
| 28 | Quantities & specification of valves and D I specials | : | Approximate quantities of DI resilient seated sluice valves & DI fitting shall be as given in schedule of quantity in NIT. DI resilient seated sluice valve shall be allowed of makes approved by E-in-C, PHED, CG up to the date of calling notice inviting tender. |
| 29 | Excavation in Concrete Road : | : | All excavation in concrete road shall be done compulsorily by using blade type concrete cutter or needle cutter (needle size not more than 32 mm dia) for cutting of concrete portion. |
| 30 | Interest Claim | : | No interest claim shall be entertained in this contract because of any reason. |
| 31 | IEC and Training |  | Contrctor should arrange for IEC work and training to VWSC members/gram panchayat members for operation and maintainance of piped water supply scheme. |

(As per letter no. 5277/TS/CE/PHED/2012 dated 14.11.2013 of Chief Engineer, P.H.E.D. Raipur Zone Raipur)

The contractor has to ensure that at least 25% off all the wages paid to laboures shall be made through digital mode Like Bhim/UPI in the respective bank account of the laboures. CG govt. Finance Department Circular no. 22/2018, No. 72/finance/budget 2/2018, New Raipur, Dated 17.04.2018

|  |  |  | Executive Engineer and member secretary, DWSM, MOHLA-MANPUR-A.CHOWKI |
| --- | --- | --- | --- |

ANNEXURE “F”

Schedule of Quantity

| RISK & COST FOR PIPED WATER SUPPLY SCHEME |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AT VILLAGE -MARKATOLA(GP) .GP-MARKATOLA BLOCK -MOHLA, DISTRICT – MOHLA-MANPUR-A.CHOWKI |  |  |  |  |  |  |  |
| Schedule of Quantity Providing, Laying & Jointing of UPVC Pipe line, OHT, Boundary wall, Chlorinator Room. Chlorinator System, Switch Room, Pump, FHTCs, Pumping Main. |  |  |  |  |  |  |  |
| S. No | SUB HEAD |  |  | QTY |  | UNIT |  |
| 1 | 2 |  |  | 3 |  | 5 |  |
| Schedule NO.- 01 |  |  |  |  |  |  |  |
| RISING MAIN |  |  |  |  |  |  |  |
| 1 | Earth work in excavation for pipe trench in ordinary soil areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 1.08 |  | cum. |  |
|  | CG,PHE, SOR Item no. 18.15 P/225 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 6 | x 0.6 x0.30 = | 1.08 |  |  |  |  |
|  |  | Total | 1.08 |  |  |  |  |
| 2 | Earth work in excavation for pipe trench in Hard soil areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 1.08 |  | cum. |  |
|  | CG,PHE, SOR Item no.18.16 P/226 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 6 | x 0.60 x 0.30 = | 1.08 |  |  |  |  |
|  |  | Total | 1.08 |  |  |  |  |
| 3 | Earth work in excavation for pipe trench in Laterite soil areas including dressing, watering, ramming and disposal of excavated earth lead up to 50m and lift up to 1.5m, disposal earth to be leveled, neatly dressed. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.18.17 P/226 |  |  | 1.08 |  | cum. |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 6 | x 0.60 x 0.3 = | 1.08 |  |  |  |  |
|  |  | Total | 1.08 |  |  |  |  |
| 4 | Earth work in excavation for pipe trench in all of rocks in areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 0.72 |  | cum. |  |
|  | Soft rock with or without blasting or bituminous pavement / cement concrete road. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.18.19.1 P/226 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 6 | x 0.60 x 0.2 = | 0.72 |  |  |  |  |
|  |  | Total | 0.72 |  |  |  |  |
| 5 | Demolishing cement concrete manually / by mechanical means including disposal of material within 50 m lead as per direction of engineer-in-charge. |  |  | 6.00 |  | cum. |  |
|  | Nominal concrete 1:3:6 or richer mix (i/c equivalent design mix) |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.18.29.1 P/227 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 50 | x 0.6 x0.20 = | 6.00 |  |  |  |  |
|  |  | Total | 6.00 |  |  |  |  |
| 6 | Providing, laying and jointing following P.V.C. pipes with solvent cement joint for 6, 8 and 10 kg/ sq. cm. pressures including testing of joints, cost of jointing materials etc. complete in all respect. |  |  | 6.00 |  | mtr. |  |
|  | CG,PHE, SOR Item no. 7.1 P/108 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 10 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 6 |  |  |  |  |
|  |  | Total | 6.00 |  |  |  |  |
| 7 | Providing, Solvent Cement Joints to PVC Pipes and fittings of 6, 8 and 10 Kg/Sq cm. Pressure including testing of joints and cost of jointing materials (i.e. socket, coupler & solvent cement) |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.3 P/109 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 10 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 10 |  |  |  |  |
|  |  | Total | 10.00 | 10 |  | each |  |
| 8 | Providing and laying in position including testing following PVC bends suitable for 6, 8 and 10 Kg/Sq. cm. pressure pipes. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.5 P/109 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 10 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 2 |  |  |  |  |
|  |  | Total | 2.00 | 2 |  | each |  |
| 9 | Providing and laying in position including testing following PVC flanged tail pieces suitable for 6, 8 and 10 Kg./Sq. cm. Pressure pipes. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.7 P/110 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 10 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 2 |  |  |  |  |
|  |  | Total | 2.00 | 2 |  | each |  |
| 10 | Providing and laying in position including testing PVC coupler suitable for 6, 8 and 10 Kg/Sq. cm. Pressure pipes. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.9 P/110 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 10 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 3 |  |  |  |  |
|  |  | Total | 3.00 | 3 |  | each |  |
| 11 | Filling available excavated earth in trenches plinth sides of foundation is layers not exceeding 20 cm. In depth i/c consolidation of earth layer by ramming watering lead upto 50 m and lift upto 1.5 mt. I/c all kind of soils. |  |  | 3.96 |  | cum. |  |
|  | CG,PHE, SOR Item no.18.22 P/227 |  |  |  |  |  |  |
|  | Marakatola |  | 3.96 |  |  |  |  |
|  |  | Total | 3.96 |  |  |  |  |
| 12 | Providing and laying mechanically mixed cement concrete with 20 mm nominal size graded stone aggregat. C M-15 |  |  | 6.00 |  | cum. |  |
|  | CG,PHE, SOR Item no. 18.41 P/229 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 50 | x 0.6 x0.20 = | 6.00 |  |  |  |  |
|  |  | Total | 6.00 |  |  |  |  |
| 13 | Providing & fixing following cast iron double flanged single door reflux (non return) valves including jointing & testing with cost of jointing material such as bolts,nuts and rubber insertion all complete as per IS :5312 (Part I) Class PN-1.0 |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no 8.4a P/117 |  |  |  |  |  |  |
|  | 50 mm dia |  |  |  |  |  |  |
|  | Marakatola |  | 2 |  |  |  |  |
|  |  | Total | 2.00 | 2.00 |  | each |  |
| 14 | Provision for Jointing of TW to Rising Main with cost of Material/ specials such as GI Union / CI Flange,GI Reducer UPVC MTA FTA etc. as per requiement of site i/c cost of labour etc. complete as per approved specification and as directed by Engineer in charge. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no 19.19 P/246 |  |  |  |  |  |  |
|  | Marakatola |  | 1 |  |  |  |  |
|  |  | Total | 1.00 | 1.00 |  | Job |  |
| Schedule NO.- 02 |  |  |  |  |  |  |  |
| DISTRIBUTION NETWORK |  |  |  |  |  |  |  |
| S. No | SUB HEAD |  |  | QTY |  | UNIT |  |
| 1 | 2 |  |  | 3 |  | 5 |  |
| 1 | Earth work in excavation for pipe trench in ordinary soil areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 4.14 |  | cum. |  |
|  | CG,PHE, SOR Item no. 18.15 P/225 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 23 | x 0.60 x0.3 = | 4.14 |  |  |  |  |
|  |  | Total | 4.14 |  |  |  |  |
| 2 | Earth work in excavation for pipe trench in Hard soil areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 4.14 |  | cum. |  |
|  | CG,PHE, SOR Item no.18.16 P/226 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 23 | x 0.60 x 0.30 = | 4.14 |  |  |  |  |
|  |  | Total | 4.14 |  |  |  |  |
| 3 | Earth work in excavation for pipe trench in Laterite soil areas including dressing, watering, ramming and disposal of excavated earth lead up to 50m and lift up to 1.5m, disposal earth to be leveled, neatly dressed. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.18.17 P/226 |  |  | 4.14 |  | cum. |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 23 | x 0.60 x 0.30 = | 4.14 |  |  |  |  |
|  |  | Total | 4.14 |  |  |  |  |
| 4 | Earth work in excavation for pipe trench in all of rocks in areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 2.76 |  | cum. |  |
|  | Soft rock with or without blasting or bituminous pavement / cement concrete road. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.18.19.1 P/226 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 23 | x 0.60 x 0.20 = | 2.76 |  |  |  |  |
|  |  | Total | 2.76 |  |  |  |  |
| 5 | Demolishing cement concrete manually / by mechanical means including disposal of material within 50 m lead as per direction of engineer-in-charge. |  |  | 48.00 |  | cum. |  |
|  | Nominal concrete 1:3:6 or richer mix (i/c equivalent design mix) |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.18.29.1 P/227 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 400 | x 0.60 x 0.2 = | 48.00 |  |  |  |  |
|  |  | Total | 48.00 |  |  |  |  |
| 6 | Providing, laying and jointing following P.V.C. pipes with solvent cement joint for 6, 8 and 10 kg/ sq. cm. pressures including testing of joints, cost of jointing materials etc. complete in all respect. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.1 P/108 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 0.00 |  |  |  |  |
|  |  | Total | 0.00 | 0.0 |  | mtr. |  |
|  | 110 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 23.00 |  |  |  |  |
|  |  | Total | 23.00 | 23.0 |  | mtr. |  |
| 8 | Providing, Solvent Cement Joints to PVC Pipes and fittings of 6, 8 and 10 Kg/Sq cm. Pressure including testing of joints and cost of jointing materials (i.e. socket, coupler & solvent cement) |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.3 P/109 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 10.0 |  |  |  |  |
|  |  | Total | 10.00 | 10 |  | each |  |
|  | 110 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 5.0 |  |  |  |  |
|  |  | Total | 5.00 | 5 |  | each |  |
| 9 | Providing and laying in position including testing following PVC bends suitable for 6, 8 and 10 Kg/Sq. cm. pressure pipes. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.5 P/109 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 6.0 |  |  |  |  |
|  |  | Total | 6.00 | 6 |  | each |  |
|  | 110 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 3.0 |  |  |  |  |
|  |  | Total | 3.00 | 3 |  | each |  |
| 10 | Providing and laying in position including testing following PVC Tees, suitable for 6, 8 and 10 Kg/Sqm. Pressure pipes. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.6 P/110 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 5.0 |  |  |  |  |
|  |  | Total | 5.00 | 5 |  | each |  |
|  | 110 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 2.0 |  |  |  |  |
|  |  | Total | 2.00 | 2 |  | each |  |
| 20 | Providing & laying in position including testing following cast iron plain ended standard specials confirming to IS/5531/1988 (Reaffirmed 2002). |  |  |  |  |  |  |
|  | Cast Iron Adopter (Flange Spigot) |  |  |  |  |  |  |
|  | 80 mm dia |  |  |  |  |  |  |
|  | Marakatola |  | 4.0 |  |  |  |  |
|  |  | Total | 4.00 | 4 |  | each |  |
| 11 | Providing and laying in position including testing following PVC flanged tail pieces suitable for 6, 8 and 10 Kg./Sq. cm. Pressure pipes. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.7 P/110 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 4.0 |  |  |  |  |
|  |  | Total | 4.00 | 4 |  | each |  |
|  | 110 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 2.0 |  |  |  |  |
|  |  | Total | 2.00 | 2 |  | each |  |
| 12 | Providing and laying in position including testing PVC coupler suitable for 6, 8 and 10 Kg/Sq. cm. Pressure pipes. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.9 P/110 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 3.0 |  |  |  |  |
|  |  | Total | 3.00 | 3 |  | each |  |
|  | 110 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 1.0 |  |  |  |  |
|  |  | Total | 1.00 | 1 |  | each |  |
| 13 | Providing and laying in position including testing following PVC end Cap (plugs) suitable for 6, 8 and 10 Kg/Sq cm. Pressure pipes. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.8 P/110 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 6.0 |  |  |  |  |
|  |  | Total | 6.00 | 6 |  | each |  |
| 14 | Providing and fixing PVC Djoint (Detachable joint) in PVC pipe line suitable for classes up to 10kg/sqcm Pressure pipes. i/c nut bolt, cutting of pipe, testing of joints etc complete. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.7.12 P/111 |  |  |  |  |  |  |
|  | 90 mm Dia |  |  |  |  |  |  |
|  | Marakatola |  | 3 |  |  |  |  |
|  |  | Total:- | 3.0 | 3 |  | Mtr |  |
| 22 | Providing laying and jointing of following galvanized Iron (MS) Pipes with specials (such as bends, elbows, tees etc) class light, medium & heavy including testing of joints, cost of pipes, specials and jointing materials all complete. Pipes and sockets conforming to IS:1239/2011 Part- II. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 6.1 P/89 |  |  |  |  |  |  |
|  | 100 mm dia class mudiam (For culvert crossing) |  |  |  |  |  |  |
|  | Marakatola |  | 50 |  |  |  |  |
|  |  | Total | 50.0 | 50 |  | RM |  |
| 23 | Filling available excavated earth in trenches plinth sides of foundation is layers not exceeding 20 cm. In depth i/c consolidation of earth layer by ramming watering lead upto 50 m and lift upto 1.5 mt. I/c all kind of soils. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.18.22 P/227 |  |  |  |  |  |  |
|  | Qty As per Item No. 1 to 4 |  |  |  |  |  |  |
|  | Marakatola |  | 15.18 |  |  |  |  |
|  |  | Total | 15.18 | 15.18 |  | cum. |  |
| 24 | Providing and laying mechanically mixed cement concrete with 20 mm nominal size graded stone aggregat. C M-15 |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 18.41 P/229 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 400 | x 0.6 x 0.20 = | 48 |  |  |  |  |
|  |  | Total | 48.00 | 48.00 |  | cum. |  |
| 26 | Providing & fixing of following Ductile iron double flanged sluice valves as per I.S.:14846-2000 fitted with cap including jointing & testing with cost of jointing material such as bolts, nuts, rubber insertions etc. all complete. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no8.4.41 P/73 |  |  |  |  |  |  |
|  | 80 mm dia |  |  |  |  |  |  |
|  | Marakatola |  | 3 |  |  |  |  |
|  |  | Total | 3.00 | 3.00 |  | each |  |
|  | 100 mm dia |  |  |  |  |  |  |
|  | Marakatola |  | 1 |  |  |  |  |
|  |  | Total | 1.00 | 1.00 |  | each |  |
| 27 | CONSTRUCTION OF RCC SLUICE VALVE CHAMBER SIZE 1.0mX1.0mX1.0m |  |  |  |  |  |  |
|  | Marakatola |  | 4 |  |  |  |  |
|  |  | Total | 4.00 | 4.00 |  | As per Sub Estimate |  |
| 29 | Provision for jointing of Rising main to supmp well/OHT and OHT to Distribution pipe line with cost of material/specials such as Bends, MTA as per rerquirement of site i/c cost of labour with excavation, labour as per requirement complete as per approved specification and as directed by Engineer in charge. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no 19.20 P/246 |  |  |  |  |  |  |
|  | Marakatola |  | 1.00 |  |  |  |  |
|  |  | Total | 1.00 | 1.00 |  | Job |  |
| Schedule NO.- 03 |  |  |  |  |  |  |  |
|  | FHTCs (Funtional Household Tap Connection |  |  |  |  |  |  |
| 1 | Providing and constructing one stand post as per type design with excavation 15 cm thick PCC 1:3:6 bedding 20 mm thick PCC 1:2:4 convert for platform of 1.5mx1.5m with side curb and bucket rest of 80 mm dia. 160mm dia PVC pipe central post duly filled therein with C.C. 1:2:4, 2.2m long, 15 mm dia medium G.I. pipe. From point of tapping to stand post additional 1620 (20 mm dia) composite pipe 6.0 m long. Providing and fixing of 15 mm dia, one steel water tap, one flow control valve SS, 5 LPM complete together with all labour and material charges as per drawing and as directed by Engineer-in-charge when good foundation in available. Rate includes draining arrangement by excavating open gutters complete |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No.253/19.35 |  |  |  |  |  |  |
|  | Marakatola |  | 1 |  |  |  |  |
|  |  | Total | 1.00 | 1.00 |  | Each |  |
| 2 | Earth work in excavation for pipe trench in ordinary soil in areas including dressing, watering and ramming & disposal of excavated earth lead up to 50 m- & lift up to 1.5 m. disposal earth to be leveled, neatly dressed |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No.226/18.19 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 20 | x0.25x0.15 | 0.750 |  |  |  |  |
|  |  | Total | 0.75 | 0.75 |  | cum. |  |
| 3 | Earth work in excavation for pipe trench in all kinds of rocks in areas including dressing, stacking of useful material and disposal of unserviceable one up to 50 m lead and lift up to 1.5 m. (a) Soft rock with or without blasting or bituminous pavement /cement concrete road. |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No.226/18.19 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 20 | x0.25x0.15 | 0.750 |  |  |  |  |
|  |  | Total | 0.75 | 0.75 |  | cum. |  |
| 4 | Demolishing cement concrete manually / by mechanical means including disposal of material within 50 m lead as per direction of engineer-in-charge. |  |  |  |  |  |  |
|  | Nominal concrete 1:3:6 or richer mix (i/c equivalent design mix) |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.18.29.1 P/227 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 5 | x0.25x0.25 | 0.313 |  |  |  |  |
|  |  | Total | 0.31 | 0.31 |  | cum. |  |
| 5 | Providing and fixing Polyethylene- Aluminum Polyethylene (PE-AL-PE) Composite Pressure pipes Conforming to IS: 15450-2004 U.V. Stablished with carbon black having thermal stability for hot & cold water supply, capable to with stand temperature up to 800 C including all specials and fittiings of composite material (engineering plastic. gland and brass insert wherever required) e.g. elbws, tees. reducers, couplers and connectors wiith clamp at 1m spacing. This includes testing of joints complete as per the directions of engineer- incharge (Concealed work including cutting chases& making good the wall etc ) |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No.254/19.37.2 |  |  |  |  |  |  |
|  | 1620 ( 20mm OD ) Pipe |  |  |  |  |  |  |
|  | Marakatola |  | 700.00 |  |  |  |  |
|  |  | Total | 700.00 | 700 |  | Rm |  |
| 6 | Filling available excavated earth in trenches, lead up to 50m and lift up to 1.5m in all kind of soil excluding watering and ramming. |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No. 227/18/23 |  |  |  |  |  |  |
|  | (B) in Hard Soil for Road |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 20 | x0.25x0.30 | 1.500 |  |  |  |  |
|  |  | Total | 1.50 | 1.50 |  | cum. |  |
| 7 | Providing & laying mechanically mixed cement concrete 20mm nominal size graded crushed stone excluding cost of centering & shuttering. |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No. 229/18/41.1.1 |  |  |  |  |  |  |
|  | In Plinth & foundation |  |  |  |  |  |  |
|  | For Cutting 1:3:6 (M-10) |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 5 | x0.25x0.25 | 0.313 |  |  |  |  |
|  |  | Total | 0.31 | 0.31 |  | cum. |  |
| Schedule NO.- 04 |  |  |  |  |  |  |  |
| Providing and installation of Submersible pump |  |  |  |  |  |  |  |
| 1 | Supplying & Installation of Energy efficiant five star BEE rating ISI Marked required capacity of Three Phase, 50 Hz, 415V, deep well submersible pump Steel body, suitable for 6"tube well with Control Panel Starter suitable for Submersible pump with dry run protection, single phase preventer, connections, including clamps, bore cap etc. as required as per specifications but excluding pipe and connection cable. |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No. 354/34.20.106 (iv) |  |  |  |  |  |  |
|  | 5 H.P. with 10to11stages,HeadMt.101-40 Discharge LPM 60-270 |  |  |  |  |  |  |
|  | Marakatola |  | 1.00 |  |  |  |  |
|  |  | Total | 1.00 | 1 |  | each |  |
| 2 | Providing laying and jointing of following galvanized Iron (MS) Pipes with specials (such as bends, elbows, tees etc) class light, medium & heavy including testing of joints, cost of pipes, specials and jointing materials all complete. Pipes and sockets conforming to IS:1239/2011 Part-II. |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No. 89/6.1 |  |  |  |  |  |  |
|  | 50mm Dia |  |  |  |  |  |  |
|  | Marakatola |  | 50.00 |  |  |  |  |
|  |  | Total | 50.00 | 50 |  | RM |  |
| 3 | Supplying and laying of submersible flat cable ISImarked 3 core copper wire of suitable size with proper clamping of approved make. |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No. 354/34.20.107 (ii) |  |  |  |  |  |  |
|  | 4.0 Sq.mm.multi strand |  |  |  |  |  |  |
|  | Marakatola |  | 100.00 |  |  |  |  |
|  |  | Total | 100.00 | 100 |  | Per meter |  |
| 4 | Supplying and laying of approved make stainless steel wire rope 6 mm thick complete with binding for support of pump and motor |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No. 354/ 34.20.109 |  |  |  |  |  |  |
|  | Marakatola |  | 100.00 |  |  |  |  |
|  |  | Total | 100.00 | 100 |  | Per meter |  |
| Schedule NO.- 05 |  |  |  |  |  |  |  |
| RCC OVER HAED TANK (MARKATOLA) |  |  |  |  |  |  |  |
| S. No | SOR Re. | SUB HEAD |  | QTY |  | UNIT |  |
| 1 | 2 | 3 |  | 4 |  | 6 |  |
| RCC OVER HAED TANK |  |  |  |  |  |  |  |
| 1 | Designing (structurally & aesthetically), and constructing RCC elevated service reservoirs of following capacity with RCC staging consisting of columns, internal and external bracings spaced vertically as per staging of the ESR. including excavation in all types of strata, foundation concrete, cement plaster with water proofing compound to the inside face of the container including refilling & disposing off the surplus stuff within a lead of 50 meters, all labour and material charges including lowering, laying, erecting, hoisting and jointing of pipe assembly of inlet, outlet, scour, overflow and bypass arrangements as per departmental design, providing and fixing accessories such as Aluminum Ladder, C.I. manhole frame and covers water level indicators, lightening conductor, G.I. pipe railing around walk way and top slab, providing RCC staircase from ground level to balcony level along with columns and from balcony level roof top level along with container wall, M.S.grill gate of 2 mtr. height with locking arrangement of approved design RCC chambers for all valves, ventilating shafts, providing and applying three coats of weather coat paints to the structure including roof slab epoxy painting to internal surface & anti termite treatment for underground parts of the structure and giving satisfactory water tightness test as per I.S. code, The job to include painting the name of the scheme and other details on the reservoir as per thedirections of Engineer-in-Charge. |  |  |  |  |  |  |
|  | Construction of R.C.C. over head tank 40 KL capacity on 12 meter staging including drawing, design, soil SBC testing with all vertical pipe fitting and specials. |  |  |  |  |  |  |
|  | CGPHESOR Page no. 295 Item no 14.20 (23.2.3) |  |  |  |  |  |  |
|  | R.C.C. over head tank 25 KL cost |  |  | 1 |  | Each |  |
|  | Add for 15000 Liters |  |  | 15000 |  | LTR |  |
|  | 40 KL 12 mtr Staging |  |  |  |  |  |  |
| Schedule NO.- 06 |  |  |  |  |  |  |  |
| SWITCH ROOM |  |  |  |  |  |  |  |
| S.No | Item |  |  | Qty. |  | Unit |  |
| 1 | Excavation for all types and sizes of foundations, trenches and drains or for any other purpose including disposal of excavated stuff upto 1.5 m lift and lead upto 50m (at least 5m away from the excavated area), including dressing and leveling of pits. |  |  |  |  |  |  |
|  | In all types of soils. |  |  | 1.68 |  | Cum |  |
| 2 | Excavation for all types and sizes of foundations, trenches and drains or for any other purpose including disposal of excavated stuff upto 1.5 m lift and lead upto 50m (at least 5m away from the excavated area), including dressing and leveling of pits. |  |  |  |  |  |  |
|  | In ordinary rocks. |  |  | 1.68 |  | Cum |  |
| 3 | Providing and laying nominal mix plain cement concrete with crushed stone aggregate using concrete mixer in all works upto plinth level excluding cost of form work. |  |  |  |  |  |  |
|  | 1:3:6 ( 1 cement : 3 coares sand :6 graded stone aggregate 40 mm nominal size). (Base cone). M.10 - 1:3:6 |  |  | 0.72 |  | Cum. |  |
| 4 | Providing and placing in position reinforcement for R.C.C. work including straightening, cutting, bending, binding etc. complete as per drawings including cost of binding wire in foundation and plinth all complete: |  |  |  |  |  |  |
|  | Cold twisted bar / Hot rolled deformed steel |  |  | 50 |  | Kg. |  |
| 5 | Providing and laying design mix reinforced cement concrete with crushed graded stone aggregate 20mm nominal size using batching plant, transit mixer and concrete pump, in all works upto plinth level excluding cost of form work |  |  |  |  |  |  |
|  | M- 20 Grade |  |  | 0.542 |  | Cum |  |
| 6 | Providing and fixing form work including centring, shuttering, strutting, staging, propping bracing etc. complete and including its removal at all levels, for: |  |  |  |  |  |  |
|  | (i) Beams, lintels, cantilevers & walls |  |  | 4.92 |  | Sqm. |  |
| 7 | Brick work with modular fly-ash lime bricks (FaLG bricks) confirming to IS:12894-2002 of class designation 4.0 in foundation and plinth in : |  |  |  |  |  |  |
|  | (a) Cement Mortar 1:6 (1 Cement : 6 Coarse Sand) |  |  | 5.15 |  | Cum |  |
| 8 | Extra for brick work in superstructure above plinth level for every floor or part thereof in addition to rate for foundation and plinth: |  |  | 2.25 |  | cum |  |
| 9 | Providing and laying damp proof course (upto 50mm thick) with plain cement concrete 1:2:4 (1 cement : 2 coarse sand : 4 graded crushed stone aggregate 20mm nominal size) including form work. |  |  | 0.084 |  | Cum |  |
| 10 | Providing and filling in plinth with sand/ Crusher dust and hard moorum under floor in layers not exceeding 20cm in depth consolidating each deposited layer by ramming and watering, including dressing etc. complete. |  |  | 0.43 |  | Cum. |  |
| 11 | Providing and making 6mm thick cement plaster of mix: |  |  |  |  |  |  |
|  | (In Cement Mortar 1:6 (1 cement : 6 fine sand) |  |  | 23.3 |  | Cum |  |
| 12 | Structural steel work in single section including cutting, hoisting, fixing in position and applying a priming coat of red oxide zinc chromate primer.(Door frame) |  |  |  |  |  |  |
|  | (b) In Flat Iron. |  |  | 25 |  | kg. |  |
| 13 | Providing and making 15mm thick cement plaster on the rough side of single or half brick wall of mix: |  |  |  |  |  |  |
|  | (In Cement Mortar 1:6 (1 cement : 6 fine sand) |  |  | 9.4 |  | Sqm. |  |
| 14 | Painting new work ( two or more coats) to give even shade with |  |  |  |  |  |  |
|  | Premium synthetic enamel paint |  |  | 4.23 |  | sqm |  |
| 15 | Providing and fixing steel door/ window with M.S. sheet 1mm thick, frame of angle iron, diagonal braces of angle/ flat iron of suitable size, 3.00 mm M.S. gusset plates at junctions and corners, all necessary fittings complete including applying a priming coat of red oxide zinc chromate |  |  | 60 |  | Kg. |  |
|  | primer. |  |  |  |  |  |  |
| 16 | Cement concrete flooring with cement concrete 1:2:4 (1 cement : 2 coarse sand : 4 graded stone aggregate 20mm) finished with a floating coat of neat cement. |  |  |  |  |  |  |
|  | 50 mm thick |  |  | 1.44 |  | sqm. |  |
| 17 | Providing and laying design mix reinforced cement concrete with crushed graded stone aggregate 20mm nominal size using batching plant, transit mixer and concrete pump, in all works upto plinth level excluding cost of form work. |  |  |  |  |  |  |
|  | M-20 Grade |  |  | 0.121 |  | Cum |  |
| 18 | Distempering with acrylic washable distemper to give an even shade, On new work (Two or more coats) |  |  | 36 |  | Sqm. |  |
| 19 | Provision for electrical fittings. |  |  | L.S. |  |  |  |
|  | Marakatola |  | 1.00 |  |  |  |  |
|  |  | Total | 1.00 | 1.00 |  | each |  |
| Schedule NO.- 07 |  |  |  |  |  |  |  |
| BOUNDARY WALL |  |  |  |  |  |  |  |
| 1 | Excavation for all types and sizes of foundations, trenches and drains or for any other purpose including disposal of excavated stuff. upto 1.5 m lift and lead upto 50m (at least 5m away from the excavated area), including dressing and leveling of pits |  |  |  |  |  |  |
|  | (PWD SOR, .No. 1.1.1/P-9) |  |  |  |  |  |  |
|  | 15X0.90X0.90X0.90 = 10.94 |  |  | 10.94 |  | Cum |  |
| 2 | Providing and laying nominal mix plain cement concrete with crushed stone aggregate using concrete mixer in all works upto plinth level excluding cost of form work. |  |  |  |  |  |  |
|  | (PWD SOR, .No. 3.1.3/P-23) |  |  |  |  |  |  |
|  | 15X0.90X0.90X0.10 = 1.22 |  |  | 1.22 |  | Cum. |  |
| 3 | Providing and placing in position cold twisted steel and hot rolled deformed steel reinforcement for R.C.C. work including cutting, bending, binding etc. complete as per drawings including cost of binding wire and including all wastages etc. complete. |  |  |  |  |  |  |
|  | (PWD SOR, .No. 3.12.1/P-24) |  |  |  |  |  |  |
|  | 100mm dia tar steel @ 0.15m. c/c |  |  |  |  |  |  |
|  | Jalli - 2no.X 2 X6X0.80X0.62 = 160.70kg |  |  |  |  |  |  |
|  | Column -27no.X4X3.20X0.62 = 214.27kg |  |  |  |  |  |  |
|  | Stirrups - 27no.X17no.X0.70X0.398 =127.87kg |  |  |  |  |  |  |
|  | Beam – 2X78.30X0.62 = 97.09 |  |  |  |  |  |  |
|  | 2X79.30X0.89 = 141.15 |  |  |  |  |  |  |
|  | 2X78.30X0.89 = 139.37g |  |  |  |  |  |  |
|  | Stirrups – 522X 0.90X0.398 = 186.98kg |  |  |  |  |  |  |
|  | 1067.43kg |  |  | 1067 |  | Kg. |  |
| 4 | Providing and fixing formwork i/c centering, shuttering, strutting, staging, propping, bracing etc. complete and including its removal at all levels, |  |  |  |  |  |  |
|  | Foundations, footings bases of columns plinth beam, curtain wall in any shape and size and all type of wall below plinth level. |  |  |  |  |  |  |
|  | (PWD SOR, .No. 2.1.1/P-16 & 2.1.5/ P-16) |  |  |  |  |  |  |
|  | Foundations-27X4X0.90X0.10 = 9.72 m2 |  |  |  |  |  |  |
|  | 27X4X0.85X0.30 = 27.54 |  |  |  |  |  |  |
|  | 27X4X0.20X0.25 = 05.40 |  |  |  |  |  |  |
|  | Beam – 2X78.30X0.30 = 46.98 |  |  |  |  |  |  |
|  | 89.64 |  |  | 89.64 |  | Sqm. |  |
|  | colunm – 27X4X0.20X2.10 = 45.36 |  |  | 45.36 |  | Sqm. |  |
| 5 | Providing and laying nominal mix reinforced cement concrete with crushed stone aggrigat using concrete mixer in all works upto plinth level excluding from work. 1:1:1.5:3 cement concrete |  |  |  |  |  |  |
|  | (PWD SOR, .No. 3.2.1/P-23 |  |  |  |  |  |  |
|  | 27X0.85X0.85X0.30 = 5.852 m3 |  |  |  |  |  |  |
|  | 27X[(0.202+0.852) /2] X0.25m. = 2.573 m3 |  |  |  |  |  |  |
|  | 27X0.20X0.20X2.35 = 2.538m3 |  |  |  |  |  |  |
|  | Beam – 1X 78.30X0.20X0.30 = 4.698 |  |  |  |  |  |  |
|  | 15.661 |  |  | 15.66 |  | Cum |  |
| 6 | Brick work with modular fly-ash lime bricks (FaLG bricks) confirming to IS:12894-2002 of class designation 4.0 in foundation and plinth in : |  |  |  |  |  |  |
|  | (PWD SOR, .No. 7.5.4/P-45 |  |  |  |  |  |  |
|  | Cement Mortar 1:6 (1cement:6 sand) |  |  |  |  |  |  |
|  | 1no.X73.30X0.20X1.80m 26.388 |  |  | 26.39 |  | cum |  |
| 7 | Extra for brick work in superstructure above plinith level for every floor or part theireof in addition to rate for foundation plinth: |  |  | 26.39 |  | cum |  |
|  | (PWD SOR, .No. 7.6/P-45 |  |  |  |  |  |  |
| 8 | 15mm thick cement plaster on the rough side of single or half brick wall of mix: |  |  |  |  |  |  |
|  | (PWD SOR, .No. 11.3.4/P-103 |  |  |  |  |  |  |
|  | Cement Mortar 1:6 (1cement:6 sand) |  |  |  |  |  |  |
|  | 2X73.30X1.80 = 263.88 |  |  | 263.9 |  | cum |  |
| 9 | Finishing walls with water proofing cement paint of required shade to give an even shade. |  |  |  |  |  |  |
|  | (PWD SOR, .No. 14.14.1/P-133 |  |  |  |  |  |  |
|  | In side- 81.40X2.10 = 170.94 |  |  |  |  |  |  |
|  | Outer -83.00X2.10 = 174.30 |  |  |  |  |  |  |
|  | Swith room – = 69.76 Total 415 M2 |  |  | 415 |  | Sqm. |  |
| 10 | Steel work welded in built up sections/ramed work including cutting, hoistiong, fixing position and applying a priming coat of red oxide zinc chromate primer. |  |  |  |  |  |  |
|  | (PWD SOR, .No. 9.6.1/P-80 |  |  |  |  |  |  |
|  | Steel gate – approximate weight – 200kg |  |  | 200 |  | kg |  |
| 11 | Painting on new work (two or more coats) to give an even shade with: |  |  |  |  |  |  |
|  | (PWD SOR, .No. 14.9.1/P-132 |  |  |  |  |  |  |
|  | Sating synthetic enamel paint (top most approved branded quality) |  |  |  |  |  |  |
|  | 2X5.0X2.0 = 20.00sqm. |  |  | 20 |  | sqm. |  |
| 12 | Providing and filling in plinth with sand/Crusher dust and hard moorum under floor in laryers not exceeding 20cm in depth consolidating each deposited layer by ramming and watering, including dressing etc. complete. |  |  |  |  |  |  |
|  | (PWD SOR, .No. 1.18/P-11 |  |  |  |  |  |  |
|  | 21.60X21.60 = 466.56 |  |  |  |  |  |  |
|  | (-) 3.50X3.50 = 12.65 |  |  |  |  |  |  |
|  | (-) 0.785X6.50 = 33.16 420.75X0.30 =130.50 |  |  |  |  |  |  |
|  | 19.683-(2.187+5.852+2.573+0.270) =8.80 |  |  |  |  |  |  |
|  | 139.3 |  |  | 139.3 |  | cum. |  |
| 13 | Providing and fixing broken glass 100 mm high in spacing not more than 40 mm both ways and laid in 50 mm thick (average) cement mortar 1:4 (1 cement : 4 coarse sand ) over compound walls, prapet walls and the like. |  |  | 18.65 |  | sqm |  |
|  | (PWD SOR, .No. 9.38/P-84 |  |  |  |  |  |  |
|  | Marakatola |  | 1.00 |  |  |  |  |
|  |  | Total | 1.00 | 1.00 |  | each |  |
| Schedule NO.- 08 |  |  |  |  |  |  |  |
| CHLORINATOR SYSTEM |  |  |  |  |  |  |  |
| 1 | Providing, erecting, commissioning and giving test & trial for a period of one month including one year free maintenance after commissioning of Electro chlorinator capable of generating chlorine from common salt by electrolysis using electrodes in form of sodium hypo chlorite solution containing 6-8 gms/lit of available chlorine in batch or continuous process and capable of providing 8 hrs storage of hypochlorite in case of power failure. The electro chlorinator shall comprise of following: ● Electrolytic cell consisting dimensionally stable electrodes made from Gr I Titanium sheet with multi metal oxide coating. Electrolyzer tank made from PVC-FRP or Acrylic ● Power pack consisting of transformer rectifier for generating suitable DC current from AC supply along with the control switch for dosing pumps etc. through MCB's contacts, relays and wiring. ● Control panel for the electro chlorinator consisting of DC voltage and current display income phase status unit on-off switches fuses etc. ● Dosing tank of suitable capacity made from PVC/FRP. ● Dosing pumps of specials quality (1W+1S) suitable to handle hypo chlorite solution. ● Entire chlorine solution pipeline shall be of PVC. ● Chlorine test kit suitable to measure residual chlorine up to 5 ppm. |  |  |  |  |  |  |
|  | 24 gms/hr |  |  | 1job |  | each |  |
|  | Marakatola |  | 1.00 |  |  |  |  |
|  |  | Total | 1.00 | 1.00 |  | each |  |
| Schedule NO.- 09 |  |  |  |  |  |  |  |
| CHLORINATOR ROOM |  |  |  |  |  |  |  |
| 1 | Excavation for all types and sizes of foundations, trenches and drains or for any other purpose including disposal of excavated stuff upto 1.5 m lift and lead upto 50m (at least 5m away from the excavated area), incl. dressing & leveling of pits. |  |  |  |  |  |  |
|  | In all types of soil |  |  |  |  |  |  |
|  | Steps -- |  |  |  | 7.13 |  | Cum. |
| 2 | Providing and filling in plinth with sand/ Crusher dust and hard moorum under floor in layers not exceeding 20cm in depth consolidating each deposited layer by ramming and watering, including dressing etc. complete. |  |  |  |  |  |  |
|  | Code -- 1.18 , Page -11 |  |  |  | 1.44 |  | Cum. |
| 3 | Providing and laying nominal mix cement concrete with crushed stone aggregate using concrete mixer in foundation, plinth and at ground level excl. cost of form work. |  |  |  |  |  |  |
|  | HALL -- |  |  |  | 0.68 |  | Cum. |
| 4 | Providing and fixing formwork including centering, shuttering, strutting, staging, propping bracing etc. complete and including its removal at all levels, for: |  |  |  |  |  |  |
|  | Foundations, footings, base of columns and plinth beam in any shape and size. |  |  |  |  |  |  |
|  | Ground beam Sides --- |  |  |  | 7.92 |  | Sqm. |
| 5 | Columns, Pillars, Piers & likes- rectangular |  |  |  |  |  |  |
|  | or square in shape |  |  |  | 12.64 |  | Sqm |
|  | Code -- 2.1.5 , Page -- 16 . |  |  |  |  |  |  |
| 6 | Suspended floors, roofs, access platform, |  |  |  |  |  |  |
|  | balconies (plain surfaces) and shelves |  |  |  |  |  |  |
|  | (cast in situ) Code -- 2.1.8 , Page -- 15 . |  |  |  |  |  |  |
|  | Lintal Beam Side - |  |  |  | 3.84 |  |  |
|  | Chhajja -- |  |  |  | 1.62 |  |  |
|  | MAIN SLAB -- |  |  |  | 7.29 |  |  |
|  |  |  |  |  | 12.75 |  | Sqm. |
| 7 | Providing and laying nominal mix reinforcement cement concrete with crushed stone aggregate using concrete mixer in all works upto floor five level excl. cost of reinforcement and form work. |  |  |  |  |  |  |
|  | 1:1½:3 (1 cement : 1½ coarse sand : 3 graded stone aggregate 20mm nominal size). |  |  |  |  |  |  |
|  | Code -- 3.2.1, Page -- 23 . |  |  |  | 0.49 |  |  |
|  | 4x(0.9+0.30)/2x0.45 =1.57 cum. |  |  |  | 1.57 |  |  |
|  | 4x0.2x0.2x0.50= 0.08 cum |  |  |  | 0.08 |  |  |
|  | Ground Beam |  |  |  | 0.58 |  |  |
|  | Lintel |  |  |  | 0.38 |  |  |
|  | Chhajja -- |  |  |  | 0.13 |  |  |
|  | Main Slab |  |  |  | 0.58 |  |  |
|  |  |  |  |  | 3.80 |  | Cum |
| 8 | Providing and placing in position reinforcement for R.C.C. work incl. straightening, cutting, bending, binding etc. complete as per drawings incl.cost of binding wireall complete: |  |  |  |  |  |  |
|  | Cold twisted bar / Hot rolled deformed steel |  |  |  |  |  |  |
|  | Code -- 3.12.4 , Page -- 24 . |  |  |  |  |  |  |
|  | Taken 80Kg per Cum. Of R.C.C. Qty. i.e. |  |  |  | 304.13 |  | kg |
| 9 | Brick work with modular fly-ash lime bricks (FALG Bricks) confirming to IS:12894-2002 of class designation 40 in found & plinth in: |  |  |  |  |  |  |
|  | Cement Mortar 1:5 (1 cement : 5 coarse sand) |  |  |  |  |  |  |
|  | Code -- 7.5.3 , Page -- 45 . |  |  |  | 0.33 |  | Cum. |
| 10 | Brick work with modular fly-ash lime bricks (FaLG Bricks) confirming to IS:12894-2002 of class designation 4.0 in foundation and plinth in: |  |  |  |  |  |  |
|  | Extra for brick work in superstructure above plinth level for every floor or part thereof in addition to rate for foundation and plinth:Cement Mortar 1:6 (1 cement : 6 coarse sand) |  |  |  |  |  |  |
|  | Code -- 7.6, Page -- 45 . |  |  |  |  |  |  |
|  | Deduction as openings --- D |  |  |  | 0.38 |  |  |
|  | Window -- W -- |  |  |  | 0.43 |  |  |
|  |  |  |  |  | 0.81 |  |  |
|  | After Deduction Net Brick work Qty. = |  |  |  | 4.95 |  | cum |
| 11 | Providing and laying damp proof course (upto 50mm thick) with plain cement concrete 1:2:4 (1 cement : 2 coarse sand : 4 graded crushed stone aggregate 20mm nominal size) including formwork. |  |  |  |  |  |  |
|  | Code -- 3.13 , Page -- 24 . |  |  |  | 0.10 |  |  |
|  |  |  |  |  | 0.10 |  | cum |
| 12 | Cement concrete flooring with cement concrete 1:2:4 (1 cement : 2 coarse sand : 4 graded stone aggregate 20mm) finished with a floating coat of neat cement.. 50mm thick |  |  |  |  |  |  |
|  | Code --12.3.2 Page -- 111 . |  |  |  | 5.76 |  | Sqm |
| 13 | 6mm thick cement plaster of mix:In |  |  |  |  |  |  |
|  | Cement mortar 1:3 (1 cement : 3 fine sand) |  |  |  |  |  |  |
|  |  |  |  |  | 9.00 |  | Sqm. |
| 14 | 12mm thick cement plaster of mix: |  |  |  |  |  |  |
|  | (d) In Cement mortar. 1:5 . |  |  |  |  |  |  |
|  | ( 1 Cement : 5 sand ) |  |  |  |  |  |  |
|  | Code -- 11.2.3 , Page -- 103 . |  |  |  |  |  |  |
|  | Net Qty. of inner walls plaster work |  |  |  | 24.75 |  | Sqm |
| 15 | 15mm Th. Cement plaster on the rough |  |  |  |  |  |  |
|  | side of single or half brick wall --- |  |  |  |  |  |  |
|  | in cement mortar 1:5 ( 1 cement : 5 sand ) |  |  |  |  |  |  |
|  | Code -- 11.3.3 , Page -- 108 . |  |  |  |  |  |  |
|  | Net Qty. of outer walls plaster work |  |  |  | 34.03 |  | Sqm |
| 16 | Providing and fixing steel door/window with M.S. sheet 1mm thick, frame and angle iron diagonal braces including applying a priming coat of red oxide zinc chromate primer. |  |  |  |  |  |  |
|  |  |  |  |  | 4.05 |  |  |
|  | Taken 120kg. i.e. |  |  |  | 120.00 |  | kg |
| 17 | Applying one coat of cement primer on wall surface (applied @ 0.80 litrs/10 sqm) complete. |  |  |  |  |  |  |
|  | Code -- 14.12 , Page -- 133. |  |  |  |  |  |  |
|  | Qty. Same as item No. -- 12+13+14 |  |  |  | 67.78 |  | Sqm. |
| 18 | Wall painting with acrylic premium emulsion (plastic) paint of required shade to give an even shade. |  |  |  |  |  |  |
|  | On new work (Two or more coats) Qty. Same as item No. -- 12+13+14 |  |  |  |  |  |  |
|  | Code -- 14.10.1 , Page -- 133 . |  |  |  | 67.78 |  | Sqm |
| 19 | Painting on new work (two or more coats) |  |  |  |  |  |  |
|  | to give an even shade with: |  |  |  |  |  |  |
|  | Satin synthetic enamel paint (top most |  |  |  |  |  |  |
|  | approved branded quality) |  |  |  |  |  |  |
|  | Code -- 14.22.1 , Page -- 134 . |  |  |  | 4.05 |  | Sqm |
|  | Marakatola |  | 1.00 |  |  |  |  |
|  |  | Total | 1.00 |  | 1.00 |  | each |
| Schedule NO.- 10 |  |  |  |  |  |  |  |
| ELECTRIC CONNECTION |  |  |  |  |  |  |  |
| 1 | Providing and stretching 3 phase transmission line by Cspdcl and making electrical connection for tube wells and pump house for pumping machinery including all necessary charges, Cspdcl demand charges etc. 1 nos. for submersible pump. |  |  |  |  | - |  |

| AT VILLAGE - DHANEGAON BLOCK -MOHLA, DISTRICT – MOHLA-MANPUR-A.CHOWKI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Schedule of Quantity Providing, Laying & Jointing of HDPE pipe & FHTC |  |  |  |  |  |
| Schedule NO.- 01 |  |  |  |  |  |
| DISTRIBUTION NETWORK |  |  |  |  |  |
| S. No | SUB HEAD |  |  | QTY | UNIT |
| 1 | 2 |  |  | 3 | 5 |
| 1 | Earth work in excavation for pipe trench in ordinary soil areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 1.80 | cum. |
|  | CG,PHE, SOR Item no. 18.15 P/225 |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 10 | x 0.60 x0.3 = | 1.80 |  |  |
|  |  | Total | 1.80 |  |  |
| 2 | Earth work in excavation for pipe trench in Hard soil areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 1.80 | cum. |
|  | CG,PHE, SOR Item no.18.16 P/226 |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 10 | x 0.60 x 0.30 = | 1.80 |  |  |
|  |  | Total | 1.80 |  |  |
| 3 | Earth work in excavation for pipe trench in Laterite soil areas including dressing, watering, ramming and disposal of excavated earth lead up to 50m and lift up to 1.5m, disposal earth to be leveled, neatly dressed. |  |  |  |  |
|  | CG,PHE, SOR Item no.18.17 P/226 |  |  | 1.80 | cum. |
|  | Dhanegaon |  |  |  |  |
|  | 10 | x 0.60 x 0.30 = | 1.80 |  |  |
|  |  | Total | 1.80 |  |  |
| 4 | Earth work in excavation for pipe trench in all of rocks in areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 1.20 | cum. |
|  | Soft rock with or without blasting or bituminous pavement / cement concrete road. |  |  |  |  |
|  | CG,PHE, SOR Item no.18.19.1 P/226 |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 10 | x 0.60 x 0.20 = | 1.20 |  |  |
|  |  | Total | 1.20 |  |  |
| 5 | Demolishing cement concrete manually / by mechanical means including disposal of material within 50 m lead as per direction of engineer-in-charge. |  |  | 6.00 | cum. |
|  | Nominal concrete 1:3:6 or richer mix (i/c equivalent design mix) |  |  |  |  |
|  | CG,PHE, SOR Item no.18.29.1 P/227 |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 50 | x 0.60 x 0.2 = | 6.00 |  |  |
|  |  | Total | 6.00 |  |  |
| 7 | Providing, Laying, Jointing & field testing of HDPE pipes (High Density Polyethylene Pipes) confirming to IS 4984/14151/12786/13488 with necessary jointing material like mechanical connector of jointing pipes by heating to the ends of pipes with the help of Teflon coated electric mirror/ heater to the required temperature and then pressing the ends together against each other, to form a monolithic & leak proof joint by thermosetting process. It may be required to be done with Jack/ Hydraulic Jacks/ Butt fusion machine. (50 mm & above fusion jointed & below 50mm mechanical jointed) |  |  |  |  |
|  | PHE SOR Page no. 253/19.35 |  |  |  |  |
|  | 63 mm dia HDPE 6 kg./cm2 |  |  |  |  |
|  | Dhanegaon |  | 8 |  |  |
|  |  | Total:- | 8.0 | 8 | Mtr |
|  | 75 mm dia HDPE 6 kg./cm2 |  |  |  |  |
|  | Dhanegaon |  | 2 |  |  |
|  |  | Total:- | 2.0 | 2 | Mtr |
| 15 | Providing and laying including testing Bend 90̊ confirming to specification. |  |  |  |  |
|  | Phe Sor Page no. 126/ 9.2 |  |  |  |  |
|  | 63 mm dia |  |  |  |  |
|  | Dhanegaon |  | 6 |  |  |
|  |  | Total:- | 6.0 | 6 | Mtr |
|  | 75 mm dia |  |  |  |  |
|  | Dhanegaon |  | 5 |  |  |
|  |  | Total:- | 5.0 | 5 | Mtr |
| 16 | Providing and laying including testing Equal Tee confirming to IS specification. |  |  |  |  |
|  | Phe SorPage no. 127/9.4 |  |  |  |  |
|  | 63 mm dia |  |  |  |  |
|  | Dhanegaon |  | 4 |  |  |
|  |  | Total:- | 4.0 | 4 | Mtr |
|  | 75 mm dia |  |  |  |  |
|  | Dhanegaon |  | 4 |  |  |
|  |  | Total:- | 4.0 | 4 | Mtr |
| 17 | Providing and laying including testing Reducer: confirming to IS specifications. |  |  |  |  |
|  | Phe Sor Page no. 128/9.6 |  |  |  |  |
|  | 75 mm dia |  |  |  |  |
|  | Dhanegaon |  | 3 |  |  |
|  |  | Total:- | 3.0 | 3 | Mtr |
| 18 | Providing butt fusion welded joint/joining by heating to the ends with the help of Teflon coated electric mirror/heater ends together etc. by thermosetting processes to HDPE Pipe and specials. (6 kg. 8 kg. 10 kg.) (50 mm & above fusion jointed & below 50 mm mechanical jointed) including testing |  |  |  |  |
|  | Phe Sor Page no. 128/9.7 |  |  |  |  |
|  | 63 mm dia |  |  |  |  |
|  | Dhanegaon |  | 6 |  |  |
|  |  | Total:- | 6.0 | 6 | Mtr |
|  | 75 mm dia |  |  |  |  |
|  | Dhanegaon |  | 5 |  |  |
|  |  | Total:- | 5.0 | 5 | Mtr |
| 19 | Providing and laying including testing End Cap confirming to IS specifications. |  |  |  |  |
|  | Phe SorPage no. 128/9.8 |  |  |  |  |
|  | 63 mm dia |  |  |  |  |
|  | Dhanegaon |  | 5 |  |  |
|  |  | Total:- | 5.0 | 5 | Mtr |
| 20 | Providing & fixing of following Cast iron double flanged sluice valves as per I.S.:14846-2000 fitted with cast iron cap including jointing & testing with cost of jointing material such as bolts, nuts, rubber insertions etc. all complete |  |  |  |  |
|  | 50mm dia |  |  |  |  |
|  | Dhanegaon |  | 2 |  |  |
|  |  | Total:- | 2.0 | 2 | Mtr |
|  | 65mm dia |  |  |  |  |
|  | Dhanegaon |  | 1 |  |  |
|  |  | Total:- | 1.0 | 1 | Mtr |
| 22 | Providing laying and jointing of following galvanized Iron (MS) Pipes with specials (such as bends, elbows, tees etc) class light, medium & heavy including testing of joints, cost of pipes, specials and jointing materials all complete. Pipes and sockets conforming to IS:1239/2011 Part- II. |  |  |  |  |
|  | CG,PHE, SOR Item no. 6.1 P/89 |  |  |  |  |
|  | 100 mm dia class mudiam (For culvert crossing) |  |  |  |  |
|  | Dhanegaon |  | 50 |  |  |
|  |  | Total | 50.0 | 50 | RM |
| 23 | Filling available excavated earth in trenches plinth sides of foundation is layers not exceeding 20 cm. In depth i/c consolidation of earth layer by ramming watering lead upto 50 m and lift upto 1.5 mt. I/c all kind of soils. |  |  |  |  |
|  | CG,PHE, SOR Item no.18.22 P/227 |  |  |  |  |
|  | Qty As per Item No. 1 to 4 |  |  |  |  |
|  | Dhanegaon |  | 6.60 |  |  |
|  |  | Total | 6.60 | 6.60 | cum. |
| 24 | Providing and laying mechanically mixed cement concrete with 20 mm nominal size graded stone aggregat. C M-15 |  |  |  |  |
|  | CG,PHE, SOR Item no. 18.41 P/229 |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 50 | x 0.6 x 0.20 = | 6 |  |  |
|  |  | Total | 6.00 | 6.00 | cum. |
| 27 | CONSTRUCTION OF RCC SLUICE VALVE CHAMBER SIZE 1.0mX1.0mX1.0m |  |  |  |  |
|  | Dhanegaon |  | 3 |  |  |
|  |  | Total | 3.00 | 3.00 | As per Sub Estimate |
| 29 | Provision for jointing of Rising main to supmp well/OHT and OHT to Distribution pipe line with cost of material/specials such as Bends, MTA as per rerquirement of site i/c cost of labour with excavation, labour as per requirement complete as per approved specification and as directed by Engineer in charge. |  |  |  |  |
|  | CG,PHE, SOR Item no 19.20 P/246 |  |  |  |  |
|  | Dhanegaon |  | 1.00 |  |  |
|  |  | Total | 1.00 | 1.00 | Job |
| Schedule NO.- 02 |  |  |  |  |  |
|  | FHTCs (Funtional Household Tap Connection |  |  |  |  |
| 1 | Providing and constructing one stand post as per type design with excavation 15 cm thick PCC 1:3:6 bedding 20 mm thick PCC 1:2:4 convert for platform of 1.5mx1.5m with side curb and bucket rest of 80 mm dia. 160mm dia PVC pipe central post duly filled therein with C.C. 1:2:4, 2.2m long, 15 mm dia medium G.I. pipe. From point of tapping to stand post additional 1620 (20 mm dia) composite pipe 6.0 m long. Providing and fixing of 15 mm dia, one steel water tap, one flow control valve SS, 5 LPM complete together with all labour and material charges as per drawing and as directed by Engineer-in-charge when good foundation in available. Rate includes draining arrangement by excavating open gutters complete |  |  |  |  |
|  | PHE SOR P.No /Item No.253/19.35 |  |  |  |  |
|  | Dhanegaon |  | 3 |  |  |
|  |  | Total | 3.00 | 3.00 | Each |
| 2 | Earth work in excavation for pipe trench in ordinary soil in areas including dressing, watering and ramming & disposal of excavated earth lead up to 50 m- & lift up to 1.5 m. disposal earth to be leveled, neatly dressed |  |  |  |  |
|  | PHE SOR P.No /Item No.226/18.19 |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 0 | x0.25x0.15 | 0.000 |  |  |
|  |  | Total | 0.00 | 0.00 | cum. |
| 3 | Earth work in excavation for pipe trench in all kinds of rocks in areas including dressing, stacking of useful material and disposal of unserviceable one up to 50 m lead and lift up to 1.5 m. (a) Soft rock with or without blasting or bituminous pavement /cement concrete road. |  |  |  |  |
|  | PHE SOR P.No /Item No.226/18.19 |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 0 | x0.25x0.15 | 0.000 |  |  |
|  |  | Total | 0.00 | 0.00 | cum. |
| 4 | Demolishing cement concrete manually / by mechanical means including disposal of material within 50 m lead as per direction of engineer-in-charge. |  |  |  |  |
|  | Nominal concrete 1:3:6 or richer mix (i/c equivalent design mix) |  |  |  |  |
|  | CG,PHE, SOR Item no.18.29.1 P/227 |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 5 | x0.25x0.25 | 0.313 |  |  |
|  |  | Total | 0.31 | 0.31 | cum. |
| 5 | Providing and fixing Polyethylene- Aluminum Polyethylene (PE-AL-PE) Composite Pressure pipes Conforming to IS: 15450-2004 U.V. Stablished with carbon black having thermal stability for hot & cold water supply, capable to with stand temperature up to 800 C including all specials and fittiings of composite material (engineering plastic. gland and brass insert wherever required) e.g. elbws, tees. reducers, couplers and connectors wiith clamp at 1m spacing. This includes testing of joints complete as per the directions of engineer- incharge (Concealed work including cutting chases& making good the wall etc ) |  |  |  |  |
|  | PHE SOR P.No /Item No.254/19.37.2 |  |  |  |  |
|  | 1620 ( 20mm OD ) Pipe |  |  |  |  |
|  | Dhanegaon |  | 280.00 |  |  |
|  |  | Total | 280.00 | 280 | Rm |
| 6 | Filling available excavated earth in trenches, lead up to 50m and lift up to 1.5m in all kind of soil excluding watering and ramming. |  |  |  |  |
|  | PHE SOR P.No /Item No. 227/18/23 |  |  |  |  |
|  | (B) in Hard Soil for Road |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 0 | x0.25x0.30 | 0.000 |  |  |
|  |  | Total | 0.00 | 0.00 | cum. |
| 7 | Providing & laying mechanically mixed cement concrete 20mm nominal size graded crushed stone excluding cost of centering & shuttering. |  |  |  |  |
|  | PHE SOR P.No /Item No. 229/18/41.1.1 |  |  |  |  |
|  | In Plinth & foundation |  |  |  |  |
|  | For Cutting 1:3:6 (M-10) |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 5 | x0.25x0.25 | 0.313 |  |  |
|  |  | Total | 0.31 | 0.31 | cum. |

ANNEXURE - “G”

BREAK-UP SCHEDULE FOR VALUATION OF WORK

| S.No. | Description | Amount of Tender (Rs. Lakh) | Cumulative percentage |
| --- | --- | --- | --- |
| 1 | Providing, laying, jointing, interconnection, testing and commissioning of 90/110 mm dia UPVC class 06/10 kg/sqcm Pipe, 63/75 mm dia HDPE 06 kg/sqcm Pipewith suitable joints, valve specials and all fittings including all allied civil works, complete with 03 months trial run, operation & maintenance for 06 months under Single Village of Piped Water Supply Scheme under Jal Jeevan Mission For Village Markatola Block Mohla District- Mohla-Manpur-A.Chowki . | 4.17 | 24.24% |
| 2 | Providing, laying, jointing, interconnection, testing and commissioning of Rising Main | 0.44 | 2.55% |
| 3 | Construction of R.C.C. over head tank | 3.90 | 22.68% |
| 4 | Construction of Switch Room | 0.50 | 2.90% |
| 5 | Construction of boundary wall around the Over Head tank (as per schedule). | 1.82 | 10.57% |
| 6 | Installation of Electro chlorinator | 2.81 | 16.32% |
| 7 | Construction of chlorinator room size | 0.55 | 3.20% |
| 8 | Providing FHTC in individual households & govt. institutions | 1.35 | 7.84% |
| 9 | Provision for Pump | 0.97 | 5.64% |
| 10 | Provision for Electric connection from CSPDCL, | 0.70 | 4.07% |
| 11 | Provision for Road Cutting | 0.00 | 0.00% |
| Total Rs. |  | 17.22 | 100% |

| S.No. | Description | Amount of Tender (Rs. Lakh) | Cumulative percentage |
| --- | --- | --- | --- |
| 1 | Providing, laying, jointing, interconnection, testing and commissioning of 90/110 mm dia UPVC class 06/10 kg/sqcm Pipe, 63/75 mm dia HDPE 06 kg/sqcm Pipewith suitable joints, valve specials and all fittings including all allied civil works, complete with 03 months trial run, operation & maintenance for 06 months under Single Village of Piped Water Supply Scheme under Jal Jeevan Mission For Village Dhanegaon Block Mohla District- Mohla-Manpur-A.Chowki . | 1.43 | 66.63% |
| 8 | Providing FHTC in individual households & govt. institutions | 0.71 | 33.37% |
| Total Rs. |  | 2.14 | 100% |

ANNEXURE -  “G-I”

Schedule of Payment of Rising main and Distribution pipe line

| S.No. | Description | Schedule of payment |
| --- | --- | --- |
| 1 | Rising main pipe line and distribution pipe line | As per actual work and sanctioned tender percentage above/below with respect/to PHE SOR with effect from 01.06.2020 with upto date amendment till tender date. |

ANNEXURE -  “G-II”

Break-Up Schedule of payment for construction of  Over HeadTank

Amount of OHT = ………………….00 x (1+sanctioned tender %)

100

| S.No. | Particulars | Percentage of |
| --- | --- | --- |
| 1 | After Casting of Levelling Course | 5% |
| 2 | After Foundation Including Column Upto Ground Level | 10% |
| 3 | After Casting 50% R.C.C. Staging | 10% |
| 4 | After Full Staging | 10% |
| 5 | After Ring Beam & Bottom Slab Casting & Supply & Erection Of Pipes & Special | 25% |
| 6 | After Casting Vertical Walls of Tank | 12% |
| 7 | After Casting Stair Case/Ladder And Top Dome/Slab Finishing (Including Railing, Valve Chamber, Water Level Indicator, Lightening Arrestor, Colour Washing, Oil Painting Etc. Complete) | 12% |
| 8 | After Testing Of Complete Work | 10% |
|  | TOTAL | 100% |

Remark:-1- Drawing & design of OHT will be supplied by department, for which Rs. 500/- will be deducted from the contractor’s bill.

- The contractor shall be required to conduct proper investigations with full field tests for finding bearing capacity & other details at his own cost.  No payment will be made to contractor for carrying out the test. In case S.B.C test is conducted by department then the amount for this shall  be deducted from the contractor’s bill.

ANNEXURE - “G-III"

Break-Up Schedule of payment for construction of

Switch room

| S.No. | Description | Schedule of payment |
| --- | --- | --- |
| 1 | Construction switch room size | As per actual work and sanctioned tender percentage above/below with respect/to PWD SOR with effect from 01-01-2015with upto date amendment till tender date |

ANNEXURE - “G-IV"

Break-Up Schedule of payment for construction of

Bounday wall

| S.No. | Description | Schedule of payment |
| --- | --- | --- |
| 1 | Construction of boundary wall around the over head tank. | As per actual work and sanctioned tender percentage above/below with respect/to PWD SOR with effect from 01-01-2015with upto date amendment till tender date |

ANNEXURE - “G-V"

Break-Up Schedule of payment for Supply of

Electro Chlorinator as per PHE USOR June 2020

Amount of Electro Chlorinator =  281000.00×(1± sanctioned tender100%)

| S.No. | Particulars | Percentage of |
| --- | --- | --- |
| 1 | Supply of Electro Chlorinator | 50% |
| 2 | Installation of Electro Chlorinator | 30% |
| 3 | Testing of Electro Chlorinator | 20% |

ANNEXURE -  "G-VI"

Break-Up Schedule of payment for construction of

Chlorinator room

| S.No. | Description | Schedule of payment |
| --- | --- | --- |
| 1 | Construction of chlorinator room | As per actual work and sanctioned tender percentage above/below with respect/to PWD SOR with effect from 01-01-2015with upto date amendment till tender date |

ANNEXURE - “G-VII"

Break-Up Schedule of payment for providing FHTC in individual households Govt. institutions

| S.No. | Description | Schedule of payment |
| --- | --- | --- |
| 1 | Providing FHTC in individual households & Govt. institutions | As per actual work and sanctioned tender percentage above/below with respect/to PHE USOR with effect from 01.06.2020 with upto date amendment till tender date |

ANNEXURE - “G-VIII"

Break-Up Schedule of payment for Provision for supply & installation of submersible pump

| S.No. | Description | Schedule of payment |
| --- | --- | --- |
| 1 | Provision for supply & installation of submersible pump | As per actual work and sanctioned tender percentage above/below with respect/to PHE USOR with effect from 01.06.2020 with upto date amendment till tender date except for supply of submersible pump. For supply of submersible pump only rate approved by CSIDC will be paid. |

ANNEXURE - “G-IX"

Break-Up Schedule of payment for Provision for Electric connection from CSPDCL, ROAD CUTTING,

| S.No. | Description | Schedule of payment |
| --- | --- | --- |
| 1 | Provision for Electric connection from CSPDCL, ROAD CUTTING, | Demand note raised by CSPDCL, ROAD CUTTING, /Cencern Department will be reimburced to the contractor & no percentage above or below will be applicable in this item. |

ANNEXURE “H”

// PRE-CONTRACT INTEGRITY PACT //

1.	GENERAL

1.1	This pre-bid contract Agreement (herein after called the Integrity Pact) is made on………………day of the month ………….20……….., between, the Government of Chhattisgarh acting  through Shri………………………………………….(Designation of the officer, Department) Government of Chhattisgarh (hereinafter called the “BUYER”, which expression shall mean and include, unless the context otherwise requires, his successors in the office and assigns) and the First Party, proposes to procure (name of the Stores/Equipment/Work/Service) and M/s …………………… …………represented by Shri……………………………………Chief Executive Officer (hereinafter called the “BIDDER’/Seller”, which expression shall mean and include unless the context otherwise requires, his successors and permitted assigns) and the Second Party, is willing to offer/has offered.

1.2	WHEREAS the BIDDER is a Private Company/Public Company/Government Undertaking/Partnership/Registered Export Agency, constituted in accordance with the relevant law in the matter and the BUYER is a Ministry/Department of the Government, performing its function on behalf of the Government of Chhattisgarh.

2.	OBJECTIVES

NOW, THEREFORE, the BUYER and the BIDDER agree to enter into this pre-contract agreement, hereinafter referred to as Integrity Pact, to avoid all forms of corruption by following a system that is fair, transparent and free from any influence / prejudiced dealings prior to, during and subsequent to the Contract to be entered into with a view to :-

2.1	Enabling the BUYER to obtain the desired Stores/Equipment/Work/Service at a competitive price in conformity with the defined specification by avoiding the high cost and the distortionary impact of corruption on public procurement, and

2.2	Enabling BIDDERs to abstain from bribing or indulging in any corrupt practices in order to secure the contract by providing assurance to them that their competitors will also abstain from bribing any corrupt practices and the BUYER will commit to prevent corruption, in any form, by its official by following transparent procedures.

3.	COMMITMENTS OF THE BUYER

The BUYER commits itself to the following :-

3.1	The BUYER undertakes that no official of the BUYER, connected directly or  indirectly with the contract, will demand, take promise for or accept, directly or through intermediaries, any bribe, consideration, gift, reward, favour or any material or immaterial benefit or any other advantage from the BIDDER, either for themselves or for any person, organization or third party related to the contract in exchange for an advantage in the bidding process, bid evaluation, contracting or implementation process related to the contract.

3.2	The BUYER will, during the pre-contract stage, treat BIDDERs alike, and will provide to all BIDDERs the same information and will not provide any such information to any particular BIDDER which could afford an advantage to that particular BIDDER in comparison to the other BIDDERs.

3.3	All the officials of the BUYER will report the appropriate Government office any attempted or completed breaches of the above commitments as well as any substantial suspicion of such a breach.

In cash  any such preceding misconduct on the part of such official(s) is reported by the BIDDER to the BUYER with the full and verifiable facts and the same prima facie found to be correct by the BUYER,  necessary disciplinary proceedings, or any other action as deemed fit, including criminal proceedings may be initiated by the BUYER and such a person shall be debarred from further dealings related to the contract process. In such a cash while an enquiry is being conducted by the BUYER the proceedings under the contract would not be stalled.

4.	COMMITMENTS OF BIDDERS

The BIDDER commits itself to take all measures necessary to prevent corrupt practices, unfair means an illegal activities during any stage of  its bid or during any pre-contract or post –contract stage in order to secure the contract or in furtherance to secure it and in particular commit itself to the following :-

4.1. 	The BIDDER will not offer, directly or through intermediaries, any bribe, gift, consideration, reward, favor, any material or immaterial benefit or other advantage, commission, fees, brokerage or inducement to any official of the BUYER, connected directly or indirectly with the biding process, or to any person, organization or third party related to the contract in exchange for any advantage in the bidding, evaluation, contracting and implementation of the contract.

4.2. 	The BIDDER further undertakes that it has not given, offered or promised to give, directly or indirectly any bribe, gif, consideration, reward, favor, any material or immaterial benefit or other advantage, commission, fees, brokerage, or inducement to any official of the BUYER or otherwise in procuring the Contract of forbearing to do or having done any act in relation to the obtaining or execution of the contract or any other contract with the Government for showing or forbearing to show favor or disfavor to any person in relation to the contract or any other contract with the Government.

4.3	The BIDDER further confirms and declares to the BUYER that the BIDDER in the original Manufacture /Integrator /Authorized government sponsored export entity of the stores and has not engaged any individual or firm or company whether Indian or foreign to intercede, facilitate or in any way to recommend to the BUYER or any of its functionaries, whether officially or unofficially to the award of the contract to the BIDDER, nor has any amount been paid, promised or intended to be paid to any such individual, firm or company in respect of any such intercession, facilitation or recommendation.

4.4	The BIDDER, either while presenting the bid or during pre-contract negotiations or before signing the contract, shall disclose any payment he has made, is committed to or intends to make to officials of the BUYER or their family members, agents, brokers or any other intermediaries in connection with the contract and the details of services agreed upon for such payments.

4.5	The BIDDER will not collude with other parties interested in the contract to impair the transparency, fairness and progress of the bidding process, bid evaluation, contracting and implementation of the contract.

4.6	The BIDDER will not accept any advantage in exchange for any corrupt practice, unfair means and illegal activities.

4.7	The BIDDER shall not use improperly, for purpose of competition or personal gain, or pass on to others, any information provided by the BUYER as part of the business relationship, regarding plans, technical proposal and business details, including information contained in any electronic data carrier. The BIDDER also undertakes to exercise due and adequate care lest any such information is divulged.

4.8	The BIDDER commits to refrain from giving any complaint directly or through any other manner without supporting it with full and verifiable facts.

4.9	The BIDDER shall not instigate or cause to instigate any third person to commit any of the acts mentioned above.

5.	PREVIOUS TRANSGRESSION

5.1	The BIDDER declares that no previous transgression occurred in the last three years immediately before signing of this Integrity Pact with any other company in any country in respect of any corrupt practices envisaged hereunder or with any Public Sector Enterprise in India or any Government Department in India that could justify BIDDER’s exclusion from the tender process.

5.2	If the BIDDER makes incorrect statement on this subject, BIDDER can be disqualified from the tender process or the contract, if already awarded can be terminated for such reason.

6.	EARNEST MONEY (SECURITY DEPOSIT)

6.1	Every  BIDDER while submitting commercial bid, shall deposit an amount as specified in RFP as Earnest Money/Security Deposit, with the BUYER through any of the following instruments :

(i)	Bank Draft or a Pay Order in favor of ………………………….

(ii)	A confirmed guarantee by an Indian Nationalized Bank, promising payment of the guaranteed sum to the ………………(BUYER) ……………..on demand within three working days without any demur whatsoever and without seeking any reasons whatsoever. The demand for payment by the BUYER shall be treated as conclusive proof of payment.

(iii) 	Any other mode or through any other instrument ( to be specified in the RFP).

6.2	The Earnest Money/Security Deposit shall be valid up to a period of five years or the complete conclusion of the contractual of the contractual obligations to the complete satisfaction of both the BIDDER and BUYER, including warranty period, whichever is later.

6.3	In the case of successful BIDDER a clause would also be incorporated in the Article pertaining to Performance Bond in the Purchase Contract that the provisions of Sanctions for violation shall be applicable for forfeiture of Performance Bond in case of a decision by the BUYER to forfeit the same without assigning any reason for imposing sanction for violation of this Pact.

6.4	No interest shall be payable by the BUYER to the BIDDER on Earnest Money/Security Deposit for the period of its currency.

7.	SANCTIONS FOR VIOLATIONS

7.1	Any breach of the aforesaid provisions by the BIDDER or any one employed by it or acting on its behalf (whether with or without the knowledge of the BIDDER) shall entitle the BUYER to take all or any one of  the following actions, wherever required :-

(i)	To immediately call off the pre contract negotiations without assigning any reason or giving any compensation to the BIDDER. However, the proceedings with the other BIDDER(s) would continue.

(ii)	To forfeit fully or partially the Earnest Money Deposit (in pre-contract stage) and/or Security Deposit /Performance Bond (after the contract is signed), as decided by the BUYER and the BUYER shall not be required to assign any reason therefore.

(iii)	To immediately cancel the contract, if already signed, without giving any compensation to the BIDDER.

(iv)	To recover all sums already paid by the BUYER, and in case of the Indian BIDDER with interest thereon at 2% higher than the prevailing Prime Lending Rate while in case of a BIDDER from a country other than India with Interest thereon at 2% higher than the LIBOR. If any outstanding payment is due to the BIDDER from the BUYER in connection with any other contract such outstanding payment could also be utilized to recover the aforesaid sum and interest.

(v)	To encase the advance bank guarantee and performance bond/warranty bond, if furnished by the BIDDER, in order to recover the payments, already made by the BUYER, along with interest.

(vi)	To cancel all or any contracts with the BIDDER and the BIDDER shall be liable to pay compensation for any loss or damage to the BUYER resulting from such cancellation/rescission and the BUYER shall be entitled to deduct the amount so payable from the money(s) due to the BIDDER.

(vii)	To debar the BIDDER from participating in future bidding processes of the Government of Chhattisgarh for a minimum period of five years, which may be further extended at the discretion of the BUYER.

(viii)	To recover all sums paid in violation of this Pact by BIDDER(s) to any middlemen or agent or broken with a view to securing the contract.

(ix)	In cases where irrevocable Letters of Credit have been received in respect of any contract signed by the BUYER with the BIDDER, the same shall not be opened.

(x)	If the BIDDER or any employee of the BIDDER or any person acting on behalf of the BIDDER, either directly or indirectly, is closely related to any of the officers of the BUYER, or alternatively, if any close relative of an officer of the BUYER has financial interest/stake in the BIDDER’s firm, the same shall be disclosed by the BIDDER at the time of filling of tender. Any failure to disclose the interest involved shall entitle the BUYER to rescind the contract without payment of any compensation to the BIDDER.

The term ‘close relative’ for this purpose would mean spouse whether residing with the Government servant or not, but not include a spouse separated from the Government servant by a decree or order of a competent court; son or daughter or step son or step daughter and wholly dependent upon the Government servant or of whose custody the Government servant has been deprived of by or under any law; any other person related, whether by blood or marriage, to the Government servant or to the Government servant’s wife or husband and wholly dependant upon Government servant.

(xi)	The BIDDER shall not lend to or borrow any money from or enter into any monetary dealings or transactions, directly or indirectly, with any employee of the BUYER, and if he does so, the BUYER shall be entitled forthwith to rescind the contract and all other contracts with the BIDDER. The BIDDER shall be liable to pay compensation for any loss or damage to the BUYER resulting from such rescission and the BUYER shall be entitled to deduct the amount so payable from the money(s) due to the BIDDER.

7.2	The decision of the BUYER to the effect that a breach of the provisions of this pact has been committed by the BIDDER shall be final and conclusive on the BIDDER. However, the BIDDER can approach the Monitor(s) appointed for the purposes of this Pact.

8.	FALL CLAUSE

8.1	The BIDDER undertakes that if has not supplied/is not supplying similar product/systems or subsystems at a price lower than that offered in the present bid in respect of any other Department of the Government of Chhattisgarh or PSU and if it is found at any stage that similar product/systems or sub systems was supplied by the BIDDER to any other Department of the Government of Chhattisgarh or a PSU at a lower price, then that very price, with due allowance for elapsed time, will be applicable to the present case and the difference in the cost would be refunded by the BIDDER to the BUYER, if the contract has already been concluded.

9.	INDEPENDENT MONITORS

9.1	The BUYER will appoint Independent Monitors (hereinafter referred to as Monitors) for this Pact.

9.2	The task of the Monitors shall be to review independently and objectively, whether and to what extent the parties comply with the obligations under this Pact.

9.3	The Monitors shall not be subject to instructions by the representatives of the parties and perform their functions neutrally and independently.

9.4	Both the parties accept that the Monitors have the right to access all the documents relating to the project/procurement, including minutes of meetings. The Monitor shall be under contractual obligation to treat the information and documents of the BIDDER/Sub contractor/s) with confidentiality

9.5	As soon as the Monitor notices, or has reason to believe, a violation of this Pact he will so inform the Authority designated by the BUYER.

9.6	The Monitor will submit a written report to the designated Authority of BUYER/ Secretary in the Department/within 8 to 10 weeks from the date of reference or intimation to him by the BUYER/BIDDER and, should the occasion arise, submit proposals for correcting problematic situations.

10.	FACILITATION OF INVESTINGATION

In case of any allegation of violation of any provisions of this Pact or payment of commission, the BUYER or its agencies shall be entitled to examine all the documents including the Books of Account of the BIDDER shall provide necessary information of the relevant documents and shall extend all possible help for the purpose of such examination.

11.	LAY AND PLACE OF JURISDICTION

This Pact is subject to Indian Law, the place of performance and jurisdiction shall be the seat of the BUYER.

12.	OTHER LEGAL ACTIONS

The actions stipulated in this Integrity Pact are without prejudice to any other legal action that may follow in accordance with the provisions of the any other law in force relating to any civil or criminal proceedings.

13.	VALIDITY

13.1	The validity of this Integrity Pact shall be from the date of its signing and extend up to 5 years or the complete execution of the contract to the satisfaction of both the BUYER and the BIDDER/Seller whichever is later. In cash BIDDER is unsuccessful; this Integrity Pact shall expire after six months from the date of the signing of the contract.

13.2	If one or several provisions of this Pact turn out to be invalid; the remainder of this Pact shall remain valid. In such case, the parties will strive to come to an agreement to their original intentions.

14.	The parties hereby sign this Integrity Pact at……………………….on……………….

BUYER							BIDDER.

Name of  the Officer						CHIEF EXECUTIVE OFFICER

Designation

Department/PSU

Witness						Witness

1)	……………………………………….		 1)	……………………………

……………………………………….			……………………………

2)	……………………………………….		2)	……………………………

……………………………………….			……………………………

ANNEXURE “I”

Special Conditions

      - "Additional performance security (APS) shall be deposited by the successful bidder at the time of signing of agreement when the bid amount is seriously unbalanced i.e. less than the estimated cost i.e. less than the estimated cost by more than 10%. In such an event the successful bidder will deposit the Additional Performance Security (APS) to the extent of difference of 90% of the PAC and bid amount in the shape of Bank Guarantee, issued by approved Scheduled Bank, for agreemented period in favour of the Executive Engineerand member secretary, DWSM, MOHLA-MANPUR-A.CHOWKI  before signing the agreement. The same shall be released along with the normal S.D. after completion of the work. If the contractor fails to complete the work or left the work in complete, this Additional Performance Security (APS), shall be forfeited by the department, & the agreement shall be terminated and action shall be taken in accordance with clause 3 of the agreement. In case the tenderer/contracto'r refuses to deposit Additional Performance Security (APS) then his bid will be rejected by the sanctioning authority and earnest money shall be forfeited."

      - The contractor will be responsible for taking demand note from CSPDCL, ROAD CUTTING,   and the amount of this will be reimbursed by the department.

      - Before installation of power pump , verification of source and yield test should be done by the contractor, accordingly the capacity of pump will be decidedand will be approved by Engineer-in charge.

      - Minimum Diameter of pipe to be used for drinking water supply is 63 mm .

|  | Executive Engineer and member secretary, DWSM, MOHLA-MANPUR-A.CHOWKI (C.G.) |
| --- | --- |

ANNEXURE “J”

CENTRALISED PROCEDIRES

OF THE GOVERNMENT OF CHHATTISGARH

FOR SUSPENSION, DEMOTION, NON-RENEWAL & DE-REGISTRATION OF CONTRACTORS/FIRMS, 2014

_________________________________________________________________________

- To Replace Appendix-1 to 4 of PWD Orders No. F5-8/19/2013/Nivida dated 5.8.2013

- RFP to specifically refer to the aforesaid orders as amended

_________________________________________________________________________

1.	Nomenclature and Coming Into Force

1.1	These Procedures shall be called "Centralised Procedures of the Government of Chhattisgarh for Suspension, Demotion, Non-Renewal and De-Registration of Contractors/Firms, 2014".

1.2	These Procedures shall come into force with effect from the date of consequential changes made to previous orders PWD No. F5-8/19/2013/Nivida dated 5.8.2013 on the subject.

2.	Scope of Procedures

2.1	These Procedures lay down substantive and procedural aspects for demotion, non-renewal and de-registration (including suspension) of registered contractors/firms, including the consequences of such demotion, non-renewal, de-registration and suspension, in relation to contracts covered by these procedures.

2.2	These procedures shall apply to all contracts awarded by state agencies participating in the unified registration system, for e-registration through the website of the state public works department, and the said participating agencies shall include their constituent, attached and subordinate offices, whether referred to as a "Department", "Wing", "Organisation", "Board" or similar nomericlature in common usage.

2.3	For the purposes of these procedures, "contracts" shall mean contracts or agreements under the works department manual entered into by the state agencies to which these procedures are applicable (hereinafter referred to as "state agencies")

2.4	These procedures shall not apply to debarment and suspension (or blacklisting, banning etc., by whatever name called) by states public sector enterprises, registered societies, autonomous institutes or similar separate legal entities, even if functioning under the administrative control of the Government of Chhattisgarh.

2.5	These procedures shall be incorporated by specific reference in contracts entered into by relevant agencies with from the date of notification of these procedures.

All prior contracts (i.e. cases where the last date of filling a proposal, tender, quotation, bid etc. by whatever name called, predates the date of notification of these procedures, including contracts entered into before the date of notification of these procedures), shall however continue to be governed by the specific clauses of contract and instructions issued by the government of chhattisgarh in force prior to notification of these procedures. For cases where the last date of filling a proposal, tender, quotation, bid etc., by whatever name called has already been schedule and notification, and is subsequent to the date of notification of these procedures, an addendum to the request for proposal (or Notice Inviting Tender, request for quotation, Invitation To Bid etc. by whatever name called) shall be issued by procuring officials, to ensure that acceptance of the terms and conditions prescribed under these procedures is obtained from all participating bidders.

2.6	"Entities" shall include individuals, as well as companies, trusts, societies or other associations of individuals with whom the state agencies have entered into contracts, or intend to enter into contracts, or could enter into contracts.

3.	Cross-Debarment

3.1	Any order for demotion, non-renewal, suspension or de-registration issued by the designed Authority provided for under these Procedures shall have immediate effect as provided for under these Procedures shall have immediate effect as provided for under these Procedures shall have immediate effect as provided herein on on contracts awarded and processed by the state agencies.

3.2	Any order for suspension, banning, demotion, blacklisting, de-registration or debarment issued by any authority in the Government of India or other State Government, or by any other entity not covered under the scope of these Procedures, shall not affect the eligibility of contractors or firms participation in the Unified Registration system, Unless the Designated Authority under these Procedures has issued its own order of suspension, non-renewal, demotion or de-registration, pursuant to or consequent or based upon the order issued by such an external agency, after satisfying itself that the order of suspension, non-renewal, demotion or de-registration against such a contractor/firm is appropriate and warranted in order to protect government or public interest in the facts and circumstances of the specific case.

4.	Satisfaction of Designated Authority

4.1	Proceedings for suspension, demotion, or de-registration shall be in the nature of summary administrative decisions by the Designated Authority, aimed at protecting government of public interest.

4.2	Accordingly, the Designated Authority shall exercise due diligence normally expected of an administrative action for issue of an order of suspension, non-renewal, demotion or de-registration against a contractor/firm is appropriate and warranted in order to protect government of public interest in the facts and circumstances of the specific case.

5.	Designated Authorities

5.1	The authority competent to issue and order of suspension non-renewal demotion or de-registration, including a notice or proposed demotion or de-registration shall be the authority so specified in the unified Registration system.

5.2	The orders of the Designated Authority shall have effect across contracts awarded and processed by the state agencies.

5.3	The Designated Authority shall perform his/her function independently of any superior officers, Board, Council or Committees in the Government. Normally therefore, there should not be any separate of prior requirement of bringing any case for proposed suspension, no-renewal, demotion or deregistration before any committee by any state agency, the Designated Authority shall recuse himself/herself from the deliberations or decisions of such board etc. in order to maintain his/her independence in decision making.

5.4	The Designate Authority shall be free and be empowered to consult any appropriate Department that he/she may deem appropriate (such as legal or finance divisions or departments) before arriving at a decision for suspension non-renewal, demotion or de-registration, However the advise, recommendation or comments of any such Department shall neither be binding upon the designated Authority and nor shall it form the sole basis of the decision of the Designated Authority.

5.5	To the extent practicable the designated Authority shall decide upon a case within  thirty days of the proposal being submitted to it for de-registration, non-renewal, suspension or demotion, provided he/she has received all  documents that he/she deems relevant.

6.	Appeals

6.1	Appeals against orders of suspension and final orders for demotion, non-renewal or de-registration issued by the designated Authority shall lie only with the Appellate Authority specified in the Unified Registration system. The Appellate Authority shall discharge his/her functions independently of any superior officers, Boards, Councils or Committees in the Government. No appeals shall lie against a notice of proposed demotion, non-renewal or de-registration.

6.2	Therefore, normally there should not be any separate or prior requirement of bringing any case for proposed suspension, non-renewal, demotion or de-registration before any Board, Council or Committe. However, if any case is brought before any Board, Council or Committee, any member of the Appellate Authority represented on such boards etc shall recuse himself/herself from the deliberations or decisions of such Board etc in order to maintain his/her independence in decision-making.

6.3	The Appellate Authority shall be free and be empowered to consult any Department that it deems appropriate (such as legal or finance divisions or Department) before deciding an appeal. However, the advice, recommendation or comments of any such Department shall neither be

binding upon the Appellate Authority and nor shall it form the sole basics of the decision taken in appeal by the Appellate Authority.

6.4	The order of suspension, non-renewal, demotion or de-registration (including notice of proposed demotion or de-registration) of the Designated Authority, or the orders in appeal issued by the Appellate Authority, shall not form part of the scope of issued that can be brought before arbitrator(s) or arbitral tribunal, notwithstanding anything contained in an arbitration agreement in relation to Authority or the Appellate Authority shall not form part of the scope of issues that can be referred to the Independent External Monitor under any “Integrity Pact” signed by covered entities under these procedures, notwithstanding anything contained in the Integrity Pact.

6.5	To the extent practicable, the Appellate Authority shall decide an appeal within thirty days of the appeal being submitted to it against de-registration, non- renewal suspension or demotion. Provided he/she has received all documents that he/she deems relevant from the appellant and from respondents.

6.6	The Appellate Authority shall have the power to pend the processing of a contract for reasons it may deem fit.

7. 	Grounds for Demotion, Non –Renewal and De-Registration

7.1	The Designated, Authority may de-register a contractor/firm (referred herein below as “entity”) for any sufficient reason, including any one or more of but not limited to the grounds listed below;

(i)	Conviction for, or the commencement of an investigation under the criminal Procedure code, 1973 or the filing of a chargesheet by an investigative agency in a criminal court of competent jurisdiction, or a civil judgment in respect of commission or alleged commission of fraud or an offence under any law in force in India or elsewhere, either directly or indirectly, by an entity, its proprietor, employee, partner, agent or representative in connection with (a) obtaining, or (b) attempting to obtain, or (c) performing a government contract or agreement:

Provided that in case of commencement of criminal investigation this clauses shall constitute a valid ground for de-registration only if the said fraud or offence is in relation to a contract or agreement entered into with agent representative or any other intermediary in relation to any contract to witch these procedures apply;

(l)	Engagement of any individual or entity to intercede, facilitates, or recommends the award of a contact, excluding legal representatives employed directly by the entity;

(m)	Collusion to impair transparency, fairness or progress of the contracting process;

(n)	Complaining without full and verifiable facts; or

(o)	Instigating or causing any third person to commit any of the above.

7.3	A designated Authority may not renew the registration of a contractor/ firm for any sufficient reason, including any one or more of, but not limited to, the grounds listed below:

(i)	Failure to meet any volume of work criterion specified from time to time under the unified registration system as published on the website for the system, for the particular class of registration for the last three financial years ending with the financial year immediately prior to the date of submission of proposal for non-renewal to the Designated Authority; and

(ii)	Any of the grounds specified in paragraphs 7.1or 7.2 above.

8. 	Grounds for Suspension

8.1	Suspension of an entity can be ordered by the Designated Authority, where it determines that continuation of dealing is not considered desirable in government or public interest, pending completion of proceeding into allegation or facts related to any of the grounds enumerated in paragraphs 7.1 and 7.2 above, and where it is considered necessary to forthwith order such discontinuation without prior notice of suspension to the entity.

8.2	In all such cases of suspension it shall be incumbent upon the Designated Authority to put the suspended entity on notice and to grant an early opportunity of post-decisional hearing.

9. 	Effect of Suspension, Non-renewal, De-Registration and Demotion

9.1	A final order of de-registration or non-renewal in respect of a contractor/firm shall result in immediate ineligibility of the contractor/firm and its affiliates for all classes of contracts (or for classes of contracts higher than the resultant demoted class in cases of demotion) from participating in future bide or contracts or agreements for a minimum period of two years and a maximum period of ten years with effect from the date of demotion or non-renewal or de-registration, including ineligibility from evaluation in ongoing cases where a contract or agreement is yet to be finally signed such ineligibility shall be without compensation from or liability to the state agency similarly, a de-registration or non-renewal contractor/firm and its affiliates shall be ineligible for award of a contract, including receipt of an order under a rate contract, and for entering into any contract covered by these procedures, in cases where an order for de-registration or non-renewal has come into effect before the signing of such contract.

9.2	Where the de-registration, non-renewed, demoted or suspended contractor/firm has already emerged as the most preferred bidder using price and/or technical criteria as specified in the request for proposal (or Notice Inviting Tender or Invitation To Bid or request for Quotation by whatever name called), the  procurement process shall be continued beating the de-registered, suspended demoted or non-renewed contractor/firm as ineligible, notwithstanding any appeal, unless the appellate authority has pended the procurement process.

9.3	A de-registration or non-renewed contractor/firm shall not be eligible to receive Request for proposal (oi Notice Inviting Tender or Invitation to Bid or Request for Quotation etc. by whatever name called) from the state agencies in cases of limited and/or restricted tendering where such issue is provided for under tender procedures.

9.4	In any case where the entity is suspended, de-registered, non-renewed or demoted subsequent to such issue, it shall not be considered eligible for the purposes of any technical, field or commercial evoluation or for award or signing of contract undertaken in pursuance to the Request for proposal (or Notice Inviting Tender or Request for Quotation or Invitation To Bid etc. by whatever name called).

9.5	Mere initiation of a criminal or other investigation or inquiry by any authority in the Government of India or other state Governments, or by any other entity not covered under the scope of these procedures or the mere filling of a    chargesheet or any other formal proceeding against an entity by an enforcement agency exercising power of a court, shall not render an contractor/firm ineligible/ unless the designated authority under these procedures has issued its own order of suspension, non-renewal demotion or de-registration, pursuant to or consequent or based upon the order issued by such an external agency, after satisfying itself that the order of suspension, non- renewal, demotion or de-registration against such a contractor/firm is appropriate and warranted in order to protect government or public interest in the facts and circumstances of the case, similarly, a notice of proposed demotion or de-registration shall not render a contractor/firm ineligible, unless a final order for demotion, non-renewal has been issued by the Designated Authority under these procedures prior to conclusion of the contract.

9.6	The above consequences on ineligibility of a contractor/firm shall be in addition to any criminal liability that may arise out of any laws in force in India, and also in addition to any other pecuniary consequences and civil liabilities, including penalties, costs or liabilities as may be imposed by procuring officials, as well as forfeiture of earnest money deposits, encashment of bank guarantees and performance bonds, risk and cost purchase, recovery of certain sums from the erring entity, and liability for compensation for losses or damages as may be provided for under contract or agreement, as may be provided for in the relevant Notice Inviting Tender, Request For Proposal Request For Quotation, Invitation to Bid and any such documents.

9.7	Notwithstanding anything contairred in the contract, procuring officers shall not renew, expand or extend current contracts with suspended their duration or modify them to the advantage of a contractor/firm, unless prior approval of the authority one level above the authority competent to approve the same under the contract is taken for such renewal, modification or extension: Provided further that in respect of de-registered contractors/firms, the level of prior approval shall be the state Government.

9.8	Termination of contracts, if considered appropriate by procuring officials, shall follow procedures as provided for under the contract and other relevant instructions of the Government.

10.	Notice of Proposal for demotion of De-Registration ( and Order of Suspension or non-Renewal)

10.1	Demotion of de-registration shall be imitated by the Designated Authority upon receipt of information or proposal by putting a contractor/firm on notice:

(i)	That  demotion or de-registration is being contemplated;

(ii)	Of the reason for proposed demotion or de-registration relied upon under paragraph 7 of these Procedures for the proposed demotion or de-registration;

(iii)	Stating the period of demotion  of de-registration and the proposed start and end dates for the period of demotion and de-registration;

(iv)	That, within fifteen days of receipt of the notice, the contractor /firm may information and arguments in connection contesting the proposed demotion/de-registration.

10.2	An order for suspension or non-renewal by the Designated Authority shall afford an opportunity for a post - decisional hearing to the suspended contractor/ firm, including therein:

(i)	The fact that suspension had been ordered forthwith:

(ii)	Of the reasons for suspension or non-renewal- relied upon under paragraph 7 read with paragraph 8 of these procedures:

(iii)	Stating the period of suspension or non-renewal with proposed effective end dates;

(iv)	Advising that suspension or non-renewal is effective only in respect of state agencies

(v)	That, within fifteen days of receipt of the notice, the entity may submit in writing, either in person or through a representative, information and arguments in connection contesting the suspension or non-renewal.

11.	Final Order for Demotion/De-Registratin

11.1	if the Designated Authority decides to issue a final order of demotion or de-registration, whether after the issue of an initial order of suspension or non-renewal or otherwise after issue of notice of proposed demotion or de-registration, the contractor/firm concerned shall expeditiously be given notice of such final order.

(i) Referring to the notice(s) of the order of suspension or non-renewal, if any and     notice of proposed demotion or de-registration.

(ii) Specifying the reason for demotion or de-registration; and

(iii) Starting the period of demotion or de-registration including effective start and end dates.

11.2	if pursuant to the notice of demotion or de-registration, final orders of demotion or de-registration are not imposed, the Designated Authority shall promptly notify the contractor/firm by speed post or by registered post (acknowledgment due), in addition to the manner of publication specified under these procedures.

11.3	Upon completion of the period of de-registration or demotion, a de-registration contractor/firm that has been de-registration or demoted, as well as a contractor/firm whose registration has not been renewed under these procedures shall need to make a fresh application of enlistment or empanelment as a registration of contractor/firms.

12. 	Period of Demotion, De-Registration, Suspension and Non-Renewal

12.1	Demotion and de-registration shall be for a period, commensurate with the seriousness of the reason(s), but not less than two years and not more than ten years in any case, including subsequent extension of demotion or de-registration order, if any;

Provided that in case the period of demotion for an entity is less than the remaining period of registration, the order of demotion shall have the effect of restricting the renewal of registration or re-registration upon completion of registraion to the demoted class for the balance period of demotion;

12.2	The period of suspension shall not exceed one year from the date of issue of the orders for suspension.

12.3	If suspension precedes a demotion or de-registration, the suspension period shall be included in determining the total demotion or de-registration period.

12.4	The Designated Authority may extend the demotion, non-renewal or de-registration for an additional period within the cumulative maximum permissible period of ten years, if it determines that an extension is necessary to protect government or public interest. However, demotion, non-renewal or de-registration may not be extended solely on the basis of the facts and circumstances upon which the initial order were issued. If demotion, non-renewal or de-registration for an additional period is considered necessary, the procedure prescribed under paragraph 10 of these procedures shall be following for such extension.

12.5	The Appellate Authority may reduce the period of demotion, non-renewal, suspension or de-registration, at the affected contractor/firm’s request, supported by documentation if it is satisfied upon examination of all facts and circumstances of the case, including all of the following:-

(i)	That such reduction shall be in government or public interest;

(ii)	That such reduction shall be appropriate for seasons such as newly discovered

material evidence, and/or reversal of the facts, circumstances, enquiry, investigation, conviction or judgment, if any upon which the original debarment or suspension was based; and

(iii)	That the contractor/firm, subsequent to demotion, non-renewal, de-registration or suspension, has put in place sufficient remedial measures, including compliance mechanisms, effective standards of conduct and other internal control systems that are relevant to facts and circumstances of the case.

12.6	Except in case falling under paragraph 12.5(ii) above, reconsideration requests shall not be entertained period to two years from the date of demotion, suspension, non-renewal or de-registration.

13. 	Affiliates and Anti-Circumvention

13.1	Demotion, non-renewal, de-registration and suspension shall always, by implication, extend to “affiliates” of a demoted or non-renewal or de-registration or suspended contractor/firm.

13.2	The demoted, non-renewal, de-registered or suspended contractor/firm and another entity to which such orders can extend shall be presumed to be “affiliates” for the purpose of these procedures if:-

(i)	If either one controls, or has the power to control the other; or

(ii)	A third party controls, or has the power to control both.

13.3	Indices of control include, but are not limited to, interlocking management or ownership, identity of interests amongst employees, owners or members, or shared facilities.

13.4	Particular care needs to be taken by procuring officers to ensure that a demoted or non-renewed or de-registered or suspended contractor/firm does not transact contracts or agreements under a different name or division, either through a transfer of assets of a demoted, non-renewed, de-registered, or suspended contractor/firm to another legal entity or otherwise.

14. 	Consolidated list of Demoted, Non-renewed, De-Registered and Suspended Contractors/Firms

14.1	 A nodal officer specifically notified by the government shall ensure that an updated list of demoted, non-renewed, de-registered and suspended entities, as informed by the Designated Authority, is always maintained on the official website of the unified Registration system.

14.2	In particular, such nodal officer shall be responsible for:-

(i)	Compiling and maintaining a current, consolidated and searchable list of all demoted, non-renewed, de-registered or suspended entities (including their known affiliates), complete in all respects such as scope, period and coverage of orders of demotion, de-registration, non-renewal or suspension; and

(ii)	Providing within such list the name and telephone number of Designated Authorities and Appellate Authority; and	Providing within such list the official contact person responsible for its maintenance and distribution.

12. 	Responsibilities of Procuring Officials

12.1	Procuring officials of the state agencies of the Government shall ensure full comp liance with the provision of these procedures, and shall be responsible in particular, for the following;

(i)	That appropriate procedures are established to implement the substantive 	and procedural aspects of these procedures;

(ii)	That their action are coordinated with internal and external agencies and offices so as to enable receipt of prompt information relevant to potential breaches of entities obligations that may require cases to be placed before Designated Authorities for their consideration for suspension, non-renewal,  demotion or de-registration; and

(iii)	That ineligibility of demoted, non-renewed, de-registered or suspended entities is given effect to forthwith in accordance with the terms of the orders for demotion, non-renewal, de-registration or suspension, especially through a careful watch on the consolidated list as provided for under Rule-14 above.

16	Clarification and Amendments

16.1	All clarifications and amendments to these procedures shall be issued under the authority of the General Administration Department.

17.	In case of  legal issuessuit/petition can be filed only in High Court Bilaspur or appropriate court within chhattisgarh.

ANNEXURE “K”

INFORMATION & INSTRUCTIONS

TO THE BIDDERS FOR

ONLINE ELECTRONIC GOVERNMENT PROCUREMENT SYSTEM (e-GPS)

Guidelines for bidders on using integrated eProcurement System Govt. of Chhattisgarh. https://eproc.cgstate.gov.in

Note:  These conditions will over-rule the conditions  stated  in  the  tender document(s), wherever relevant and applicable.

1. Vendor / Bidder Registration on the e-Procurement System:

All the Users / Bidders (Manufacturers / Contractors / Suppliers / Vendors / Distributors etc.)  registered  with  and  intending to  participate  in  the  Tenders  of  various Govt. Departments  /  Agencies /  Corporations  /  Boards  / Undertakings  under  Govt.  of Chhattisgarh processed using the Integrated e-Procurement System are required to get registered on the centralized portal https://eproc.cgstate.gov.in and get approval on specific class (e.g. A, B, C, D, UGE, UDE)  from Public Works Department (in case to participate in tenders restricted to vendors / bidders in a particular class).

The non – registered users / bidders who are also eligible to participate in the tenders floated using the e-Procurement system are also required to be registered online on the e-Procurement system.

For  more  details,  please  get  in  touch  with  e-Procurement system integrator, M/s. Mjunction  Services  Limited,  Raipur – 492  001 on Toll free 1800 258 2502 or  email helpdesk.eproc@cgswan.gov.in.

2. Digital Certificates:

The bids submitted online must be signed digitally with a valid Class II / Class – III Digital Signature Certificate to establish the identity of the bidders submitting the bids online.  The  bidders  may  obtain pair  of Encryption  &  Signing Class – II /  Class – III Digital Certificate issued by an approved Certifying Authority (CA) authorized by the Controller of Certifying Authorities (CCA), Government of India.

Note: It may take up to 7 to 10 working days for issuance of Class-II / Class-III Digital Certificate,  Therefore  the  bidders  are  advised  to  obtain  it  at  the  earliest.  It  is compulsory to possess a valid Class-II / Class-III Digital Certificate while registering online  on  the  above  mentioned  e-Procurement  portal.  A Digital  Certificate  once mapped  to  an  account  /  registration  cannot  be  remapped  with  any  other  account / registration however it may be inactivated / deactivated.

Important Note: bid under preparation / creation for a particular tender may only be submitted using the same digital certificate that is used for encryption to encrypt the bid data during the bid preparation / creation / responding stage. However bidder may prepare / create and  submit  a  fresh  bid using  his/her  another  /  reissued  /  renewed Digital Certificate only within the stipulated date and time as specified in the tender.

In case, during the process of a particular bid preparation / responding for a tender, the bidder loses his/her Digital Certificate because of any reason they may not be able to submit

he same bid under preparation online,  Hence the bidders are advised to keep their Digital Certificates secure to be used whenever required and comply with IT Act 2000 & its amendments and CVC guidelines.

The digital certificate issued to the authorized user of an individual / partnership firm / private limited company / public limited company / joint venture and used for online bidding will be considered as equivalent to a no-objection certificate / power of attorney to the user.

Unless the certificate is revoked, it will be assumed to represent adequate authority of the specific individual to bid on behalf of the organization / firm for online tenders as per Information Technology Act 2000. This authorized user will be required to obtain a valid Class-II / Class-III Digital Certificate. The Digital Signature executed through the use of Digital Certificate of this authorized user will be binding on the organization

/  firm.  It  shall  be  the  responsibility  of management  /  partners  of  the  concerned organization / firm to inform the Certifying Authority, if the authorized user changes, and apply for a fresh digital certificate for the new authorized user.

3. Online Payment: As the bid is to be submitted only online, bidders are required to make online payment(s) of the Registration fee / Transaction or Service fees / EMD using the online payments gateway services integrated into the e-Procurement system using various payment modes like Credit Card / Debit Card / Internet Banking / Cash Card / NEFT / RTGS etc.

For  the  list  of  available online modes  of  electronic  payments  that  are  presently accepted on the online payments gateway services, please refer the link ‘Payments accepted online’ on the eProcurement portal https://eproc.cgstate.gov.in.

4.  Setup  of User’s Computer  System: In order  to  operate  on  the  e-Procurement system for  a  bidder /  user,  the computer system /  desktop  /  laptop of  the  bidder  is required to have Java ver. 7.25 or latest update, Internet explorer 9 & above or any other latest browser. A detailed step by step document on the same is available on the home page.

5. Publishing of N.I.T.: For the tenders processed using the e-Procurement system, only  a  brief advertisement notice related  to  the tender  shall  be  published  in  the newspapers  and  the detailed notice  shall  be  published  only  on  the  e-Procurement system. Bidders can view the detailed notice, tender document and the activity time schedule for all the tenders processed using the e-Procurement system on the portal https://eproc.cgstate.gov.in.

6.  Tender’s Critical  Dates &  Time:  The bidders  are  strictly  advised  to  follow  the tender time for their side for tasks / activities and responsibilities to participate in the tender, as all the activities / tasks of each tender are locked before the start time & date and after the end time & date for the relevant activity of the tender as set by the concerned department official.

7.  Download Tender  Document(s):  The tender  document  and  supporting document(s) if any can be downloaded only online. The tender document(s) will be available for download to concerned bidders after online publishing of the tender and up to the stipulated date & time as set in the tender.

8. Submit Online Bids: bidders have to submit their bid online after successful filling of forms within the specified date and time as set in the tender. The encrypted bid data of only those bidders who have submitted their bids within the stipulated date & time will be accepted by the e-Procurement system. It is expected

that  the  bidder  complete  his  bid  ad  submit  within  timeline,  a bidder  who  has  not submitted his bid within the stipulated date & time will not be available during opening.

9. Submission of Earnest Money Deposit: The bidders shall submit their Earnest Money Deposit

Either as usual in a physically sealed Earnest Money Deposit envelope and the same should reach the concerned office OR Online using payment gateway as stated in the Notice Inviting Tender. Bidders also have to upload scanned copy of Earnest Money Deposit instrument OR Online Payment /NEFT/RTGS receipt along with the reference details online.

10.Opening of Tenders: The concerned department official receiving the tenders or his duly authorized officer shall first open the online Earnest Money Deposit envelope of all the bidders and verify the same uploaded by the bidders. He / She shall check for the validity of Earnest Money Deposit as required. He / She shall also verify the scanned documents  uploaded  by  the bidders,  if  any,  as  required.  In  case, the requirements are incomplete, the next i.e. technical and commercial envelopes of the concerned bidders received online shall not be opened.

The  concerned official shall  then open  the  other subsequent envelopes  submitted online by the bidders in the presence of the bidders or their authorized representatives who  choose  to  be  present  in  the bid opening process  or  may  view  opened  details online.

11.  Briefcase: Bidders are privileged  to  have  an  online briefcase  to  keep  their documents online and the same can be attached to multiple tenders while responding, this will facilitate bidders to upload their documents once in the briefcase and attach the same document to multiple bids submitting.

For any further queries / assistance, bidders may contact:

1. The  Service  Integrator  of  e-Procurement system,  M/s.  Mjunction  Service  Ltd.  on

Help Desk Toll free No. 1800 258 2502 or email helpdesk.eproc@cgswan.gov.in .

2. Sr. Manager, Chhattisgarh Infotech & Biotech Promotion

Society (CHiPS) on Tel. No. 0771 - 5014128 or email: pro-chips@nic.in.

|  | Executive Engineer and member secretary , DWSM, MOHLA-MANPUR-A.CHOWKI (C.G.) |
| --- | --- |

**********

ANNEXURE “L”

fufonk izi= izkIr Bsdsnkjksa dh ik=rk ekin.M gsrq foHkkx esa izpfyr fufonk izi= ¼QkeZ ^^,**½ esa  ,isf.MDl 2-10 ds ,usDtj Special Condition G-2 III

- ANNEXURE – 2.10 G III

“Additional performance security (APS) shall be deposited by the successful bidder at the time of signing of agreement when the bid amount is seriously unbalanced i.e. less than the estimated cost by more than 10%. In such an event the successful bidder will deposit the Additional performance security (APS) to the extent of difference of 90% of the PAC and bid amount in the shape of F.D.R., issued by approved scheduled Bank, for agreement period in favour of the Executive Engineer before signing the agreement. The same shall be released along with the normal S.D. after completion of the work. If the contractor fails to complete the work or left the work incomplete, this Additional performance security (APS), shall be forfeited by the department in favour of Govt. of Chhattisgarh & the agreement shall be terminated and action shall be taken in accordance with clause 3 of the agreement. In case the tenderer/contractor refuses to deposit Additional performance security (APS) then his bid will be rejected by the sanctioning authority and earnest money shall be forfeited.”

- If the tenderer, whose tender has been accepted, and after signing the agreement. (i) does not start regular actual physical items of work within 25% (twenty five percent) of the time allowed for completion, or (ii) abnormally slowdown the work or (iii) abandons the work, or (iv) merely goes on applying for extension of time; the Executive Engineer shall serve a “show cause” notice with details to the contractor in this regard and if the contractor does not reply, or if his reply is considered not satisfactory (at the sole discretion of the Executive Engineer), his earnest money and the performance security money in this regard shall be forfeited in favour of Govt. If the contractor has committed a similar default on earlier occasion(s) in previous three consecutive years the contractor shall be debarred from participating in any future tender of any P.H.E.D. Division in the State of Chhattisgarh for a period of 2 (two) years from the date of such order, by the authority which had registered him/her.

Such orders & action shall be final binding and conclusive.

- Detailed program of construction:

    - Withing 12 days of issue of order to start work, the contractor shall submit in the prescribed Proforma a detailed construction program month wise mentioning start annd completion of each item/event involved in the due performance of the contract. For contract more than 10 Crores Contractor shall also submit detailed program month wise for

      - Materials procurement,

      - Detailing of construction plants & equipments,

      - Cash flow/revised Cash flow.

    - The Contractor shall submit in the first week of each month a statement of “target vis-à-vis actual performance” of each item/event with slippage, if any; mentioning reason of slippage annd proposal for revised construction program to complete the same in targeted date or validity extended date. Failure to submit this monthly statement for 4(four) month can be treated as “fundamental Breach of Contract” and can result in invoking clause 3 of the condition of contract.

- Cess @ 1% (One percent) shall be deducted at source, from every bill of contractor by the Executive Engineer under “building and other construction for Workers Welfare, Cess Act 1996”.

- It is mandatory for the controctor (S) to get himself/themselevs registered with “C.G. Building and other construction Welfare Board” as soon as the work order is issued to him/them for the work amounting to Rs. 10.00 (Ten) lakh and above and submit a copy the same to the concern Executive Engineer, otherwish no payment will be made under the contract.

- Contractors are advised to go through the Notice inviting tenders & the tender/PQ document thoroughly. Certificates, annexure. Enclosures as mentioned in the document will have to be submitted by the tenderers strictly in the prescribed format, at the time of submission of Technical/Finacial bid, failing which the contractor shall disqualify for the work and his finacial offer shall not be opened and no representation, appeal or objection, what so ever in this regard shall entertained by the department.

- It is mandatory to submit Manual by the contractor the list of on-going works/works in hand. If any work is found delayed beyond one year form the stipulated date of completion due to contractor’s fault, the contractor will be disqualified for the reason of poor performance.

- Contractor(s)/Firm(s) who has been blacklisted/debarred by any state government department, shall have no right to participate in tender.

- In case of conflict between “General condition of contract and the special condition” the trems of special conditions shall prevail.

ANNEXURE “M”

SINGLE TENDER

Annexure-1

Affidavit

I…………………………………………..S/o……………………………………………. Aged ……………. Years …………………….. resident …………………………… of ………………………………. (address …………………………………………………….) (For and on behalf of ………………………………………………………………….), do here by and herewith solemnly affirm/state on oath that :-

- All documents and Information furnished are correct in all respects to the best of mv knowledge and behalf.

- I have not suppressed or omitted any information as is required.

- I hereby authorize the P.H.E. officials to get all the documents verified from appropriate source (s)

- I am enclosing dully noterised agreement wise detail for balance work in hand on this date affidavit

Deponent

(……………………………)

Authorized signatory

For and on behalf of

……………………….

(affix seal)

Verification

I …………………………………………………………………….. do here by affirm that contents stated in Para 1 to 3 above are true to the best of my knowledge and believe and are based on my/our record.

Verified that this …………………….date of ……………………..200 at (Place)…………………… Seal of attestation by a Public Deponent.

Notary with date (…………………………………..)

Authorized Signature/

For and on behalf of……………………

(affix seal)

Annexure – 2

Information regarding minimum one similar work (D.I. Pipe line works)

| S.No. | Project Name | Name of Employee | Value of contract | Contract No. | Date of issue of work order | Stipulated Date Completion | Actual Date of completion | Value of work done | Remarks Remarks explaining reasons for Delay, if any and the amount of deductions due to delay also mention if any claim or dispute is pending in any forum. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |

Annexure – 3

Work performed on all classes of Civil Construction Works over the last 10 years.

| S.No. | Project Name | Name of Employee | Description of work. | Value of contract | Contract No. | Date of issue of work order | Stipulated Date Completion | Actual Date of completion | Year wise value of work done as per certificate of employer Rs. In Lacs. |  |  |  |  |  | Remarks Remarks explaining reasons for Delay, if any and the amount of deductions due to delay also mention if any claim or dispute is pending in any forum. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 12 | 16 |

Annexure – 4

Existing commitments and on going of civil engineering construction works.

| S. No. | Project Name | Description of work. | Contract No. & Year | Name & address of the employer | Value of contract (Rs. Lakhs) | Date of issue of work order | Stipulated period of completion in months | Anticipated date of completion ** | Value of work done up to date of issue of N.I.T. (Rs. Lakhs) | Probable value of works remaining to be completed (Rs. Lakhs) | Anticipated months required for completion of balance works. | Value of claims or dispute if any, pending |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  |  |  |  |  |  |  |  |  |  |

Annexure-N

Payment of wages through digital mode

Annexure-O

Annexure-P

GST order copy

Annexure-Q

Order Copy

Annexure-R

Switch Room

Boundary Wall

Chlorinator Room

FHTC

L-Section

Quaility Assiramce Plan (QAP) along with sampling plan of the Flow Control Valves