Schedule of Quantity

| RISK & COST FOR PIPED WATER SUPPLY SCHEME |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AT VILLAGE -MARKATOLA(GP) .GP-MARKATOLA BLOCK -MOHLA, DISTRICT – MOHLA-MANPUR-A.CHOWKI |  |  |  |  |  |  |  |
| Schedule of Quantity Providing, Laying & Jointing of UPVC Pipe line, OHT, Boundary wall, Chlorinator Room. Chlorinator System, Switch Room, Pump, FHTCs, Pumping Main. |  |  |  |  |  |  |  |
| S. No | SUB HEAD |  |  | QTY |  | UNIT |  |
| 1 | 2 |  |  | 3 |  | 5 |  |
| Schedule NO.- 01 |  |  |  |  |  |  |  |
| RISING MAIN |  |  |  |  |  |  |  |
| 1 | Earth work in excavation for pipe trench in ordinary soil areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 1.08 |  | cum. |  |
|  | CG,PHE, SOR Item no. 18.15 P/225 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 6 | x 0.6 x0.30 = | 1.08 |  |  |  |  |
|  |  | Total | 1.08 |  |  |  |  |
| 2 | Earth work in excavation for pipe trench in Hard soil areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 1.08 |  | cum. |  |
|  | CG,PHE, SOR Item no.18.16 P/226 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 6 | x 0.60 x 0.30 = | 1.08 |  |  |  |  |
|  |  | Total | 1.08 |  |  |  |  |
| 3 | Earth work in excavation for pipe trench in Laterite soil areas including dressing, watering, ramming and disposal of excavated earth lead up to 50m and lift up to 1.5m, disposal earth to be leveled, neatly dressed. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.18.17 P/226 |  |  | 1.08 |  | cum. |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 6 | x 0.60 x 0.3 = | 1.08 |  |  |  |  |
|  |  | Total | 1.08 |  |  |  |  |
| 4 | Earth work in excavation for pipe trench in all of rocks in areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 0.72 |  | cum. |  |
|  | Soft rock with or without blasting or bituminous pavement / cement concrete road. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.18.19.1 P/226 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 6 | x 0.60 x 0.2 = | 0.72 |  |  |  |  |
|  |  | Total | 0.72 |  |  |  |  |
| 5 | Demolishing cement concrete manually / by mechanical means including disposal of material within 50 m lead as per direction of engineer-in-charge. |  |  | 6.00 |  | cum. |  |
|  | Nominal concrete 1:3:6 or richer mix (i/c equivalent design mix) |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.18.29.1 P/227 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 50 | x 0.6 x0.20 = | 6.00 |  |  |  |  |
|  |  | Total | 6.00 |  |  |  |  |
| 6 | Providing, laying and jointing following P.V.C. pipes with solvent cement joint for 6, 8 and 10 kg/ sq. cm. pressures including testing of joints, cost of jointing materials etc. complete in all respect. |  |  | 6.00 |  | mtr. |  |
|  | CG,PHE, SOR Item no. 7.1 P/108 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 10 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 6 |  |  |  |  |
|  |  | Total | 6.00 |  |  |  |  |
| 7 | Providing, Solvent Cement Joints to PVC Pipes and fittings of 6, 8 and 10 Kg/Sq cm. Pressure including testing of joints and cost of jointing materials (i.e. socket, coupler & solvent cement) |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.3 P/109 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 10 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 10 |  |  |  |  |
|  |  | Total | 10.00 | 10 |  | each |  |
| 8 | Providing and laying in position including testing following PVC bends suitable for 6, 8 and 10 Kg/Sq. cm. pressure pipes. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.5 P/109 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 10 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 2 |  |  |  |  |
|  |  | Total | 2.00 | 2 |  | each |  |
| 9 | Providing and laying in position including testing following PVC flanged tail pieces suitable for 6, 8 and 10 Kg./Sq. cm. Pressure pipes. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.7 P/110 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 10 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 2 |  |  |  |  |
|  |  | Total | 2.00 | 2 |  | each |  |
| 10 | Providing and laying in position including testing PVC coupler suitable for 6, 8 and 10 Kg/Sq. cm. Pressure pipes. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.9 P/110 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 10 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 3 |  |  |  |  |
|  |  | Total | 3.00 | 3 |  | each |  |
| 11 | Filling available excavated earth in trenches plinth sides of foundation is layers not exceeding 20 cm. In depth i/c consolidation of earth layer by ramming watering lead upto 50 m and lift upto 1.5 mt. I/c all kind of soils. |  |  | 3.96 |  | cum. |  |
|  | CG,PHE, SOR Item no.18.22 P/227 |  |  |  |  |  |  |
|  | Marakatola |  | 3.96 |  |  |  |  |
|  |  | Total | 3.96 |  |  |  |  |
| 12 | Providing and laying mechanically mixed cement concrete with 20 mm nominal size graded stone aggregat. C M-15 |  |  | 6.00 |  | cum. |  |
|  | CG,PHE, SOR Item no. 18.41 P/229 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 50 | x 0.6 x0.20 = | 6.00 |  |  |  |  |
|  |  | Total | 6.00 |  |  |  |  |
| 13 | Providing & fixing following cast iron double flanged single door reflux (non return) valves including jointing & testing with cost of jointing material such as bolts,nuts and rubber insertion all complete as per IS :5312 (Part I) Class PN-1.0 |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no 8.4a P/117 |  |  |  |  |  |  |
|  | 50 mm dia |  |  |  |  |  |  |
|  | Marakatola |  | 2 |  |  |  |  |
|  |  | Total | 2.00 | 2.00 |  | each |  |
| 14 | Provision for Jointing of TW to Rising Main with cost of Material/ specials such as GI Union / CI Flange,GI Reducer UPVC MTA FTA etc. as per requiement of site i/c cost of labour etc. complete as per approved specification and as directed by Engineer in charge. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no 19.19 P/246 |  |  |  |  |  |  |
|  | Marakatola |  | 1 |  |  |  |  |
|  |  | Total | 1.00 | 1.00 |  | Job |  |
| Schedule NO.- 02 |  |  |  |  |  |  |  |
| DISTRIBUTION NETWORK |  |  |  |  |  |  |  |
| S. No | SUB HEAD |  |  | QTY |  | UNIT |  |
| 1 | 2 |  |  | 3 |  | 5 |  |
| 1 | Earth work in excavation for pipe trench in ordinary soil areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 4.14 |  | cum. |  |
|  | CG,PHE, SOR Item no. 18.15 P/225 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 23 | x 0.60 x0.3 = | 4.14 |  |  |  |  |
|  |  | Total | 4.14 |  |  |  |  |
| 2 | Earth work in excavation for pipe trench in Hard soil areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 4.14 |  | cum. |  |
|  | CG,PHE, SOR Item no.18.16 P/226 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 23 | x 0.60 x 0.30 = | 4.14 |  |  |  |  |
|  |  | Total | 4.14 |  |  |  |  |
| 3 | Earth work in excavation for pipe trench in Laterite soil areas including dressing, watering, ramming and disposal of excavated earth lead up to 50m and lift up to 1.5m, disposal earth to be leveled, neatly dressed. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.18.17 P/226 |  |  | 4.14 |  | cum. |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 23 | x 0.60 x 0.30 = | 4.14 |  |  |  |  |
|  |  | Total | 4.14 |  |  |  |  |
| 4 | Earth work in excavation for pipe trench in all of rocks in areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 2.76 |  | cum. |  |
|  | Soft rock with or without blasting or bituminous pavement / cement concrete road. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.18.19.1 P/226 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 23 | x 0.60 x 0.20 = | 2.76 |  |  |  |  |
|  |  | Total | 2.76 |  |  |  |  |
| 5 | Demolishing cement concrete manually / by mechanical means including disposal of material within 50 m lead as per direction of engineer-in-charge. |  |  | 48.00 |  | cum. |  |
|  | Nominal concrete 1:3:6 or richer mix (i/c equivalent design mix) |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.18.29.1 P/227 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 400 | x 0.60 x 0.2 = | 48.00 |  |  |  |  |
|  |  | Total | 48.00 |  |  |  |  |
| 6 | Providing, laying and jointing following P.V.C. pipes with solvent cement joint for 6, 8 and 10 kg/ sq. cm. pressures including testing of joints, cost of jointing materials etc. complete in all respect. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.1 P/108 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 0.00 |  |  |  |  |
|  |  | Total | 0.00 | 0.0 |  | mtr. |  |
|  | 110 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 23.00 |  |  |  |  |
|  |  | Total | 23.00 | 23.0 |  | mtr. |  |
| 8 | Providing, Solvent Cement Joints to PVC Pipes and fittings of 6, 8 and 10 Kg/Sq cm. Pressure including testing of joints and cost of jointing materials (i.e. socket, coupler & solvent cement) |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.3 P/109 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 10.0 |  |  |  |  |
|  |  | Total | 10.00 | 10 |  | each |  |
|  | 110 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 5.0 |  |  |  |  |
|  |  | Total | 5.00 | 5 |  | each |  |
| 9 | Providing and laying in position including testing following PVC bends suitable for 6, 8 and 10 Kg/Sq. cm. pressure pipes. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.5 P/109 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 6.0 |  |  |  |  |
|  |  | Total | 6.00 | 6 |  | each |  |
|  | 110 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 3.0 |  |  |  |  |
|  |  | Total | 3.00 | 3 |  | each |  |
| 10 | Providing and laying in position including testing following PVC Tees, suitable for 6, 8 and 10 Kg/Sqm. Pressure pipes. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.6 P/110 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 5.0 |  |  |  |  |
|  |  | Total | 5.00 | 5 |  | each |  |
|  | 110 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 2.0 |  |  |  |  |
|  |  | Total | 2.00 | 2 |  | each |  |
| 20 | Providing & laying in position including testing following cast iron plain ended standard specials confirming to IS/5531/1988 (Reaffirmed 2002). |  |  |  |  |  |  |
|  | Cast Iron Adopter (Flange Spigot) |  |  |  |  |  |  |
|  | 80 mm dia |  |  |  |  |  |  |
|  | Marakatola |  | 4.0 |  |  |  |  |
|  |  | Total | 4.00 | 4 |  | each |  |
| 11 | Providing and laying in position including testing following PVC flanged tail pieces suitable for 6, 8 and 10 Kg./Sq. cm. Pressure pipes. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.7 P/110 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 4.0 |  |  |  |  |
|  |  | Total | 4.00 | 4 |  | each |  |
|  | 110 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 2.0 |  |  |  |  |
|  |  | Total | 2.00 | 2 |  | each |  |
| 12 | Providing and laying in position including testing PVC coupler suitable for 6, 8 and 10 Kg/Sq. cm. Pressure pipes. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.9 P/110 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 3.0 |  |  |  |  |
|  |  | Total | 3.00 | 3 |  | each |  |
|  | 110 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 1.0 |  |  |  |  |
|  |  | Total | 1.00 | 1 |  | each |  |
| 13 | Providing and laying in position including testing following PVC end Cap (plugs) suitable for 6, 8 and 10 Kg/Sq cm. Pressure pipes. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 7.8 P/110 |  |  |  |  |  |  |
|  | 90 mm dia UPVC 6 kg./cm2 |  |  |  |  |  |  |
|  | Marakatola |  | 6.0 |  |  |  |  |
|  |  | Total | 6.00 | 6 |  | each |  |
| 14 | Providing and fixing PVC Djoint (Detachable joint) in PVC pipe line suitable for classes up to 10kg/sqcm Pressure pipes. i/c nut bolt, cutting of pipe, testing of joints etc complete. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.7.12 P/111 |  |  |  |  |  |  |
|  | 90 mm Dia |  |  |  |  |  |  |
|  | Marakatola |  | 3 |  |  |  |  |
|  |  | Total:- | 3.0 | 3 |  | Mtr |  |
| 22 | Providing laying and jointing of following galvanized Iron (MS) Pipes with specials (such as bends, elbows, tees etc) class light, medium & heavy including testing of joints, cost of pipes, specials and jointing materials all complete. Pipes and sockets conforming to IS:1239/2011 Part- II. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 6.1 P/89 |  |  |  |  |  |  |
|  | 100 mm dia class mudiam (For culvert crossing) |  |  |  |  |  |  |
|  | Marakatola |  | 50 |  |  |  |  |
|  |  | Total | 50.0 | 50 |  | RM |  |
| 23 | Filling available excavated earth in trenches plinth sides of foundation is layers not exceeding 20 cm. In depth i/c consolidation of earth layer by ramming watering lead upto 50 m and lift upto 1.5 mt. I/c all kind of soils. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.18.22 P/227 |  |  |  |  |  |  |
|  | Qty As per Item No. 1 to 4 |  |  |  |  |  |  |
|  | Marakatola |  | 15.18 |  |  |  |  |
|  |  | Total | 15.18 | 15.18 |  | cum. |  |
| 24 | Providing and laying mechanically mixed cement concrete with 20 mm nominal size graded stone aggregat. C M-15 |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no. 18.41 P/229 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 400 | x 0.6 x 0.20 = | 48 |  |  |  |  |
|  |  | Total | 48.00 | 48.00 |  | cum. |  |
| 26 | Providing & fixing of following Ductile iron double flanged sluice valves as per I.S.:14846-2000 fitted with cap including jointing & testing with cost of jointing material such as bolts, nuts, rubber insertions etc. all complete. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no8.4.41 P/73 |  |  |  |  |  |  |
|  | 80 mm dia |  |  |  |  |  |  |
|  | Marakatola |  | 3 |  |  |  |  |
|  |  | Total | 3.00 | 3.00 |  | each |  |
|  | 100 mm dia |  |  |  |  |  |  |
|  | Marakatola |  | 1 |  |  |  |  |
|  |  | Total | 1.00 | 1.00 |  | each |  |
| 27 | CONSTRUCTION OF RCC SLUICE VALVE CHAMBER SIZE 1.0mX1.0mX1.0m |  |  |  |  |  |  |
|  | Marakatola |  | 4 |  |  |  |  |
|  |  | Total | 4.00 | 4.00 |  | As per Sub Estimate |  |
| 29 | Provision for jointing of Rising main to supmp well/OHT and OHT to Distribution pipe line with cost of material/specials such as Bends, MTA as per rerquirement of site i/c cost of labour with excavation, labour as per requirement complete as per approved specification and as directed by Engineer in charge. |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no 19.20 P/246 |  |  |  |  |  |  |
|  | Marakatola |  | 1.00 |  |  |  |  |
|  |  | Total | 1.00 | 1.00 |  | Job |  |
| Schedule NO.- 03 |  |  |  |  |  |  |  |
|  | FHTCs (Funtional Household Tap Connection |  |  |  |  |  |  |
| 1 | Providing and constructing one stand post as per type design with excavation 15 cm thick PCC 1:3:6 bedding 20 mm thick PCC 1:2:4 convert for platform of 1.5mx1.5m with side curb and bucket rest of 80 mm dia. 160mm dia PVC pipe central post duly filled therein with C.C. 1:2:4, 2.2m long, 15 mm dia medium G.I. pipe. From point of tapping to stand post additional 1620 (20 mm dia) composite pipe 6.0 m long. Providing and fixing of 15 mm dia, one steel water tap, one flow control valve SS, 5 LPM complete together with all labour and material charges as per drawing and as directed by Engineer-in-charge when good foundation in available. Rate includes draining arrangement by excavating open gutters complete |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No.253/19.35 |  |  |  |  |  |  |
|  | Marakatola |  | 1 |  |  |  |  |
|  |  | Total | 1.00 | 1.00 |  | Each |  |
| 2 | Earth work in excavation for pipe trench in ordinary soil in areas including dressing, watering and ramming & disposal of excavated earth lead up to 50 m- & lift up to 1.5 m. disposal earth to be leveled, neatly dressed |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No.226/18.19 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 20 | x0.25x0.15 | 0.750 |  |  |  |  |
|  |  | Total | 0.75 | 0.75 |  | cum. |  |
| 3 | Earth work in excavation for pipe trench in all kinds of rocks in areas including dressing, stacking of useful material and disposal of unserviceable one up to 50 m lead and lift up to 1.5 m. (a) Soft rock with or without blasting or bituminous pavement /cement concrete road. |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No.226/18.19 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 20 | x0.25x0.15 | 0.750 |  |  |  |  |
|  |  | Total | 0.75 | 0.75 |  | cum. |  |
| 4 | Demolishing cement concrete manually / by mechanical means including disposal of material within 50 m lead as per direction of engineer-in-charge. |  |  |  |  |  |  |
|  | Nominal concrete 1:3:6 or richer mix (i/c equivalent design mix) |  |  |  |  |  |  |
|  | CG,PHE, SOR Item no.18.29.1 P/227 |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 5 | x0.25x0.25 | 0.313 |  |  |  |  |
|  |  | Total | 0.31 | 0.31 |  | cum. |  |
| 5 | Providing and fixing Polyethylene- Aluminum Polyethylene (PE-AL-PE) Composite Pressure pipes Conforming to IS: 15450-2004 U.V. Stablished with carbon black having thermal stability for hot & cold water supply, capable to with stand temperature up to 800 C including all specials and fittiings of composite material (engineering plastic. gland and brass insert wherever required) e.g. elbws, tees. reducers, couplers and connectors wiith clamp at 1m spacing. This includes testing of joints complete as per the directions of engineer- incharge (Concealed work including cutting chases& making good the wall etc ) |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No.254/19.37.2 |  |  |  |  |  |  |
|  | 1620 ( 20mm OD ) Pipe |  |  |  |  |  |  |
|  | Marakatola |  | 700.00 |  |  |  |  |
|  |  | Total | 700.00 | 700 |  | Rm |  |
| 6 | Filling available excavated earth in trenches, lead up to 50m and lift up to 1.5m in all kind of soil excluding watering and ramming. |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No. 227/18/23 |  |  |  |  |  |  |
|  | (B) in Hard Soil for Road |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 20 | x0.25x0.30 | 1.500 |  |  |  |  |
|  |  | Total | 1.50 | 1.50 |  | cum. |  |
| 7 | Providing & laying mechanically mixed cement concrete 20mm nominal size graded crushed stone excluding cost of centering & shuttering. |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No. 229/18/41.1.1 |  |  |  |  |  |  |
|  | In Plinth & foundation |  |  |  |  |  |  |
|  | For Cutting 1:3:6 (M-10) |  |  |  |  |  |  |
|  | Marakatola |  |  |  |  |  |  |
|  | 5 | x0.25x0.25 | 0.313 |  |  |  |  |
|  |  | Total | 0.31 | 0.31 |  | cum. |  |
| Schedule NO.- 04 |  |  |  |  |  |  |  |
| Providing and installation of Submersible pump |  |  |  |  |  |  |  |
| 1 | Supplying & Installation of Energy efficiant five star BEE rating ISI Marked required capacity of Three Phase, 50 Hz, 415V, deep well submersible pump Steel body, suitable for 6"tube well with Control Panel Starter suitable for Submersible pump with dry run protection, single phase preventer, connections, including clamps, bore cap etc. as required as per specifications but excluding pipe and connection cable. |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No. 354/34.20.106 (iv) |  |  |  |  |  |  |
|  | 5 H.P. with 10to11stages,HeadMt.101-40 Discharge LPM 60-270 |  |  |  |  |  |  |
|  | Marakatola |  | 1.00 |  |  |  |  |
|  |  | Total | 1.00 | 1 |  | each |  |
| 2 | Providing laying and jointing of following galvanized Iron (MS) Pipes with specials (such as bends, elbows, tees etc) class light, medium & heavy including testing of joints, cost of pipes, specials and jointing materials all complete. Pipes and sockets conforming to IS:1239/2011 Part-II. |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No. 89/6.1 |  |  |  |  |  |  |
|  | 50mm Dia |  |  |  |  |  |  |
|  | Marakatola |  | 50.00 |  |  |  |  |
|  |  | Total | 50.00 | 50 |  | RM |  |
| 3 | Supplying and laying of submersible flat cable ISImarked 3 core copper wire of suitable size with proper clamping of approved make. |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No. 354/34.20.107 (ii) |  |  |  |  |  |  |
|  | 4.0 Sq.mm.multi strand |  |  |  |  |  |  |
|  | Marakatola |  | 100.00 |  |  |  |  |
|  |  | Total | 100.00 | 100 |  | Per meter |  |
| 4 | Supplying and laying of approved make stainless steel wire rope 6 mm thick complete with binding for support of pump and motor |  |  |  |  |  |  |
|  | PHE SOR P.No /Item No. 354/ 34.20.109 |  |  |  |  |  |  |
|  | Marakatola |  | 100.00 |  |  |  |  |
|  |  | Total | 100.00 | 100 |  | Per meter |  |
| Schedule NO.- 05 |  |  |  |  |  |  |  |
| RCC OVER HAED TANK (MARKATOLA) |  |  |  |  |  |  |  |
| S. No | SOR Re. | SUB HEAD |  | QTY |  | UNIT |  |
| 1 | 2 | 3 |  | 4 |  | 6 |  |
| RCC OVER HAED TANK |  |  |  |  |  |  |  |
| 1 | Designing (structurally & aesthetically), and constructing RCC elevated service reservoirs of following capacity with RCC staging consisting of columns, internal and external bracings spaced vertically as per staging of the ESR. including excavation in all types of strata, foundation concrete, cement plaster with water proofing compound to the inside face of the container including refilling & disposing off the surplus stuff within a lead of 50 meters, all labour and material charges including lowering, laying, erecting, hoisting and jointing of pipe assembly of inlet, outlet, scour, overflow and bypass arrangements as per departmental design, providing and fixing accessories such as Aluminum Ladder, C.I. manhole frame and covers water level indicators, lightening conductor, G.I. pipe railing around walk way and top slab, providing RCC staircase from ground level to balcony level along with columns and from balcony level roof top level along with container wall, M.S.grill gate of 2 mtr. height with locking arrangement of approved design RCC chambers for all valves, ventilating shafts, providing and applying three coats of weather coat paints to the structure including roof slab epoxy painting to internal surface & anti termite treatment for underground parts of the structure and giving satisfactory water tightness test as per I.S. code, The job to include painting the name of the scheme and other details on the reservoir as per thedirections of Engineer-in-Charge. |  |  |  |  |  |  |
|  | Construction of R.C.C. over head tank 40 KL capacity on 12 meter staging including drawing, design, soil SBC testing with all vertical pipe fitting and specials. |  |  |  |  |  |  |
|  | CGPHESOR Page no. 295 Item no 14.20 (23.2.3) |  |  |  |  |  |  |
|  | R.C.C. over head tank 25 KL cost |  |  | 1 |  | Each |  |
|  | Add for 15000 Liters |  |  | 15000 |  | LTR |  |
|  | 40 KL 12 mtr Staging |  |  |  |  |  |  |
| Schedule NO.- 06 |  |  |  |  |  |  |  |
| SWITCH ROOM |  |  |  |  |  |  |  |
| S.No | Item |  |  | Qty. |  | Unit |  |
| 1 | Excavation for all types and sizes of foundations, trenches and drains or for any other purpose including disposal of excavated stuff upto 1.5 m lift and lead upto 50m (at least 5m away from the excavated area), including dressing and leveling of pits. |  |  |  |  |  |  |
|  | In all types of soils. |  |  | 1.68 |  | Cum |  |
| 2 | Excavation for all types and sizes of foundations, trenches and drains or for any other purpose including disposal of excavated stuff upto 1.5 m lift and lead upto 50m (at least 5m away from the excavated area), including dressing and leveling of pits. |  |  |  |  |  |  |
|  | In ordinary rocks. |  |  | 1.68 |  | Cum |  |
| 3 | Providing and laying nominal mix plain cement concrete with crushed stone aggregate using concrete mixer in all works upto plinth level excluding cost of form work. |  |  |  |  |  |  |
|  | 1:3:6 ( 1 cement : 3 coares sand :6 graded stone aggregate 40 mm nominal size). (Base cone). M.10 - 1:3:6 |  |  | 0.72 |  | Cum. |  |
| 4 | Providing and placing in position reinforcement for R.C.C. work including straightening, cutting, bending, binding etc. complete as per drawings including cost of binding wire in foundation and plinth all complete: |  |  |  |  |  |  |
|  | Cold twisted bar / Hot rolled deformed steel |  |  | 50 |  | Kg. |  |
| 5 | Providing and laying design mix reinforced cement concrete with crushed graded stone aggregate 20mm nominal size using batching plant, transit mixer and concrete pump, in all works upto plinth level excluding cost of form work |  |  |  |  |  |  |
|  | M- 20 Grade |  |  | 0.542 |  | Cum |  |
| 6 | Providing and fixing form work including centring, shuttering, strutting, staging, propping bracing etc. complete and including its removal at all levels, for: |  |  |  |  |  |  |
|  | (i) Beams, lintels, cantilevers & walls |  |  | 4.92 |  | Sqm. |  |
| 7 | Brick work with modular fly-ash lime bricks (FaLG bricks) confirming to IS:12894-2002 of class designation 4.0 in foundation and plinth in : |  |  |  |  |  |  |
|  | (a) Cement Mortar 1:6 (1 Cement : 6 Coarse Sand) |  |  | 5.15 |  | Cum |  |
| 8 | Extra for brick work in superstructure above plinth level for every floor or part thereof in addition to rate for foundation and plinth: |  |  | 2.25 |  | cum |  |
| 9 | Providing and laying damp proof course (upto 50mm thick) with plain cement concrete 1:2:4 (1 cement : 2 coarse sand : 4 graded crushed stone aggregate 20mm nominal size) including form work. |  |  | 0.084 |  | Cum |  |
| 10 | Providing and filling in plinth with sand/ Crusher dust and hard moorum under floor in layers not exceeding 20cm in depth consolidating each deposited layer by ramming and watering, including dressing etc. complete. |  |  | 0.43 |  | Cum. |  |
| 11 | Providing and making 6mm thick cement plaster of mix: |  |  |  |  |  |  |
|  | (In Cement Mortar 1:6 (1 cement : 6 fine sand) |  |  | 23.3 |  | Cum |  |
| 12 | Structural steel work in single section including cutting, hoisting, fixing in position and applying a priming coat of red oxide zinc chromate primer.(Door frame) |  |  |  |  |  |  |
|  | (b) In Flat Iron. |  |  | 25 |  | kg. |  |
| 13 | Providing and making 15mm thick cement plaster on the rough side of single or half brick wall of mix: |  |  |  |  |  |  |
|  | (In Cement Mortar 1:6 (1 cement : 6 fine sand) |  |  | 9.4 |  | Sqm. |  |
| 14 | Painting new work ( two or more coats) to give even shade with |  |  |  |  |  |  |
|  | Premium synthetic enamel paint |  |  | 4.23 |  | sqm |  |
| 15 | Providing and fixing steel door/ window with M.S. sheet 1mm thick, frame of angle iron, diagonal braces of angle/ flat iron of suitable size, 3.00 mm M.S. gusset plates at junctions and corners, all necessary fittings complete including applying a priming coat of red oxide zinc chromate |  |  | 60 |  | Kg. |  |
|  | primer. |  |  |  |  |  |  |
| 16 | Cement concrete flooring with cement concrete 1:2:4 (1 cement : 2 coarse sand : 4 graded stone aggregate 20mm) finished with a floating coat of neat cement. |  |  |  |  |  |  |
|  | 50 mm thick |  |  | 1.44 |  | sqm. |  |
| 17 | Providing and laying design mix reinforced cement concrete with crushed graded stone aggregate 20mm nominal size using batching plant, transit mixer and concrete pump, in all works upto plinth level excluding cost of form work. |  |  |  |  |  |  |
|  | M-20 Grade |  |  | 0.121 |  | Cum |  |
| 18 | Distempering with acrylic washable distemper to give an even shade, On new work (Two or more coats) |  |  | 36 |  | Sqm. |  |
| 19 | Provision for electrical fittings. |  |  | L.S. |  |  |  |
|  | Marakatola |  | 1.00 |  |  |  |  |
|  |  | Total | 1.00 | 1.00 |  | each |  |
| Schedule NO.- 07 |  |  |  |  |  |  |  |
| BOUNDARY WALL |  |  |  |  |  |  |  |
| 1 | Excavation for all types and sizes of foundations, trenches and drains or for any other purpose including disposal of excavated stuff. upto 1.5 m lift and lead upto 50m (at least 5m away from the excavated area), including dressing and leveling of pits |  |  |  |  |  |  |
|  | (PWD SOR, .No. 1.1.1/P-9) |  |  |  |  |  |  |
|  | 15X0.90X0.90X0.90 = 10.94 |  |  | 10.94 |  | Cum |  |
| 2 | Providing and laying nominal mix plain cement concrete with crushed stone aggregate using concrete mixer in all works upto plinth level excluding cost of form work. |  |  |  |  |  |  |
|  | (PWD SOR, .No. 3.1.3/P-23) |  |  |  |  |  |  |
|  | 15X0.90X0.90X0.10 = 1.22 |  |  | 1.22 |  | Cum. |  |
| 3 | Providing and placing in position cold twisted steel and hot rolled deformed steel reinforcement for R.C.C. work including cutting, bending, binding etc. complete as per drawings including cost of binding wire and including all wastages etc. complete. |  |  |  |  |  |  |
|  | (PWD SOR, .No. 3.12.1/P-24) |  |  |  |  |  |  |
|  | 100mm dia tar steel @ 0.15m. c/c |  |  |  |  |  |  |
|  | Jalli - 2no.X 2 X6X0.80X0.62 = 160.70kg |  |  |  |  |  |  |
|  | Column -27no.X4X3.20X0.62 = 214.27kg |  |  |  |  |  |  |
|  | Stirrups - 27no.X17no.X0.70X0.398 =127.87kg |  |  |  |  |  |  |
|  | Beam – 2X78.30X0.62 = 97.09 |  |  |  |  |  |  |
|  | 2X79.30X0.89 = 141.15 |  |  |  |  |  |  |
|  | 2X78.30X0.89 = 139.37g |  |  |  |  |  |  |
|  | Stirrups – 522X 0.90X0.398 = 186.98kg |  |  |  |  |  |  |
|  | 1067.43kg |  |  | 1067 |  | Kg. |  |
| 4 | Providing and fixing formwork i/c centering, shuttering, strutting, staging, propping, bracing etc. complete and including its removal at all levels, |  |  |  |  |  |  |
|  | Foundations, footings bases of columns plinth beam, curtain wall in any shape and size and all type of wall below plinth level. |  |  |  |  |  |  |
|  | (PWD SOR, .No. 2.1.1/P-16 & 2.1.5/ P-16) |  |  |  |  |  |  |
|  | Foundations-27X4X0.90X0.10 = 9.72 m2 |  |  |  |  |  |  |
|  | 27X4X0.85X0.30 = 27.54 |  |  |  |  |  |  |
|  | 27X4X0.20X0.25 = 05.40 |  |  |  |  |  |  |
|  | Beam – 2X78.30X0.30 = 46.98 |  |  |  |  |  |  |
|  | 89.64 |  |  | 89.64 |  | Sqm. |  |
|  | colunm – 27X4X0.20X2.10 = 45.36 |  |  | 45.36 |  | Sqm. |  |
| 5 | Providing and laying nominal mix reinforced cement concrete with crushed stone aggrigat using concrete mixer in all works upto plinth level excluding from work. 1:1:1.5:3 cement concrete |  |  |  |  |  |  |
|  | (PWD SOR, .No. 3.2.1/P-23 |  |  |  |  |  |  |
|  | 27X0.85X0.85X0.30 = 5.852 m3 |  |  |  |  |  |  |
|  | 27X[(0.202+0.852) /2] X0.25m. = 2.573 m3 |  |  |  |  |  |  |
|  | 27X0.20X0.20X2.35 = 2.538m3 |  |  |  |  |  |  |
|  | Beam – 1X 78.30X0.20X0.30 = 4.698 |  |  |  |  |  |  |
|  | 15.661 |  |  | 15.66 |  | Cum |  |
| 6 | Brick work with modular fly-ash lime bricks (FaLG bricks) confirming to IS:12894-2002 of class designation 4.0 in foundation and plinth in : |  |  |  |  |  |  |
|  | (PWD SOR, .No. 7.5.4/P-45 |  |  |  |  |  |  |
|  | Cement Mortar 1:6 (1cement:6 sand) |  |  |  |  |  |  |
|  | 1no.X73.30X0.20X1.80m 26.388 |  |  | 26.39 |  | cum |  |
| 7 | Extra for brick work in superstructure above plinith level for every floor or part theireof in addition to rate for foundation plinth: |  |  | 26.39 |  | cum |  |
|  | (PWD SOR, .No. 7.6/P-45 |  |  |  |  |  |  |
| 8 | 15mm thick cement plaster on the rough side of single or half brick wall of mix: |  |  |  |  |  |  |
|  | (PWD SOR, .No. 11.3.4/P-103 |  |  |  |  |  |  |
|  | Cement Mortar 1:6 (1cement:6 sand) |  |  |  |  |  |  |
|  | 2X73.30X1.80 = 263.88 |  |  | 263.9 |  | cum |  |
| 9 | Finishing walls with water proofing cement paint of required shade to give an even shade. |  |  |  |  |  |  |
|  | (PWD SOR, .No. 14.14.1/P-133 |  |  |  |  |  |  |
|  | In side- 81.40X2.10 = 170.94 |  |  |  |  |  |  |
|  | Outer -83.00X2.10 = 174.30 |  |  |  |  |  |  |
|  | Swith room – = 69.76 Total 415 M2 |  |  | 415 |  | Sqm. |  |
| 10 | Steel work welded in built up sections/ramed work including cutting, hoistiong, fixing position and applying a priming coat of red oxide zinc chromate primer. |  |  |  |  |  |  |
|  | (PWD SOR, .No. 9.6.1/P-80 |  |  |  |  |  |  |
|  | Steel gate – approximate weight – 200kg |  |  | 200 |  | kg |  |
| 11 | Painting on new work (two or more coats) to give an even shade with: |  |  |  |  |  |  |
|  | (PWD SOR, .No. 14.9.1/P-132 |  |  |  |  |  |  |
|  | Sating synthetic enamel paint (top most approved branded quality) |  |  |  |  |  |  |
|  | 2X5.0X2.0 = 20.00sqm. |  |  | 20 |  | sqm. |  |
| 12 | Providing and filling in plinth with sand/Crusher dust and hard moorum under floor in laryers not exceeding 20cm in depth consolidating each deposited layer by ramming and watering, including dressing etc. complete. |  |  |  |  |  |  |
|  | (PWD SOR, .No. 1.18/P-11 |  |  |  |  |  |  |
|  | 21.60X21.60 = 466.56 |  |  |  |  |  |  |
|  | (-) 3.50X3.50 = 12.65 |  |  |  |  |  |  |
|  | (-) 0.785X6.50 = 33.16 420.75X0.30 =130.50 |  |  |  |  |  |  |
|  | 19.683-(2.187+5.852+2.573+0.270) =8.80 |  |  |  |  |  |  |
|  | 139.3 |  |  | 139.3 |  | cum. |  |
| 13 | Providing and fixing broken glass 100 mm high in spacing not more than 40 mm both ways and laid in 50 mm thick (average) cement mortar 1:4 (1 cement : 4 coarse sand ) over compound walls, prapet walls and the like. |  |  | 18.65 |  | sqm |  |
|  | (PWD SOR, .No. 9.38/P-84 |  |  |  |  |  |  |
|  | Marakatola |  | 1.00 |  |  |  |  |
|  |  | Total | 1.00 | 1.00 |  | each |  |
| Schedule NO.- 08 |  |  |  |  |  |  |  |
| CHLORINATOR SYSTEM |  |  |  |  |  |  |  |
| 1 | Providing, erecting, commissioning and giving test & trial for a period of one month including one year free maintenance after commissioning of Electro chlorinator capable of generating chlorine from common salt by electrolysis using electrodes in form of sodium hypo chlorite solution containing 6-8 gms/lit of available chlorine in batch or continuous process and capable of providing 8 hrs storage of hypochlorite in case of power failure. The electro chlorinator shall comprise of following: ● Electrolytic cell consisting dimensionally stable electrodes made from Gr I Titanium sheet with multi metal oxide coating. Electrolyzer tank made from PVC-FRP or Acrylic ● Power pack consisting of transformer rectifier for generating suitable DC current from AC supply along with the control switch for dosing pumps etc. through MCB's contacts, relays and wiring. ● Control panel for the electro chlorinator consisting of DC voltage and current display income phase status unit on-off switches fuses etc. ● Dosing tank of suitable capacity made from PVC/FRP. ● Dosing pumps of specials quality (1W+1S) suitable to handle hypo chlorite solution. ● Entire chlorine solution pipeline shall be of PVC. ● Chlorine test kit suitable to measure residual chlorine up to 5 ppm. |  |  |  |  |  |  |
|  | 24 gms/hr |  |  | 1job |  | each |  |
|  | Marakatola |  | 1.00 |  |  |  |  |
|  |  | Total | 1.00 | 1.00 |  | each |  |
| Schedule NO.- 09 |  |  |  |  |  |  |  |
| CHLORINATOR ROOM |  |  |  |  |  |  |  |
| 1 | Excavation for all types and sizes of foundations, trenches and drains or for any other purpose including disposal of excavated stuff upto 1.5 m lift and lead upto 50m (at least 5m away from the excavated area), incl. dressing & leveling of pits. |  |  |  |  |  |  |
|  | In all types of soil |  |  |  |  |  |  |
|  | Steps -- |  |  |  | 7.13 |  | Cum. |
| 2 | Providing and filling in plinth with sand/ Crusher dust and hard moorum under floor in layers not exceeding 20cm in depth consolidating each deposited layer by ramming and watering, including dressing etc. complete. |  |  |  |  |  |  |
|  | Code -- 1.18 , Page -11 |  |  |  | 1.44 |  | Cum. |
| 3 | Providing and laying nominal mix cement concrete with crushed stone aggregate using concrete mixer in foundation, plinth and at ground level excl. cost of form work. |  |  |  |  |  |  |
|  | HALL -- |  |  |  | 0.68 |  | Cum. |
| 4 | Providing and fixing formwork including centering, shuttering, strutting, staging, propping bracing etc. complete and including its removal at all levels, for: |  |  |  |  |  |  |
|  | Foundations, footings, base of columns and plinth beam in any shape and size. |  |  |  |  |  |  |
|  | Ground beam Sides --- |  |  |  | 7.92 |  | Sqm. |
| 5 | Columns, Pillars, Piers & likes- rectangular |  |  |  |  |  |  |
|  | or square in shape |  |  |  | 12.64 |  | Sqm |
|  | Code -- 2.1.5 , Page -- 16 . |  |  |  |  |  |  |
| 6 | Suspended floors, roofs, access platform, |  |  |  |  |  |  |
|  | balconies (plain surfaces) and shelves |  |  |  |  |  |  |
|  | (cast in situ) Code -- 2.1.8 , Page -- 15 . |  |  |  |  |  |  |
|  | Lintal Beam Side - |  |  |  | 3.84 |  |  |
|  | Chhajja -- |  |  |  | 1.62 |  |  |
|  | MAIN SLAB -- |  |  |  | 7.29 |  |  |
|  |  |  |  |  | 12.75 |  | Sqm. |
| 7 | Providing and laying nominal mix reinforcement cement concrete with crushed stone aggregate using concrete mixer in all works upto floor five level excl. cost of reinforcement and form work. |  |  |  |  |  |  |
|  | 1:1½:3 (1 cement : 1½ coarse sand : 3 graded stone aggregate 20mm nominal size). |  |  |  |  |  |  |
|  | Code -- 3.2.1, Page -- 23 . |  |  |  | 0.49 |  |  |
|  | 4x(0.9+0.30)/2x0.45 =1.57 cum. |  |  |  | 1.57 |  |  |
|  | 4x0.2x0.2x0.50= 0.08 cum |  |  |  | 0.08 |  |  |
|  | Ground Beam |  |  |  | 0.58 |  |  |
|  | Lintel |  |  |  | 0.38 |  |  |
|  | Chhajja -- |  |  |  | 0.13 |  |  |
|  | Main Slab |  |  |  | 0.58 |  |  |
|  |  |  |  |  | 3.80 |  | Cum |
| 8 | Providing and placing in position reinforcement for R.C.C. work incl. straightening, cutting, bending, binding etc. complete as per drawings incl.cost of binding wireall complete: |  |  |  |  |  |  |
|  | Cold twisted bar / Hot rolled deformed steel |  |  |  |  |  |  |
|  | Code -- 3.12.4 , Page -- 24 . |  |  |  |  |  |  |
|  | Taken 80Kg per Cum. Of R.C.C. Qty. i.e. |  |  |  | 304.13 |  | kg |
| 9 | Brick work with modular fly-ash lime bricks (FALG Bricks) confirming to IS:12894-2002 of class designation 40 in found & plinth in: |  |  |  |  |  |  |
|  | Cement Mortar 1:5 (1 cement : 5 coarse sand) |  |  |  |  |  |  |
|  | Code -- 7.5.3 , Page -- 45 . |  |  |  | 0.33 |  | Cum. |
| 10 | Brick work with modular fly-ash lime bricks (FaLG Bricks) confirming to IS:12894-2002 of class designation 4.0 in foundation and plinth in: |  |  |  |  |  |  |
|  | Extra for brick work in superstructure above plinth level for every floor or part thereof in addition to rate for foundation and plinth:Cement Mortar 1:6 (1 cement : 6 coarse sand) |  |  |  |  |  |  |
|  | Code -- 7.6, Page -- 45 . |  |  |  |  |  |  |
|  | Deduction as openings --- D |  |  |  | 0.38 |  |  |
|  | Window -- W -- |  |  |  | 0.43 |  |  |
|  |  |  |  |  | 0.81 |  |  |
|  | After Deduction Net Brick work Qty. = |  |  |  | 4.95 |  | cum |
| 11 | Providing and laying damp proof course (upto 50mm thick) with plain cement concrete 1:2:4 (1 cement : 2 coarse sand : 4 graded crushed stone aggregate 20mm nominal size) including formwork. |  |  |  |  |  |  |
|  | Code -- 3.13 , Page -- 24 . |  |  |  | 0.10 |  |  |
|  |  |  |  |  | 0.10 |  | cum |
| 12 | Cement concrete flooring with cement concrete 1:2:4 (1 cement : 2 coarse sand : 4 graded stone aggregate 20mm) finished with a floating coat of neat cement.. 50mm thick |  |  |  |  |  |  |
|  | Code --12.3.2 Page -- 111 . |  |  |  | 5.76 |  | Sqm |
| 13 | 6mm thick cement plaster of mix:In |  |  |  |  |  |  |
|  | Cement mortar 1:3 (1 cement : 3 fine sand) |  |  |  |  |  |  |
|  |  |  |  |  | 9.00 |  | Sqm. |
| 14 | 12mm thick cement plaster of mix: |  |  |  |  |  |  |
|  | (d) In Cement mortar. 1:5 . |  |  |  |  |  |  |
|  | ( 1 Cement : 5 sand ) |  |  |  |  |  |  |
|  | Code -- 11.2.3 , Page -- 103 . |  |  |  |  |  |  |
|  | Net Qty. of inner walls plaster work |  |  |  | 24.75 |  | Sqm |
| 15 | 15mm Th. Cement plaster on the rough |  |  |  |  |  |  |
|  | side of single or half brick wall --- |  |  |  |  |  |  |
|  | in cement mortar 1:5 ( 1 cement : 5 sand ) |  |  |  |  |  |  |
|  | Code -- 11.3.3 , Page -- 108 . |  |  |  |  |  |  |
|  | Net Qty. of outer walls plaster work |  |  |  | 34.03 |  | Sqm |
| 16 | Providing and fixing steel door/window with M.S. sheet 1mm thick, frame and angle iron diagonal braces including applying a priming coat of red oxide zinc chromate primer. |  |  |  |  |  |  |
|  |  |  |  |  | 4.05 |  |  |
|  | Taken 120kg. i.e. |  |  |  | 120.00 |  | kg |
| 17 | Applying one coat of cement primer on wall surface (applied @ 0.80 litrs/10 sqm) complete. |  |  |  |  |  |  |
|  | Code -- 14.12 , Page -- 133. |  |  |  |  |  |  |
|  | Qty. Same as item No. -- 12+13+14 |  |  |  | 67.78 |  | Sqm. |
| 18 | Wall painting with acrylic premium emulsion (plastic) paint of required shade to give an even shade. |  |  |  |  |  |  |
|  | On new work (Two or more coats) Qty. Same as item No. -- 12+13+14 |  |  |  |  |  |  |
|  | Code -- 14.10.1 , Page -- 133 . |  |  |  | 67.78 |  | Sqm |
| 19 | Painting on new work (two or more coats) |  |  |  |  |  |  |
|  | to give an even shade with: |  |  |  |  |  |  |
|  | Satin synthetic enamel paint (top most |  |  |  |  |  |  |
|  | approved branded quality) |  |  |  |  |  |  |
|  | Code -- 14.22.1 , Page -- 134 . |  |  |  | 4.05 |  | Sqm |
|  | Marakatola |  | 1.00 |  |  |  |  |
|  |  | Total | 1.00 |  | 1.00 |  | each |
| Schedule NO.- 10 |  |  |  |  |  |  |  |
| ELECTRIC CONNECTION |  |  |  |  |  |  |  |
| 1 | Providing and stretching 3 phase transmission line by Cspdcl and making electrical connection for tube wells and pump house for pumping machinery including all necessary charges, Cspdcl demand charges etc. 1 nos. for submersible pump. |  |  |  |  | - |  |

| AT VILLAGE - DHANEGAON BLOCK -MOHLA, DISTRICT – MOHLA-MANPUR-A.CHOWKI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Schedule of Quantity Providing, Laying & Jointing of HDPE pipe & FHTC |  |  |  |  |  |
| Schedule NO.- 01 |  |  |  |  |  |
| DISTRIBUTION NETWORK |  |  |  |  |  |
| S. No | SUB HEAD |  |  | QTY | UNIT |
| 1 | 2 |  |  | 3 | 5 |
| 1 | Earth work in excavation for pipe trench in ordinary soil areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 1.80 | cum. |
|  | CG,PHE, SOR Item no. 18.15 P/225 |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 10 | x 0.60 x0.3 = | 1.80 |  |  |
|  |  | Total | 1.80 |  |  |
| 2 | Earth work in excavation for pipe trench in Hard soil areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 1.80 | cum. |
|  | CG,PHE, SOR Item no.18.16 P/226 |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 10 | x 0.60 x 0.30 = | 1.80 |  |  |
|  |  | Total | 1.80 |  |  |
| 3 | Earth work in excavation for pipe trench in Laterite soil areas including dressing, watering, ramming and disposal of excavated earth lead up to 50m and lift up to 1.5m, disposal earth to be leveled, neatly dressed. |  |  |  |  |
|  | CG,PHE, SOR Item no.18.17 P/226 |  |  | 1.80 | cum. |
|  | Dhanegaon |  |  |  |  |
|  | 10 | x 0.60 x 0.30 = | 1.80 |  |  |
|  |  | Total | 1.80 |  |  |
| 4 | Earth work in excavation for pipe trench in all of rocks in areas including dressing, watering and ramming and disposal of excavated earth lead up to 50 meters and lift up to 1.5m, isposal earth to be levelled, neatly dressed. |  |  | 1.20 | cum. |
|  | Soft rock with or without blasting or bituminous pavement / cement concrete road. |  |  |  |  |
|  | CG,PHE, SOR Item no.18.19.1 P/226 |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 10 | x 0.60 x 0.20 = | 1.20 |  |  |
|  |  | Total | 1.20 |  |  |
| 5 | Demolishing cement concrete manually / by mechanical means including disposal of material within 50 m lead as per direction of engineer-in-charge. |  |  | 6.00 | cum. |
|  | Nominal concrete 1:3:6 or richer mix (i/c equivalent design mix) |  |  |  |  |
|  | CG,PHE, SOR Item no.18.29.1 P/227 |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 50 | x 0.60 x 0.2 = | 6.00 |  |  |
|  |  | Total | 6.00 |  |  |
| 7 | Providing, Laying, Jointing & field testing of HDPE pipes (High Density Polyethylene Pipes) confirming to IS 4984/14151/12786/13488 with necessary jointing material like mechanical connector of jointing pipes by heating to the ends of pipes with the help of Teflon coated electric mirror/ heater to the required temperature and then pressing the ends together against each other, to form a monolithic & leak proof joint by thermosetting process. It may be required to be done with Jack/ Hydraulic Jacks/ Butt fusion machine. (50 mm & above fusion jointed & below 50mm mechanical jointed) |  |  |  |  |
|  | PHE SOR Page no. 253/19.35 |  |  |  |  |
|  | 63 mm dia HDPE 6 kg./cm2 |  |  |  |  |
|  | Dhanegaon |  | 8 |  |  |
|  |  | Total:- | 8.0 | 8 | Mtr |
|  | 75 mm dia HDPE 6 kg./cm2 |  |  |  |  |
|  | Dhanegaon |  | 2 |  |  |
|  |  | Total:- | 2.0 | 2 | Mtr |
| 15 | Providing and laying including testing Bend 90̊ confirming to specification. |  |  |  |  |
|  | Phe Sor Page no. 126/ 9.2 |  |  |  |  |
|  | 63 mm dia |  |  |  |  |
|  | Dhanegaon |  | 6 |  |  |
|  |  | Total:- | 6.0 | 6 | Mtr |
|  | 75 mm dia |  |  |  |  |
|  | Dhanegaon |  | 5 |  |  |
|  |  | Total:- | 5.0 | 5 | Mtr |
| 16 | Providing and laying including testing Equal Tee confirming to IS specification. |  |  |  |  |
|  | Phe SorPage no. 127/9.4 |  |  |  |  |
|  | 63 mm dia |  |  |  |  |
|  | Dhanegaon |  | 4 |  |  |
|  |  | Total:- | 4.0 | 4 | Mtr |
|  | 75 mm dia |  |  |  |  |
|  | Dhanegaon |  | 4 |  |  |
|  |  | Total:- | 4.0 | 4 | Mtr |
| 17 | Providing and laying including testing Reducer: confirming to IS specifications. |  |  |  |  |
|  | Phe Sor Page no. 128/9.6 |  |  |  |  |
|  | 75 mm dia |  |  |  |  |
|  | Dhanegaon |  | 3 |  |  |
|  |  | Total:- | 3.0 | 3 | Mtr |
| 18 | Providing butt fusion welded joint/joining by heating to the ends with the help of Teflon coated electric mirror/heater ends together etc. by thermosetting processes to HDPE Pipe and specials. (6 kg. 8 kg. 10 kg.) (50 mm & above fusion jointed & below 50 mm mechanical jointed) including testing |  |  |  |  |
|  | Phe Sor Page no. 128/9.7 |  |  |  |  |
|  | 63 mm dia |  |  |  |  |
|  | Dhanegaon |  | 6 |  |  |
|  |  | Total:- | 6.0 | 6 | Mtr |
|  | 75 mm dia |  |  |  |  |
|  | Dhanegaon |  | 5 |  |  |
|  |  | Total:- | 5.0 | 5 | Mtr |
| 19 | Providing and laying including testing End Cap confirming to IS specifications. |  |  |  |  |
|  | Phe SorPage no. 128/9.8 |  |  |  |  |
|  | 63 mm dia |  |  |  |  |
|  | Dhanegaon |  | 5 |  |  |
|  |  | Total:- | 5.0 | 5 | Mtr |
| 20 | Providing & fixing of following Cast iron double flanged sluice valves as per I.S.:14846-2000 fitted with cast iron cap including jointing & testing with cost of jointing material such as bolts, nuts, rubber insertions etc. all complete |  |  |  |  |
|  | 50mm dia |  |  |  |  |
|  | Dhanegaon |  | 2 |  |  |
|  |  | Total:- | 2.0 | 2 | Mtr |
|  | 65mm dia |  |  |  |  |
|  | Dhanegaon |  | 1 |  |  |
|  |  | Total:- | 1.0 | 1 | Mtr |
| 22 | Providing laying and jointing of following galvanized Iron (MS) Pipes with specials (such as bends, elbows, tees etc) class light, medium & heavy including testing of joints, cost of pipes, specials and jointing materials all complete. Pipes and sockets conforming to IS:1239/2011 Part- II. |  |  |  |  |
|  | CG,PHE, SOR Item no. 6.1 P/89 |  |  |  |  |
|  | 100 mm dia class mudiam (For culvert crossing) |  |  |  |  |
|  | Dhanegaon |  | 50 |  |  |
|  |  | Total | 50.0 | 50 | RM |
| 23 | Filling available excavated earth in trenches plinth sides of foundation is layers not exceeding 20 cm. In depth i/c consolidation of earth layer by ramming watering lead upto 50 m and lift upto 1.5 mt. I/c all kind of soils. |  |  |  |  |
|  | CG,PHE, SOR Item no.18.22 P/227 |  |  |  |  |
|  | Qty As per Item No. 1 to 4 |  |  |  |  |
|  | Dhanegaon |  | 6.60 |  |  |
|  |  | Total | 6.60 | 6.60 | cum. |
| 24 | Providing and laying mechanically mixed cement concrete with 20 mm nominal size graded stone aggregat. C M-15 |  |  |  |  |
|  | CG,PHE, SOR Item no. 18.41 P/229 |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 50 | x 0.6 x 0.20 = | 6 |  |  |
|  |  | Total | 6.00 | 6.00 | cum. |
| 27 | CONSTRUCTION OF RCC SLUICE VALVE CHAMBER SIZE 1.0mX1.0mX1.0m |  |  |  |  |
|  | Dhanegaon |  | 3 |  |  |
|  |  | Total | 3.00 | 3.00 | As per Sub Estimate |
| 29 | Provision for jointing of Rising main to supmp well/OHT and OHT to Distribution pipe line with cost of material/specials such as Bends, MTA as per rerquirement of site i/c cost of labour with excavation, labour as per requirement complete as per approved specification and as directed by Engineer in charge. |  |  |  |  |
|  | CG,PHE, SOR Item no 19.20 P/246 |  |  |  |  |
|  | Dhanegaon |  | 1.00 |  |  |
|  |  | Total | 1.00 | 1.00 | Job |
| Schedule NO.- 02 |  |  |  |  |  |
|  | FHTCs (Funtional Household Tap Connection |  |  |  |  |
| 1 | Providing and constructing one stand post as per type design with excavation 15 cm thick PCC 1:3:6 bedding 20 mm thick PCC 1:2:4 convert for platform of 1.5mx1.5m with side curb and bucket rest of 80 mm dia. 160mm dia PVC pipe central post duly filled therein with C.C. 1:2:4, 2.2m long, 15 mm dia medium G.I. pipe. From point of tapping to stand post additional 1620 (20 mm dia) composite pipe 6.0 m long. Providing and fixing of 15 mm dia, one steel water tap, one flow control valve SS, 5 LPM complete together with all labour and material charges as per drawing and as directed by Engineer-in-charge when good foundation in available. Rate includes draining arrangement by excavating open gutters complete |  |  |  |  |
|  | PHE SOR P.No /Item No.253/19.35 |  |  |  |  |
|  | Dhanegaon |  | 3 |  |  |
|  |  | Total | 3.00 | 3.00 | Each |
| 2 | Earth work in excavation for pipe trench in ordinary soil in areas including dressing, watering and ramming & disposal of excavated earth lead up to 50 m- & lift up to 1.5 m. disposal earth to be leveled, neatly dressed |  |  |  |  |
|  | PHE SOR P.No /Item No.226/18.19 |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 0 | x0.25x0.15 | 0.000 |  |  |
|  |  | Total | 0.00 | 0.00 | cum. |
| 3 | Earth work in excavation for pipe trench in all kinds of rocks in areas including dressing, stacking of useful material and disposal of unserviceable one up to 50 m lead and lift up to 1.5 m. (a) Soft rock with or without blasting or bituminous pavement /cement concrete road. |  |  |  |  |
|  | PHE SOR P.No /Item No.226/18.19 |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 0 | x0.25x0.15 | 0.000 |  |  |
|  |  | Total | 0.00 | 0.00 | cum. |
| 4 | Demolishing cement concrete manually / by mechanical means including disposal of material within 50 m lead as per direction of engineer-in-charge. |  |  |  |  |
|  | Nominal concrete 1:3:6 or richer mix (i/c equivalent design mix) |  |  |  |  |
|  | CG,PHE, SOR Item no.18.29.1 P/227 |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 5 | x0.25x0.25 | 0.313 |  |  |
|  |  | Total | 0.31 | 0.31 | cum. |
| 5 | Providing and fixing Polyethylene- Aluminum Polyethylene (PE-AL-PE) Composite Pressure pipes Conforming to IS: 15450-2004 U.V. Stablished with carbon black having thermal stability for hot & cold water supply, capable to with stand temperature up to 800 C including all specials and fittiings of composite material (engineering plastic. gland and brass insert wherever required) e.g. elbws, tees. reducers, couplers and connectors wiith clamp at 1m spacing. This includes testing of joints complete as per the directions of engineer- incharge (Concealed work including cutting chases& making good the wall etc ) |  |  |  |  |
|  | PHE SOR P.No /Item No.254/19.37.2 |  |  |  |  |
|  | 1620 ( 20mm OD ) Pipe |  |  |  |  |
|  | Dhanegaon |  | 280.00 |  |  |
|  |  | Total | 280.00 | 280 | Rm |
| 6 | Filling available excavated earth in trenches, lead up to 50m and lift up to 1.5m in all kind of soil excluding watering and ramming. |  |  |  |  |
|  | PHE SOR P.No /Item No. 227/18/23 |  |  |  |  |
|  | (B) in Hard Soil for Road |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 0 | x0.25x0.30 | 0.000 |  |  |
|  |  | Total | 0.00 | 0.00 | cum. |
| 7 | Providing & laying mechanically mixed cement concrete 20mm nominal size graded crushed stone excluding cost of centering & shuttering. |  |  |  |  |
|  | PHE SOR P.No /Item No. 229/18/41.1.1 |  |  |  |  |
|  | In Plinth & foundation |  |  |  |  |
|  | For Cutting 1:3:6 (M-10) |  |  |  |  |
|  | Dhanegaon |  |  |  |  |
|  | 5 | x0.25x0.25 | 0.313 |  |  |
|  |  | Total | 0.31 | 0.31 | cum. |