- Power operated Brush Cutter / Weed Slasher:

Required Specifications:

* Engine/ Motor- Type- 2Stroke.

* Power - 30 cc to 50 cc.

*Fuel - Type - Petrol, Capacity - 0.6 L to 1.6 L.

* Clutch: Type-Centrifugal, Material : Mild Steel, Dia: 75-80 mm.

* Drive Shaft: Length 1500 mm long, Dia: 26 mm to 28 mm, Type: Fixed/Ditachable.

*  Cutting Head: Boron Steel Metal Blade of 2T type or Nylon spool with 3mm to 3.5mm wiere thickness.

* Weight:More than 6kg depending on engine & build.

*  Starting Mechanism: Recoil (manual pull).

*  Handles: U type, double shoulder strap.

*  Safety Features:  Blade guard/ Shield, Throttele lock-off or switch, Safety Harness, Meshed Helmet, Goggles.

* Accessories: Replacement parts - Blades, Trimmer Heads, Tool kits.

* Maintainence: Ease of changing Air filter, Spark plug.

- Post Hole digger:

Required Specifications:

* Power Type: 2 Stroke.

* Engine / Power: 2 HP

* Auger (Drill Bit Size):Diameter of auger 8 inches.

* Depth Capability: 2 feet.

* Fuel Capacity: 0.5 L to 1 L.

* Weight of the Engine:7kg.

* Shaft Minimum Wieght: 2kg.

* Clucth Type: Mild Steel.

*  Safety & Durabilit: Replaceable cutting tip or blade on auger, Shear bolt or clutch protection, Build quality (frame, welding, reinforced structure)

- HDP/HTP Power Sprayer

Required Specifications:

* Power Type: 4 Stroke.

* Fuel Type:Petrol.

* Fuel Capacity: 1.8 L to 3.0 L.

* Engine / Power: 3 HP

* Delivery Hose: 100 meters with 5 layer (50 X 2 Mtr)

* Power Rating, KW:2.10Kw/ 2.8 HP @ 3600 RPM.

* Max.torque, Nm: 7.8 Nm @ 2800 RPM.

* Pump Type: 22 Pump

* Piston Configuration:3-piston triplex system made of heavy-duty stainless steel.

*Minimum Discharage Rate:13870.0 ml/minute at 25 kg/cm2

*Maximum Discharage Rate: 1420.0 ml/min at 10kg/cm2

* Discharge rated pressure: 14450.0 ml/min at 15kg/cm2

* Operating Speed:  Standard revolutions per minute (RPM) of 800 to 1400.

*  Weight of the Sprayer:  When mounted with a motor, engine, and frame, the total weight ranges from 25 kg to 35 kg