# KARNATAKA CO-OPERATIVE OILSEEDS GROWERS’ FEDERATION Ltd.,

No. 11, Blue Cross Chambers, Infantry Road Cross, Bangalore – 01

Telephones: 080 - 25550268, 25583061, 99868 48181

e-mail: pi@kof.coop, purchase@kof.coop and  palm@kof.coop

Technical specification for supply of 3 phase Decanter Centrifuge for palm oil recovery/dewatering and supervision of erection and installation.

The list of the Machineries required at Palm Oil Complex, KIADB Indl.Area, Sarathi Village, Kurubarahalli, Harihar

Bowl

| Maximum main speed: | 4200 rpm |
| --- | --- |
| Operating speed: | 3150 rpm (best suitable for palm oil application) |
| Bowl length: | 1512 mm |
| Bowl diameter: | 360 mm |
| Beach angle: | Suitable |
| Solids discharge type: | 10 bushings |
| Solids transportation aids, cylindrical section: | Ribs |
| Solids transportation aids, taper section: | Ribs |
| Material - bowl: | SS 316 |
| Liquid outlet, type: | Power tubes, x light phase + x heavy phase, LP/HP close to end hub |
| Liquid outlet light phase, radius: | 98 mm |
| Liquid outlet heavy phase, radius: | 103 mm |

Conveyor

| Conveyor type: | Suitable for palm oil application |
| --- | --- |
| Material - flights: | SS 316 |
| Feed Zone type: | Special for this application |

Wear Protection

| Bowl solids discharge: | Wear saddles / bushing in tungsten carbide |
| --- | --- |
|  | steep cone, 6 pieces |
| Conveyor flights: | Tungsten Carbide Tiles on flights near small end side & Tungsten carbide hard facing on flights near large end side |
| Conveyor feed zone: | Tungsten Carbide wear liners |
| Frame / casing: | Wear liner in upper casing |

Frame and Casing

| Frame/Casing type: | Box Beam Integral Casing |
| --- | --- |
| Material - casing / cover: | SS 316 |
| Inside surface of casing: | Liners of stainless steel at discharge areas |
| Material – Frame: | Mild Steel with SS cladding at product wetted surface |
| Upper Casing type: | 3 Phase |
| Outlet Funnels: | For Heavy and Light phase provided |

Motors & drive parts

| Main drive motor rating: | 22 kW |
| --- | --- |
| Back drive: | Countershaft Transmission |
| Motor type | TEFC Squirrel cage induction motor |
| Power supply | 3 phase 415 V 50 Hz |
| IP rating | IP 55 |
| Control Panel |  |
| Panel | VFD based Panel |

| Capacity | 15 Mts/Hr |
| --- | --- |
| Flow Rate | 8-10 Meter cube/Hour |

DIMENSIONS:

| SL No. | Particulars | Dimension |
| --- | --- | --- |
| 01 | Length | 3540 mm |
| 02 | Width | 1304 mm |
| 03 | Height | 990 mm |
| 04 | Weight | 2150 Kgs |

Scope of supply

| Item | Description | Quantity |
| --- | --- | --- |
| 1 | 3-Phase Decanter | 1 |
| 2 | VFD Based Panel | 1 |
| 3 | Liquid (Both heavy & Light phases) outlet funnel | 1 |
| 4 | Set of standard Tools | 1 |
| 5 | Lubricants included for commissioning | 1 |
| 6 | Intermediate service kits for main and conveyor bearings | 1 each |