NIT No: 55  /PR/SBPDCL /2026

SOUTH BIHAR POWER DISTRIBUTION CO. LTD, PATNA

(STORES AND PURCHASE)

###### TENDER   SPECIFICATION FOR

9 Meter 300 Kg W/L  PSC Pole

Tender Cost Rs.11,800.00  including GST@18%

# SOUTH BIHAR POWER DISTRIBUTION CO LTD: PATNA

## DEPARTMENT OF STORES AND PURCHASE

General conditions for submission of tenders for supply of materials:

- Online tender for Domestic Competitive bidding is invited from technically & financially sound reputed manufacturers registered under the Companies Act 1956 /2013 or any other act for supply of 9 m 300 Kg W/L PSC Pole  conforming to the relevant IS/IEC and as well as per technical specifications. The material has to be type tested as per relevant IS/IEC standard with latest amendment whichever applicable. The quantity to be procured is indicated as detailed below:-

| S. N. | Item | Unit | Quantity of SBPDCL | Quantity of NBPDCL | Total quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | 9 m 300 Kg W/L PSC Pole | No | 15,000 | nil | 15,000 |

The requirement mentioned in the tender notice & General Condition is tentative. The quantity may increase or decrease (±30%)  at the time of finalization of tender subject to availability of fund and requirement.

The Company reserves the right to cancel part or whole quantity of the material under any head under special circumstances.

- Joint Ventures (JVs) / Associate and subletting is not allowed.

- Scope of Work :

(a) The scope of work in brief shall include design, manufacture, supply, loading at factory, transportation to site / stores,  unloading at site/stores including guaranteed obligation  of  complete  supply  of  materials  in  conformity  to  the  technical  specification enclosed herewith. Pre-dispatch inspection will be carried out as per NIT.

The monthly delivery schedule for the quantities which can be delivered should be indicated as per clause of Delivery Schedule.

- Following shall be the  qualification requirements:-

- TECHNO COMMERCIAL REQUIREMENTS:-

A1. Technical:-

- Test report  (PQR):-The bidder has to submit  test report for sample Concrete cube as per relevant IS/IEC standard  from NABL accredited  laboratory otherwise offer shall be rejected. The test certificate issued within last (5) five years from last date of submission of bid from NABL accredited laboratory will be accepted.

- Supply experience:- The bidder must have supplied at least 15,000 nos. 9 m PSC pole within the last 5 (five) years from last date of submission of bid  , to any Indian Govt. Company related to Power / Indian Power Distribution Companies / Indian Govt. Organization or in EPC/Turnkey project in India  if end user is any Indian Govt. Company related to Power  /Indian Power Distribution Companies/Indian Govt. Organization,  the scope shall also be  considered. In this regard, purchase order issued within last 5 (five) years from last date of submission of bid will be accepted.

In case of direct supply to any Indian Govt. Company related to Power/ Indian Power Distribution Companies/ Govt. Organization:-Bidder shall have to submit copies of un-priced/priced purchase orders and proof of actual delivery of materials to the client such as duly acknowledged delivery challans or excise duty/ GST, gate passes or  Work completion Certificate or, client’s certificate or, Notarized affidavit of self-declaration for successfully supply of aforesaid required quantity .

In case of supply to any Indian Govt. Company related to Power /Indian Power Distribution Companies / Govt. Organization through EPC/Turnkey project:-Firm has to submit copies of un-priced/priced purchase order/ work order and  documentary evidence that end user is any Indian Govt. Company related to Power (as detailed above) / Govt. Organization and proof of actual delivery of materials to the client such as  as duly acknowledged delivery challans or excise duty/ GST, gate passes or   Work completion Certificate or, client’s certificate or, Notarized affidavit of self-declaration for successfully supply of aforesaid required quantity.

Note: Bidder must upload the duly filled Annexure-I with details of uploaded purchase order against the supply experience.

- Registration Certificate: - Documentary evidence regarding category of the manufacturer (Micro / Small / Medium / Large) like Registration Certificate issued by SSI / NSIC / Directorate of Industries etc.

- Entity Registration:

- The bidder shall be a legal entity registered or incorporated in India as per the Companies Act 1956/2013 or any other act.

Certificate of Incorporation/Memorandum of Association/ Partnership deed etc. must be furnished for the same.

- Bidder's Annual Production Capacity:

The bidder’s Annual production capacity shall not be less than the twice the tendered quantity required to be supplied. The bidder must submit Annual reports/certificate issued by MSME/Charted Accountant/Chartered Engineer/NSIC/ Directorate of Industries, clearly mentioning the annual production during the last 3 (three) Financial years upto March 2025.

- In-house inspection/testing facilities:

- Confirmation regarding availability of in-house inspection/ testing facilities or access through other government approved agencies for performing inspection/ testing needs to be provided.

The Bidder shall give details of its Testing facilities at manufacturer’s work with copies of calibration/re-calibration certificates of the major testing equipment

A2 . Commercial:-

- Audited Annual Accounts:-  Audited Annual Accounts along with Audit Report during the last Five (5) financial years i.e. upto 2024-25 to establish the capability of the bidder to successfully perform the tender and deliver the materials.

- MAAT (Minimum Average Annual Turnover):-The vendor must have MAAT of at least Five Crore only for the best three (3) financial years out of last five (5) financial years. i.e. up to March 2025.

Audited Annual Accounts of the last 5 (five) financial years ending March, 2025 needs to be submitted.

For M.S.E.(Micro & Small Enterprises) and  Start-ups of Bihar, vendor must have turnover of at least 50% of the MAAT requirement as mentioned above (i.e. INR 2.50 Crore only) for any one year out of last five (5) financial years up to FY 2024-25. Required Audited Annual Accounts must be accompanied with bid documents.

- Net Worth: - Net Worth of last three financial year shall be positive up to FY 2024-25 .

- Audited Annual Accounts:-  Audited Annual Accounts during the last Five (5) financial years upto March, 2025 to establish the capability of the bidder to successfully perform the tender and deliver the materials.

- Copies of PAN Card, GST Registration certificate.

- CA Certificate regarding Turnover & Net worth of last 5 FY upto 2024-25 needs to be submitted.

All documents as indicated herein is required to establish bidder's financial and technical qualification must be submitted as part of techno-commercial bid. Bids shall not be considered if the past documentary credentials of the bidder submitted by him are found un-satisfactory.

A3.  Following Notarized affidavit to be submitted by bidder failing which their tender shall not be accepted:-

- A  Notarized Affidavit to the effect that the sample Concrete cube has been tested as per relevant IS at Govt. Lab /NABL accredited Lab and the test certificate and all other documents enclosed with the tender are true and genuine must be uploaded with the tender documents

(b)	 Bidder has to submit a notarized affidavit for the effect that they are presently not blacklisted by any State / Centre Utilities / PSUs / govt. entities on the date of submission of bid.

(c)    Bidder has to submit a notarized affidavit for the effect that they are not debarred/ blacklisted from BSPHCL and its subsidiary companies (i.e. NBPDCL/SBPDCL/BSPTCL/BSPGCL)on the date of submission of bid.

(d)Bidder has to submit a notarized affidavit to the effect that they are not de-empanelled or their vendor approval has not cancelled by BSPHCL and its subsidiary companies (i.e. NBPDCL/SBPDCL/BSPTCL/BSPGCL) in last two years from last date of submission of bid.”

(e) Notarized Affidavit for(1)all documents submitted in the bid are true and genuine and (2)all documents to be submitted during the execution of contract, if awarded, will be true and genuine.

A4.  Others:-

- Debarment/Blacklisting Notifications issued by one subsidiary of BSPHCL shall automatically extend also to other subsidiaries.

- Firms black listed by any State / Centre Utilities / PSUs / govt. entitieswill also stand black listed till their black listing is revoked.

- Following details are also to be submitted:-

a)    Number of Years in Production.

b)    Certification of Design to the relevant standard.

c)    Existence of Local office (if applicable).

d)    Existence of service network and parts stock.

e)    Following Good Manufacturing Practices (GMP).

f)    Location of factories/ manufacturing units.

- Both part of tender(Part-1 Technical & Commercial part and Part – II Price part) can be Downloaded/Uploaded from the website - 	www.eproc2.bihar.gov.in. Tender cost of Rs10,000/-(Rupees Ten  Thousand only)with GST@18%and bid Processing fee as per provisions of eproc portal (in addition to cost of tender)  are  mandatory to be paid through online mode i.e. Internet Payment gateway (Credit/Debit Card), Net banking, NEFT/RTGS. Bids along with necessary online payments must be submitted through e-payment portal www.eproc.bihar.gov.in before date and time specified in the Tender Inviting Notice or its subsequence extension if any. The department does not take any responsibility for the delay/Non Submission of tender/Non Reconciliation of online payment caused due to Non-availability of Internet Connection, network Traffic/Holiday or any other reason. Copy of receipt of online payment is to be submitted to  ESE(Purchase),SBPDCL ,Patna as per schedule.

M.S.E.(Micro & Small Enterprises) and  Start-ups Unit of Bihar  is exempted for submission of Tender Cost of Rs.10,000/-(Rupees Ten Thousand only) with GST@18%.

- Deposit of Earnest Money(EMD)/

(a) An Earnest money of Rs. 14,05,500.00 only should be deposited to Electrical Superintending Engineer(Purchase), SBPDCL, Patna in the form of Bank Draft or Bank Guarantee issued by Nationalized Bank / Scheduled bank of India & drawn in favour of Sr. Manager (F&A ), SBPDCL payable at Patna valid for 45 days beyond the bid validity period in prescribed enclosed format as per date and time specified in the Tender Inviting Notice or its subsequence extension if any , failing which the tender will not be  considered. It may also be deposited through E-payment mode in SBPDCL’s SB A/C No. 201001371988 (Indusind Bank, Boring Road Patna, IFS Code No. –INDB0000393).

(b) Cheques are not acceptable towards deposit of Earnest Money. Cheques on account of Earnest Money will be considered without Earnest Money and the tender will be not considered.

(c) No Earnest Money will be accepted after opening of the tender. Earnest money deposited with other NIT is not adjustable with this NIT. No Bank commission or interest will be paid on Earnest Money.

(d)If discom concludes that bid evaluation and award of contract cannot be completed within original validity due to reasons beyond control, it will ask for consent of bidders to extend validity period.

(e)The request and the responses thereto shall be made in writing or by e-mail to all participating bidders;

(f)A bidder agreeing to the extension request shall also agree to suitable extension of validity period of EMD. Such a bidder shall not be required to or permitted to modify its bid;

(g)A bidder may refuse the request to extend the bid validity which would lead to his disqualification without forfeiting his EMD. In such a case the bid shall not be further considered for evaluation and award and EMD shall be returned.

- MODIFICATION AND WITHDRAWAL OF BID

a)	The bidders may modify or withdraw their bids after submission but before expiry of the last date & time for submission of bids. Multiple submissions of bids are permitted.

b)    	The intimation for bid withdrawal/ modification received after last date &time for bid submission shall be ignored and bid already submitted shall be deemed to be a validly submitted bid.

c) 	No bid will be withdrawn/ modified after the last date & time for submission of bid till bid validity period.

d) 	Withdrawal or modification of bid after bid submission date and time during bid validity period will result in forfeiture of EMD.

- Definition:-

- Local content

Local content” means the amount of value added in Bihar which shall, unless otherwise prescribed by the Competent Authority laid down under this policy, be the total value of the items procured (excluding net domestic indirect taxes) minus the value of content brought from outside Bihar as a proportion of the total value, in percent.

The local enterprise at the time of tender or bidding shall be required to indicate percentage of local content and provide self-certification that the item offered (quantifying the infrastructure such as the production unit, manpower, electricity consumption, raw material & land possession/

building premises) meets the local content requirement for local enterprise as the case may be. They shall also give details of the location(s) at which the local value addition is made.

- Local Enterprise

  - Registered in the State of Bihar

  - Production facility in the State of Bihar.

  - Minimum Local content-30%

- Special Category of enterprises

  - Bihar Micro and Small Enterprises (BMSEs)

    - Registered in the State of Bihar

    - Valid Udyam Registration

    - Shall meet the minimum local content requirement of 30%

  - Start-up: Start-ups as defined in Bihar Start-up Policy 2022 as amended from time to time).

- Margin of purchase preference :- Maximum extent to which the price quoted by a “Local Enterprise” may be above the L1 for the purpose of purchase preference

- Non-Local Enterprise :-An enterprise, whose goods offered for procurement, has the local content less than 30 %.

- EXEMPTION:- The tenderers of the following categories are exempted from deposit of Earnest Money subject to the conditions laid down below and such firm must upload the registration certificate for exemption as well as to submit the same to Electrical Superintending Engineer(Purchase), before the scheduled date and time failing which the tender will be rejected.

- M.S.E.(Micro & Small Enterprises) and  Start-ups Unit of Bihar  for manufacture of the items for which the tender is submitted. The tender in such case must be supported with a certified copy of an up to date/valid certificate of their registration failing which the tender will be rejected.

- The State Govt. /Govt. of India undertakings.

Tenders not accompanied with adequate amount of Earnest Money/ required exemption certificate will be rejected out rightly.

- Margin  of  Purchase Preference for Local Enterprises of Bihar  :-

Price preference will be given to Local Enterprises of Bihar as per Notification No.M-4-03/2021 8550/F Patna dated 07.08.2024 of Finance Department, GoB. "Margin of purchase preference" means the maximum extent to which the price quoted by a "Local Enterprise' may be above the L-l for the purpose of purchase preference .The margin of purchase preference for Local Enterprise shall be 15% for the procurement of goods .

- Local content requirement

The minimum local content requirement for the local enterprise for goods related procurement is 30%. In support of Local content, bidder has to submit following:-

(i) The local enterprise at the time of tender or bidding shall be required to indicate percentage of local content and provide self certification that the item offered (quantifying the infrastructure such as the production unit, manpower, electricity consumption, raw material & land possession/ building premises) meets the local content requirement for local enterprise as the case may be. They shall also give details of the location(s) at which the local value addition is made.

(ii) In case of procurement for the value in excess of Rs. 5 crores, the local enterprise shall be required to provide a certificate from the auditor of the company (in the case of companies) or from a practicing cost accountant or chartered accountant (in respect of enterprises other than companies) giving the percentage of local content quantifying the infrastructure such as the production unit, manpower, electricity consumption, raw material & land possession/ building premises.

- Contract Performance Guarantee (CPG):- The successful vendor shall be required to submit CPG before execution of Contract Agreement in shape of Bank Guarantee issued by any Nationalised / Scheduled Banks of India on non-judicial stamp paper of Rs.1,000/-(Rupees One Thousand) only or value of paper as prevailing at that time  in  the prescribed format equivalent to ten  percent (10%)  ( 5% in case of M.S.E.(Micro & Small Enterprises) & Start-ups  Unit of Bihar )  of order value to protect interest of the DISCOM in case of breach of contract. The CPG should be valid for the period of 30 (Thirty) months from the date of issue of Purchase Order which will be further extended till expiry of the warranty period of last consignment.

No Bank commission or interest shall be paid by the DISCOMs.

The 10% ( 5% in case of M.S.E.(Micro & Small Enterprises) & Start-ups  Unit of Bihar )   CPG of Total Order Value should be in favour of Sr. Manager(F&A), North Bihar Power Distribution Company Limited/ South Bihar Power Distribution Company Limited and submitted to the Sr. Manager (F&A) NBPDCL / SBPDCL for acceptance after confirmation from issuing Bank.

The CPG shall be discharged and returned to the Agency within thirty (30) days of expiry of the claim period of last consignment. If the vendor fails to remedy any reported defect within warranty period, the DISCOM shall be entitled to either en-cash the CPG or to remedy the defect at the expense of the vendor.

Details of SBPDCL Account is given below:-

A/c name SBPDCL Capital

A/c no: 0352101042077

IFSC CNRB0000352

Canara Bank Main Branch, Patna

Note: If the rate quoted by the L-1 bidder is more than or less than by twenty five percent (25 %) of the Company’s cost estimate, this will be treated as abnormally high/ low. In this case, successful bidder/s will be required to enhance the Performance security (i.e. CPG) by additional 10% at bidder's expense to a level sufficient to protect the company interests against likely financial losses in the event of default under the contract.

- DELIVERY: The material are required to be delivered within 06 (Six) months from the date of issue of Purchase Order as per the following delivery schedule.

| Month | Delivery Schedule |
| --- | --- |
| Within 2nd months | 20 % of ordered quantity |
| During 3rd month | 20 % of ordered quantity |
| During 4th month | 20 % of ordered quantity |
| During 5th month | 20 % of ordered quantity |
| During 6th month | 20 % of ordered quantity |

Early delivery shall be preferred. The proforma for schedule of delivery is enclosed with the Tender; Tenderers are requested to fill up the proforma giving a clear schedule of month wise delivery capability.

- NBPDCL/SBPDCL reserves the right to cancel the lowest or any of the tender either in whole or in part of full or default all of the tenderers without assigning any reason.

- Price: - The Ex-Works rate  &Freight per unit must be quoted in words and figures in the prescribed proforma. The price [Ex-Works rate  & Freight] is firm. For landed price, per Unit GST shall be taken as 18% or as applicable in addition to the quoted unit Ex works rate & unit freight. Firm shall have to quote unit  Ex-works& unit Freight up to F.O.R. destination and other charges if any. However, applicable GST shall be paid by SBPDCL/NBPDCL.

If the difference is found between the rate quoted in words & figures, then the rate quoted in words will be considered.

- Evaluation of Bid & Clarifications on bid:-

During bid evaluation, SBPDCL may, if needed, seek clarification (s) from Bidder (s) in support of compliance to QR. SBPDCL shall be fully authorized to seek any QR related clarifications from the bidders during evaluation of bids, if required, without need of any further approval.

No change in stipulated QR shall be made after opening of bids and in case QR needs modifications, re-tendering shall be done. Evaluation shall be done only on the basis of uploaded documents by the Bidders

The bidder shall not be permitted to submit any additional document during clarification stage, which enhances his original submitted credentials.

Any post-bid change in the price or substance of the bid, if given by the bidder, shall not be accepted in case of Open tender and Limited tender

Bidder shall have to fill the Price sheet of Financial Bid online. L-1 bidder will be decided on the lowest landed rate by the participating bidder.

If there are more than one bidder who have quoted lowest landed rate in the tender, final lowest bidder(L-1 bidder) will be decided by draw of lots in presence of all such lowest bidders or their authorized representative.

- Insurance - The Insurance will have to be underwritten with the Company underwriters against its open policy in all cases. As such the Insurance premium should not be quoted by the bidder and should not be included in the quoted price.

- Routine Tests: Routine Test will have to be conducted on 100% quantity by the firm before material is offered for pre-dispatch inspection, according to relevant standard, and tests certificate in triplicate will have to be submitted to ESE (S&P). Full quantity of the materials shall be subjected to the test prescribed in the relevant standard (latest edition) before supply is affected. Untested material will not be accepted.

- Packing: -Material shall be delivered suitably packed as per ISS. It should be robust enough for rough handling during transportation by Rail/Road. Further it is to clarify here that all the charges towards packing, forwarding, loading, unloading etc shall be borne by the supplier.

- Forfeiture of Earnest Money& Contract Performance Bank Guarantee:-

It should be clearly understood that in the event of the vendor failing to accept the e-mailed and detailed order if it is placed within the validity period of the offer or/and execute the full quantity, then the full amount of Earnest Money or/and Contract Performance Bank Guarantee will be forfeited and the DISCOMs decision in this respect will be final and binding on the vendor.

- Standard activity wise indicative timeline of NBPDCL/SBPDCL for material / equipment procurement:-

| Activity Particulars | Maximum Permissible Period | Start Date | End Date |
| --- | --- | --- | --- |
| Letter from Supplier with request for material inspection post manufacturing of material& its readiness for inspection from date of PO | 30 days | T | T+30 days |
| Intimation of Inspection &Raising of Inspection by NBPDCL/SBPDCL. | 4 days | T+30 days | T+34 days |
| Inspection of Material by NBPDCL / SBPDCL official(s)/Authorize Representative & submission of Inspection Report | 7 days | T+34 days | T+41 days |
| Issuance of Dispatch Instruction (D.I.) with Dispatch Schedule | 7 days | T+41 days | T+48 days |
| Delivery of Material at its destination as per NBPDCL / SBPDCL requirement (included in DI) | 10 days | T+48 days | T+58 days |

where, T is the date of issuance of Purchase Order (P.O.) to the vendor/ supplier

- Progress Report: The suppliers will have to submit monthly progress report regularly until completion of delivery to the Electrical Superintending Engineer (S&P),North Bihar Power Distribution Co. Ltd./ South Bihar Power Distribution Co. Ltd, Patna by the Ist week of every month following the month in which order is placed.

- Terms of Payment: -

    - 90% payment along with full amount of taxes, duties and freight &keeping 10% retention money, on submission of triplicate copy of bills with all relevant documents (Sample check list) enclosed in annexure by the vendor after receipt of the material / equipment in good condition. After successful completion of delivery of entire order or modified quantity as decided during its delivery schedule, 10 % retention money may be released after deduction of penalty.

    - Payment will be released normally within 15 working days from the date of submission of bill along with all required relevant document(s) to Sr. Manager (Finance), NBPDCL/SBPDCL.

    - Payment to be made by Sr. Manager (Finance), NBPDCL / SBPDCL.

    - Payment will be made only after execution of contract agreement.

    - In case the vendor / supplier is micro or small enterprises
covered under the MSME development act, 2006, payment will be made as
per provision made in the act i.e. 45 days from the acceptance of
material provided the supplier submit the bill with all relevant
records minimum 15 working days in advance.

    - Deviation from standard terms of payment of NBPDCL / SBPDCL is not acceptable.

- Penalty

    - Retention amount @10% of the bill value excluding GST will be deducted from each bill of the vendor/supplier/agency to ensure adherence to the scheduled delivery period of entire ordered quantity or as modified.

    - After completion of delivery schedule, or as the final quantity of delivery of material as decided, the complete delivery cycle for the material / equipment will be reviewed to assess the applicability of penalty @0.1% per working day plus applicable GST, if any, on the value of material delayed subject to maximum ceiling of 10% of the total ordered value of PO plus applicable GST, if any.

    - Material / equipment delivered in physically damaged condition or, incomplete is liable to be rejected. Such item shall be lifted back by vendor / supplier within 15 days from receipt of rejection notice from NBPDCL / SBPDCL and have to supply back the material within maximum upto next 30 days.

    - Material will be deemed to have been delivered only when all its components parts are also delivered. If certain components are not delivered in time the material will be considered as delayed with such time when missing parts are delivered.

    - The period for levy of penalty will be considered based on the reasons of delay which may be attributable to completely vendor/supplier or completely DISCOM or partly vendor/supplier or partly DISCOM. Penalty shall not be imposed for the delay not attributable to supplier.

Note: As per terms & conditions of penalty, if value of deducted penalty along with GST exceeds maximum ceiling value of 10% of the total ordered value of PO, then the excess amount of applicable tax, GST on the penalty shall be deposited additionally by the supplier within one month from the date of notice to the supplier. However, the maximum ceiling limit of penalty against Late Delivery(LD) shall be as per the NIT. If the aforesaid amount of tax, GST is not deposited within one month, the required amount will be realized from the CPG.

- Validity Period:-The tenderers should specify the validity period of their offer which should be for a minimum period of 180 days from the last date of submission of bid.

- Raw Materials:- No raw materials will be arranged by the Company. Tenderers will have to arrange their requirements of raw materials themselves.

The purchaser reserves the right to inspect the raw material procured by the supplier for the purpose of manufacture of tendered material to check the genuineness of raw materials. The purchaser or his 	representative can demand the original invoices in respect of all the raw materials.

- Quantity to be offered against the tender:

Bidder should quote for minimum 100 % of the tendered quantity failing which the tender will be rejected.

- Distribution of Quantity- L-1 bidder will be decided on the lowest landed rate by the participating bidder. For landed rate ,Ex works and freight shall be as per rate quoted by the bidder while GST shall be 18% or as applicable for the item. Tender shall be evaluated considering GST@18% irrespective of GST rate quoted by bidder.

- If L-1 is a "Local Enterprise", then Ll is the selected as supplier for 100% quantity.

- If L-1  is a "Non-Local Enterprise" then the following procedure for purchase preference shall be followed:-

- 75% of the order quantity shall be awarded to L-1.

- The lowest bidder among the "Local Enterprise" within the margin of purchase preference will be invited to match the L-1  price for the remaining 25% of the quantity, and contract for that quantity shall be awarded to such "Local Enterprise" subject to matching the L-1  price. In case such lowest eligible "Local Enterprise" fails to match the L-1 price or accepts less than the offered quantity, the next higher "Local Enterprise" within the margin of purchase preference shall be invited to match the L-1  price for remaining quantity and so on, and contract shall be awarded accordingly. In case some quantity is still left uncovered by "Local Enterprise", then such balance quantity may also be ordered to the L-1  bidder.

Note: In case more than one local enterprises have quoted the same price and are within the margin of purchase preference, the applicable order quantity mentioned above shall be equally divided amongst those.

- Jurisdiction of court:-The Courts/Council/Tribunal of Patna shall alone have an exclusive jurisdiction to decide any difference/dispute/cases for and against NBPDCL/SBPDCL/Vendor arising out of or in respect of the NIT, or contract agreement, or purchase order.

- Arbitration/ Resolution of disputes

All disputes or differences in respect of which the decision, if any, of the vendorand/or the SBPDCL/NBPDCL has not become final or binding as aforesaid shall be settled by arbitration in the manner provided herein below.

    - The arbitration shall be conducted by three (3) arbitrators, one each to be nominated by the vendorand the SBPDCL/NBPDCL and the third to be appointed by both the arbitrators in accordance with the Indian Arbitration Act, 1996 as amended from time to time. For this purpose a panel of five arbitrators shall be provided by Managing Director of the SBPDCL/NBPDCLfrom which SBPDCL/NBPDCLand Vendor will choose one each and the third Arbitrator shall be chosen by two Arbitrators from the same list. If either of the parties fails to appoint its arbitrator within sixty (60) days after receipt of a notice from the other party invoking the Arbitration Clause, the arbitrator appointed by the party invoking the arbitration Clause shall become the sole arbitrator to conduct the arbitration.

    - The language of the arbitration proceedings and that of the documents and communications between the parties shall be English. The arbitration shall be conducted in accordance with the provisions of the Indian Arbitration and Conciliation Act, 1996 or any statutory modification thereof.

    - The governing law shall be the Law of India and CivilCourt of Patna shall have an exclusive jurisdiction over such matters of disputes.

    - The decision of the majority of the arbitrators shall be final and binding upon the parties. In the event of any of the aforesaid arbitrators dying, neglecting, resigning or being unable to act for any reason, it will be lawful for the party concerned to nominate another arbitrator in place of the outgoing arbitrator.

    - During settlement of disputes and arbitration proceedings, both parties shall be obliged to carry out their respective obligations under the Contract.

- Extension/Repeat order: -

(i) The circumstances may arise where due to time constraints or any other factor, the Repeat Order against an existing order is to be placed without inviting further offer of price. Such orders are called Repeat Orders which may be placed against previous orders (except those placed on single vendor) without further going through the normal inviting price offer after taking approval of competent authority as per the DoP.

(ii) Repeat Order may also be placed by SBPDCL against the previous orders placed by NBPDCL and vice versa after approval of competent authority.

(iii) Repeat order will be placed and the proposal for repeat orders will be on existing terms, conditions and specifications.

(iv) Value of Repeat Order shall not exceed hundred percent (100%) of the original ordered quantity. However, the repeat order can be for one more additional unit in case of the sanctioned quantity is in fraction.

(v) Repeat Order shall not be placed more than once and shall be placed within twelve (12) months of the placement of the original order.

(vi) The performance of the vendor must be satisfactory.

If Extension/ Repeat order is placed as per terms & conditions of the NIT, then it is mandatory for the firm to execute the placed extension/repeat order as per terms & conditions of the original P.O. without any deviation otherwise action like BG forfeiting/ debarment/ blacklisting shall be taken against the firm.

- Guarantee:- The equipment/material shall be guaranteed for a period of 24 months from the date of delivery.  If during the guarantee period, the materials are found defective or sub-standard, the same will have to be replaced free of cost by the vendor within 30 days of intimation. If the defective materials are not replaced/rectified as per above guarantee clause, the DISCOM shall recover an equivalent amount alongwith 15% supervision charges from any of the bills of the supplier. Failure to however guaranteed obligation may render the supplier for blacklisting/debarment and/or forfeiture of all security and payment.

- Contract Agreement:- The vendor will have to execute a contract agreement with the North Bihar Power Distribution Company Limited &South Bihar Power Distribution Company Limited separately  in  the prescribed Proforma within 15 (Fifteen) days from the date of issue of  P.O. Contract agreement will be executed after submission of CPG BG against the P.O.

- Pre Dispatch Inspection:-  Pre-dispatch inspection shall be done by authorized inspecting officer of DISCOMs/authorized inspecting agency of DISCOMs. If offered quantity is found to be less than 95% or the materials is rejected during inspection in both cases the actual expenses incurred will be deducted from vendors bill which will be not less than Rs. 50,000.00 (Rupee Fifty Thousand) only. Material will have to be delivered only after approval of Inspection Report and issue of Dispatch Instructions by E.S.E. (P). The vendor may depute its one representative with authorized inspecting officer of DISCOM for the purpose.

- Import License

Import License cannot be arranged by the NBPDCL/SBPDCL for such or those materials which are under banned category of import. When import is desired to be effected by the vendor(s) under their own quota/license, the value of quota specifically available against the tender should be stated. In case of availability of suitable indigenous make materials, the same may be preferred.

- Debarment/ blacklisting of agencies

      - List of vendors shall be subjected to continuous review of performance. Following may result in debarment/blacklisting as the case may be:

- non-performance with regard to quality and supply schedule based on objective criteria,

- Attempt to bribe a utility official,

- Found engaging in corrupt or fraudulent practices,

- Non-compliance to the materials/equipment failure rate in any contract,

- Established misbehavior and threatening officials of the utility,

- Black listing by other state, central utilities or other government entities.

      - Black listing shall be for a period ranging from three (3) to six (6) years. Corrupt and fraudulent practices shall attract harshest punishment.

      - Debarment/Blacklisting Notifications issued by one subsidiary of BSPHCL shall automatically extend to other subsidiaries. Firms black listed by other state governments, utilities and GOI will also stand black listed till their black listing is revoked.

If debarred/ blacklisted Vendor feel that they have justifiable reasons to defend their case, they may appeal to MD , NBPDCL/ SBPDCL before going for litigation

- Termination of contract:- In case the supplier fails to deliver the stores /materials or any consignment thereof within contracted period  of delivery or in case the stores / materials are found  not in accordance with the prescribed specification and the approved samples, the Company shall exercise it’s discretionary power either :-

  - To recover from the supplier, the damages as provided in the penalty      clause of general condition of tender for supply.

  - To purchase elsewhere after giving due notice to the contractor on account  and at the risk of the contractor  without canceling  the contract.

  - Firm may be debarred/ Blacklisted

  - To cancel the contract reserving Company’s right to recover damages.

Note : -Withstanding the powers , under (a) , (b), (c) & (d) referred   above are in addition to the rights and remedy available to the Company under the General law of India relating to contracts .

- In the event of purchase of stores of similar description  , the option of the Company shall be final . In the event of action taken under

(a) or (b) above , the supplier shall be liable for any loss  which the Company sustain on the account  but the supplier shall not be entitled to any saving on such purchases made against default.

- The decision  of the Company shall be final as regards the acceptability of the stores supplied by the supplier and the Company shall not be required to give any reason  in writing or otherwise at any time for the rejection of stores/materials.

- The Company reserves the right to cancel whole or part of the awarded contract without assigning any reason of those firms which will be found defaulter for delay in supply of materials or the supply of sub-standard quality of the materials. The Company also reserves the right to distribute the order at the approved rate among other valid tenderers, in case the tenderer having quoted the approved rate is not found capable of supplying the total tendered quantity in the stipulated period of delivery schedule.

- RISK AND COST TENDER :

- Whenever DISCOM decides to terminate the contract at the Risk and Cost of the Vendor the first step will be to ensure joint measurement/ weight of the completed portion of the work/ supply during the final 48 Hours’ notice period.

- In case the Vendor refuses to co-operate in measurement/weight, the DISCOM shall undertake Ex Parte measurement or joint measurement/weight along with third party.

- The DISCOM shall be free to complete the left over portion of the supply not done either on its own or through engagement of another Vendor at the Risk and Cost of the original Vendor.

- In CASE to finalize, another Vendor shall be allowed. The original vendor shall not be allowed.

- The DISCOMs shall be free to recover the extra cost incurred on completion of the balance supply through forfeiting CPG and in case the same is found to be insufficient from other works of the Vendor under the DISCOM or with any other subsidiary of BSPHCL or even from the other contracts of the Vendor with any department of the state Government.

- CORRUPT OR FRAUDULENT PRACTICE:

The Purchaser requires that the Bidders observe the highest standard of ethics during the procurement and execution of the Project. In pursuance of this policy, the Purchaser:

a) Defines, for the purposes of this provision, the terms set forth below as follows:

(i) “Corrupt practice” means behavior on the part of officials in the public or private sectors by which they improperly and unlawfully enrich themselves and/ or those close to them, or induce others to do so, by misusing the position in which they are placed, and it includes the offering, giving, receiving, or soliciting of anything of value to influence the action of any such official in the procurement process or in contract execution; and

(ii) “Fraudulent practice” means a misrepresentation of facts in order to influence a procurement process or the execution of a contract to the detriment of the Purchaser, and  includes collusive practice amount Bidders (prior to or after Bid submission ) designed to establish Bid prices at artificial non-competitive levels and to deprive the Purchaser of the benefits of free and open competition.

b) Purchaser will reject a proposal for award if it determines that the Bidder recommended for award has engaged in corrupt or fraudulent practice in competing for the contract in question.

c) Purchaser will declare a firm ineligible, either indefinitely or for a stated period of time, to be awarded  an contract if he at any time determines that the firm is engaged in corrupt or fraudulent practice in competing for, or in executing, the Contract.

- EVENTS OF DEFAULT:-

Each of the following events or occurrences shall constitute an event of default (“Event of Default”) under the Contract:

(a) Vendor fails or refuses to pay any amount due under the Contracts.

(b) Vendor fails or refuses to deliver Commodities conforming to his Bid document/ specifications, or fails to deliver Commodities and, or execute the works assigned to them within the period specified in P.O or any extension thereof.

(c) Vendor becomes insolvent or unable to pay its debts when due, or commits any act of bankruptcy, such as filing any petition in any bankruptcy, winding-up or reorganization proceeding, or acknowledges in writing its insolvency or inability to pay its debts; or the Vendor’s creditors file any petition relating to bankruptcy of Vendor;

(d) Vendor otherwise fails or refuses to perform or observe any term or condition of the Contract and such failure is not remediable or, if remediable, continues for a period of 30 days after receipt by the Vendor of notice of such failure from company (SBPDCL).

- CONSEQUENCES OF DEFAULT:

- If an Event of Default occurs and would be continuing, Purchaser shall terminate the Contract by written notice.

- In the Event of Default, Purchaser may, without prejudice to any other right granted to it by law, or the Contract, take any or all of the following action:

- Forfeit the Contract Performance Bank Guarantee, if submitted or any submitted Bank Guarantee;

- Withheld of any other payment to fulfil the Risk & Cost tender clause of the NIT;

- Recover any losses and / or additional expenses, Purchaser may incur as a result of  Seller’s default.

- Debarment/ Blacklisting of the firm shall be taken.

- Force Majeure

    - “Force Majeure” shall mean any event beyond the reasonable control of the employer or of the Vendor, as the case may be, and which is unavoidable notwithstanding the reasonable care of the party affected, and shall include, without limitation, the following:

War, hostilities or warlike operations (whether war be declared or not), invasion, act of foreign enemy and civil war, rebellion, revolution, insurrection, mutiny, usurpation of government, conspiracy, riot and civil commotion, epidemics, earthquake, quarantine restrictions, freight embargoes, landslide, volcanic activity, flood or cyclone, or other inclement weather condition, nuclear and pressure waves or other natural or physical disaster

    - Neither party shall be considered to be in default or in breach of his obligations under the Contract to the extent that performance such obligation is prevented by any circumstances of Force majeure, which arises after date of Notification of Award.

    - If either party is prevented, hindered or delayed from or in performing any of its obligations under the Contract by an event of Force Majeure, then it shall notify the other in writing of the occurrence of such event and the circumstances thereof within fourteen (14) days after the occurrence of such event. The Engineer-in-Charge shall be the point of contact in such case.

    - The party who has given such notice shall be excused from the performance or punctual performance of its obligations under the Contract for so long as the relevant event of Force Majeure continues and to the extent that such party’s performance is prevented, hindered or delayed. The Time for Completion shall be extended.

    - The vendor who are agreeable to accept the above terms of general conditions will only be considered and other will outright be rejected.

- POST DELIVERY SAMPLE CHECK:-

After delivery of above material randomly selected and sealed samples may be sent to NABL accredited laboratory like CPRI/ NTH/ERDA/NPL and tested for quality check at own cost of SBPDCL. If adverse report reported by laboratory whole lot will be rejected and submitted security money will be forfeited. Supplier may be blacklisted/ debarred. Sample of the supplied material shall be selected & sealed in presence of the supplier before sending it to the lab for quality check provided the supplier appears for the selection/sealing process on intimation by SBPDCL. In case, the supplier is absent on the scheduled date for selection/sealing process, the selection/sealing will be done by SBPDCL on its own.

- Deviation from specification:-

No any deviation from specifications of material and the terms & conditions of the tender shall be allowed. The vendor who are agreeable to accept the above terms of general conditions will only be considered and other will outrightly be rejected.

- GTP & Drawing Approval: The supplier shall submit the GTP & drawing for approval of the purchaser within seven(7) days from issue of P.O. The same will normally be approved within seven(7) days of submission.

- General :-SBPDCL will ascertain to its satisfaction whether the bids are responsive as per the Qualification Requirement specified above to execute the order satisfactorily. SBPDCL shall be the sole judge in this regard and the interpretation of SBPDCL regarding the qualification requirement and other terms & conditions of this tender shall be final and binding on the bidders/suppliers. Conditional Bids shall not be acceptable.

Encl:

1. Condition of tender- Annexure ‘A’

2. Performa of Bank Guarantee for Bid -Annexure ‘B’

3. Monthly delivery schedule -Annexure ‘C’

4. Tender Performa for Tech. & Commercial Part-I, - Annexure-D

5. Past Performance of supply-Annexure-E

6.Contract Performance Guarantee- Annexure-F

7. Sample check list of documents required for Payment- Annexure-G

8.Tender Performa for Price Bid: Annexure-H

9.  Format for Supply Experience: Annexure-I

10.Technical Specification:-Annexure-J

Sd/-

Electrical Superintending Engineer(Purchase)

SBPDCL

Annexure ‘A’

SOUTH BIHAR POWER DISTRIBUTION CO. LTD., PATNA

DEPARTMENT OF STORES & PURCHASE

CONDITION OF TENDER

The following conditions are to be strictly fulfilled by the firm while submitting their tenders.

1. Exemption will be granted as per general condition.

2. Tenderer has to accept the penalty clause as per tender otherwise tender will be considered as invalid .

3. The offers, which are not kept valid for acceptance for at least 180 days from the date of opening of the tenders, will be rejected.

4. Incomplete tenders which do not contain full details, technical particulars, drawing literature, price , delivery period, etc. will be rejected.

5. Prices are specifically asked to be submitted as F.O.R. destination and if not so furnished the tender shall not be considered as valid and will be rejected.

6. Tender which is not submitted in the company’s prescribed form shall be rejected. Any additional particulars can be furnished in the accompanying letter of statements.

7. Performa for schedule of Requirement and deliveries is enclosed with the tender. Tenderer are requested to fill up the Performa giving clearly the monthly delivery schedule.

8. Tenders uploaded after the due date and time will be rejected.

9. Prices must be indicated both in figures and words failing which the tender will be rejected.

10. In case of any error, omission, commission, confusion, typographical mistakes, SBPDCL’s interpretation will prevail.

Electrical Superintending Engineer(Purchase)

SBPDCL

Annexure ‘B’

SOUTH BIHAR POWER DISTRIBUTION CO. LTD., PATNA

TENDER PROFORMA FOR TECHINCAL & COMMERCIAL PART-I

1.	Name and full address of the tenderer:

2.	Name of manufacturer:

3.	Name of material:

4.	Tender notice no. with due date:

5.	Quantity offered:

6.	Deliver schedule:

a)	Monthly delivery schedule:

7.		Validity of the offer (it should not be less than 180 days)

8.	Amount of Earnest Money paid with no. & date of Bank draft:

9.	Acceptance of the terms of payment.

10.	Notarized affidavit (must be submitted)

(I) towards true & genuineness of tender document

(II)Towards Blacklisting /Debarred

11.	Acceptance of repeat extension order

12.	Any deviation from the specifications of material and the terms & conditions of the tender

13.	Acceptance of penalty clause.

14. 	Guarantee

15.	Type Test Report as per relevant IS

16.	Quantity if any, yet to be delivered against pending P.O. delivery Schedule , details of up-to-date supply with quantity and date.

17.	Past Performance (copy of order received and execution details)

18.	Drawing & literature, if any.

19.	Details of technical capability manufacturing capacity per annum.

20.	Details of plant and manufacturing capacity per annum.

21.	Enclosures with the tender should be specified

22.	Name of their Banks with financial arrangement with them.

a)         Cash Credit limit

b)	Raw material inventory limit Rs.

c)	Finished goods hypothecation limit Rs.

d)	Bank guarantee limit Rs.

e)	IDBI limit if any Rs.

f) Any other financial facility given to them by the Bankers

(Letter in original from their Banks may be furnished).

23. Remarks if any.

Name and Signature of

authorized person of the firm/ Tenderer

Annexure ‘C’

SOUTH BIHAR POWER DISTRIBUTION CO. LTD., PATNA

PROFORMA OF BANK GUARANTEE FOR BID GUARANTEE(EMD)

(To be stamped in accordance with stamp Act)

The non-judicial stamp paper should be in the name of issuing bank

Ref……………….							Bank Guarantee

No……….								Date……………………

To

Sr. Manager(F&A)

South Bihar Power Distribution Company Limited

VidyutBhawan

JawaharLal Nehru Marg (Bailey Road)

Patna-800021

Dear Sirs,

In accordance with invitation to bid under your specification No…………… …………… M/s…………………….. having its Registered/Head office at …………………..(hereinafter called the ‘Bidder’) wish to participate in the said Bid or ……….. and you, as a special favour have agreed to accept an irrevocable land unconditional Bank Guarantee for an amount of ………………valid up to …………..on behalf of Bidder in lieu of the Bid deposit required to be made by the bidder, as a Condition precedent for participation in the Said Bid.

We, the…………………………Bank at…………………having our Head office at……………..(Local address) guarantee and undertake to pay immediately on demand by South Bihar Power Distribution Company Limited the Amount of …………………………………………………..

( in words & figures)

Without any reservation protest demur and recourse. Any such demand made by said ‘Owner” shall be conclusive and binding on us irrespective of any dispute or difference raised by the Bidder.

This Guarantee shall be irrevocable and shall remain valid upto and including………………….and claim period is six months beyond validity period.@........................................If any further extension of this exceeding one year) on receiving instruction from M/s …………………………… on whose behalf this guarantee is issued.

In witness whereof the Bank, through its authorized officer, has set its hand and stamp on this…………………………………………………. Day of………………………..20………. at ……………………………….

WITNESS:

………………………………………….			……………………………………….

(Signature)						(Signature)

………………………………………..			……………………………………

(Name)						(Name)

…………………………………………..		……………………………………

(Official Address)						    (Official Address)

@ This date shall be forty-five (45) days after the last date for which the bid is valid.

ANNEXURE- “D”

SOUTH BIHAR POWER DISTRIBUTION CO. LTD., PATNA

PERFROMA FOR SCHEDULE OF REQUIREMENT & SPECIFIED DELIVERIES FOR MONTHWISE DELIVERY SCHEDULE.

| SI. No. | Item No. | Quantity required | Quantity offered | 1st Month | 2nd Month | 3rd Month | 4th Month | 5th Month | 6th Month |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Signature of tenderer/ authorized

Signatory with name, designation & Seal

Notice :-

- Priorities of dispatches will be notified by the purchaser at the time of pre-dispatch inspection.

Name and Signature ofauthorized person of the firm/ Tenderer.

ANNEXURE- “E”

SOUTH BIHAR POWER DISTRIBUTION CO. LTD., PATNA

DEPARTMENT OF STORES & PURCHASE

Past Performance of Supply

Name and Address of the Bidder :

| SI. No. | P.O./ LOI No. & Date | Quantity Ordered | P.O. place by | Delivery Schedule | Qty. Delivered | Quantity Pending | Reason for Non supply | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |

Name and Signature of

authorized person of the firm/ Tenderer.

ANNEXURE- “F”

SOUTH BIHAR POWER DISTRIBUTION CO. LTD., PATNA

DEPARTMENT OF STORES AND PUCHASE.

Contract Performance Guarantee

Bank Guarantee No: _______________________

Date: __________________________________

NIT No: ________________________________

P.O No: ________________________________

Name of contract- "-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------“

To,

Senior Manager (F&A)

South Bihar Power Distribution Co. Ltd./North Bihar Power Distribution Co. Ltd,

Vidyut Bhawan, Bailey Road,

Patna-800021 (Bihar)

Dear Ladies and/or Gentleman,

Amount of Guarantee: Rs __________________/-

Guarantee cover from: _______________________/-

Last date of Lodge of Claim: ____________________

We refer to the P.O No.  _______________________________  dated ___________  by  South Bihar Power Distribution Co. Ltd. (SBPDCL)/ North Bihar Power Distribution Co. Ltd(NBPDCL) , having its Registered Office at VidyutBhawan, Bailey Road, Patna-800021 (Bihar) ("the Employer"/“SBPDCL/NBPDCL”'/”Owner”), (which expression shall unless repugnant to the subject or context include his successors and assigns)  and to ________________________________ having its principal place of business at __________________________________________ and ______________________________________ ("the supplier") concerning  (Name of work) …………………………………………………………………………………………………………………………………………………………………………………………………………………………. against NIT No: ____/PR/SBPDCL/2026,

By this letter we, _____________________________ (insert name & address of the issuing bank) ______________________ having its Registered/Head Office at -----(insert address of registered office of the bank) (which expression shall unless repugnant to the subject or context include his successors and assigns) organized under the laws of India do hereby irrevocably guarantee payment to the "SBPDCL/NBPDCL/Owner" up to Rs. _______________/- (Rupees ______________________ only) i.e., ten  percent (10%) (5% in case of SSI/ Start up unit of Bihar)  of the contract value (Supply works) valid for the period of 30 (Thirty) months from the date of issue of Purchase Order which will be further extended till expiry of the warranty period of last consignment i.e., up to and inclusive of _________________ (Date).

Our liability under the guarantee shall be valid till ____________________. (Date)

We undertake to make payment under this letter of Guarantee upon receipt by us of your first written demand signed by the Employer duly authorized officer or the authorized officer of SBPDCL/NBPDCL, Owner declaring the firm to be in default under the contract and without cavil or argument any sum or sums within the above named limits, without your need to prove or show grounds or reasons for your demand and without the right of the Contractor to dispute or question such demand. Such payment shall be made by us on the same banking day as request without demur and delay and without referring the matter back to the Contractor for what so reason it may be

Our liability under this letter of Guarantee shall be to pay to the  SBPDCL/NBPDCL Owner whichever is the lesser of the sum so requested or the amount then guaranteed hereunder in respect of any demand duly made hereunder prior to expiry of the letter of Guarantee, without being entitled to inquire whether or not this payment is lawfully demanded. However, Our liability under this guarantee shall be up to Rs. _____________________________________. (Rupees ____________________________only)

This letter of Guarantee shall remain in full force and shall be valid from the date of issue until for the period of 30 (Thirty ) months from the date of issue of Purchase Order

which will be further extended till expiry of the warranty period of last consignment i.e. up to and inclusive of ______________ and shall be extended from time to time for such period (not) exceeding one year), as may be desired by SBPDCL.

Except for the documents herein specified, no other documents or other action shall be required. Notwithstanding any law or regulation.

Our liability under this letter of Guarantee shall become null and void immediately upon expiry of claim period, whether it is returned or not, and no claim may be made hereunder after such expiry or after the aggregate of the sums paid by us to the Employer shall equal the sums guaranteed hereunder, whichever is the earlier.

All notices to be given under shall be given by registered (airmail) posts to the addressee at the address herein set out or as otherwise advised by and between the parties hereto i.e. between bank and SBPDCL Owner.

We hereby agree that any part of the contract may be amended, renewed, extended, modified, compromised, released or discharged by mutual agreement between you and the contractor, and this security may be exchanged or surrendered without in any way impairing or affecting our liabilities hereunder without notices to us and without the necessity for any additional endorsement, consent or guarantee by us, provided, however, that the sum guaranteed shall not be increased or decreased.

No action, event or condition which by any applicable law should operate to discharge us from liability hereunder shall have any effect and we hereby waive any right we may have to apply such law so that in all respects our liability hereunder shall be irrevocable and, except as stated herein, unconditional in all respects.

Notwithstanding anything contained herein:

- Our Liability under this Bank Guarantee shall not exceed Rs. ___________/-(Rupees ____________________________________ only).

- The Bank Guarantee shall be valid up to _______________ and claim period six months after validity period.

- We are liable to pay the guaranteed amount or any part thereof under this guarantee only and only if a claim / demand is served upon us on or before ________________ (claim period).

- We hereby unconditionally agree that the SBPDCL shall have power to invoke or encash this letter of Guarantee any time during its validity period.

For and on behalf of the Bank

[Signature of the authorized signatory(ies)]

Signature------------------

Name -------------------------

Designation --------------------

PO Number ----------------------

Contract Number (s): Tel --------------------- Mobile -------------

Fax Number -------------------------------

email -------------------------------

Common Seal of the Bank --------------------------------

Witness:

Signature ---------------------------

Name -----------------------------

Address -------------------------

Contract Number (s): Tel. --------------------- Mobile ----------------

email -----------------

Note:

- For the purpose of executing the Bank Guarantee, the non-judicial stamp papers of appropriate value shall be purchased in the name of Bank who issues the 'Bank Guarantee'.

- The Bank Guarantee shall be signed on all the pages by the Bank Authorities indicating their PO nos. and should invariably be witnessed.

3.The Bank Guarantee should be in accordance with the Performa as provided. However, in case the issuing bank insists for additional paragraph for limitation of liability, the following may be added at the end of the Performa of the Bank Guarantee [i.e., end paragraph of the Bank Guarantee preceding the signature(s) of the issuing authority (ies) of the Bank Guarantee

ANNEXURE- “G”

SOUTH BIHAR POWER DISTRIBUTION CO. LTD., PATNA

DEPARTMENT OF STORES AND PUCHASE.

Sample check list of documents required for Payment

| S. No. | Document Particulars | Approval required from SBPDCL? (yes or no) | Enclosed (yes or no) |
| --- | --- | --- | --- |
| 1 | Bills (In Triplicate) | No |  |
| 2 | Copy of purchase order | No |  |
| 3 | Copy of contract agreement | No |  |
| 4 | Acceptance of bank guarantee | Yes |  |
| 5 | Copy of approval of inspection report | Yes |  |
| 6 | Copy of approval of routine test | Yes |  |
| 7 | Copy of type test report (as per Purchase Order) | Yes |  |
| 8 | Copy of despatch allocation | No |  |
| 9 | Copy of GST Registration certificate | No |  |
| 10 | Copy of Challan& Lorry Receipt | No |  |
| 11 | Complete SRV issued by the consignee | No |  |

Note:- 1. To be customized for each material / equipment.

Annexure ‘H’

SOUTH BIHAR POWER DISTRIBUTION CO. LTD., PATNA

TENDER PROFORMA FOR PRICE PART

| NIT No- |  | :- |  |
| --- | --- | --- | --- |
| Name of material |  | :- | As per NIT |
| Quantity |  | :- | As per NIT |
| 1 | Name of the tenderer | :- |  |
| 2 | Full address of the tenderer | :- |  |
| 3 | Name of Manufacturer | :- |  |
| 4 | Name of materials | :- |  |
| 5 | Quantity offered with monthly quantity delivery schedule and name of the month of commencement of deliver (must be supported by manufacturing capacity) (Minimum quantity shall be offered as per NIT) | :- |  |
|  |  |  | Price to be quoted by bidder per unit |
| 6 | Price breakup |  | ……… ………. |
| (a) | Ex-factory price | :- |  |
| (b) | Freight | :- |  |
| (c) | GST | :- |  |
| (d) | Other Charges, if any | :- |  |
| (e) | Discount if any | :- |  |
| (f) | Landed Rate per Unit (Inclusive of Ex-factory price, freight and all applicable taxes & duties) | :- |  |

ANNEXURE- “I”

SOUTH BIHAR POWER DISTRIBUTION CO. LTD., PATNA

DEPARTMENT OF STORES & PURCHASE

Past Performance of Supply

Name and Address of the Bidder :

Format for Supply Experience

| S.N. | Name of material | P.O. No. with Date | Ordered Quantity, in no | Supplied Quantity, in no | Supplied in FY | Name of end user (govt organisation only) | Type of supply (Direct/turnkey project) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total |  |  |  |  |  |  |  |

Note: Copy of each document must be submitted in support of the above.

Name and Signature of

authorized person of the firm/ Tenderer.

- AFFIDAVIT FORMAT

(To be stamped in accordance with Stamp Act, the Non-Judicial Stamp Paper of Rs 100 should be in the name of Firm)

(To whom so ever it may concern)

I, name-………….. Designation………….. of Firm name with full address……………do hereby solemnly affirm and declare as follow:-

- The material has been type tested as per relevant IS/IEC standard whenever applicable by …… (name of laboratory) …………and the test certificate enclosed with the tender are true and genuine.

(b)	 Our Firm/ Company ………….………… is presently not blacklisted by any State / Centre Utilities / PSUs / govt. entities on the date of submission of bid.

(c)    Our Firm/ Company ………………….… is not debarred/ blacklisted from BSPHCL and its subsidiary companies (i.e. NBPDCL/SBPDCL/BSPTCL/BSPGCL) on the date of submission of bid.

(d) Our Firm/ Company ………………  is not de-empanelled or their vendor approval has not cancelled by BSPHCL and its subsidiary companies (i.e. NBPDCL/SBPDCL/BSPTCL/BSPGCL) in last two years from last date of submission of bid.”

(e) (1) all documents submitted in the bid are true and genuine and (2) all documents to be submitted during the execution of contract, if awarded, will be true and genuine.

(f) Firm/ Company will pay the actual losses caused to utility equipment due to defects / mal-operation of their equipment (if any). The quantification of loss by utility shall be final in this regard.

- That the above statements are true and correct to the best of our knowledge. Sworn and signed on this ….. Day of………2026. If it is found wrong then suitable action as per terms & conditions of NIT may be taken against our firm.

ANNEXURE- “J”

South  Bihar Power Distribution Company Limited, Patna

(DEPARTMENT OF STORES AND PURCHASE)

TECHNICAL SPECIFICATION FOR PRESTRESSED CEMENT CONCRETE POLES

###### PSC Poles

PSC poles shall be of solid rectangular section with an overall length of 9.0M suitable for use in 11 KV & 33KV overhead power lines, LT power lines and double pole structures associated with the lines and for 11/0.4 KV substations.

The poles shall be designed for a working load of 300 kg, applied at 600 mm below the top, with a minimum factor of safety of 2.5.

###### Applicable Standards

Except when they conflict with specific requirements in this Specification, the PSC poles shall comply with the relevant provisions made in the following Indian Standards or the latest versions thereof.

  - IS: 1678, Specification for prestressed concrete poles for overhead power, traction and telecommunication lines.

  - IS : 2905,  Method of test for concrete poles for overhead power and telecommunication lines.

  - IS: 7321, Code of Practice for selection, handling and erection of concrete poles for overhead power and telecommunication lines.

  - IS 1343 (1980): Code of Practice for Prestressed Concrete

  - IS 456 (2000): Plain and Reinforced Concrete

3.0  	Terminology

For the purpose of this specification, following definitions shall apply:-

# 3.1	Average Permanent Load

That fraction of the working load which may be considered of long duration over a period of one year.

3.2	Load Factor

The ratio of ultimate transverse load to the transverse load at first crack.

3.3	Transverse

The direction of the line bisecting the angle contained by the conductor at the  pole.  In the case of a straight run, this will be normal to the run of the line.

  - Transverse Load at First Crack

For design, the transverse load at first crack shall be taken as not less than the value of the working load.

3.5	Working Load

The maximum load in the transverse direction, that is ever likely to occur, including the wind pressure on the pole.  This load is assumed to act at a point 600mm below the top with the butt end of the pole planted to the required depth as intended in the design.

3.6	Ultimate Failure

The condition existing when the pole ceases to sustain a load increment owing to either crushing of concrete, or snapping of the prestressing tendon or permanent stretching of the steel in any part of the pole.

3.7	Ultimate Transverse Load

The load at which failure occurs, when it is applied at a point 600 mm below the top and perpendicular to the axis of the pole along the transverse direction with the butt end of the pole planted to the required depth as intended in the design.

4.0	 Application

# These  poles  may be used for double  pole  structures  of distribution  transformers, on both side  of various crossings to maintain necessary clearances, wherever required, as per REC Construction  Standards  F-2  to  F-4 and for special locations in 11  KV  and  L.T. Lines, such as road crossings etc.

5.0 Material

5.1   Cement

# The  cement used in the manufacture of prestressed  concrete poles  shall be ordinary or rapid hardening Portland cement  conforming  to  IS: 269 - 1976 (Specification for ordinary  and  low heat  Portland  cement) or IS : 8041  E-1978  (Specification  for rapid hardening Portland cement).

5.2 Aggregates

Aggregates used for the manufacture of pre-stressed concrete poles shall confirm to IS : 383 (Specification for coarse and fine aggregates from natural sources for concrete).  The nominal maximum size of aggregates shall in no case exceed 12 mm .

5.3 Water

Water should be free from chlorides, sulphates, other salts and organic matter.  Potable water will be generally suitable.

5.4 Admixture

Admixture should not contain Calcium Chloride or other chlorides and salts which are likely to promote corrosion of pre-stressing steel. The admixture shall conform to IS 9103.

5.5 Pre-Stressing Steel

The pre-stressing steel wires including those used as untensioned wires should conform to IS: 1785 (Part-I) (Specification for plain hard-drawn steel wire for prestressed concrete, Part-I cold drawn stress relieved wire), IS: 1785 (Part-II) (Specification for plain hard-drawn steel wire) or IS:6003 (Specification for indented wire for prestressed concrete). The type designs given in the annexure are for plain wires of 4 mm diameter with a guaranteed ultimate strength of 175 kg/mm2 and for plain wires of 5 mm diameter with a guaranteed ultimate strength of 160 kg/mm2.

All  prestressing steel shall be free  from splits, harmful scratches, surface flaws, rough, aged and imperfect edges and other defects likely to impair its use in prestressed concrete.

5.6 Concrete Mix

# The  concrete mix shall be designed to  the  requirements laid  down for controlled concrete (also called design  mix  concrete)  in  IS : 1343 - 1980 (Code of  practice  for  prestressed concrete)  and  IS : 456 - 1978 (Code of practice for  plain  and reinforced  concrete),  subject to the following  special  conditions;

# a)Minimum works cube strength at 28 days should be  at  least 420 Kg/cm2.

# b)The  concrete strength at transfer should be at  least  210 Kg/cm2.

# c)The mix should contain at least 380 Kg. of cement per  cubic meter of concrete.

d)The mix should contain as low a water content as is consistent  with  adequate workability. If it becomes necessary  to  add water to increase the workability, the cement content also should be  raised in such a way that the original value of water  cement ratio is maintained.

6.0 Design Requirements

The poles shall be designed for the following requirements:

- The poles shall be planted directly in the ground with a planting depth as per IS : 1678. Wherever, planting depth is  required to be increased beyond the specified limits or alternative arrangements are required to be made,  on account of ground conditions e.g. water logging etc., the same shall be in the scope of the bidder at no extra cost to owner. The bidder shall furnish necessary design calculations/details of alternative arrangements in this regard.

- The working load on the poles should correspond to those that are likely to come on the pole during their service life.

- The factor of safety shall not be less than 2.5

- The average permanent load shall be 40% of the working load.

- The F.O.S. against first load shall be 1.0.

- At average permanent load, permissible tensile stress in concrete shall be 30 kg/cm2.

- At the design value of first crack load, the modulus of rupture shall not exceed 53.0 kg/cm2 for M-40.

- The ultimate moment capacity in the longitudinal direction should be at least one fourth of that in the transverse direction.

- The maximum compressive stress in concrete at the time of transfer of pre-stress should not exceed 0.8 times the cube strength.

- The concrete strength at transfer shall not be less than half of the 28 days strength ensured in the design, i.e. 420 × 0.5 = 210 kg/cm²

For  model  check calculations on the design of  poles,  referred  to  in  the annexure, a reference may be made  to  the  REC  “Manual  on  Manufacturing of solid PCC poles,  Part-I  -  Design Aspects”

6.1	Dimensions and Reinforcements

The cross-sectional dimensions and reinforcement details of PSC poles shall conform to the approved drawings and annexure. The pole shall be of tapered rectangular section, with dimensions varying from top to bottom as specified in the approved design.

The prestressing steel shall consist of high tensile wires conforming to relevant Indian Standards and shall be arranged symmetrically about the centroidal axis of the pole to ensure uniform stress distribution and stability under transverse loading.

The number, diameter, spacing, and positioning of prestressing wires and untensioned reinforcement shall strictly conform to the approved design drawings. Adequate cover shall be maintained to protect the steel against corrosion and environmental effects.

Provision of holes for fixing cross-arms, insulators, stay fittings, and other accessories shall be made in accordance with REC Specification No. 15/1979. The holes shall be:

- Accurately located as per standard spacing

- Cleanly formed during casting (no drilling after casting unless approved)

- Free from cracks and surface damage

The diameter and spacing of holes shall be suitable for standard line hardware and shall not adversely affect the structural strength of the pole..

6.2	Manufacture

6.2.1	All pre-stressing wires and reinforcements shall be accurately fixed as shown in drawings and maintained in position during manufacture.  The un-tensioned reinforcement, as indicated in the drawings, should be held in position by the use of stirrups which should go round all the wires.

6.2.2	All wires shall be accurately stretched with uniform prestress in each wire.  Each wire or a group of wires shall be anchored positively during casing. Care should be taken to see that the anchorages do not yield before the concrete attains the necessary strength.

6.3	Cover

The cover of concrete measured from the outside of pre-stressing tendon shall be normally 20 mm.

6.4	Welding and Lapping of Steel

The high tensile steel wire shall be continuous over the entire length of the tendon.  Welding shall not be allowed in any case.  However, jointing or coupling may be permitted provided the strength of the joint or coupling is not less than the strength of each individual wire.

6.5	Compacting

Concrete shall be compacted by spinning, vibrating, shocking or other suitable mechanical means.  Hand compaction shall not be permitted.

6.6	Curing

The concrete shall be covered with a layer of sacking, canvass, hessian or similar absorbent material and kept constantly wet up to the time when the strength of concrete is at least equal to the minimum strength of concrete at transfer of prestress.  Thereafter, the pole may be removed from the mould and watered at intervals to prevent surface cracking of the unit, the interval should depend on the atmospheric humidity and temperature.

The prestressing wires shall be de-tensioned only after the concrete has attained the specified strength at transfer (i.e. 210 kg/cm2 for M40 grade concrete ).  The cubes cast for the purpose of determining the strength at transfer should be cured, as far as possible, under conditions similar to those under which the poles are cured.  The transfer stage shall be determined based on the daily tests carried out on concrete cubes till the specified strength indicated above is reached.  Thereafter the test on concrete shall be carried out as detailed in IS : 1343 (Code of practice for prestressed concrete).  The manufacturer shall supply, when required by the owner or his representative, result of compressive test conducted in accordance with IS : 456 (Code of practice for plain and reinforced concrete) on concrete cubes made from the concrete used for the poles.  If the owner so desired, the manufacturer shall supply cubes for test purposes and such cubes shall be tested in accordance with IS : 456 (Code of practice for plain and reinforced concrete).

The detensioning shall be done by slowly releasing the wires, without imparting shock or sudden load to the poles.  The rate of detensioning may be controlled by any suitable means either mechanical (screw type) or hydraulic.

The poles shall not be detensioned or released by cutting the prestressing wires using flames or bar croppers while the wires are still under tension.

6.7	Lifting Eye-Hooks or Holes

Separate eye-hooks or holes shall be provided for handling the transport, one each at a distance of 0.15 times the overall length, from either end of the pole.  Eye-hooks, if provided, should be properly anchored and should be on the face that has the shorter dimension of the cross-section.  Holes, if provided for lifting purposes, should be perpendicular to the broad face of the pole.

6.8 	Holes for Cross Arms etc

Sufficient number of holes shall be provided in the poles for attachment of cross arms and other equipments.

7.0  	Stacking & Transportation

Stacking should be done in such a manner that the broad side of the pole is vertical.  Each tier in the stack should be supported on timber sleeper located as 0.15 times the overall length, measured from the end.  The timber supported in the stack should be aligned in a vertical line.

Poles should be transported with their broad faces placed vertically and in such a manner that shocks are avoided.  Supports should be so arranged that they are located approximately at a distance equal to 0.15 times the overall length from the ends.  The erection of the pole should be carried out in such a way that the erection loads are applied so as to cause moment with respect to the major axis, i.e. the rope used for hoisting the pole should be parallel to the broader face of the pole.

8.0	Earthing

8.1	Earthing shall be provided by having length of 8 SWG GI wire embedded in concrete during manufacture and the ends of the wires left projecting from the pole to a length of 100mm at 250 mm from top and 150 mm below ground level.

8.2 Earth wire shall not be allowed to come in contract with the prestressing wires.

9.0	Tests

9.1	Transverse Strength Test

9.1.1Poles made from ordinary Portland cement shall be tested only on the completion of 28 days and poles made from rapid hardening cement only on the completion of 14 days, after the day of manufacture.

9.1.2The poles may be tested in either horizontal or vertical position.  If tested in horizontal position, provisions shall be made to compensate for the overhanging weight of the pole, for this purpose, the overhanging portion of the pole may be supported on a movable trolley or similar device.

9.1.3The pole shall be rigidly supported at the butt end for a distance equal to the agreed depth of planting

    - Load shall be applied at a point 600 mm from the top of the pole and shall be steadily and gradually increased to the design value of the transverse load at first crack.  The deflection at this load shall be measured.

    - A prestressed concrete pole shall be deemed not to have passed the test if  cracks wider than 0.1 mm appear at a stage prior to the application of the design transverse load at first crack and the observed ultimate transverse load is less  than the design ultimate transverse load.

    - The load shall then be reduced to zero and increased gradually to a load equal to the first crack load plus 10% of the minimum ultimate transverse load, and held up for 2 minutes.  This procedure shall be repeated until the load reaches the value of 80 per cent of the minimum ultimate transverse load and thereafter increased by 5 per cent of the minimum ultimate transverse load until failure occurs.  Each time the load is applied, it shall be held for 2 minutes.  The load applied to prestressed concrete pole at the point of failure shall be measured to the nearest five Kilograms.

    - The pole shall be deemed not to have passed the test if the observed ultimate transverse load is less than the design ultimate transverse load.

9.2	Measurement of Cover

    - After completion of the transverse strength test, the sample pole shall be taken and checked for cover.  The cover of the pole shall be measured at 3 points, one within 1.8 meter from the butt end of the pole, the second within 0.6 meters from the top and the third at an intermediate point and the mean value compared with the specified value.

    - The mean value of the measured cover should not differ by more than (+) 1 mm from the specified cover.  The individual values should not differ by more than (+) 3 mm from the specified value.

    - If these requirements are not met, the workmanship with reference to aligning of the end plates and prestressing wires and assembly of moulds should be improved and inspection at pre-production stage tightened suitably.

- Sampling and Inspection

10.1	Scale of Sampling

10.1.1	Lot:  In any batch, all poles of the same class and same dimensions shall be grouped together to constitute a lot.

    - Sub-lot:  If the number of poles in a lot exceed 500, the lot shall be divided into a suitable number of sub lots such that the number of poles in any sub-lot shall not exceed 500.  The acceptance or otherwise of a sub-lot shall be determined on the basis of the performance of samples selected from it.

    - The number of poles to be selected from a lot or a sub-lot shall depend upon its size and shall be in accordance with Col.1 and 2 of the following table:

## SAMPLE SIZE AND CRITERION FOR CONFORMITY

| Size of lot or sub-lot | Dimensional Requirement |  | No. of poles for transverse strength test |
| --- | --- | --- | --- |
|  | Sample Size | Permissible No. of defective samples |  |
| Upto 100 | 10 | 1 | * |
| 101 to 200 | 15 | 1 | 3 |
| 201 to 300 | 20 | 2 | 4 |
| 301 to 500 | 30 | 3 | 5 |

*The number of poles to be tested shall be subject to the agreement between the owner and supplier.

    - These poles shall be selected at random.  In order to ensure randomness, all the poles in the lot or the sub-lot may be arranged in a serial order and starting from any random pole, every rth pole may be included in the sample, r being the integral part of N/n where N is the size of the lot or the sub-lot and n is the sample size.

  - Number of Tests

10.2.1           All the poles as selected in 10.1.3 shall be tested for overall length, cross-section and uprightness.  The tolerance shall be (+) 15 mm on overall length (+), 3 mm on cross sectional dimensions and 0.5 per cent on uprightness.

    - The number of poles to be tested for transverse strength test shall be accordance with Col. 4 of the above table.  These poles may be selected from those already tested in 10.2.1

  - Criteria for Conformity

    - A lot or sub-lot shall be considered as conforming to this specification if the conditions under 10.3.2  and 10.3.2 are satisfied.

    - The number of poles which does not satisfy the requirements of overall length, cross-section and uprightness shall not exceed the corresponding number given in Col.3 of Table in 10.1.2. If the number of such poles exceeds the corresponding number, all poles in the lot or sub-lot shall be tested for these requirements and those not satisfying the requirements shall be rejected.

    - All the poles tested for transverse strength test shall satisfy the requirements of the test.  If one or more poles fail, twice the number of poles originally tested shall be selected from those already selected and subjected to the test.  If there is no failure among these poles, the lot or the sub-lot shall be considered to have satisfied the requirements of this test.

- Marking:-

- The following engraving should be done on the poles during its casting at 3 meters height from bottom:-

| Sr. no. | Particulars |
| --- | --- |
| (i) | Name of Manufacturer |
| (ii) | (S&P) NBPDCL/SBPDCL |
| (iii) | P.O. No. with date |
| (iv) | Transverse strength of pole in Kg |
| (v) | Date of Casting |
| (vi) | Pole Sl. No. |
| (vii) | Blue Colour indelible depth marker at 1.5 Meters from bottom to verify planting depth |

The above engraving upon manufactured Pole is mandatory and it must be clearly visible. If it is found during inspection and delivery that any particulars mentioned above is not engraved upon manufactured Pole then that Pole will be out rightly rejected.

12.0	           Drawings

  - The drawings submitted by the Contractor shall be approved/commented by the Owner as the case may be within thirty (30) days of receipt of drawings in Owner's design office. If the designs/drawings are commented by the Owner, the Contractor shall submit revised design/drawings duly incorporating all comments within fifteen (15) days of date of issue of comments.

  - Pole accessories drawings like name plate, danger plate, anti-climbing device etc. shall be prepared by the Contractor and submitted to the Owner in three copies along-with one re-producible, for record. These drawings shall be prepared in A4 size only.

  - While submitting the drawings, the Contractor shall clearly indicate on each drawing Specification No., Name of the line, letter reference no. and date on which the submission are made. The same practice is also to be followed while submitting distribution copies.

  - The Contractor shall furnish the drawings in six(6) copies to the Owner for necessary distribution within fifteen (15) days after the receipt of the approval from the Owner.