NIT No: 58/PR/SBPDCL /2026

SOUTH BIHAR POWER DISTRIBUTION CO. LTD, PATNA

(STORES AND PURCHASE)

###### TENDER   SPECIFICATION FOR

11 KV 3 element outdoor type oil immersed combined metering unit of CT &   PT of 0.2s accuracy class

Tender Cost Rs. 11,800.00 with Applicable GST @18%

# NORTH BIHAR POWER DISTRIBUTION CO LTD: PATNA

## DEPARTMENT OF STORES AND PURCHASE

General conditions for submission of tenders for supply of materials:

- Online tender for Domestic Competitive bidding is invited from technically & financially sound reputed manufacturers registered under the Companies Act 1956 /2013 or any other act for supply of 11 KV 3 element outdoor type oil immersed combined metering unit of CT & PT of 0.2s accuracy class conforming to the relevant IS-16227:2016 and as well as per technical specifications through. The equipment has to be type tested as per relevant IS/IEC standard with latest amendment whichever applicable. The quantity to be procured is indicated as detailed below:-

| Name of Materials | Type of materials | Ratio | Quantity of NBPDCL | Quantity of SBPDCL | Total Quantity |
| --- | --- | --- | --- | --- | --- |
| 11 KV 3 element outdoor type oil immersed combined metering unit of CT & PT of 0.2s accuracy class | 11 KV Consumer Metering Unit (Cat-C) | 100/5A | 70 Nos. | Nil | 70 Nos. |
|  |  | 50/5A | 150 Nos. | 50 Nos. | 200 Nos. |
|  |  | 25/5A | 650 Nos. | 150 Nos. | 800 Nos. |
|  |  | 10/5A | 550 Nos. | 150 Nos. | 700 Nos. |
| Total |  |  | 1420 Nos. | 350 Nos. | 1770 Nos. |

The requirement mentioned in the tender notice & General Condition is tentative. The quantity may increase or decrease (±30%)  at the time of finalization of tender subject to requirement.

The Company reserves the right to cancel part or whole quantity of the material under any head under special circumstances.

- Joint Ventures (JVs) / Associate and subletting is not allowed.

- Scope of Work : The scope of work in brief shall include design, manufacture, type testing, inspection, supply, loading at factory, transportation to site / stores,  unloading at site/stores including guaranteed obligation  of  complete  supply  of  materials  in  conformity  to  the  technical  specification enclosed herewith.

The monthly delivery schedule for the quantities which can be delivered should be indicated as per clause 13 (Delivery Schedule).

- Following shall be the  qualification requirements:-

- TECHNO COMMERCIAL REQUIREMENTS:-

A1. Technical:-

- Type Test(PQR):- Tender material has to be type tested for at least one ratio of tendered material as per relevant IS-16227:2016/IEC standard from CPRI/NTH/ERDA/NPL. The test certificate issued within last (5) five years from last date of submission of bid from CPRI/NTH/ERDA/NPL will be accepted.

- Supply Experience: The bidder must have supplied at least 885 nos. 11 KV combined metering unit within the last 5 (five) years from last date of submission of bid  to any Govt. Power Utility / Indian Power Distribution Company/ Govt. Organization or in EPC/Turnkey project  if end user is any Govt. Power Utility / Govt. Organization  the scope shall also be  considered:

In case of direct supply to any Govt. Power Utility/ Govt. Organization /Indian Power Distribution Company:-Bidder shall have to submit copies of un-priced/priced purchase orders and proof of actual delivery of materials to the client such as Work completion Certificate or execution status of each order supported by duly acknowledged delivery challans/ excise duty/ GST/  gate passes or, client’s certificate or, Notarized affidavit of self-declaration for successfully supply of aforesaid required quantity of 11 KV metering unit.

In case of supply to any Govt. Power Utility / Govt. Organization/ Indian Power Distribution Company through EPC/Turnkey project:-Firm has to submit copies of un-priced/priced purchase order/ work order and  documentary evidence that end user is any Govt. Power Utility / Govt. Organization and proof of actual delivery of materials to the client such as  Work completion Certificate or  execution status of each order supported by duly acknowledged delivery challans / excise duty/ GST/  gate passes or, client’s certificate or, Notarized affidavit of self-declaration for successfully supply of aforesaid required quantity of 11 KV metering unit.

- Bidder's Annual Production Capacity:

The bidder’s Annual production capacity shall not be less than the two times of tendered quantity required to be supplied. The bidder must submit Annual reports/certificate issued by MSME/Charted Accountant/Chartered Engineer/NSIC/ Directorate of Industries , clearly mentioning the annual production with an affidavit in this regard during the last 3 (three) Financial years ending March 2025.

- Registration Certificate: - Documentary evidence regarding category of the manufacturer (Micro / Small / Medium / Large) like Registration Certificate issued by SSI / NSIC / Directorate of Industries etc.

- Entity Registration:

- The bidder shall be a legal entity registered or incorporated in India as per the Companies Act 1956/2013 or any other act.

Certificate of Incorporation/Memorandum of Association/ Partnership deed etc must be furnished for the same.

- In-house inspection/testing facilities:

- Confirmation regarding availability of in-house inspection/ testing facilities or access through other government approved agencies for performing inspection/ testing needs to be provided.

The Bidder shall give details of its Testing facilities at manufacturer’s work with copies of calibration/re-calibration certificates of the major testing equipment

A2 . Commercial:-

- Minimum Average Annual Turnover(MAAT) :-

- The vendor must have MAAT of at least INR Ten Crores only for the best three (3) financial years out of last five (5) financial years ending March 2025.

Audited Annual Accounts of the last 5 (five) financial years up to FY 2024-25 needs to be submitted.

- For SSI unit of Bihar, vendor must have turnover of at least 50% of the MAAT requirement as mentioned above (i.e. INR 5 Crores only) for any one year out of last five (5) financial years up to FY 2024-25  . Required Audited balance sheet must be accompanied with bid documents along with CA Certificate indicating  turnover of last 5 FY upto FY 2024-25.

- Net Worth: - Net Worth of last three financial year up to FY 2024-25  shall be positive.

- Audited Annual Accounts:- Audited Annual Accounts for the last Five (5) financial years up to FY 2024-25  (along with Auditor’s report & notes to Accounts) to establish the capability of the bidder to successfully perform the tender and deliver the materials or IT Return duly acknowledged by the Income tax department for the last Five (5) years.

- Copies of PAN Card, GST Registration certificate alongwith GST R-9 of last 5 FY upto FY 2024-25.

- CA certificate regarding turnover & net worth for last five financial year up to FY 2024-25 .

All documents as indicated herein is required to establish bidder's financial and technical qualification must be submitted as part of techno-commercial bid. Bids shall not be considered if the past documentary credentials of the bidder submitted by him are found un-satisfactory.

A3.  Following Notarized affidavit to be submitted by bidder failing which their tender shall not be accepted:-

- A Notarized Affidavit to the effect that the material has been type tested as per relevant IS/IEC standard whenever applicable by CPRI/ NTH/ERDA/NPL and the test certificate enclosed with the tender are true and genuine must be uploaded with the tender documents.

(b)	 Bidder has to submit a notarized affidavit for the effect that they are presently not blacklisted by any State / Centre Utilities / PSUs / govt. entities on the date of submission of bid.

(c)    Bidder has to submit a notarized affidavit for the effect that they are not debarred/ blacklisted/de-empanelled from BSPHCL and its subsidiary companies (i.e. NBPDCL/SBPDCL/BSPTCL/BSPGCL)on the date of submission of bid.

(d)Bidder has to submit a notarized affidavit to the effect that they are not de-empanelled or their vendor approval has not cancelled by BSPHCL and its subsidiary companies (i.e. NBPDCL/SBPDCL/BSPTCL/BSPGCL) in last two years from last date of submission of bid.”

(e) Notarized Affidavit for(1)all documents submitted in the bid are true and genuine and (2)all documents to be submitted during the execution of contract, if awarded, will be true and genuine.

A4.  Others:-

- Debarment/Blacklisting/ de-empanelled Notifications issued by any subsidiary of BSPHCL shall automatically extend also to other subsidiaries.

- Firms black listed by any State / Centre Utilities / PSUs / govt. entitieswill also stand black listed till their black listing is revoked.

- Following details are also to be submitted:-

a)    Number of Years in Production.

b)    Certification of Design to the relevant standard.

c)    Existence of Local office (if applicable).

d)    Existence of service network and parts stock.

e)    Following Good Manufacturing Practices (GMP).

f)    Location of factories/ manufacturing units.

g)	Banker’s Certificate for various Fund based credit limit & utilization.

- Both part of tender (Part-1 Technical & Commercial part and Part – II Price part) can be Downloaded/Uploaded from the website - 	www.eproc2.bihar.gov.in. Tender cost of Rs. 11,800.00 including 18% applicable GST and bid Processing fee as per provisions of eproc portal (in addition to cost of tender)  are  mandatory to be paid through online mode i.e. Internet Payment gateway (Credit/Debit Card), Net banking, NEFT/RTGS. Bids along with necessary online payments must be submitted through e-payment portal www.eproc2.bihar.gov.in before date and time specified in the Tender Inviting Notice or its subsequence extension if any. The department does not take any responsibility for the delay/Non Submission of tender/Non Reconciliation of online payment caused due to Non-availability of Internet Connection, network Traffic/Holiday or any other reason. Copy of receipt of online payment is to be submitted to department (S&P), NBPDCL, Patna as per schedule.

M.S.E.(Micro & Small Enterprises) and  Start-ups Unit of Bihar  is exempted for submission of Tender Cost.

- Deposit of Earnest Money(EMD)/

(a) An Earnest money of Rs. 25,19,035.00 only should be deposited to Electrical Superintending Engineer(Purchase), SBPDCL, Patna in the form of Bank Draft or Bank Guarantee issued by Nationalized Bank / Scheduled bank of India & drawn in favour of Sr. Manager (F&A ), SBPDCL payable at Patna valid for 45 days beyond the bid validity period in prescribed enclosed format as per date and time specified in the Tender Inviting Notice or its subsequence extension if any , failing which the tender will not be  considered. It may also be deposited through E-payment mode in SBPDCL’s SB A/C No. 201001371988 (Indusind Bank, Boring Road Patna, IFS Code No. –INDB0000393).

(b) Cheques are not acceptable towards deposit of Earnest Money. Cheques on account of Earnest Money will be considered without Earnest Money and the tender will be not considered.

(c) No Earnest Money will be accepted after opening of the tender. Earnest money deposited with other NIT is not adjustable with this NIT. No Bank commission or interest will be paid on Earnest Money.

(d)If discom concludes that bid evaluation and award of contract cannot be completed within original validity due to reasons beyond control, it will ask for consent of bidders to extend validity period.

(e)The request and the responses thereto shall be made in writing or by e-mail to all participating bidders;

(f)A bidder agreeing to the extension request shall also agree to suitable extension of validity period of EMD. Such a bidder shall not be required to or permitted to modify its bid;

(g)A bidder may refuse the request to extend the bid validity which would lead to his disqualification without forfeiting his EMD. In such a case the bid shall not be further considered for evaluation and award and EMD shall be returned.

- MODIFICATION AND WITHDRAWAL OF BID

a)	The bidders may modify or withdraw their bids after submission but before expiry of the last date & time for submission of bids. Multiple submissions of bids are permitted.

b)    	The intimation for bid withdrawal/ modification received after last date &time for bid submission shall be ignored and bid already submitted shall be deemed to be a validly submitted bid.

c) 	No bid will be withdrawn/ modified after the last date & time for submission of bid till bid validity period.

d) 	Withdrawal or modification of bid after bid submission date and time during bid validity period will result in forfeiture of EMD.

- Definition:-

- Local content

Local content” means the amount of value added in Bihar which shall, unless otherwise prescribed by the Competent Authority laid down under this policy, be the total value of the items procured (excluding net domestic indirect taxes) minus the value of content brought from outside Bihar as a proportion of the total value, in percent.

The local enterprise at the time of tender or bidding shall be required to indicate percentage of local content and provide self-certification that the item offered (quantifying the infrastructure such as the production unit, manpower, electricity consumption, raw material & land possession/

building premises) meets the local content requirement for local enterprise as the case may be. They shall also give details of the location(s) at which the local value addition is made.

- Local Enterprise

  - Registered in the State of Bihar

  - Production facility in the State of Bihar.

  - Minimum Local content-30%

- Special Category of enterprises

  - Bihar Micro and Small Enterprises (BMSEs)

    - Registered in the State of Bihar

    - Valid Udyam Registration

    - Shall meet the minimum local content requirement of 30%

  - Start-up: Start-ups as defined in Bihar Start-up Policy 2022 as amended from time to time).

- Margin of purchase preference :- Maximum extent to which the price quoted by a “Local Enterprise” may be above the L1 for the purpose of purchase preference

- Non-Local Enterprise :-An enterprise, whose goods offered for procurement, has the local content less than 30 %.

- EXEMPTION:- The tenderers of the following categories are exempted from deposit of Earnest Money subject to the conditions laid down below and such firm must upload the registration certificate for exemption as well as to submit the same to Electrical Superintending Engineer(Purchase), before the scheduled date and time failing which the tender will be rejected.

- M.S.E.(Micro & Small Enterprises) and  Start-ups Unit of Bihar  for manufacture of the items for which the tender is submitted. The tender in such case must be supported with a certified copy of an up to date/valid certificate of their registration failing which the tender will be rejected.

- The State Govt. /Govt. of India undertakings.

Tenders not accompanied with adequate amount of Earnest Money/ required exemption certificate will be rejected out rightly.

- Margin  of  Purchase Preference for Local Enterprises of Bihar  :-

Price preference will be given to Local Enterprises of Bihar as per Notification No.M-4-03/2021 8550/F Patna dated 07.08.2024 of Finance Department, GoB. "Margin of purchase preference" means the maximum extent to which the price quoted by a "Local Enterprise' may be above the L-l for the purpose of purchase preference .The margin of purchase preference for Local Enterprise shall be 15% for the procurement of goods .

- Local content requirement

The minimum local content requirement for the local enterprise for goods related procurement is 30%. In support of Local content, bidder has to submit following:-

(i) The local enterprise at the time of tender or bidding shall be required to indicate percentage of local content and provide self certification that the item offered (quantifying the infrastructure such as the production unit, manpower, electricity consumption, raw material & land possession/ building premises) meets the local content requirement for local enterprise as the case may be. They shall also give details of the location(s) at which the local value addition is made.

(ii) In case of procurement for the value in excess of Rs. 5 crores, the local enterprise shall be required to provide a certificate from the auditor of the company (in the case of companies) or from a practicing cost accountant or chartered accountant (in respect of enterprises other than companies) giving the percentage of local content quantifying the infrastructure such as the production unit, manpower, electricity consumption, raw material & land possession/ building premises.

- Contract Performance Guarantee (CPG):- The successful vendor shall be required to submit CPG before execution of Contract Agreement in shape of Bank Guarantee issued by any Nationalised / Scheduled Banks of India on non-judicial stamp paper of Rs.1,000/-(Rupees One Thousand) only or value of paper as prevailing at that time  in  the prescribed format equivalent to ten  percent (10%)  ( 5% in case of M.S.E.(Micro & Small Enterprises) & Start-ups  Unit of Bihar )  of order value to protect interest of the DISCOM in case of breach of contract. The CPG should be valid for the period of 70 (Seventy) months from the date of issue of Purchase Order which will be further extended till expiry of the warranty period of last consignment.

No Bank commission or interest shall be paid by the DISCOMs.

The 10% ( 5% in case of M.S.E.(Micro & Small Enterprises) & Start-ups  Unit of Bihar )   CPG of Total Order Value should be in favour of Sr. Manager(F&A), North Bihar Power Distribution Company Limited/ South Bihar Power Distribution Company Limited and submitted to the Sr. Manager (F&A) NBPDCL / SBPDCL for acceptance after confirmation from issuing Bank.

The CPG shall be discharged and returned to the Agency within thirty (30) days of expiry of the warranty period of last consignment. If the vendor fails to remedy any reported defect within warranty period, the DISCOM shall be entitled to either en-cash the CPG or to remedy the defect at the expense of the vendor.

Details of SBPDCL Account is given below:-

A/c name SBPDCL Capital

A/c no: 0352101042077

IFSC CNRB0000352

Canara Bank Main Branch, Patna

Note: If the rate quoted by the L-1 bidder is more than or less than by twenty five percent (25 %) of the Company’s cost estimate, this will be treated as abnormally high/ low. In this case, successful bidder/s will be required to enhance the Performance security (i.e. CPG) by additional 10% at bidder's expense to a level sufficient to protect the company interests against likely financial losses in the event of default under the contract.

- DELIVERY: The material are required to be delivered within 07 (seven) months from the date of issue of Purchase Order as per the following delivery schedule.

Within 2 (two) months 	          - 	 at least 20 % of the ordered quantity

During 3rd (Third) month          -	 at least 20 % of the ordered quantity

During 4th (Fourth) month        -	 at least 15 % of the ordered quantity

During 5th (Fifth) month           -	 at least 15 % of the ordered quantity

During 6th (Sixth) month          -	 at least 15 % of the ordered quantity

During 7th (Seventh) month      -	 at least 15 % of the ordered quantity

Early delivery shall be preferred.

The proforma for schedule of delivery is enclosed with the Tender; Tenderers are requested to fill up the proforma giving a clear schedule of month wise delivery capability.

The authority reserves the right to cancel the order/contract in part or full for default or delayed supply or supply of sub standard materials without assigning any reason and action like forfeiture of CPG and blacklisting/debarment of the firm shall also be taken.

- Price: - The Ex-Works rate &Freight per unit must be quoted in words and figures in the prescribed proforma. For landed price, per Unit GST shall be taken as 18% or as applicable in addition to the quoted unit Ex works rate & unit freight. The price[Ex-works rate] is variable as per relevant IEEMA Circular .  Firm shall have to quote unit Ex-works& unit Freight up to F.O.R. destination and other charges if any. However, applicable GST shall be paid by NBPDCL/SBPDCL.

If the difference is found between the rate quoted in words & figures, then the rate quoted in words will be considered.

Note:- The price should be quoted after considering  the benefit of input credit in GST or any other assistance available to the supplier and the benefit available to them will be retained by them. A certificate/ undertaking to this effect should also accompany the tender.

- Evaluation of Bid & Clarifications on bid:-

During bid evaluation, SBPDCL may, if needed, seek clarification (s) from Bidder (s) in support of compliance to QR. SBPDCL shall be fully authorized to seek any QR related clarifications from the bidders during evaluation of bids, if required, without need of any further approval.

No change in stipulated QR shall be made after opening of bids and in case QR needs modifications, re-tendering shall be done. Evaluation shall be done only on the basis of uploaded documents by the Bidders

The bidder shall not be permitted to submit any additional document during clarification stage, which enhances his original submitted credentials.

Any post-bid change in the price or substance of the bid, if given by the bidder, shall not be accepted in case of Open tender and Limited tender

Bidder shall have to fill the Price sheet of Financial Bid online. L-1 bidder will be decided on the lowest landed rate by the participating bidder for each item.

If there are more than one bidder who have quoted lowest landed rate in the tender, final lowest bidder(L-1 bidder) will be decided by draw of lots in presence of all such lowest bidders or their authorized representative.

- Insurance - The Insurance will have to be underwritten with the Company underwriters against its open policy in all cases. As such the Insurance premium should not be quoted by the bidder and should not be included in the quoted price.

- (I) Post-delivery Type Test:-

One type test (of any ratio) against each P.O. shall be done separately for each DISCOM. Sample against each PO of NBPDCL & SBPDCL will be selected and sent for post delivery type test. Post Delivery Type test will be done separately against each purchase order of both DISCOM, NBPDCL & SBPDCL. The purchaser will select random sample/samples from any lot of delivered / supplied item in the Stores of DISCOM in presence of representative of manufacturer and the sample will be jointly sealed at site which will be sent for type test from Central Power Research Institute (CPRI).The transportation and test will be carried out in the presence of NBPDCL/ SBPDCL officials and firm’s representative. The results of this type test/test sample must confirm the relevant IS & approved GTP. The Vendor shall bear this transportation, loading, unloading ,testing charges etc.

Loading at Store, safe transportation to CPRI, unloading at CPRI, if after testing reconditioning of material is required then safe transportations to vendors workshop, unloading and reconditioning of material and then safe transport back to Purchaser Store shall be in the scope of vendor.

- From the submitted bill of firm against delivered material, 15% of bill value as 'keep-back' shall be retained till obtaining of type test report from CPRI. It will be released after obtaining of successful Type Test report or against submission of Bank Guarantee (BG) of equivalent ‘keep-back’ amount upto maximum 15% of the total ordered value.

- The BG submitted against aforesaid type test report shall be initially valid till 12 months from the date of purchase order or 90 days beyond the submission of type test report, whichever is later.

- All Routine tests will have to be conducted on the materials according to relevant standard (latest edition).  In case the order is placed the routine test certificate along with the soft copy of Routine test conducted will have to be furnished with supply of material. Untested material will not be accepted Vendor should have all the test facilities available in India only.

- In case of sample failing in aforesaid type tests, following actions will be taken:

- Short closure of the Contract.

- For the quantity of material utilised, the firm shall have to extend Guarantee Period for additional 50% period (i.e. additional two & half years over five (5) years original Guarantee period).

- 10% additional BG against utilised material value shall be submitted by the  supplier which will be valid through the extended guarantee period (i.e., seven & half years).

- All un-utilised materials supplied against the Purchase Order shall be taken back by supplier at their own cost and risk after refund of payment already made against these materials.

- Supplier shall reimburse the cost of type test fee and associated expenditure such as transportation etc.

- In addition, firm shall be penalised for supplying bad quality material by way of debarment for two years.

Note:- All the above activities such as extension of Guarantee period as mentioned above, submission of 10 % additional PBG of utilized material value, refund of paid amount against un-utilized materials, reimbursement of cost of type test fee and associated expenditure such as transportation etc. shall have to be completed by the supplier within 30 days from date of issue of notice for the same.

- If the supplier fails to comply as mentioned in the above para 17.I.A. (a),(b), (c),(d),(e) &(f) , following action shall be taken:-

          - 15% Retention amount/ Bank Guarantee against type test and 10% retention amount against Penalty shall be forfeited.

          - CPG amount will be forfeited.

          - Balance amount shall be recovered from the bills of other works/PO or CPG against other work/PO of the vendor under NBPDCL/SBPDCL or with any other subsidiary of BSPHCL or even from the other contracts of the vendor

          - Firm shall be blacklisted for 3 years.

          - If any balance amount is still left, legal action like filling of Certificate case for recovery of balance paid amount will be done under Bihar and Orissa Public demand Recovery Act.

Note: All the above mentioned terms & condition shall be the part of the Contract agreement to be executed between the firm & DISCOM after placement of Purchase Order.

16. (II) Routine Tests: Routine Test will have to be conducted on 100% quantity by the firm before material is offered for pre-dispatch inspection, according to relevant standard, and tests certificate in triplicate will have to be submitted to department (S&P). Full quantity of the materials shall be subjected to the test prescribed in the relevant standard (latest edition) before supply is affected. Untested material will not be accepted.

- Packing: -Material shall be delivered suitably packed as per ISS. It should be robust enough for rough handling during transportation by Rail/Road. Further it is to clarify here that all the charges towards packing, forwarding, loading, unloading etc shall be borne by the supplier.

- Forfeiture of  Contract Performance Bank Guarantee:-

It should be clearly understood that in the event of the vendor failing to accept and execute the detailed purchase order communicated through email / India Post, Courier or any other communicating medium if it is placed within the validity period of the offer, then the full amount of contract performance bank guarantee (CPG) will be forfeited and the DISCOMs decision in this respect will be final and binding on the vendor. CPG will also be forfeited in case it is found that the bidder has submitted forged document in offered bid and thereafter.

- Standard activity wise indicative timeline of NBPDCL/SBPDCL for material / equipment procurement:-

| Activity Particulars | Timeline | Start Date (P.O date) | End Date |
| --- | --- | --- | --- |
| Letter from vendor with request for material inspection after manufacturing of material & its readiness for inspection from date of PO | 15days | T | T+15 days |
| Intimation of Inspection & Raising of Inspection by NBPDCL/SBPDCL | 4 days | T+ 15 days | T+19 days |
| Inspection of Material by NBPDCL/SBPDCL official(s)/ Authorize Representative & Submission of Inspection Report. | 8 days | T+19 days | T+27 days |
| Issuance of Dispatch Instruction (D.I.) with Dispatch Schedule | 4 days | T+27 days | T+31days |
| Delivery of Material at its its destination as per NBPDCL/SBPDCL requirement (included in DI) | 10 days | T+31 days | T+41days |

where, T is the date of issuance of Purchase Order (P.O.) to the vendor/ supplier

- Progress Report: The suppliers will have to submit monthly progress report regularly until completion of delivery to the Electrical Superintending Engineer (S&P),North Bihar Power Distribution Co. Ltd./ South Bihar Power Distribution Co. Ltd, Patna by the Ist week of every month following the month in which order is placed.

- Terms of Payment: -

    - 75% payment along with full amount of taxes, duties and freight keeping 10% retention money & 15% against type test report, on submission of triplicate copy of bills with all relevant documents (Sample check list) enclosed in annexure by the vendor after receipt of the material / equipment in good condition. After successful completion of delivery of entire order or modified quantity as decided during its delivery schedule, 10 % retention money may be released after deduction of penalty.

    - 15% of bill amount shall be keeping back up to submission of type test report. This 15% keep back amount shall be released against the submission of equivalent amount of BG.

    - Payment will be released normally within 15 working days from the date of submission of bill along with all required relevant document(s) to Sr. Manager (Finance), NBPDCL/SBPDCL.

    - Payment to be made by Sr. Manager (Finance), NBPDCL / SBPDCL.

    - Payment will be made only after execution of contract agreement.

    - In case the vendor / supplier is micro or small enterprises
covered under the MSME development act, 2006, payment will be made as
per provision made in the act i.e. 45 days from the acceptance of
material provided the supplier submit the bill with all relevant
records minimum 15 working days in advance.

    - Deviation from standard terms of payment of NBPDCL / SBPDCL is not acceptable.

- Penalty

    - Retention amount @10% of the bill value excluding GST will be deducted from each bill of the vendor/supplier/agency to ensure adherence to the scheduled delivery period of entire ordered quantity or as modified.

    - After completion of delivery schedule, or as the final quantity of delivery of material as decided, the complete delivery cycle for the material / equipment will be reviewed to assess the applicability of penalty @0.1% per working day plus applicable GST, if any, on the value of material delayed subject to maximum ceiling of 10% of the total ordered value of PO plus applicable GST, if any.

    - Material / equipment delivered in physically damaged condition or, incomplete is liable to be rejected. Such item shall be lifted back by vendor / supplier within 15 days from receipt of rejection notice from NBPDCL / SBPDCL and have to supply back the material within maximum upto next 30 days.

    - Material will be deemed to have been delivered only when all its components parts are also delivered. If certain components are not delivered in time the material will be considered as delayed with such time when missing parts are delivered.

    - The period for levy of penalty will be considered based on the reasons of delay which may be attributable to completely vendor/supplier or completely DISCOM or partly vendor/supplier or partly DISCOM. Penalty shall not be imposed for the delay not attributable to supplier.

    - No penalty will be applicable in late delivery of sample material sent for type test.

Note: As per terms & conditions of penalty, if value of deducted penalty along with GST exceeds maximum ceiling value of 10% of the total ordered value of PO, then the excess amount of applicable tax, GST on the penalty shall be deposited additionally by the supplier within one month from the date of notice to the supplier. However, the maximum ceiling limit of penalty against Late Delivery (LD) shall be as per the NIT. If the aforesaid amount of tax, GST is not deposited within one month, the required amount will be realized from the CPG.

- Validity Period:-The tenderers should specify the validity period of their offer which should be for a minimum period of 180 days from the last date of submission of bid.

    - Bidders should specify the validity period of their offer/bid which should be valid for a minimum period of 180 days from the last date of submission of bid. The offers which are not valid for at least 180 days from the last date of submission of bid shall be rejected out rightly.

    - If DISCOM concludes that bid evaluation and award of contract cannot be completed within original validity due to reasons beyond control, it will ask for consent of bidders to extend validity period.

    - The request and the responses thereto shall be made in writing or by e-mail to all participating bidders;

    - A bidder agreeing to the extension request shall also agree to suitable extension of validity period of EMD. Bidders shall not be allowed to modify their bids.

    - If any bidder refuses the request to extend the bid validity, the bidder shall be disqualified from the ongoing Tender and accordingly Tender will be finalized among rest participating bidders.

- Raw Materials:-No raw materials will be arranged by the Company Tenderers will have to arrange their requirements of raw materials themselves.

The purchaser reserves the right to inspect the raw material procured by the supplier to check the genuineness/quality of raw materials for the purpose of manufacture of the meter. The purchaser or its 	representative can demand the original invoices in respect of all the raw 	materials.

- Quantity to be offered against the tender:

Bidder should quote for minimum 100 % of the tendered quantity failing which the tender will be rejected.

- Distribution of Quantity- L-1 bidder will be decided on the lowest landed rate by the participating bidder for each item. For landed rate ,Ex works and freight shall be as per rate quoted by the bidder while GST shall be 18% or as applicable for the item. Tender shall be evaluated considering GST@18% irrespective of GST rate quoted by bidder.

- If L-1 is a "Local Enterprise", then L-1 is the selected as supplier for 100% quantity.

- If L-1 is a "Non-Local Enterprise" then the following procedure for purchase preference shall be followed:-

- 75% of the order quantity shall be awarded to L-1.

- The lowest bidder among the "Local Enterprise" within the margin of purchase preference will be invited to match the L-1 price for the remaining 25% of the quantity, and contract for that quantity shall be awarded to such "Local Enterprise" subject to matching the L-1 price. In case such lowest eligible "Local Enterprise" fails to match the L-1 price or accepts less than the offered quantity, the next higher "Local Enterprise" within the margin of purchase preference shall be invited to match the L-1 price for remaining quantity and so on, and contract shall be awarded accordingly. In case some quantity is still left uncovered by "Local Enterprise", then such balance quantity may also be ordered to the L-1 bidder.

Note: In case more than one local enterprises have quoted the same price and are within the margin of purchase preference, the applicable order quantity mentioned above shall be equally divided amongst those.

- Jurisdiction of court:-The Courts/Council/Tribunal of Patna shall alone have an exclusive jurisdiction to decide any differences/dispute/claim for and against north Bihar Power Distribution Co. Ltd./ South Bihar Power Distribution Co. Ltd contractor/suppliers arising out in respect of the said nit, contract agreement or purchase order.

- Arbitration/ Resolution of disputes

All disputes or differences in respect of which the decision, if any, of the vendor and/or the NBPDCL/SBPDCL has not become final or binding as aforesaid shall be settled by arbitration in the manner provided herein below.

    - The arbitration shall be conducted by three (3) arbitrators, one each to be nominated by the vendor and the NBPDCL/SBPDCL and the third to be appointed by both the arbitrators in accordance with the Indian Arbitration Act, 1996 as amended from time to time. For this purpose a panel of five arbitrators shall be provided by Managing Director of the NBPDCL/SBPDCL from which NBPDCL/SBPDCL and Vendor will choose one each and the third Arbitrator shall be chosen by two Arbitrators from the same list. If either of the parties fails to appoint its arbitrator within sixty (60) days after receipt of a notice from the other party invoking the Arbitration Clause, the arbitrator appointed by the party invoking the arbitration Clause shall become the sole arbitrator to conduct the arbitration.

    - The language of the arbitration proceedings and that of the documents and communications between the parties shall be English. The arbitration shall be conducted in accordance with the provisions of the Indian Arbitration and Conciliation Act, 1996 or any statutory modification thereof.

    - The governing law shall be the Law of India and Civil Court of Patna shall have an exclusive jurisdiction over such matters of disputes.

    - The decision of the majority of the arbitrators shall be final and binding upon the parties. In the event of any of the aforesaid arbitrators dying, neglecting, resigning or being unable to act for any reason, it will be lawful for the party concerned to nominate another arbitrator in place of the outgoing arbitrator.

    - During settlement of disputes and arbitration proceedings, both parties shall be obliged to carry out their respective obligations under the Contract.

- Extension/Repeat order: -

(i) The circumstances may arise where due to time constraints or any other factor, the Repeat Order against an existing order is to be placed without inviting further offer of price. Such orders are called Repeat Orders which may be placed against previous orders (except those placed on single vendor) without further going through the normal inviting price offer after taking approval of competent authority as per the DoP.

(ii) Repeat Order may also be placed by SBPDCL against the previous orders placed by NBPDCL and vice versa after approval of competent authority.

(iii) Repeat order will be placed and the proposal for repeat orders will be on existing terms, conditions and specifications.

(iv) Value of Repeat Order shall not exceed hundred percent (100%) of the original ordered quantity. However, the repeat order can be for one more additional unit in case of the sanctioned quantity is in fraction.

(v) Repeat Order shall not be placed more than once and shall be placed within twelve (12) months of the placement of the original order.

(vi) The performance of the vendor must be satisfactory.

If Extension/ Repeat order is placed as per terms & conditions of the NIT, then it is mandatory for the firm to execute the placed extension/repeat order as per terms &conditions of the original P.O. without any deviation otherwise action like BG forfeiting/ debarment/ blacklisting shall be taken against the firm.

Post delivery type test against original purchase order will remain applicable for repeat purchase order also.

- Guarantee:- The equipment/material shall be guaranteed for a period 60 months from the date of delivery.  If during the guarantee period, the materials are found defective or sub-standard, the same will have to be replaced free of cost by the vendor within 30 days of intimation. If the defective materials are not replaced/rectified as per above guarantee clause, the DISCOM shall recover an equivalent amount along with 15% supervision charges from any of the bills of the supplier. Failure to however guaranteed obligation may render the supplier for blacklisting/debarment and/or forfeiture of all security and payment. The outage period i.e. period from the date of failure till unit is repaired / replaced shall not be counted for arriving at the guarantee period.

- Contract Agreement:- The vendor will have to execute a contract agreement with the North Bihar Power Distribution Company Limited/South Bihar Power Distribution Company Limited separately  in  the prescribed Proforma within 15 (Fifteen) days from the date of issue of  P.O.

- Pre Dispatch Inspection:-  Pre-dispatch inspection shall be done by authorized inspecting officer of DISCOMs/authorized inspecting agency of DISCOMs. If offered quantity is found to be less than 95% or the materials is rejected during inspection in both cases the actual expenses incurred will be deducted from vendor’s bill which will be not less than Rs. 50,000.00 (Rupee Fifty Thousand) only. Material will have to be delivered only after approval of Inspection Report and issue of Dispatch Instructions by (S&P) department. The vendor may depute its one representative with authorized inspecting officer of DISCOM for the purpose.

- Import License

Import License cannot be arranged by the NBPDCL/SBPDCL for such or those materials which are under banned category of import. When import is desired to be effected by the vendor(s) under their own quota/license, the value of quota specifically available against the tender should be stated. In case of availability of suitable indigenous make materials, the same may be preferred.

- Debarment/ blacklisting/de-empanelled of agencies

- Supplier/bidder will be subjected to continuous review of performance. Following may result in debarment/blacklisting as the case may be:

- Non-performance with regard to quality and supply schedule based on objective criteria,

- Non-compliance of replacement of defective material within Guarantee period as per Guarantee Clause,

- Attempt to bribe a utility official,

- Found engaging in corrupt or fraudulent practices,

- Non-compliance to the materials/equipment failure rate in any contract,

- Established misbehavior and threatening officials of the utility,

- Black listing by other state, central utilities or other government entities.

- Black listing may be for a period ranging from three (3) to six (6) years. Corrupt and fraudulent practices shall attract harshest punishment.

- Debarment/Blacklisting/de-empanelled Notifications issued by any subsidiary of BSPHCL shall be automatically extended to other subsidiaries. Firms black listed by other state government organization, Utilities and GOI will also stand black listed till their black listing is revoked.

If debarred/ blacklisted/de-empanelled Vendor feels that they have justifiable reasons to defend their case, they may appeal to MD, SBPDCL before going for litigation

- Termination of contract:- In case the supplier fails to deliver the materials or any consignment thereof within contracted period  of delivery or in case the materials are found  not in accordance with the prescribed specification and the approved samples, the Company shall exercise it’s discretionary power either :-

  - To recover from the supplier, the damages as provided in the penalty      clause of general condition of tender for supply.

  - To purchase elsewhere after giving due notice to the contractor on account and at the risk of the contractor without canceling the contract.

  - Firm may be debarred/ Blacklisted

  - To cancel the contract reserving Company’s right to recover damages.

Note : -Withstanding the powers , under (a) , (b), (c) & (d) referred   above are in addition to the rights and remedy available to the Company under the General law of India relating to contracts .

- In the event of purchase of stores of similar description, the option of the Company shall be final. In the event of action taken under

(a) or (b) above , the supplier shall be liable for any loss  which the Company sustain on the account  but the supplier shall not be entitled to any saving on such purchases made against default.

- The decision of the Company shall be final as regards the acceptability of the stores supplied by the supplier and the Company shall not be required to give any reason in writing or otherwise at any time for the rejection of stores/materials.

- The Company reserves the right to cancel whole or part of the awarded contract without assigning any reason of those firms which will be found defaulter for delay in supply of materials or the supply of sub-standard quality of the materials. The Company also reserves the right to distribute the order at the approved rate among other valid tenderers, in case the tenderer having quoted the approved rate is not found capable of supplying the total tendered quantity in the stipulated period of delivery schedule.

- RISK AND COST TENDER :

- Whenever DISCOM decides to terminate the contract at the Risk and Cost of the Vendor the first step will be to ensure joint measurement/ weight of the completed portion of the work/ supply during the final 48 Hours’ notice period.

- In case the Vendor refuses to co-operate in measurement/weight, the DISCOM shall undertake Ex Parte measurement or joint measurement/weight along with third party.

- The DISCOM shall be free to complete the left over portion of the supply not done either on its own or through engagement of another Vendor at the Risk and Cost of the original Vendor.

- In CASE to finalize, another Vendor shall be allowed. The original vendor shall not be allowed.

- The DISCOMs shall be free to recover the extra cost incurred on completion of the balance supply through forfeiting CPG and in case the same is found to be insufficient from other works of the Vendor under the DISCOM or with any other subsidiary of BSPHCL or even from the other contracts of the Vendor with any department of the state Government.

- CORRUPT OR FRAUDULENT PRACTICE:

The Purchaser requires that the Bidders observe the highest standard of ethics during the procurement and execution of the Project. In pursuance of this policy, the Purchaser:

a) Defines, for the purposes of this provision, the terms set forth below as follows:

(i) “Corrupt practice” means behavior on the part of officials in the public or private sectors by which they improperly and unlawfully enrich themselves and/ or those close to them, or induce others to do so, by misusing the position in which they are placed, and it includes the offering, giving, receiving, or soliciting of anything of value to influence the action of any such official in the procurement process or in contract execution; and

(ii) “Fraudulent practice” means a misrepresentation of facts in order to influence a procurement process or the execution of a contract to the detriment of the Purchaser, and  includes collusive practice amount Bidders (prior to or after Bid submission ) designed to establish Bid prices at artificial non-competitive levels and to deprive the Purchaser of the benefits of free and open competition.

b) Purchaser will reject a proposal for award if it determines that the Bidder recommended for award has engaged in corrupt or fraudulent practice in competing for the contract in question.

c) Purchaser will declare a firm ineligible, either indefinitely or for a stated period of time, to be awarded  an contract if he at any time determines that the firm is engaged in corrupt or fraudulent practice in competing for, or in executing, the Contract.

- EVENTS OF DEFAULT:-

Each of the following events or occurrences shall constitute an event of default (“Event of Default”) under the Contract:

(a) Vendor fails or refuses to pay any amount due under the Contracts.

(b) Vendor fails or refuses to deliver Commodities conforming to his Bid document/ specifications, or fails to deliver Commodities and, or execute the works assigned to them within the period specified in P.O or any extension thereof.

(c) Vendor becomes insolvent or unable to pay its debts when due, or commits any act of bankruptcy, such as filing any petition in any bankruptcy, winding-up or reorganization proceeding, or acknowledges in writing its insolvency or inability to pay its debts; or the Vendor’s creditors file any petition relating to bankruptcy of Vendor;

(d) Vendor otherwise fails or refuses to perform or observe any term or condition of the Contract and such failure is not remediable or, if remediable, continues for a period of 30 days after receipt by the Vendor of notice of such failure from company (SBPDCL).

- CONSEQUENCES OF DEFAULT:

- If an Event of Default occurs and would be continuing, Purchaser shall terminate the Contract by written notice.

- In the Event of Default, Purchaser may, without prejudice to any other right granted to it by law, or the Contract, take any or all of the following action:

- Forfeit the Contract Performance Bank Guarantee, if submitted or any submitted Bank Guarantee;

- Withheld of any other payment to fulfil the Risk & Cost tender clause of the NIT;

- Recover any losses and / or additional expenses, Purchaser may incur as a result of  Seller’s default.

- Debarment/ Blacklisting of the firm shall be taken.

- Force Majeure

    - “Force Majeure” shall mean any event beyond the reasonable control of the employer or of the Vendor, as the case may be, and which is unavoidable notwithstanding the reasonable care of the party affected, and shall include, without limitation, the following:

War, hostilities or warlike operations (whether war be declared or not), invasion, act of foreign enemy and civil war, rebellion, revolution, insurrection, mutiny, usurpation of government, conspiracy, riot and civil commotion, epidemics, earthquake, quarantine restrictions, freight embargoes, landslide, volcanic activity, flood or cyclone, or other inclement weather condition, nuclear and pressure waves or other natural or physical disaster.

    - Neither party shall be considered to be in default or in breach of his obligations under the Contract to the extent that performance such obligation is prevented by any circumstances of Force majeure, which arises after date of Notification of Award.

    - If either party is prevented, hindered or delayed from or in performing any of its obligations under the Contract by an event of Force Majeure, then it shall notify the other in writing of the occurrence of such event and the circumstances thereof within fourteen (14) days after the occurrence of such event. The Engineer-in-Charge shall be the point of contact in such case.

    - The party who has given such notice shall be excused from the performance or punctual performance of its obligations under the Contract for so long as the relevant event of Force Majeure continues and to the extent that such party’s performance is prevented, hindered or delayed. The Time for Completion shall be extended.

    - The vendor who are agreeable to accept the above terms of general conditions will only be considered and other will outright be rejected.

- POST DELIVERY SAMPLE CHECK:-

Apart from Post Delivery Type Test,  any sample may be randomly selected and sealed from the delivered material may be sent to NABL accredited laboratory like CPRI/ NTH/ERDA/NPL and tested for quality check at own cost of NBPDCL/SBPDCL at any stage till the warranty period. If any adverse report reported by laboratory, whole lot will be rejected and submitted security money will be forfeited. Supplier may be debarred/blacklisted and de-empanelled. Sample of the supplied material shall be selected & sealed in presence of the supplier before sending it to the lab for quality check provided the supplier appears for the selection/sealing process on intimation by NBPDCL/SBPDCL. In case, the supplier is absent on the scheduled date for selection/sealing process, the selection/sealing will be done by NBPDCL/SBPDCL on its own and further process of testing will be done. Result of post delivery sample check will be binding on the supplier.

- Deviation from specification:-

No any deviation from specifications of material and the terms & conditions of the tender shall be allowed. The vendors who are agreeable to accept the above terms of general conditions will only be considered and other will out rightly be rejected.

- INDEMNITY FOR LOSSES

The vendor shall indemnify utility towards losses caused to other equipment operating with such new equipment/ associated equipment due to defects/mal-operation of the equipment (for any reason) supplied by such vendor.

The vendor has to furnish an undertaking on appropriate value of Non-judicial Stamp Paper that they will pay the actual losses caused to utility equipment due to defects / mal-operation of their equipment (if any). The quantification of loss by utility shall be final in this regard.

- GTP & Drawing Approval: The supplier shall submit the GTP & drawing for approval of the purchaser within seven (7) days from Contract Agreement. The same will normally be approved within seven (7) days of submission.

- General:-SBPDCL will ascertain to its satisfaction whether the bids are responsive as per the Qualification Requirement specified above to execute the order satisfactorily. SBPDCL shall be the sole judge in this regard and the interpretation of SBPDCL regarding the qualification requirement and other terms & conditions of this tender shall be final and binding on the bidders/suppliers. Conditional Bids shall not be acceptable.

Encl:

1. Condition of tender- Annexure ‘A’

2. Tender Performa for Tech. & Commercial Part-I, - Annexure-B

3. Monthly delivery schedule -Annexure ‘C’

4. Past Performance of supply-Annexure-D

5. Check list of documents to be submitted by the bidder during tender- Annexure-E

6. AFFIDAVIT FORMAT- Annexure-F

7. BID SECURITY FORM (EMD)- Annexure-G

8.  PERFORMANCE SECURITY FORM- Annexure-H

9.Sample check list of documents required for Payment- Annexure-I

10.Tender Performa for Price Bid: Annexure-J

11.  Format for Supply Experience: Annexure-K

- 12. Check list of documents to be submitted by the bidder during tender: Annexure-L

Sd/-

Electrical Superintending Engineer(S&P)

SBPDCL

Annexure ‘A’

SOUTH BIHAR POWER DISTRIBUTION CO. LTD., PATNA

DEPARTMENT OF STORES & PURCHASE

CONDITION OF TENDER

The following conditions are to be strictly fulfilled by the firm while submitting their tenders.

1. Exemption will be granted as per general condition.

2. Tenderer has to accept the penalty clause as per tender otherwise tender will be considered as invalid.

3. The offers, which are not kept valid for acceptance for at least 180 days from the date of opening of the tenders, will be rejected.

4. Incomplete tenders which do not contain full details, technical particulars, drawing literature, price , delivery period, etc. will be rejected.

5. Prices are specifically asked to be submitted as F.O.R. destination and if not so furnished the tender shall not be considered as valid and will be rejected.

6. Tender which is not submitted in the company’s prescribed form shall be rejected. Any additional particulars can be furnished in the accompanying letter of statements.

7. Performa for schedule of Requirement and deliveries is enclosed with the tender. Tenderer are requested to fill up the Performa giving clearly the monthly delivery schedule.

8. Tenders uploaded after the due date and time will be rejected.

9. Prices must be indicated both in figures and words failing which the tender will be rejected.

10. In case of any error, omission, commission, confusion, typographical mistakes, NBPDCL’s interpretation will prevail.

Electrical Superintending Engineer(S&P)

SBPDCL

Annexure ‘B’

SOUTH BIHAR POWER DISTRIBUTION CO. LTD., PATNA

TENDER PROFORMA FOR TECHINCAL & COMMERCIAL PART-I

1.	Name and full address of the tenderer:

2.	Name of manufacturer:

3.	Name of material:

4.	Tender notice no. with due date:

5.	Quantity offered:

6.	Deliver schedule:

a)	Monthly delivery schedule:

7.	Validity of the offer (it should not be less than 180 days)

8.	Acceptance of the terms of payment.

9.	Notarized affidavit (must be submitted)

(I) towards true & genuineness of tender document

(II)Towards Blacklisting /Debarred

10.	Acceptance of repeat extension order

11.	Any deviation from the material specification and terms & conditions of tender

12	Acceptance of penalty clause.

13. 	Guarantee

14.	Type Test Report as per relevant IS

15.	Quantity if any, yet to be delivered against pending P.O. delivery Schedule, details of up-to-date supply with quantity and date.

16.	Past Performance (copy of order received and execution details)

17.	Drawing & literature, if any.

18.	Details of technical capability manufacturing capacity per annum.

19.	Details of plant and manufacturing capacity per annum.

20.	Enclosures with the tender should be specified

21.	Name of their Banks with financial arrangement with them.

a)           Cash Credit limit

b)	Raw material inventory limit Rs.

c)	Finished goods hypothecation limit Rs.

d)	Bank guarantee limit Rs.

e)	Any other financial facility given to them by the Bankers (Letter in original from their Banks may be furnished).

22. 	Remarks if any.

Name and Signature of

Authorized person of the firm/ Tenderer

ANNEXURE- “C”

SOUTH BIHAR POWER DISTRIBUTION CO. LTD., PATNA

PERFROMA FOR SCHEDULE OF REQUIREMENT & SPECIFIED DELIVERIES FOR MONTHWISE DELIVERY SCHEDULE.

| SI. No. | Item No. | Quantity required | Quantity offered | 1st Month | 2nd Month | 3rd Month | 4th Month |
| --- | --- | --- | --- | --- | --- | --- | --- |

Signature of tenderer/ authorized

Signatory with name, designation & Seal

Notice:-

- Priorities of dispatches will be notified by the purchaser at the time of pre-dispatch inspection.

Name and Signature of authorized person of the firm/ Tenderer.

ANNEXURE- “D”

SOUTH BIHAR POWER DISTRIBUTION CO. LTD., PATNA

DEPARTMENT OF STORES & PURCHASE

Past Performance of Supply

Name and Address of the Bidder:

| SI. No. | P.O./ LOI No. & Date | Quantity Ordered | P.O. place by | Delivery Schedule | Qty. Delivered | Quantity Pending | Reason for Non supply | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |

Name and Signature of

Authorized person of the firm/ Tenderer.

ANNEXURE- “E”

- Check list of documents to be submitted by the bidder during tender

| Sl. No. | Particulars |
| --- | --- |
| 1 | Name & full address of bidder along with E-mail Id & contact no. |
| 2 | Entity Registration Certificate |
| 3 | Documentary evidence regarding category of the manufacturer (Micro / Small / Medium / Large) |
| 4 | Type test certificate issued within past (5) five years as on date of NIT opening from CPRI/NTH/ERDA |
| 5 | Work Experience |
| 6 | Bidder's capability |
| 7 | Annual Production reports/ certificates issued by CA |
| 8 | Minimum Average Annual Turnover (MAAT) Certificate CA certificate regarding turnover for last five financial year up to FY 2024-25 |
| 9 | Audited Annual Accounts of last 5 financial years i.e up to financial year 2024-25 |
| 10 | CA certificate regarding net worth for last three financial year up to FY 2024-25 |
| 11 | Notarized Affidavit as per enclosed format. |
| 12 | Detailed information on any litigation or arbitration arising out of contracts completed or under execution by it over the last five years |
| 13 | In-house inspection/testing facilities |
| 14 | Copies of PAN |
| 15 | GSTIN registration certificate along with GSTR-9 of last 5 FY |
| 16 | Acceptance of all the term & conditions of NIT without any deviation. |
| 17 | Other certificates/ information as per NIT |

ANNEXURE- “F”

- AFFIDAVIT FORMAT

(To be stamped in accordance with Stamp Act, the Non-Judicial Stamp Paper of Rs 1000 should be in the name of Firm)

(To whom so ever it may concern)

I, name-………….. Designation………….. of Firm name with full address……………do hereby solemnly affirm and declare as follow:-

- The material has been type tested as per relevant IS/IEC standard whenever applicable by …… (name of laboratory) …………and the test certificate enclosed with the tender are true and genuine.

(b)	 Our Firm/ Company ………….………… is presently not blacklisted by any State / Centre Utilities / PSUs / govt. entities on the date of submission of bid.

(c)    Our Firm/ Company ………………….… is not debarred/ blacklisted from BSPHCL and its subsidiary companies (i.e. NBPDCL/SBPDCL/BSPTCL/BSPGCL) on the date of submission of bid.

(d) Our Firm/ Company ………………  is not de-empanelled or their vendor approval has not cancelled by BSPHCL and its subsidiary companies (i.e. NBPDCL/SBPDCL/BSPTCL/BSPGCL) in last two years from last date of submission of bid.”

(e) (1) all documents submitted in the bid are true and genuine and (2) all documents to be submitted during the execution of contract, if awarded, will be true and genuine.

(f) The vendor shall indemnify utility towards losses caused to other equipment operating with such new equipment/ associated equipment due to defects/mal-operation of the equipment (for any reason) supplied by such vendor.

The vendor will pay the actual losses caused to utility equipment due to defects / mal-operation of their equipment (if any). The quantification of loss by utility shall be final in this regard.

- That the above statements are true and correct to the best of our knowledge. Sworn and signed on this ….. Day of………2026. If it is found wrong then suitable action as per terms & conditions of NIT may be taken against our firm.

ANNEXURE- “G”

- FORMS

BID SECURITY FORM (EMD)

(To be stamped in accordance with Stamp Act, the Non-Judicial Stamp Paper should be in the name of the issuing Bank)

Bank Guarantee No.: ........................…

Date: ...................……...

To:

Sr. Manager (Finance)

NBPDCL,

Vidyut Bhawan , Bailey Road, Patna

(insert Name and Address of Employer)

WHEREAS  M/s.  ….  (insert  name of  Bidder)……  having  its Registered/Head  Office at

….. (insert address of the Bidder) ……….. (hereinafter called "the Bidder") has submitted its Bid for the performance of the Contract for………….(insert name of the Package)……………under………….(insert Specification No)…………… (hereinafter called "the Bid"

KNOW ALL PERSONS by these present that WE ……. (insert name & address of the issuing bank) ……… having its Registered/Head Office at ….……. (insert address of registered office of the bank) ……. (hereinafter called "the Bank"), are bound unto …… (insert name of Employer) …... (hereinafter called "the Employer") in the sum of ....................(insert amount of Bid Security in figures & words) .......................... ............................……………. for which payment well and truly to be made to the said Employer, the Bank binds itself, its successors and assigns by these presents.

Sealed with the Common Seal of the said Bank this ............... day of ............... 20.... THE CONDITIONS of this obligation are:

- If the Bidder withdraws its bid during the period of bid validity specified by the Bidder in the Bid Form; or

- If, as  per  the  requirement  of  Qualification  Requirements  the Bidder  is  required to submit a Deed of Joint Undertaking and he fails to submit the same, duly attested by Notary Public of the place(s) of the respective  executant (s)  or registered with the Indian Embassy/High Commission in that Country, within ten days from the date of intimation of post – bid discussion; or

- in the case of a successful Bidder, if the Bidder fails within the specified time limit

  - to sign the Contract Agreement,

  - to furnish the required performance security,

  - In any other case specifically provided for in NIT.

WE undertake to pay to the Employer up  to  the  above  amount upon  receipt  of  its first written demand, without the Employer having to substantiate its demand, on the same working day, without referring the matter back to the Bidder for what so reason it may be provided that in its demand the Employer will note that the amount claimed by it is due to it, owing to the occurrence of any of the above-named CONDITIONS or their combination, and specifying the occurred condition or conditions.

This guarantee will remain in full force up to and including ………... (insert date, which shall be the date 30 days after the period of bid validity) ……..., and any demand in respect thereof must reach the Bank not later than the above date.

Note:

The Bank Guarantee should be in accordance with the proforma as provided. However, in case the issuing bank insists for additional paragraph for limitation of liability, the following may be added at the end of the proforma of the Bank Guarantee [i.e., end paragraph of the Bank Guarantee preceding the signature(s) of the issuing authority (ies) of the Bank Guarantee]:

“Notwithstanding anything contained herein:

- Our liability under this Bank Guarantee shall not exceed	(value in figures)

(value in words)	].

- This Bank Guarantee shall be valid upto	(validity date)	 and the claim period shall be up to six months beyond validity period.

- We are liable to pay the guaranteed amount or any part thereof under this Bank Guarantee only & only  if  we  receive  a  written  claim  or  demand  on  or  before (claim date).”

- We hereby unconditionally agree that the SBPDCL shall have power to invoke or encash this letter of guarantee any time during its validity/claim period.

For and on behalf of the Bank

[Signature of the authorised signatory(ies)] Signature

Name

Designation

POA Number

Contact Number(s):

Tel.	Mobile

Fax Number

email

Common Seal of the Bank

Witness

Signature

Name

Address_

Contact Number(s): Tel.	Mobile

email

ANNEXURE- “H”

PERFORMANCE SECURITY FORM

Bank Guarantee No. ……………………	Date................... Contract No.....................................

…………..[Name of Contract]…………………….

To:

Sr. Manager (Finance),

North Bihar Power Distribution Compnay Ltd.,/

South Bihar Power Distribution Compnay Ltd

Vidyut Bhawan, Baiely Road, Patna.

Dear	Ladies	and/or Gentlemen,

We refer to the Contract ("the Contract") signed on ..............(insert date of the Contract)…..... between M/s. XXXXX (Name of Employer), having its Registered Office at XXXXX (Registered Address  of  Employer)  (“the  Employer”/”  XXXXX (Name of Employer)”) on  behalf of XXXX (Name of  Owner) (hereinafter referred to as ‘XXXX (Short Name of Owner)’ / ’Owner’), and M/s ................. (Name  of  Contractor)  ..................,  having    its    Principal    place    of    business    at

………....(Address of Contractor)

............................... and Registered Office at ………....(Registered address of Contractor)

........................................................................................  ("the Contractor") concerning

…….................….. (Indicate brief scope of work) ............................... for the complete execution of the …… (insert name of Package along with name of the Project)…….. [Applicable for Bank Guarantees issued by Contractor/Associate for those Contracts awarded to them]

#### Or

We refer to the Contract signed on ..............(insert date of the Contract)…..... between M/s. XXXXX (Name of Employer), having its Registered Office at XXXXX (Registered Address of Employer) (“the Employer”/” XXXXX (Name of Employer)”) on behalf of XXXX (Name of Owner) (hereinafter referred to as ‘XXXX (Short Name of Owner)’ / ’Owner’), and M/s ................. (Name of Contractor) .................., having its Principal place of

business at ………....(Address of Contractor) ............................... and Registered Office at

………....(Registered	address	of	Contractor)

........................................................................................  ("the  Contractor")  and  the  Contract  ("the Contract") signed on ..............(insert date of the Contract)…..... between XXXXX (Name of Employer) on behalf of Owner  and M/s ................. (Name of Associate) .................., having

its Principal place of business at ………....(Address of Associate) ............................... and Registered          Office          at          ………....(Registered          address          of          Associate)

........................................................................................,    the    Associate    of    the     Contractor     for

executing   the   Facilities   concerning   …….................…..  (Indicate   brief  scope  of  work)

............................... for the complete execution of the …… (insert name of package along with name of the Project)…….. [Applicable for Bank Guarantees to be issued by Contractor against those Contracts awarded to their Associate]

By this letter we, the undersigned, ………(insert name & address of the issuing bank)

………, a Bank (which expression shall include its successors, administrators, executors and assigns) organized under the laws of .................................... and having its Registered/Head Office at …..…….(insert address of registered office of the bank)……..... do hereby irrevocably guarantee payment to  the  Employer  up  to  ………………………….. i.e., Ten percent (10%) (5% in case of SSI unit of Bihar) of the Contract Price.

We undertake to make payment under this Letter of Guarantee upon receipt by us of your first written demand signed by the Employer duly authorized officer or the authorized officer of Owner declaring the Contractor to be in default under the Contract and without cavil or argument any sum or sums within the above named limits, without your need to prove or show grounds or reasons for your demand and  without  the right of the Contractor to dispute or question such demand. Such payment shall be made by us on the same banking day as request without dmur and delay and without referring the matter back to the Contractor for what so reason it may be

Our liability under this Letter of Guarantee shall be to pay to the Employer whichever is the lesser of the sum so requested or the amount then guaranteed hereunder in respect of any demand duly made hereunder prior to expiry of the Letter of Guarantee, without being entitled to inquire whether or not this payment is lawfully demanded.

This letter of Guarantee shall remain in full force and shall be valid from the date of issue until ninety (90) days beyond the Defect  Liability  Period (i.e. 70 months after supply of last consignment) of  the  Facilities  i.e.  up to and inclusive of ……………. (dd/mm/yy)

and shall be extended from time to time for such period (not exceeding one year), as may be desired by M/s. ……………………… on whose behalf this Letter of Guarantee has been given.

Except for the documents herein specified, no other documents or other action shall be required, notwithstanding any applicable law or regulation.

Our liability under this Letter of Guarantee shall become null and void immediately upon its expiry, whether it is returned or not, and no claim may be made hereunder after such expiry or after the aggregate of the sums paid by us to the Employer shall equal the sums guaranteed hereunder, whichever is the earlier.

All notices to be given under shall be given by registered (airmail) posts to the addressee at the address herein set out or as otherwise advised by and between the parties hereto.

We hereby agree that any part of the Contract may be amended, renewed, extended, modified, compromised, released or discharged by mutual agreement between you and the Contractor, and this security may be exchanged or surrendered without in any way impairing or affecting our liabilities hereunder without notices to us and without the necessity for any additional endorsement, consent or guarantee by us, provided, however, that the sum guaranteed shall not be increased or decreased.

No action, event or condition which by any applicable law should operate to discharge us from liability hereunder shall have any effect and we hereby waive any right we may have to apply such law so that in all respects our liability hereunder shall be irrevocable and, except as stated herein, unconditional in all respects.

Note:

- For the purpose of executing the Bank Guarantee, the non-judicial stamp papers of appropriate value shall be purchased in the name of Bank who issues the ‘Bank Guarantee’.

- The   Bank   Guarantee   shall   be   signed   on   all   the   pages   by   the   Bank Authorities indicating their POA nos. and should invariably be witnessed.

- The Bank Guarantee should be in accordance with the proforma as provided.

However, in case the issuing bank insists for additional paragraph for limitation of liability, the following may be added at the end of the proforma of the Bank Guarantee [i.e., end paragraph of the Bank Guarantee preceding the signature(s) of the issuing authority(ies) of the Bank Guarantee]:

Quote

“Notwithstanding anything contained herein:

- This Bank Guarantee shall be valid upto	(validity date)	 and the claim period shall be up to six months beyond validity period.

- We are liable to pay the guaranteed amount or any part thereof under this Bank Guarantee only & only  if  we  receive  a  written  claim  or  demand  on  or  before (claim date).”

- We hereby unconditionally agree that the SBPDCL shall have power to invoke or encash this letter of guarantee any time during its validity/claim period.

For  and  on  behalf  of  the Bank

[Signature of the authorised signatory(ies)] Signature

Name

Designation

POA Number

Contact Number(s): Tel.	Mobile

Fax Number

email

Common Seal of the Bank

Witness: Signature

Name

Address_

Contact Number(s): Tel.	Mobile

email

ANNEXURE- “I”

SOUTH BIHAR POWER DISTRIBUTION CO. LTD., PATNA

DEPARTMENT OF STORES AND PUCHASE.

Sample check list of documents required for Payment

| S. No. | Document Particulars | Approval required from NBPDCL? (yes or no) | Enclosed (yes or no) |
| --- | --- | --- | --- |
| 1 | Bills (In Triplicate) | No |  |
| 2 | Copy of purchase order | No |  |
| 3 | Copy of contract agreement | No |  |
| 4 | Acceptance of bank guarantee | Yes |  |
| 5 | Copy of approval of inspection report | Yes |  |
| 6 | Copy of approval of routine test | Yes |  |
| 7 | Copy of type test report (as per Purchase Order) | Yes |  |
| 8 | Copy of despatch allocation | No |  |
| 9 | Copy of GST Registration certificate | No |  |
| 10 | Copy of Challan& Lorry Receipt | No |  |
| 11 | Complete SRV issued by the consignee | No |  |

Note:- 1. To be customized for each material / equipment.

Annexure ‘J’

SOUTH BIHAR POWER DISTRIBUTION CO. LTD., PATNA

TENDER PROFORMA FOR PRICE PART

| NIT No- |  | :- |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Name of material |  | :- | 11 KV 3 element outdoor type oil immersed combined metering unit 0.5 accuracy class of CT and 0.5 accuracy class of PT of different ratio |  |  |  |  |
| Quantity |  | :- | As per NIT |  |  |  |  |
| 1 | Name of the tenderer | :- |  |  |  |  |  |
| 2 | Full address of the tenderer | :- |  |  |  |  |  |
| 3 | Name of Manufacturer | :- |  |  |  |  |  |
| 4 | Name of materials | :- |  |  |  |  |  |
| 5 | Quantity offered with monthly quantity delivery schedule and name of the month of commencement of deliver (must be supported by manufacturing capacity) (Minimum quantity shall be offered as per NIT) | :- |  |  |  |  |  |
|  |  |  | Price to be quoted by bidder per Nos. |  |  |  |  |
| 6 | Price breakup |  | Rate Per Nos. for 100/5A | Rate Per Nos. for 50/5A |  | Rate Per Nos. for 25/5A | Rate Per Nos. for 10/5A |
| (a) | Ex-factory price | :- | Not to be submitted in Technical bid | Not to be submitted in Technical bid |  | Not to be submitted in Technical bid | Not to be submitted in Technical bid |
| (b) | Freight | :- |  |  |  |  |  |
| (c) | GST | :- |  |  |  |  |  |
| (d) | IGST (As Applicable) | :- |  |  |  |  |  |
| (e) | CGST (As Applicable) | :- |  |  |  |  |  |
| (f) | SGST (As Applicable) | :- |  |  |  |  |  |
| (g) | Other Charges, if any | :- |  |  |  |  |  |
| (h) | Discount if any | :- |  |  |  |  |  |
| (i) | Landed Price per Unit (both in words and figures) (Inclusive of Ex-factory price, freight and all applicable taxes & duties) | :- |  |  |  |  |  |

ANNEXURE- “K”

SOUTH BIHAR POWER DISTRIBUTION CO. LTD., PATNA

DEPARTMENT OF STORES & PURCHASE

Past Performance of Supply

Name and Address of the Bidder :

Format for Supply Experience

| S.N. | Name of material | P.O. No. with Date | Ordered Quantity, | Supplied Quantity, | Supplied in FY | Name of end user (govt organisation only) | Type of supply (Direct/turnkey project) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total |  |  |  |  |  |  |  |

Note: Copy of each document must be submitted in support of the above.

Name and Signature of

Authorized person of the firm/ Tenderer.

ANNEXURE- “L”

- Check list of documents to be submitted by the bidder during tender

| Sl. No. | Particulars |
| --- | --- |
| 1 | Name & full address of bidder along with E-mail Id & contact no. |
| 2 | Entity Registration Certificate |
| 3 | Documentary evidence regarding category of the manufacturer (Micro / Small / Medium / Large) |
| 4 | Type test certificate |
| 5 | Work Experience : i. Copy of Purchase Order |
|  | ii. Document against completion of supply against uploaded purchase order |
| 6 | Bidder's capability |
| 7 | Report/Certificate against Bidder's Annual Production Capacity |
| 8 | Minimum Average Annual Turnover (MAAT) & Net worth Certificate from CA |
| 9 | Audited Annual Accounts with UDIN of last 5 financial years upto FY 2024-25 |
| 10 | Net Worth of the bidder for last 3 financial years upto FY 2024-25 |
| 11 | Notarized Affidavit as per enclosed format. |
| 12 | In-house inspection/testing facilities: Confirmation regarding availability of in-house inspection/ testing facilities or access through other government approved agencies for performing inspection/ testing. |
| 13 | In-house inspection/testing facilities: Copies of calibration/re-calibration certificates of the major testing equipment |
| 14 | Copies of PAN |
| 15 | GSTIN registration certificate |
| 16 | Acceptance of all the term & conditions of NIT without any deviation. |
| 17 | GSTR-9 (i.e Annual GST Return of last 5 FY upto FY 2024-25 |
| 18 | Certificate/undertaking for benefit of input credit in GST or any other assistance available as per clause-15 (note). |
| 19 | Other certificates/ information as per NIT |
| 20 | Documentary evidence regarding manufacturer of tendered material |