TECHNICAL SPECIFICATIONS &  CALCULATION SHEET FOR  LIFT C.M.C. WORK

NAME OF WORK :-   FOR  PROVIDING  COMPREHENSIVE ANNUAL MAINTENANCE CONTRACT FOR 22 No. LIFT MACHINE INSTALLED AT G.A.D. COLONY , SECTOR 17 , NAVA RAIPUR ,C.G.

1.ANNEXURE-E ( Term & Condition)

2. Calculation Sheet For Lift Machine

OFFICE OF THE EXECUTIVE ENGINEER E/M DIVISION PWD RAIPUR (C.G.)

1. ANNEXURE-E

TERMS & CONDITION

NAME OF WORK :- FOR  PROVIDING  COMPREHENSIVE ANNUAL MAINTENANCE CONTRACT FOR 22 No. LIFT MACHINE INSTALLED AT G.A.D. COLONY , SECTOR 17 , NAVA RAIPUR ,C.G.

1. Log Book

- Maintain lift maintenance log book for each lift.

- Record date of servicing, maintenance activity, faults observed, parts replace dander marks.

- Record breakdown details and rectification time.

2. Safety Requirements

- Follow manufacturer guidelines and relevant lift safety standards.

- Ensure power isolation before carrying out maintenance work.

- Use proper tools, safety belts and personal protective equipment.

- Only trained and authorized personnel should carry out maintenance.

3. Comprehensive Maintenance Schedule of lift

Monthly Checks

  - Check landing locks and ensure lift does not move with gate open.

  - Check car gate switch operation.

  - Test door operator safety (for automatic doors).

  - Check emergency call bell and emergency stop button.

  - Check car lights, landing lights and call buttons.

  - Check lift door locks.

  - Check stopping of lift with 25% overload and emergency stop operation.

  - Check leveling accuracy of lift.

  - Lubricate sheave shaft, motor shaft, deflector sheave and governor pulley bearings.

  - Clean controller contacts and check relays, fuses and flexible leads.

  - Grease motor generator bearings and lubricate AC motor bearings.

  - Inspect gearbox for noise and oil leakage.

  - Adjust brake clearance and clean brake system.

  - Clean selector contacts and check leveling.

  - Lubricate governor and check rope condition and safety switch.

  - Lubricate guides and guide shoes in hoist way.

# Quarterly Maintenance

  - Check overload relay and phase failure relay.

  - Tighten power wiring connections in switch, motor and controller.

  - Check lubrication of all equipment.

  - Inspect rope fastenings, guide clamps and limit switches.

  - Check travelling cable condition and terminations.

# Half-Yearly Maintenance

  - Inspect car frame, motor, cables and insulation.

  - Check over speed safety gear and sheave condition.

  - Check gearbox backlash, thrust bearing and alignment.

  - Clean and lubricate motor bearings and guide shoes.

  - Check gear coupling bolts, oil seals and refill oil.

  - Inspect brake shoes, springs and brake drum.

# 4. Additional Terms & Conditions of AMC

# For OEM /Authorized Dealer /Channel Partner-The Authorized Dealer and Channel Partner has to submit an agreement (MOU) Between them and OEM in which it should be clearly mentioned about technical and spare supply support given by OEM only.

# Inventory Register

A inventory of all electrical and electronic installations/equipment in the building shall be prepared. The inventory shall include details such as description, quantity, location, present condition, make, rating/capacity and diagram

# Staff Deployment

The contractor shall deploy licensed electrician and skilled/semi- skilled staff at site. Licensed electrician and skilled supervisor/attendant shall be available during working hours for monitoring and emergency requirements.

# Safety Provisions.

The contractor shall be responsible for:

- Electrical safety

- Use of PPE Like Helmet, Uniform, Gloves, Shoes etc.

- Compliance with IE Rules/CEA regulations

- Any accident: contractor shall be liable

# Penalty/ Performance Clause

Penalty for delay in attending complaints, non-availability of staff, poor maintenance. Deduction shall be made from running bills.

# Records and Documentation

The contractor must maintain:

- Complain Register

- Maintenance logbook

- Preventive maintenance check list

- Equipment history sheet

# Termination Clause

Contract can be terminated in case of:

- Breach of contract according to clause Mention in tender document.

- Persistent poor performance or failure to attend complaints within stipulated time.

- Non Deployment or shortage of required technical staff (licensed electrician, etc.).

- Violation of safety standard(IE Rules/CEA regulation)or

Responsibility for any accident.

- Non Compliance With instruction of Engineer-in-charge.

- Failure to maintain proper records (logbooks, registers) or submission

Of incorrect information.

- Repeated breakdowns due to negligent maintenance.

Unauthorized  subletting or assignment of the contract.

- Abandonment of work or stopping work without prior notice.

- Continued unsatisfactory performance despite imposition of penalties.

EXECUTIVE ENGINEER

E/M DIVISION RAIPUR (C.G.)