| Sl. no. | Item code | Specification | Unit | Qty | Quoted Make as per Specification |
| --- | --- | --- | --- | --- | --- |
| 1 | 5521006015J | COPPER BRAIDED FLEXIBLE Technical Specification: Braided flexible should be 4 layers. Material:Tin-Coated Copper/CuSn braid. Cross-section area:600sq mm. Width:40mm(+/-1mm). Thickness: 15mm(+/-1mm). Overall length:560mm(+/-5mm). Braided portion length:360mm(+/-5mm) End Termination: pressed copper lugs, Suitable for bolted bus bar connection to bur bars. Lug hole size : Undrilled. Finish: Consistent, bright tin plating,No bare copper visible. Current Rating: This braid can comfortably handle a continuous current of approximately 2800Amps. Low Resistance:The large area ensures an extremely lowelectrical resistance path. This minimizes voltage drop and power loss (I2R loss) acrosstheconnector, which is crucial for efficiency and preventing overheating in high-power Applications Technical Specifications and dimensions as mentioned in the attached drawing Make: NANA UDYOG/ Ganpati Engineering/ AK Industries/ CoppFusion/ B.B.ELECTROTECHNIC, GUJARAT/ Equivalent. | Nos. | 50 |  |

Note: 1) Bidders should compulsorily mention the make they will supply in the Quoted Make column.

2) In the event of order, the bidder shall supply 01 no. of (One) sample of “COPPER BRAIDED FLEXIBLE” along with test certificate and to be approved by KPCL before supply of total ordered quantity.

-Sd-

Executive Engineer (MSP) Jog,

Karnataka Power Corporation Ltd

Jogfalls-577435