| - SCOPE: The specifications of CAST BASALT BEND as follows; |  |  |  |
| --- | --- | --- | --- |
| Sl No | Item Code Item Name/Specification | Unit | Required Quantity |  |
| 1 | 6110403338 200NB 90 DEG. CAST BASALT BEND 200 NB 90 Degree ELBOW for Ash slurry application ,Fixed Flange at both ends, Extra Matching flange 02 nos For each bend of SIZE: 200NB, Dimensions:As per Bend Drawing Material.:MS casing bend lined with CAST BASALT with 20mm Thickness,operating Temprature-350 deg,Hard ness 8 to 9 MOHs,Specific gravity-2.9 to 3.0 gm/cc,compressive strength 4500kgs/cm2(Min),water obsorption-Nil ,As per Drawing. Make:INDURE/GES/SSC/SR INDUST | NUMBERS | 4 |  |
| 2 | 6110403427 350NB 90 DEG. CAST BASALT BEND 350 NB 90 Degree ELBOW for Ash slurry application ,Fixed Flange at both ends, Extra Matching flange 02 nos For each bend of SIZE: 200NB, Dimensions:As per Bend Drawing Material.:MS casing bend lined with CAST BASALT with 20mm Thickness,operating Temprature-350 deg,Hard ness 8 to 9 MOHs,Specific gravity-2.9 to 3.0 gm/cc,compressive strength 4500kgs/cm2(Min),water obsorption-Nil,As per Drawing.Make:INDURE/GES/SSC/SR INDUST | NUMBERS | 4 |  |
| 3 | 6110403435 350NB 45 DEG. CAST BASALT BEND 350 NB 45 Degree ELBOW for Ash slurry application ,Fixed Flange at both ends, Extra Matching flange 02 nos For each bend of SIZE: 200NB, Dimensions:As per Bend Drawing Material.:MS casing bend lined with CAST BASALT with 20mm Thickness,operating Temprature-350 deg,Hard ness 8 to 9 MOHs,Specific gravity-2.9 to 3.0 gm/cc,compressive strength 4500kgs/cm2(Min),water obsorption-Nil,As per Drawing.Make:INDURE/GES/SSC/SR INDUST | NUMBERS | 4 |  |

QUALITY, MATERIAL AND WORKMANSHIP:

- All the material shall be of best quality and of class suitable for construction of the equipment and shall withstand normal variation of the temperature payload conditions, without disturbance and deterioration or setting up of undue stresses in any part and also without effecting the strength and suitability of the various parts of the equipment for the outdoor work which they have to perform. The material shall be manufactured as per the relevant standards and drawing.

- The bidder shall furnish full description, literature, catalogue etc, of the offered materials.

Test certificates & Material test reports duly signed by authorized authority shall be produced.

Yours Faithfully

For Karnataka Power Corporation Limited

-Sd -

Executive Engineer (P)